3GPP TSG-SA4 5GMS3 Ad-Hoc Meeting                                                         S4-AHI873

The Hague, Netherlands, Nov 12-14, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Metrics collection and reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Procedures for metrics reporting are not described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add metrics reporting procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.x (new), 4.8.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

============================== CHANGE 1 ==============================

### 4.4.x. Procedures of the metrics reporting

Editor’s Note: The text below also touches on interface M7d, but it was difficult to just cut out the M5d part of the procedure. Also, the text currently re-use most stuff as-is from TS 26.247 by referencing, but it might be better to actually copy-and-edit relevant parts directly into 26.512 (e.g. to simplify the XML format). Also note that an alternative possibility is to let the Media Player be "metric-agnostic" and only emit certain events, and instead let the Metrics Handler subscribe to these and then build the metrics structure (the "CTA paradigm", although CTA is not yet ready with their work...).

These procedures shall be used by the Metrics Handler function in the Media Session Handler to control metrics reporting when such reporting is configured by the OAM via the 5G control channel.

The Metrics Handler shall subscribe to metrics configurations from the OAM according to TS 26.247 Annex L.1. When a metrics configuration is received, the Metrics Handler shall store this configuration and use it for all subsequent streaming sessions.

When a streaming session is started the Metrics Handler shall evaluate if metrics from this session shall be reported. The evaluation shall be based on the "sample percentage" and "streaming source filter" specified in the stored metrics configuration, according to TS 26.247 Annex F.

If metrics shall be reported for the session, the Metrics Handler shall request the Metrics Collector function in the Media Player to create a metrics collection job. The Metrics Collector shall return a reference to the created job, which the Metrics Handler then shall use in all subsequent actions related to this job.

The Metrics Handler shall then configure the metrics collector job with the metrics which shall be collected during the session. The format of the metrics collection job configuration shall be according to TS 26.247 Annex L.2, but note that only the "metrics" attribute in the configuration shall be used.

The Metrics Handler shall regularly request the collected metrics from the Metrics Collector according to the "reporting interval" specified in the metrics configuration. The metrics returned by the Metrics Collector shall use the format as described in TS 26.247 clause 10.6, and the Metrics Handler shall forward these to the OAM according to to TS 26.247 Annex L.1.

When the session is finished the Metrics Handler shall delete the metrics collection job.

=========================== END OF CHANGE 1 ==========================

============================== CHANGE 2 ==============================

### 4.8.x. Procedures of the metrics reporting

Editor’s Note: The text currently re-use most stuff as-is from TS 26.247 by referencing, but it might be better to actually copy-and-edit relevant parts directly into 26.512 (e.g. to simplify the XML format). Also note that an alternative possibility is to let the Media Player be "metric-agnostic" and only emit certain events, and instead let the Metrics Handler subscribe to these and then build the metrics structure (the "CTA paradigm", although CTA is not yet ready with their work...).

These procedures shall be used by the Metrics Handler function in the Media Session Handler to control metrics reporting when such reporting is configured via metadata sent in-band via the media manifest.

When a streaming session is started, the Metrics Handler shall check if the manifest contains any metrics configuration, as specified in TS 26.247 clauses 10.4 and 10.5. If such a configuration is found, the Metrics Handler shall use it for the current streaming session.

The Metrics Handler shall first evaluate if metrics from this session shall be reported. The evaluation shall be based on the "sample percentage" attribute specified in the metrics configuration, according to TS 26.247 clause 10.5.

If metrics shall be reported for the session, the Metrics Handler shall request the Metrics Collector function in the Media Player to create a metrics collection job. The Metrics Collector shall return a reference to the created job, which the Metrics Handler then shall use in all subsequent actions related to this job.

The Metrics Handler shall then configure the metrics collector job with the metrics which shall be collected during the session. The format of the metrics collection job configuration shall be according to TS 26.247 Annex L.2, but note that only the "metrics" attribute in the configuration shall be used.

The Metrics Handler shall regularly request the collected metrics from the Metrics Collector according to the "reporting interval" specified in the metrics configuration. The metrics returned by the Metrics Collector shall use the format as described in TS 26.247 clause 10.6, and the Metrics Handler shall forward these to the "server address" according to the procedures described in TS 26.247 clause 10.6.

When the session is finished the Metrics Handler shall delete the metrics collection job.

=========================== END OF CHANGE 2 ==========================