3GPP SA4-MBS SWG AH on 5GMS3	S4-AHI885

# The Hague, Netherlands, 12th-14th November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.511 | CR | - | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Proposed updates on Default profile for 5GMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | November 2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed implementation of the default profile for 5GMS unicast downlink with considerations of either applying the PSS capabilities as defined in the 5GMS3 WID or upgrading the decoding capability requirements to noawadays general support of media formats particularly on video formats. 2 options are provided under square brackets for further considerations in SA4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2 ; 3.3 ; 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change 1

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5GMS2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[5Gaudio]	3GPP TS 26.117,"5G Media Streaming (5GMS); Speech and audio profiles".

[html5]	3GPP TS 26.307, "Presentation Layer for 3GPP Services".

[vr]	3GPP TS 26.118, "3GPP Virtual reality profiles for streaming applications".

[avc]	ITU-T Recommendation H.264 (06/2019): "Advanced video coding for generic audiovisual services".

[hevc]	ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".

End of change 1

Start of change 2

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GMS	5G Media Streaming

5GMSA	5G Media Streaming Architecture

AMR	Adaptive Multi-Rate

AS	Application Server

AVC	Advanced Video Coding

EVS	Enhanced Voice Services

HEVC	High-Efficiency Video Coding

DASH	Dynamic Adaptive Streaming over HTTP

OMAF	Omnidirectional Media Application Format

TV	Television

UE	User Equipment

VR	Virtual Reality

End of change 2

Start of change 3

## 4.2	Default profile

### 4.2.1	Video decoding

[

Option A (PSS capabilities)

#### 4.2.1.1	General

If the 5GMS client supports the reception of video, the following requirements on video decoding capabilities as defined in clause 4.2.1.2 for H.264 (AVC) and clause 4.2.1.3 for H.265 (HEVC) apply.

#### 4.2.1.2	H.264 (AVC)

H.264 (AVC) Progressive High Profile Level 3.1 decoder [avc] shall be supported, wherein the maximum VCL Bit Rate is constrained to be 14Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1000 and 1200, respectively.

#### 4.2.1.3	H.265 (HEVC)

H.265 (HEVC) Main Profile, Main Tier, Level 3.1 decoder [hevc] should be supported.

When H.265 (HEVC) Main Profile decoder is supported, the client is only required to process H.265 (HEVC) Main Profile bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

End of option A

Option B (HEVC only)

If the 5GMS client supports the reception of video, H.265 (HEVC) Main Profile, Main Tier, Level 4.1 decoder [hevc] shall be supported.

The client is only required to process H.265 (HEVC) Main Profile bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

End of option B

]

### 4.2.2	Audio decoding

#### 4.2.2.1	General

If the 5GMS client supports the reception of audio, the following requirements on audio decoding capabilities as defined in clause 4.2.2.2 for Enhanced aacPlus and clause 4.2.2.3 for AMR-WB apply.

#### 4.2.2.2	Enhanced aacPlus

Enhanced aacPlus decoder should be supported as defined in 3GPP TS 26. 117 [5Gaudio], clause 5.3

#### 4.2.2.3	AMR-WB

AMR-WB decoder should be supported as defined in 3GPP TS 26. 117 [5Gaudio], clause 5.3

### 4.2.3	Speech decoding

#### 4.2.3.1	General

If the 5GMS client supports the reception of speech, the following requirements on speech decoding capabilities as defined in clause 4.2.3.2 for EVS, clause 4.2.3.3 for AMR and clause 4.2.3.4 for AMR-WB apply.

#### 4.2.3.2	EVS

EVS decoder shall be supported as defined in 3GPP TS 26.117 [5Gaudio], clause 5.2

#### 4.2.3.3	AMR

AMR decoder shall be supported as defined in 3GPP TS 26.117 [5Gaudio], clause 5.2

#### 4.2.3.4	AMR-WB

AMR-WB decoder shall be supported as defined in 3GPP TS 26.117 [5Gaudio], clause 5.2

### 4.2.4	Presentation format

Scene description in 5GMS is supported through the usage of the 3GPP HTML5 Profile as defined in 3GPP TS 26.307 [html5]. The HTML5 document may be served separately as an entry point to the 5G unicast media downlink service.

End of change 3