3GPP TSG SA WG-4 MBS SWG on 5GMS3  	S4-AHI894

The Hague, The Netherlands, 12-14 November 2019

Title: Proposed updates to 26.511

Source: Orange, Qualcomm Incorporated

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	5GMS Codecs and formats capabilities	8

4.1	Introduction	8

4.2	Video codec	8

4.2.1	H.264 (AVC)	8

4.2.1.1	Decoding	8

4.2.1.2	Encoding	8

4.2.2	H.265 (HEVC)	9

4.2.2.1	Decoding	9

4.2.2.2	Encoding	9

4.3	Audio codec	9

4.3.1	Decoding	9

4.3.2	Encoding	9

4.3	Speech codec	9

4.3.1	Decoding	9

4.3.2	Encoding	9

4.5	Subtitles	9

4.6	Presentation format	9

4.7	Encapsulation	10

4.4	Encryption	10

5	5GMS Profiles	10

5.1	Introduction	10

5.2	Downlink streaming default profile	10

5.2.1	Video	10

5.2.2	Audio	10

5.2.3	Speech	11

5.2.4	Presentation format	11

5.2.5	Encapsulation	11

5.2.5.1	CMAF Track Definition	11

5.2.5.2	CMAF Switching Set and Media Profile Definition	11

5.2.5.3	Mapping to DASH Adaptation Set	11

5.2.5.4	Playback Requirements	11

5.3	Uplink streaming default profile	11

5.4	TV profile	11

5.4.1	Video decoding	11

5.4.1.1	H.264 (AVC)	12

5.4.1.2	H.265 (HEVC)	12

5.4.1.1	H.264 (AVC)	12

5.4.1.2	H.265 (HEVC)	12

5.4.2	Audio decoding	12

5.5	360 VR profile	13

5.5.1	Video	13

5.5.1.1	Operation Points	13

5.5.1.2	DASH encapsulation	13

5.5.2	Audio	13

5.5.2.1	Operation Points	13

5.5.2.2	DASH encapsulation	13

Annex <X> (informative): Change history	14

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

The present document specifies the profiles, codecs and formats for 5G Media Streaming (5GMS) services based on the 5G Media Streaming Architecture (5GMSA) including both unicast downlink and unicast uplink. The codec and format recommendations defined for each profile apply to the 5GMS client components in the user equipment (UE) as well as the Media Application Server (AS). Profiles are defined to address specific service scenarios. A default profile is defined in the case no other profile is claimed to be supported.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5GMS2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[5Gaudio]	3GPP TS 26.117,"5G Media Streaming (5GMS); Speech and audio profiles".

[html5]	3GPP TS 26.307, "Presentation Layer for 3GPP Services".

[avc]	ITU-T Recommendation H.264 (06/2019): "Advanced video coding for generic audiovisual services".

[hevc]	ITU-T Recommendation H.265 (02/2018): "High efficiency video coding".

[tv]	3GPP TS 26.116, "Television (TV) over 3GPP Services; Video Profiles".

[vr]	3GPP TS 26.118, "3GPP Virtual reality profiles for streaming applications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GMS	5G Media Streaming

5GMSA	5G Media Streaming Architecture

AS	Application Server

DASH	Dynamic Adaptive Streaming over HTTP

OMAF	Omnidirectional Media Application Format

TV	Television

UE	User Equipment

VR	Virtual Reality

# 4	5GMS Codecs and formats capabilities

## 4.1	Introduction

This clause defines codecs and formats capabilities for video, audio, speech, subtitles, presentation format, media encapsulation and encryption.

## 4.2	Video

### 4.2.1	H.264 (AVC)

#### 4.2.1.1	Decoding

The following H.264 (AVC) media decoding capabilities are defined:

AVC-HD: the capability to decode H.264 (AVC) Progressive High Profile Level 3.1 [avc] bitstreams, for which the maximum VCL Bit Rate is constrained to be 14Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1000 and 1200, respectively.

AVC-FullHD: the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [avc] bitstreams.

AVC-UHD: the capability to decode H.264/AVC Progressive High Profile Level 5.1 [avc] bitstreams for H.264/AVC with the following additional restrictions and requirements:

- the maximum VCL Bit Rate is constrained to be 120Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

- the bitstream does not contain more than 10 slices per picture.

NOTE:	High Profile for H.264/AVC excludes Flexible macro-block order, Arbitrary slice ordering, Redundant slices, Data partition.

#### 4.2.1.2	Encoding

[TBD]

### 4.2.2	H.265 (HEVC)

#### 4.2.2.1	Decoding

The following H.265 (HEVC) media decoding capabilities are defined:

HEVC-HD: the capability to decode H.265 (HEVC) Main Profile, Main Tier, Level 3.1[hevc] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

HEVC-FullHD: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1[hevc] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

HEVC-UHD: the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1[hevc] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

#### 4.2.2.2	Encoding

[TBD]

## 4.3	Audio

### 4.3.1	Decoding

### Audio decoding capabilities are defined in

3GPP TS 26.117 [5Gaudio] clause 5.3.

### 4.3.2	Encoding

[TBD]

## 4.3	Speech codec

### 4.3.1	Decoding

### Audio decoding capabilities are defined in 3GPP TS 26.117 [5Gaudio] clause 5.2.

### 4.3.2	Encoding

[TBD]

## 4.5	Subtitles

## [TBD]

# 5	5GMS Profiles

## 5.1	Introduction

A 5GMS profile describes a set of capability requirements associated to a service scenario. A default profile is defined for minimum media requirements to be supported in case no other profile is claimed to be supported. The television (TV) profile covers live and on demand streaming of audio-visual services. The 360 Virtual Reality (VR) profile covers the live and on demand streaming of 360 content.

## 5.2	5GMSd streaming profile

### 5.2.1	Introduction

This profile defines selected UE-based 5GMS Client functionalities as defined in TS26.501 [5gmsa] as shown in Figure 5.2.1-1. The functions indicated in green are defined in this clause, namely

- Media Decapsulation

- Media Decryption

- Media Decoding

- Media Presentation and Rendering

### Figure 5.2.1-1 5GMS Client Functionalities in scope of the 5GMSd streaming profile

### 5.2.2	Media Decoding Video

[

Option A (PSS capabilities)

If the 5GMS client supports the reception of video,

- AVC-HD decoding capability shall be supported as defined in clause 4.2.1.1.

- HEVC-HD decoding capability should be supported as defined in clause 4.2.2.1.

End of option A

Option B (upgraded capabilities)

If the 5GMS client supports the reception of video,

- AVC-FullHD decoding capability shall be supported as defined in clause 4.2.1.1.

- HEVC-FullHD decoding capability shall be supported as defined in clause 4.2.2.1.

End of option B

]

### 5.2.2	Media Decoding Audio

If the 5GMS client supports the reception of audio,

- eAAC+ decoding capability should be supported as defined in 3GPP TS 26.117 [5Gaudio] clause 5.3.

- AMR-WB+ decoding capability may be supported as defined in 3GPP TS 26.117 [5Gaudio] clause 5.3.

### 5.2.3	Media Decoding Speech

If the 5GMS client supports the reception of audio,

- the EVS decoding capability shall be supported as defined in 3GPP TS 26.117 [5Gaudio] clause 5.2.

- the AMR-WB decoding capability should be supported as defined in 3GPP TS 26.117 [5Gaudio] clause 5.2.

- the AMR decoding capability may be supported as defined in 3GPP TS 26.117 [5Gaudio] clause 5.2.

### 5.2.4	Presentation format

If 5GMS client supports presentation, then 3GPP HTML5 Profile as defined in 3GPP TS 26.307 [html5] should be supported.

### 5.2.5	Decapsulation and Decryption

#### 5.2.5.1	Introduction

This clause defines the requirements in terms of decapsulation and decryption of media for a 5GMSd Media Player in combination with codecs. Media Encapsulation in 5G Media Streaming for downlink is defined based on the MPEG Common Media Application Format (CMAF) [cmaf]. Based on this, this clause documents the requirements and recommendations for the support of media profiles by the definition of device requirements.

#### 5.2.5.2	Video Profiles

Tbd

#### 5.2.5.3	Audio Profiles

If the 5GMS client supports the reception of audio, then the following shall be supported:

- the EVS playback requirements as defined in 3GPP TS 26.117 [5Gaudio] clause 7.4.2.4

If the 5GMS client supports the reception of audio, then the following should be supported:

- the AMR-WB playback requirements as defined in 3GPP TS 26.117 [5Gaudio] clause 7.3.2.4 shall be supported

- the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [5Gaudio] clause 7.5.2.4 shall be supported

If the 5GMS client supports the reception of audio, then the following may be supported:

- the AMR playback requirements as defined in 3GPP TS 26.117 [5Gaudio] clause 7.2.2.4

#### the AMR-WB+ playback requirements as defined in 3GPP TS 26.117 [5Gaudio] clause 7.6.2.4

#### Encrypted Content

If the 5GMS client supports the encrypted content and any of the video profiles as defined in clause 5.2.5.2, then the 5GMS client shall support the playback requirements for encrypted content as documented in clause 8 of CTA-WAVE 5003 [29], clause 8.12 for either

- video content encrypted according to [CMAF] clause 8, using either the 'cenc' AES-CTR scheme subsample pattern encryption scheme, as specified in [CENC], clause 10.1, or

- video content encrypted according to [CMAF] clause 8, using either the 'cbcs' AES-CBC scheme, subsample pattern encryption scheme, as specified in [CENC], clause 10.4, with the following restrictions Pattern Block length of 10 and an encrypt:skip pattern of 1:9 as defined in clause 9.6 of  [CENC].

If the 5GMS client supports the decrypted content and any of the video profiles in clause 5.2.5.2, then the 5GMS client should support the playback requirements for encrypted content as documented in clause 8 of CTA-WAVE 5003 [29], clause 8.12 for both of the cases:

- video content encrypted according to [CMAF] clause 8, using either the 'cenc' AES-CTR scheme subsample pattern encryption scheme, as specified in [CENC], clause 10.1, and

- video content encrypted according to [CMAF] clause 8, using either the 'cbcs' AES-CBC scheme, subsample pattern encryption scheme, as specified in [CENC], clause 10.4, with the following restrictions Pattern Block length of 10 and an encrypt:skip pattern of 1:9 as defined in clause 9.6 of  [CENC].

If the 5GMS client supports the encrypted content and any of the audio profiles in clause 5.2.5.3, then the 5GMS client shall support the playback requirements for encrypted content as documented in clause 8 of CTA-WAVE 5003 [29], clause 8.12 for either

- audio content encrypted according to [CMAF] clause 8, using either the 'cenc' AES-CTR scheme subsample pattern encryption scheme, as specified in [CENC], clause 10.1, or

- audio content encrypted according to [CMAF] clause 8, using either the 'cbc1' AES-CBC scheme, subsample pattern encryption scheme, as specified in [CENC], clause 9.7.

If the 5GMS client supports the encrypted content and any of the audio profiles in clause 5.2.5.3, then the 5GMS client should support the playback requirements for encrypted content as documented in clause 8 of CTA-WAVE 5003 [29], clause 8.12 for both of the cases:

- audio content encrypted according to [CMAF] clause 8, using either the 'cenc' AES-CTR scheme subsample pattern encryption scheme, as specified in [CENC], clause 10.1, and

audio content encrypted according to [CMAF] clause 8, using either the 'cbc1' AES-CBC scheme, subsample pattern encryption scheme, as specified in [CENC], clause 9.7.

### 5.2.6	Capability Discovery

#### 5.2.6.1	General

A 5GMSd is expected to support capability discovery such that 5GMS-aware applications can identify, if a specific media profile is supported. In order to identify if a media profile is supported, the 5GMS client may provide an API as defined in TS26.512 [API] in via M7d, for which the client can be queried with a specific MIME type string, if the media profile is supported.

The MIME types follow RFC6381 [add].

A 5GMSd client should support at least one of the following capability discovery mechanisms for media profiles:

- If isTypeSupported() for the media profile with argument <profile> results in a yes, then the respective media profile is supported with the requirements defined in a specific clause.

- If isTypeSupported() for the media profile with argument <codec> results in a yes, then the respective media profile is supported with the requirements defined in a specific clause.

- If a conforming CMAF header is provided for playback initialization and 5GMS Client does not throw an error response, then the respective media profile is supported with the requirements defined in a specific clause.

In the following, for each media profile mentioned in clause 5.2.5, the <profile> parameter and the <codec> parameter are provided that can be used in the capability exchange.

#### 5.2.6.2	Video Profiles

#### 5.2.6.3	Audio Profiles

For AMR,

-	the <profile> parameter is defined in TS26.117, clause 7.2.2.4 as audio/mp4 profiles='camr'

### -	the <codecs> parameter is defined in TS26.117, clause 7.2.2.4 as 'samr'

#### 5.2.6.4	Encryption Mode Discovery

#### tbd

## 5.3	Uplink streaming default profile

## [TBD]

## 5.4	TV profile

### 5.4.1	Video decoding

[

Option A (PSS capabilities)

#### 5.4.1.1	H.264 (AVC)

If the 5GMS client supports the Television profile, it shall support the H.264/AVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.4.2.6.

If the 5GMS client supports the Television profile, it should support the H.264/AVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.4.3.6.

#### 5.4.1.2	H.265 (HEVC)

If the 5GMS client supports the Television profile, it should support:

-	H.265/HEVC 720p HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.2.7,

-	H.265/HEVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.3.7,

-	H.265/HEVC UHD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.4.7,

-	H.265/HEVC Full HD HDR Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.5.8, and

-	H.265/HEVC UHD HDR Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.6.8.

End of option A

Option B (upgraded capabilities)

#### 5.4.1.1	H.264 (AVC)

If the 5GMS client supports the Television profile, it shall support the H.264/AVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.4.3.6.

#### 5.4.1.2	H.265 (HEVC)

If the 5GMS client supports the Television profile, it shall support:

-	H.265/HEVC Full HD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.3.7,

If the 5GMS client supports the Television profile, it should support:

-	H.265/HEVC UHD Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.4.7,

-	H.265/HEVC Full HD HDR Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.5.8, and

-	H.265/HEVC UHD HDR Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.6.8.

-	H.265/HEVC Full HD HDR HLG Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.7.7, and

-	H.265/HEVC UHD HDR HLG Operation Point Receiver requirements as specified in TS 26.116 [tv], clause 4.5.8.7.

End of option B

]

### 5.4.2	Audio decoding

If the 5GMS client supports the Television profile, it shall/should support eAAC+ stereo Operation point as defined in 3GPP TS 26. 117 [5Gaudio], clause 6.3.3

## 5.5	360 VR profile

### 5.5.1	Video

#### 5.5.1.1	Operation Points

If the 5GMS UE supports 360 VR video, it shall include a receiver that complies with

-	the Basic H.264/AVC Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 5.1.4.

If the 5GMS UE supports 360 VR video, it should include a receiver that complies with

-	the Main H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 5.1.5.

If the 5GMS UE supports 360 VR video, it may include a receiver that complies with

-	the Flexible H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 5.1.6.

#### 5.5.1.2	DASH encapsulation

If the 5GMS UE supports 360 VR video for DASH services, it shall include a receiver that complies with

-	the Basic Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [vr], clause 5.2.2.3.

If the 5GMS UE supports 360 VR video for DASH services, it should include a receiver that complies with

-	the Main Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [vr], clause 5.2.3.3.

If the 5GMS UE supports 360 VR video for DASH services, it may include a receiver that complies with

-	the Advanced Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [vr], clause 5.2.3.4.

### 5.5.2	Audio

#### 5.5.2.1	Operation Points

If the 5GMS UE supports 3D/VR audio, it should include a receiver that complies with

-	the 3GPP MPEG-H Audio Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 6.1.4.

#### 5.5.2.2	DASH encapsulation

If the 5GMS UE supports 3D/VR audio for DASH services, it should include a receiver that complies with

-	the OMAF 3D Audio Baseline Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [vr], clause 6.2.2.3.

### 4.4.1	Video

#### 4.4.1.1	Operation Points

If the 5GMS UE supports 360 VR video, it shall include a receiver that complies with

-	the Basic H.264/AVC Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 5.1.4.

If the 5GMS UE supports 360 VR video, it should include a receiver that complies with

-	the Main H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 5.1.5.

If the 5GMS UE supports 360 VR video, it may include a receiver that complies with

-	the Flexible H.265/HEVC Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 5.1.6.

#### 4.4.1.2	DASH encapsulation

If the 5GMS UE supports 360 VR video for DASH services, it shall include a receiver that complies with

-	the Basic Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [vr], clause 5.2.2.3.

If the 5GMS UE supports 360 VR video for DASH services, it should include a receiver that complies with

-	the Main Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [vr], clause 5.2.3.3.

If the 5GMS UE supports 360 VR video for DASH services, it may include a receiver that complies with

-	the Advanced Video Media Profile Receiver requirements for DASH as specified in TS 26.118 [vr], clause 5.2.3.4.

### 4.4.2	Audio

#### 4.4.2.1	Operation Points

If the 5GMS UE supports 3D/VR audio, it should include a receiver that complies with

-	the 3GPP MPEG-H Audio Operation Point Receiver requirements as specified in TS 26.118 [vr], clause 6.1.4.

#### 4.4.2.2	DASH encapsulation

If the 5GMS UE supports 3D/VR audio for DASH services, it should include a receiver that complies with

-	the OMAF 3D Audio Baseline Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [vr], clause 6.2.2.3.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-07 | SA4#104 | S4-190593 |  |  |  | TS draft skeleton | 0.0.1 |
| 2019-07 | SA4#104 | S4-190805 |  |  |  | TS draft skeleton | 0.0.2 |
| 2019-07 | SA4#104 | S4-190811 |  |  |  | TS skeleton improvements + VR profile requirements | 0.1.0 |