3GPP TSG-SA4 MBS AHG Meeting	 S4-AHI895

# The Hague, The Netherlands, 12-14 November, 2019					revision of S4-AHI875

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR |  | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sequence Diagrams on Uplink Streaming with Network Assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2019-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 26.501, clause 6.5 “Providing Network Assistance” describes essentially defunct functionality as it was originally based on, and intended to align with, the “Assistance Information” (AsInf) functionality as specified in FLUS. However, during SA4#106, MTSI decided to abandon stage 3 specification of AsInf functionality. On the other hand, 5GMSA also includes the ‘Network Assistance’ feature whereby the UE may request/receive bitrate recommendation and/or boost support from the network. It is proposed that the description and message sequence diagrams in clause 6.5 be correspondingly revised to reflect the appropriate uplink network assistance functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revised description on uplink network assistance, along with example sequence diagram illustrations of uplink assistance based on both the 5GMSu interaction and the RAN signaling methods. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description and sequence diagrams on Uplink Network Assistance functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5 and its subclauses |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 6.5	Providing Network Assistance

### 6.5.1	General

Two methods for network assistance functionality – the ability for the network to provide bitrate recommendation and/or bitrate boost functionalities to the UE, are support for unicast media uplink streaming service delivery. Similar to the network assistance functionality as described for downlink streaming service delivery in clause 5.X and 5.Y, these correspond to, respectively,

- Application layer signaling method based on 5GMSu interaction between the Media AF (specifically, the 5GMSu AF and the Network Assistance sub-function of the Media Session Handler, and

- RAN signaling method based on ANBR/ANBRQ interaction between the RAN and the MAC Entity of the UE.

Simplified versions of message sequence diagrams for uplink network assistance, relative to the ones for downlink network assistance as shown in clause 5.X and 5.Y, are illustrated in subclauses 6.5.2 and 6.5.3.

### 6.5.2	Uplink Network Assistance based on 5GMSu Interaction

The procedure in Figure 6.5.2-1 is a high-level illustration of uplink network assistance provided by the 5GMSu AF to the Media Session Handler

Figure 6.5.2-1: Network Assistance based on 5GMSu Interaction

Steps:

1:	An Uplink streaming session is active.

2:	The Media Session Handler determines the need to request uplink assistance.

3:	Via the M5u API, an uplink assistance request message is sent by the Media Session Handler.

4:  Via the M5u API, an uplink assistance response message is returned by the 5GMSu AF

5:  The Media Session Handler takes an appropriate action upon receiving the message in step 4.

6:  The Uplink streaming session continues as result of the uplink assistance interaction.

### 6.5.3	Uplink Network Assistance based on ANBR/ANBRQ Interaction

The procedure in Figure 6.5.3-1 is a high-level illustration of uplink network assistance provided by the RAN to the Media Session Handler via the UE’s MAC Entity.

Figure 6.5.3-1: Network Assistance based on 5GMSu Interaction

Steps:

1:	An Uplink streaming session is active.

2:	The Media Session Handler determines the need to request uplink assistance.

3:	Via UE internal interface, the Media Session Handler triggers the UE MAC Entity to send an uplink assistance request to the RAN in the form of an ANBRQ message.

4:  The RAN returns the uplink assistance response to the UE MAC Entity in the form of an ANBR message, and the UE MAC Entity in turn forwards that message to the Media Session Handler.

5:  The Media Session Handler takes an appropriate action upon receiving the message in step 4.

6:  The Uplink streaming session continues as result of the uplink assistance interaction.

END OF CHANGE