3GPP TSG-WG SA4 MBS Call	S4-AHI940

# Telco

| CR-For v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR |  | rev | 0 | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Policy template parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The external application provider configures a set of dyamic policies that may be applied to the streaming sessions offered by it. It defines these as a set of operation points for the service, each of which with corresponding QoS requirements. These policies are also shared with the Session Handler on the UE to assist the client with the selection of the proper operation point. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR defines the dynamic policy templates that are used to apply QoS and charging rules to sessions of the corresponding Provisioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 solution will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 4.X	Policy Configuration (PC) Object

The 5GS offers network assistance to multimedia sessions for which provisioning has been performed using the M1d by the content provider with the Provisioning Media AF. A new media streaming session is detected by the Session Handler, which in turn will intiate a network assistance session with the Assistance Media AF over the M5d interface.

A Policy Configuration (PC) Object provides a description of a set of operation points and their respective QoS and charging requirements. The PC Object is hosted by the Provisioning Media AF and information about the configured templates are shared with the Assistance Media AFs.

The PC Object is defined in the following table:

| Parameter | type | Description |
| --- | --- | --- |
| policyId | Integer | Provides an MNO-wide unique Identifier of the policy. |
| OperationPoints | Array of Objects |  |
| OPId | Integer | Provides an indentifier for the current operation point |
| QoS | Object | This object providesthe QoS requirements that apply to this operation point. |
| MaxGBR | Integer | The maximum value for guaranteed bitrate. Note that this implies that GBR = MBR for the operation point that is provisioned for this session. Traffic that exceeds the MaxGBR will be shaped, which may result in excessive delays and packet drops. |
| AveragingWindow | Integer | The averaging window over which the MaxGBR value is calculated. The value is provided in milliseconds. |
| MaxBurst | Integer | The maximum burst that is allowed for the session at this operation point. The MaxBurst is provided in bytes. |
| BurstAveragingWindow | Integer | The averaging window over which the burst is calculated. This value is typically an order of magnitude smaller than the AveragingWindow and is meant to allow for short-lived bursts in the traffic. |
| RouteSeletion | Object | Provides information to describe how traffic of this Provisioning is to be identified and filtered. |
| NSSAI | Integer | Identifier of the network slice used for this operation point. |
| DNN | String | Name of the Data Network that is used for this session at this operation point. |
| TrafficDescriptor | Object | Contains information about the traffic descriptors that are used to identify and filter the traffic. The traffic descriptor shall be formatted as described in 6.6.2.1-2 of TS 23.503. |
| Charging | Object | Provides information about the charging policy to be used for this operation point. |
| sponsor_identifier | String | Provides the identifier for the provider of the sponsored data connection as defined in 23.203[X]. |