3GPP TSG-SA WG4 	AHVIC-153

# June 12th 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.959 | CR | CRNum | rev | - | Current version: | 0.1.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR to 26.959 on Subjective Test Methods for Immersive Audio |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LiQuImAS |  |  |  |  |  |  | Date: |  | 2018-06-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Conclusion of “Tests 1, 2 and 3” required for completion of VRStream work item and LiQuImAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Conclusion of Tests 1, 2 and 3. Adds scope, update references, removes process related aspects not belonging to a technical specification, adds subjective test over headphones, clarifies rendering aspects of Tests 2 and 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VRStream cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************** BEGINNING OF MODIFIED CLAUSE***************

# 1	Scope

The present document specifies subjective test methodologies for 3GPP immersive audio systems including channel-based, object-based, scene-based and hybrids of these formats. The subjective evaluation methods described in the present document are applicable to audio capture, coding, transmission and rendering as indicated in their corresponding clauses.

*************** END OF MODIFIED CLAUSE***************

*************** BEGINNING OF MODIFIED CLAUSE***************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-R BS.1534-3: “Method for the subjective assessment of intermediate quality level of audio systems”.

[3]	ITU-R BS.1116-3: “Methods for the subjective assessment of small impairments in audio systems”

[4]	ITU-R BS.2051-1: “Advance sound system for programme production”

[5]	3GPP TS 26.118: “3GPP Virtual Reality profiles for streaming applications”

*************** END OF MODIFIED CLAUSE***************

*************** BEGINNING OF MODIFIED CLAUSE***************

# 5	Test Methodologies for Immersive Audio Systems of TS 26.118 (Codec Quality Characterization Test)

## 5.1	Introduction

This clause specifies the codec quality characterization test for the audio profiles in TS 26.118. The codec quality characterization test is based on the test method defined in [2].

## 5.2	Experimental Design

The experimental design of the Codec Quality Characterization Test is such that all assessors rate all test Conditions. To control for possible presentation order biases, the presentation order of the test materials is fully randomized during the experiment (double-blind test). To minimize listener fatigue, the following constraints on the experimental design are defined:

- Each Test Material shall be no longer than 12s in duration.

- No more than four Codec Operating Points shall be tested for each Test Material.

- Each experiment shall contain no more than 10 Test Materials.

## 5.3	Selection of Assessors

The selection of assessors shall follow the guidelines in [2] Clause 4.1. Only experienced assessors shall participate in the experiment and the test administrator shall employ pre- and post-screening according to [2] Clause 4.1. The final test results shall include assessments from at least 10 experienced assessors that have passed both pre- and post-screening.

## 5.4	Selection of Content

Critical audio Materials representing typical virtual reality content shall be used for this test. Each test shall include at least 3 channel-based, 3 object-based and 3 scene-based test Materials. In the even a test Material is a hybrid format, the primary category to which the test material belongs to (distributed among channels, objects and scene-based) shall be indicated in the test report.

All test Materials shall be provided as either 24-bit integer or 32-bit PCM float signals with a sampling rate of 48kHz.

## 5.5	Content Presentation

The content presentation and grading process are according to [2] Clauses 5.3 and 5.4.

## 5.6	Listening Environment

The listening environment should comply with [3] Clause 8.2 and 8.3.

## 5.7	Listening System

The listening system shall be loudspeaker-based. The loudspeaker layout is layout J described in [4] Annex 1.

## 5.8	Listening Level

The listening level is according to [2] Clause 8.

## 5.9	Anchor/Reference Conditions

All Codec Quality Characterization Tests shall include one Hidden Reference and two Anchors. The two Anchors are 3.5kHz and 7kHz low-pass filtered versions of the Reference condition, as described in [2] Clause 5.1.

The Reference and Hidden Reference conditions are the source test Materials rendered to the loudspeaker setup through the Reference Renderer of the Audio Profile under test with the coding bypassed.

## 5.10	Test Conditions

The Test Conditions shall be generated by encoding, decoding and rendering the test Materials with the target operating points of:

- 128 kbps (for First Order Ambisonics contents only)

- 256 kbps

- 384 kbps

- 512 kbps

A +/- 10% variation from the target operating points is acceptable. The actual bit-rate for each Test Condition shall be reported with an accompanying justification for the target operating point deviation. The renderer used for the Test Conditions shall be the same renderer used for the Anchor and Reference Conditions.

## 5.11	Attributes

The Codec Quality Characterization Test shall assess the Basic Audio Quality attribute described in [2] Clause 6.4.

## 5.12	Test Report and Presentation of Results

The Test Report shall provide the Mean and 95% Confidence Intervals (t-distribution) for each test Condition, Hidden Reference and Anchors. All results provided shall be post-screened results.

*************** END OF MODIFIED CLAUSE***************

*************** BEGINNING OF MODIFIED CLAUSE***************

# 6	Test Methodologies for Immersive Audio Systems of TS 26.118 (Renderer Comparison Test)

## 6.1	Introduction

This clause specifies the Renderer Comparison Test for the audio profiles in TS 26.118. The Renderer Comparison Test is loosely inspired by the Comparison Category Rating test paradigm described in [5] Annex E.

## 6.2	Experimental Design

In the Renderer Comparison Test, the assessors compare a Test Condition against Anchor Conditions on four audio quality Attributes. The presentation of the Test and Anchor Conditions is binaural using head-tracking. For each trial, the Test Condition is compared to one of the Anchor Conditions as an A v. B comparison. To control for possible presentation order biases, the test Conditions shall be presented to the assessors as sample A in exactly half of the trials. The test shall be conducted with 12 Test Materials and two Anchors for a total of 24 trials.

The test is divided in two sessions. the first session compares the Test Condition against the first Anchor and the second session compares the Test Condition against the second Anchor.

## 6.3	Selection of Assessors

The selection of assessors shall follow the guidelines in [2] Clause 4.1. Only experienced assessors shall participate in the experiment and the test administrator shall employ pre- and post-screening according to [2] Clause 4.1. The final test results shall include assessments from at least 12 experienced assessors that have passed both pre- and post-screening.

## 6.4	Selection of Content

The Rendering Comparison Test shall use Critical Audio Materials representing typical virtual reality content, with a duration longer than 6s and no longer than 12s. The Rendering Comparison Test shall include 4 channel-based, 4 object-based and 4 scene-based test Materials. In the event a test Material is a hybrid format, the primary category to which the test material belongs to (distributed among channels, objects and scene-based) shall be indicated in the test report.

All test Materials shall be provided as either 24-bit integer or 32-bit PCM float signals with a sampling rate of 48kHz.

## 6.5	Content Presentation

The Test Administration Platform shall employ a Graphical User Interface (GUI) to present the Test and Reference Conditions to the assessors as A/B samples within trials. The following are constraints on the GUI design:

- The GUI shall have an “A” and “B” switch buttons which allow the assessor to seamlessly switch the audio presentation between the A and B samples for comparison.

- The GUI shall have a “Play” button which enables Time-Synchronized Playback of the A and B samples. Within a trial, one of the samples is a bit-stream for the Test Condition and the other sample is one of the Anchor Conditions.

- The GUI shall have a “Stop” button which enables stopping the Time-Synchronized Playback of the A and B samples.

- The GUI shall present four Audio Quality Attributes for assessment: Timbre (TIM), Spatial (SPA), Artifacts (ART) and Basic Audio Quality (BAQ). In addition, the GUI shall present the possibility of comparing the Loudness (LOUD) of the A and B samples through an additional loudness scale.

- The GUI shall have a “Loop” button which enables looping the Time-Synchronized playback of the A and B samples.

- The GUI shall have a “Next” button which enables the assessor to proceed to the next trial in the experiment. For each trial, the GUI shall enable the “Next” button only after assessment of TIM, SPA and BAQ have been completed. Because all source Test Materials are normalized for Listening Level according to Clause 6.8 and the highest operating point.

In addition, the Test Administration Platform shall support a real-time implementation of the Audio Profile Renderer under test as well as a real-time implementation of the Anchor Condition (see Clause 6.9) with support for head-tracking.

Figure 1 - Example of possible GUI for Rendering Comparison Test

## 6.6	Listening Environment

For each octave-band, the maximum sound pressure level of the listening environment shall not exceed the levels in Table 1 (corresponding to an NR20 noise rating curve):

| Octave Band center frequency | 31.5Hz | 62.5Hz | 125Hz | 250Hz | 500Hz | 1kHz | 2kHz | 4kHz | 8kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum Sound Pressure Level (dBSPL) | 69 | 51 | 39 | 31 | 24 | 20 | 17 | 14 | 13 |

Table 1 - Maximum Sound Pressure Level for Listening Environment

## 6.7	Listening System

The listening system shall be headphone-based with head-tracking. Both the Test Conditions and Anchor/Reference Conditions shall be binauralized using a common HRTF set. The binauralization shall use either individualized HRTFs or HRTFs based on a head and torso simulator (HATS). The choice of HRTF set shall be indicated in the test report. The headphones shall be equalized. If individualized HRTFs are used, the headphones shall have individualized equalization. If HATS HRTFs are used, the headphones shall be equalized for the same make/model of HATS.

## 6.8	Listening Level

The listening level is according to [2] Clause 8.

## 6.9	Anchor/Reference Conditions

All Renderer Comparison Tests shall include two Anchor/Reference Conditions. The two Anchors correspond to two configurations of a Common Informative Binaural Rendering (CIBR) scheme (1st and 3rd order). The CIBR:

- Receives as an input a virtual loudspeaker representation, obtained using a documented loudspeaker rendering scheme, with speaker locations positioned according to an Equivalent Spatial Domain representation (ESD). The definition of Equivalent Spatial Domain can be found in TS 26.260 Clause 4.1.1.

- Converts the ESD representation to a 1st order or 3rd order B-format representation.

- Performs rotation of the sound field, according to a motion sensor signal

- Binauralizes the audio signal for presentation.

NOTE:	The Documented Loudspeaker Renderer is Vector Based Amplitude Panning (VBAP) (Pulkki).

A block diagram of the rendering systems for Anchor Conditions is illustrated in Figure 2.

Figure 2 - Block Diagram for Anchor Conditions

## 6.10	Test Conditions

The Rendering Comparison Test shall assess only one Test Condition per experiment. This Test Condition is such that the Audio Profile shall be configured for an Operating Point providing transparent quality for all Test Materials. In addition, the Audio Profile shall be configured to operate with its Reference Renderer. For all Test Materials, the Test Condition shall be assessed against the two Anchor Conditions.

## 6.11	Attributes

The Rendering Comparison Test shall assess the four Audio Quality Attributes: Timbre (TIM), Spatial (SPA), Artifacts (ART) and Basic Audio Quality (BAQ). In addition, the Rendering Comparison Test compares any residual Loudness (LOUD) difference between A and B samples through an additional loudness scale.

## 6.12	Test Report and Presentation of Results

The Test Report shall provide the Mean and 95% Confidence Intervals (t-distribution) for the Test Condition against each of the Anchor Conditions. All results provided shall be post-screened results.

*************** END OF MODIFIED CLAUSE***************

*************** BEGINNING OF MODIFIED CLAUSE***************

# 7	Test Methodologies for Immersive Audio Systems of TS 26.118 (Codec Quality Characterization Test with Binaural Rendering)

## 7.1	Introduction

This clause specifies the optional codec quality characterization test for the audio profiles in TS 26.118 with binaural rendering over headphones. The codec quality characterization test is based on the test method defined in [2].

## 7.2	Experimental Design

The experimental design of the Codec Quality Characterization Test is such that all assessors rate all test Conditions. To control for possible presentation order biases, the presentation order of the test materials is fully randomized during the experiment (double-blind test). To minimize listener fatigue, the following constraints on the experimental design are defined:

- Each Test Material shall be no longer than 12s in duration.

- No more than four Codec Operating Points shall be tested for each Test Material.

- Each experiment shall contain no more than 10 Test Materials.

## 7.3	Selection of Assessors

The selection of assessors shall follow the guidelines in [2] Clause 4.1. Only experienced assessors shall participate in the experiment and the test administrator shall employ pre- and post-screening according to [2] Clause 4.1. The final test results shall include assessments from at least 10 experienced assessors that have passed both pre- and post-screening.

## 7.4	Selection of Content

Critical audio Materials representing typical virtual reality content shall be used for this test. Each test should include at least 3 channel-based, 3 object-based and 3 scene-based test Materials and no more than 10 test Materials in total.

All test Materials shall be provided as either 24-bit integer or 32-bit PCM float signals with a sampling rate of 48kHz.

## 7.5	Content Presentation

The content presentation and grading process are according to [2] Clauses 5.3 and 5.4.

## 7.6	Listening Environment

For each octave-band, the maximum sound pressure level of the listening environment shall not exceed the levels in Table 1 (corresponding to an NR20 noise rating curve):

| Octave Band center frequency | 31.5Hz | 62.5Hz | 125Hz | 250Hz | 500Hz | 1kHz | 2kHz | 4kHz | 8kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum Sound Pressure Level (dBSPL) | 69 | 51 | 39 | 31 | 24 | 20 | 17 | 14 | 13 |

Table 2 - Maximum Sound Pressure Level for Listening Environment

## 7.7	Listening System

The listening system shall be headphone-based using the Common Informative Binaural Renderer (CIBR) for both the Reference and Degraded conditions. The CIBR is described in [5].

The binauralization shall use either individualized HRTFs or HRTFs based on a head and torso simulator (HATS). The choice of HRTF set shall be indicated in the test report. The headphones shall be equalized. If individualized HRTFs are used, the headphones shall have individualized equalization. If HATS HRTFs are used, the headphones shall be equalized for the same make/model of HATS.

## 7.8	Listening Level

The listening level is according to [2] Clause 8.

## 7.9	Anchor/Reference Conditions

All Codec Quality Characterization Tests shall include one Hidden Reference and two Anchors. The two Anchors are 3.5kHz and 7kHz low-pass filtered versions of the Reference condition, as described in [2] Clause 5.1.

The Reference and Hidden Reference conditions are the source test Materials binaurally rendered to headphones through the Common Informative Binaural Renderer (CIBR) described in [5].

## 7.10	Test Conditions

The Test Conditions are generated by encoding, decoding and rendering the test Materials with the target operating points of:

- 128 kbps (for First Order Ambisonics contents only)

- 256 kbps

- 384 kbps

- 512 kbps

A +/- 10% variation from the target operating points is acceptable. The actual bit-rate for each Test Condition shall be reported with an accompanying justification for the target operating point deviation. The renderer used for the Test Conditions shall be the same renderer used for the Anchor and Reference Conditions.

## 7.11	Attributes

The Codec Quality Characterization Test with Binaural Rendering shall assess the Basic Audio Quality attribute described in [2] Clause 6.4.

## 7.12	Test Report and Presentation of Results

The test report shall provide the Mean and 95% Confidence Intervals (t-distribution) for each test Condition, Hidden Reference and Anchors. All results provided shall be post-screened results (see Clause 7.3).

*************** END OF MODIFIED CLAUSE***************

*************** BEGINNING OF MODIFIED CLAUSE***************