3GPP TSG-SA4 Meeting 3GPPSA4-on VR Stream 	AHVIC-162

# 14 June 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | CRNum | rev | 0 | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Video Operation Points for TS26.118 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer HHI |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VRStream |  |  |  |  |  |  | Date: |  | 2018-04-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Updates and Refines Operation Points |  |  |  |  |  |  |  |  |
| Summary of change: |  | Detailed specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Contains some remaining editor’s note. Decision on level 5.1 and level 5.2 for flexible H.264/AVC profile necessary. |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.918: "Virtual Reality (VR) media services over 3GPP".

[3]	Recommendation ITU-R BT.709-6 (06/2015): "Parameter values for the HDTV standards for production and international programme exchange".

[4]	Recommendation ITU-R BT.2020-2 (10/2015): "Parameter values for ultra-high definition television systems for production and international programme exchange".

[5]	Recommendation ITU-T H.264 (04/2017): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2014: "Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding".

[6]	Recommendation ITU-T H.265 (12/2016): "High efficiency video coding" | ISO/IEC 23008-2:2015: "High Efficiency Coding and Media Delivery in Heterogeneous Environments – Part 2: High Efficiency Video Coding".

[7]	3GPP TS 26.244: "Transparent end-to-end packet switched streaming service (PSS); 3GPP file format (3GP)".

[8]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[9]	ISO/IEC 14496-15: 2017: "Information technology - Coding of audio-visual objects - Part 15: Carriage of network abstraction layer (NAL) unit structured video in ISO base media file format".

[10]	ISO/IEC 23001-8:2016, "Information technology -- MPEG systems technologies -- Part 8: Coding-independent code points".

[11]	Recommendation ITU-R BT.2100-1 (06/2017): "Image parameter values for high dynamic range television for use in production and international programme exchange".

[12]	ISO/IEC 23090-2, "Information technology -- Coded representation of immersive media -- Part 2: Omnidirectional media format".

=====  CHANGE  ===== (Replace the existing clause 5.1)

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3DOF	3 Degrees of freedom

BRIR	Binaural Room Impulse Response

CMP	Cubemap projection

DASH	Dynamic Adaptive Streaming over HTTP

ERP	Equirectangular projection

FOV	Field of view

HMD	Head Mounted Display

HOA	High Order Ambisonics

HRTF	Head-related transfer function

OMAF	Omnidirectional MediA Format

VR	Virtual Reality

### 4.1.3	Video Signal Representation

Commonly used video encoders cannot directly encode spherical videos, but only 2D textures. However, there is a significant benefit to reuse conventional 2D video encoders. Based on this, Figure 4.4 provides the basic video signal representation in the context of omnidirectional video in the context of the present document. By pre-processing, the spherical video is mapped to a 2D texture. The 2D texture is encoded with a regular 2D video encoder and the VR rendering metadata is encoded and provided along with the video bitstream, such that at the receiving end the inverse process can be applied to reconstruct the spherical video.

Figure 4.4: Video Signal Representation

Equirectangular projection

Mapping of spherical video to a 2D texture signal is illustrated in Figure 4.5. The most commonly used mapping from spherical to 2D is the equirectangular mapping. The mapping is bijective, i.e. it may be expressed in both directions.

| 3d-space |  | Projected picture (2D texture signal) |
| --- | --- | --- |
| Figure 4.5: Mapping of spherical video to a 2D texture signal [12]. |  |  |

Following the definitions in clause 4.1.2, the mapping of the color samples of 2D texture images onto a spherical coordinate space in angular coordinates (ϕ, θ) for use in omnidirectional video applications for which the viewing perspective is from the origin looking outward toward the inside of the sphere. The spherical coordinates are defined so that ϕ is the azimuth and θ is the elevation.

Assume a 2D texture with pictureWidth and pictureHeight, being the width and height, respectively, of a monoscopic projected luma picture, in luma samples and the center point of a sample location (i, j) along the horizontal and vertical axes, respectively, then for the equirectangular projection the sphere coordinates (, ) for the luma sample location, in degrees, are given by the following equations:

= ( 0.5 − i ÷ pictureWidth ) * 360
 = ( 0.5 − j ÷ pictureHeight ) * 180

Cubemap projection

Mapping of cubemap projected video to a 2D texture signal is illustrated in Figure 4.6. The mapping is bijective, i.e. it may be expressed in both directions.

| 3d-space |  | Projected picture (2D texture signal) |
| --- | --- | --- |
| Figure 4.6: Relation of the cube face arrangement of the projected picture to the sphere coordinates [12]. |  |  |

Assume a 2D texture with pictureWidth and pictureHeight, being the width and height, respectively, of a monoscopic projected luma picture, in luma samples and the center point of a sample location (i, j) along the horizontal and vertical axes, respectively, where i and j are in relative projected picture sample units and may have non-integer real values.

The values of pictureWidth and pictureHeight shall be such that pictureWidth is a multiple of 3, pictureHeight is a multiple of 2, and pictureWidth / 3 is equal to pictureHeight / 2.

The sphere coordinates (ϕ, θ) for the luma sample location, in degrees, are given by the following equations:

lw = pictureWidth / 3
lh = pictureHeight / 2
tmpHorVal = i − Floor( i ÷ lw ) * lw
tmpVerVal = j − Floor( j ÷ lh ) * lh
i′ = −( 2 * tmpHorVal ÷ lw ) + 1
j′ = −( 2 * tmpVerVal  ÷ lh ) + 1
w = Floor( i ÷ lw )
h = Floor( j ÷ lh )
if( w  = =  1  &&  h  = =  0 ) { // positive x front face
	x = 1.0
	y = i′
	z = j′
} else if( w  = =  1  &&  h  = =  1 ) { // negative x back face
	x = −1.0
	y = −j′
	z = −i′
} else if( w  = =  2  &&  h  = =  1 ) { // positive z top face	(5-2)
	x = −i′
	y = −j′
	z = 1.0
} else if( w  = =  0  &&  h  = =  1 ) { // negative z bottom face
	x = i′
	y = −j′
	z = −1.0
} else if( w  = =  0  &&  h  = =  0 ) { // positive y left face
	x = −i′
	y = 1.0
	z = j′
} else { // ( w  = =  2  &&  h  = =  0 ), negative y right face
	x = i′
	y = −1.0
	z = j′
}
ϕ  = Atan2( y, x ) * 180 ÷ π
θ = Asinz ÷ x2 + y2 + z2 * 180 ÷ π

In addition to regular projection, other pre-processing may be applied to the video when mapped into 2D textures. Examples include region-wise packing, stereo frame packing or rotation. The present document defines different pre- and post-processing schemes in the context of video rendering schemes.

# 5	Video Enablers

# 5.1	Video Operation Points

## 5.1.1	Definition of Operation Point

For the purpose to define interfaces to a conforming video decoder, video operation points are defined. In this case the following definitions hold:

- Operation Point: A collection of discrete combinations of different content formats including spatial and temporal resolutions, colour mapping, transfer functions, rendering metadata etc. and the encoding format.

- Receiver: A receiver that can decode and render any bitstream that is conforming to a certain Operation Point.

- Bitstream: A video bitstream that conforms to a video encoding format and certain Operation Point including VR Rendering Metadata

Figure X Video Operation Points

This clause focuses on the interoperability point to a media decoder as indicated in Figure X. This clause does not deal with the access engine and file parser which addresses aspects how the video bitstream is delivered.

In all video operation points, the VR Presentation can be rendered using a single media decoder which provides decoded signals and rendering metadata by decoding relevant SEI messages.

## 5.1.2	Parameters of Visual Operation Point

This clause defines the potential parameters of Visual Operation Points. This includes potential video decoder profile and levels with additional restrictions as well as conventional video signal parameters and VR rendering metadata. The requirements are defined from the perspective of the video decoder and renderer.

Parameters for a Visual Operation Point include

-	Codec, Profile and level requirements

-	Restrictions of regular video parameters, typically expressed in the Video Usability informations

-	Usage and restrictions of VR rendering metadata

## 5.1.3	Operation Point Summary

This specification defines several operation points for different target applications and scenarios. In particular, two legacy operation points are defined that use existing video codecs H.264/AVC and H.265/HEVC to enable distribution of up to 4K full 360 mono video signals up to 60Hz by using simple equirectangular projection.

In addition, one operation for each codec is defined that enables enhanced features, in particular stereo video, up to 8K mono, higher frame rates and HDR.

Table 5.1-1 summarizes the Operation Points, the detailed definitions are defined in the remainder of clause 5.1.

Restrictions on source formats such as resolution and frame rates, content generation and encoding guidelines are provided in Annex A.

Table 5.1-1 Summary of Operation Points

| Operation Point name | Decoder | Bit depth | Typical Spatial Resolution | Frame Rate | Colour space format | Transfer Characteristics | Projection | Rotation | RWP | Stereo |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H.264/AVC Basic | H.264/AVC HP@L5.1 | 8 | Up to 4k | Up to 60 Hz | BT.709 | BT.709 | ERP w/o padding | No | No | No |
| H.264/AVC Flexible | H.264/AVC [HP@L5.1/ HP@L5.2] | 8 | Up to 6k in mono and 4k in stereo | Up to 120 Hz | BT.709 | BT.709 | ERP w/o padding | No | Full | TaB |
| H.265/HEVC Basic | H.265/HEVC MP10@L5.1 | 8, 10 | Up to 4k | Up to 60 Hz | BT.709 BT.2020 | BT.709, | ERP w/o padding | No | No | No |
| H.265/HEVC Flexible | H.265/HEVC MP10@L5.1 | 8, 10 | Up to 8k in mono and 4k in stereo | Up to 120 Hz | BT.709 BT.2020 | BT.709, BT.2100 PQ | ERP w/o padding CMP | No | Full | TaB |

VR Rendering metadata in the Operation Points is carried in SEI messages. Receivers are expected to be able to process the VR metadata carried in SEI messages. However, the same VR metadata may be duplicated on system-level. In this case, the Receiver may rely on the system level processing to extract the relevant VR Rendering metadata rather than extracting this from the SEI message.

## 5.1.4	Basic H.264/AVC

#### 5.1.4.1 	General

This operation point targets simple deployments and legacy receivers at basic quality. A full 360-degree video signal with equirectangular projection following the 3GPP reference system may be provided to the decoding and rendering system for immediate decoding and rendering. Note that this operation point enables to distribute 4k video at regular frame rates and 3k video at higher frame rates.

Restricted coverage is supported as well, but only in a basic any backward-compatible fashion.

A Bitstream conforming to the 3GPP VR Basic H.264/AVC Operation point shall conform to the requirements in the remainder of clause 5.1.4.

A receiver conforming to the 3GPP VR Basic H.264/AVC Operation point shall support decoding and rendering a Bitstream conforming to the 3GPP VR Basic H.264/AVC Operation point. Detailed receiver requirements are provided in the remainder of clause 5.1.4.

#### 5.1.4.2	Profile and level

A Bitstream conforming to the 3GPP VR Basic H.264/AVC Operation point shall conform to H.264/AVC Progressive High Profile Level 5.1 [5] for H.264/AVC with the following additional restrictions and requirements

- the maximum VCL Bit Rate is constrained to be 120Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

- the bitstream does not contain more than 10 slices per picture

Note: High Profile for H.264/AVC excludes Flexible macro-block order (FMO), Arbitrary slice ordering (ASO), Redundant slices (RS), Data partition

Hence, for a Bitstream conforming to the 3GPP VR Basic H.264/AVC Operation point, the following applies

-	The profile_idc shall be set to 100 indicating the High profile.

-	The constrain_set0_flag, constrain_set1_flag, constrain_set2_flag and constrain_set3_flag shall all be set to 0.

-	The value of level_idc shall not be greater than 51 (corresponding to the level 5.1) and should indicate the lowest level to which the Bitstream conforms.

#### Aspect Ratios and Spatial resolutions

Picture aspect ratio 2:1 should be used for the encoded picture.

The spatial resolution of the original format in equirectangular projection (ERP) should be one of the following (expressed in luminance pixel width ×  luminance pixel height):

-	4096 × 2048, 3840 × 1920, 3072 × 1536, 2880 × 1440, 2048 × 1024.

The spatial resolution of the distribution format should be one of the following (expressed in luminance pixel width × luminance pixel height):

-	3840 × 1920, 2880 × 1440, 1920 × 960, 1440 × 720, 960 × 480, 720 × 360.

-	4096 × 2048, 3072 × 1536, 2048 × 1024, 1536 × 768, 1024 × 512.

NOTE 1:	Distribution formats do not exceed the native resolution of the Operation Point, but they may be subsampled in order to optimize distribution or adapt to the viewing conditions.

A Receiver conforming to the 3GPP VR Basic H.264/AVC Operation Point shall be capable of decoding and rendering Bitstreams that contain spatial resolutions as above.

#### 5.1.4.4	Colour information

A Bitstream conforming to the 3GPP VR Basic H.264/AVC Operation Point shall use Recommendation ITU-R BT.709 [3] colorimetry. Hence, in the VUI, the colour parameter information shall be present, i.e.

-	video_signal_type_present_flag value and colour_description_present_flag value shall be set to 1.

-	The colour_primaries value, the transfer_characteristics value and the matrix_coefficients value in the Video Usability Information shall all be set to 1.

A Receiver conforming to the 3GPP VR Basic H.264/AVC Operation Point shall be capable of decoding and rendering Bitstreams that use Recommendation ITU-R BT.709 [3] colorimetry according to the bitstream requirements documented above.

#### 5.1.4.5	Frame rates

A Bitstream conforming to the 3GPP VR Basic H.264/AVC Operation Point shall have one of the following frame rates: 24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001 Hz.

The profile and level constraints of H.264/AVC Progressive High Profile Level 5.1 require careful balance of the permitted frame rates and spatial resolutions. Table 5.1-2 provides the permitted combinations of spatial resolutions and frame rates.

Table 5.1-2 Permitted combinations of spatial resolutions and frame rates

| Spatial Resolution | Permitted Frame Rates |
| --- | --- |
| 4096 × 2048 | 24; 25; 30; 24/1.001; 30/1.001 Hz |
| 3840 × 1920 | 24; 25; 30; 24/1.001; 30/1.001 Hz |
| 3072 × 1536 | 24; 25; 30; 24/1.001; 30/1.001; 50 Hz |
| 2880 × 1440 | 24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001 Hz |
| 2048 × 1024 | 24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001 Hz |

In the VUI, the timing information may be present.

-	If the timing information is present, i.e. the value of timing_info_present_flag is set to 1, then the values of num_units_in_tick and time_scale shall be set according to the frame rates allowed above. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level.

-	The frame rate shall not change between two RAPs. fixed_frame_rate_flag value shall be set to 1.

A Receiver conforming to the 3GPP VR Basic H.264/AVC Operation Point shall be capable of decoding and rendering Bitstreams that use frame rates according to the bitstream requirements documented above.

#### 5.1.4.6	Random access point

For H.264/AVC random access point (RAP) definition refer to TS26.116 [X], clause 4.4.1.1.

RAPs shall be present in the Bitstream at least once every 5 seconds. It is recommended that RAPs occur in the video Bitstream on average at least every 2 seconds. The time interval between successive RAPs is measured as the difference between their respective decoding time values.

#### 5.1.4.7	Sequence parameter set

The following restrictions apply to the active Sequence Parameter Set (SPS):

-	gaps_in_frame_num_value_allowed_flag value shall be set to 0.

-	The Video Usability Information shall be present in the active Sequence Parameter Set. The vui_parameter_present_flag shall be set to 1.

-	The source video format shall be progressive. frame_mbs_only_flag shall be set to 1 for every picture of the Bitstream.

#### 5.1.4.8	Video usability information

In addition to the previous constraints on the VUI on colour information in clause 5.1.4.4 and on frame rates in clause 5.1.4.5, this clause contains further requirements.

The aspect ratio information shall be present, i.e.

-	The aspect_ratio_present_flag value shall be set to 1.

-	The aspect_ratio_idc value shall be set to 1 indicating a square pixel format.

There are no requirements on output timing conformance for H.264/AVC decoding (Annex C of [5]). The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

#### 5.1.4.9	Omni-directional Projection Format

This operation point uses equirectangular projection, such the video is automatically rendered in the 3GPP reference system. This is enabled by using the MPEG metadata on equirectangular projection.

A Bitstream conforming to the 3GPP VR Basic H.264/AVC Operation Point shall include the equirectangular projection SEI message (payloadType equal to 150) at every RAP. The erp_guard_band_flag shall be set to 0.

A Receiver conforming to the 3GPP VR Basic H.264/AVC Operation Point shall be able to process the information contained on equirectangular projection SEI message (payloadType equal to 150) with erp_guard_band_flag shall be set to 0.

#### 5.1.4.10	Restricted Coverage

This operation point permits the decoding and rendering of restricted coverage video signals in a rudimentary way. In this case it is expected that pixels that are projected to a non-covered region are included in the full image, but are visually differentiated from the covered region, for example using black, grey or white color.

Application or system-based signalling may support signalling the coverage region.

#### 5.1.4.12	Other VR Metadata

For a Bitstream conforming to the 3GPP VR Basic H.264/AVC Operation Point

- the equirectangular projection SEI message (payloadType equal to 150) with erp_guard_band_flag not set to 0 shall not be present.

- the sphere rotation SEI message (payloadType equal to 154) shall not be present.

- the region-wise packing SEI message (payloadType equal to 155) shall not be present.

- the frame-packing arrangement SEI message (payloadType equal to 45) shall not be present

#### 5.1.4.13	Receiver Compatibility

Receivers conforming to the 3GPP VR Basic H.264/AVC Operation Point shall support decoding and displaying 3GPP VR Basic H.264/AVC Operation Point Bitstreams.

Receivers conforming to the 3GPP VR Basic H.264/AVC Operation Point shall support all Receiver requirements in clause 5.1.4.

## 5.1.5	Flexible H.264/AVC

#### 5.1.5.1 	General

This operation targets enhanced 360 video decoding and rendering of H.264/AVC video for VR applications. Among others, this operation point supports rendering of:

- 4K mono video at up to 120 Hz frame rates

- 4K stereoscopic video at up to 60 Hz frame rates

- Higher than 4K resolutions for restricted coverage

- Rendering of certain viewports beyond 4K

A Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation point shall conform to the requirements in the remainder of clause 5.1.5.

A Receiver conforming to the 3GPP VR Flexible H.264/AVC Operation point shall support decoding and rendering a Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation point. Detailed receiver requirements are provided in the remainder of clause 5.1.5.

#### 5.1.5.2	Profile and level

A Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation point shall conform to H.264/AVC Progressive High Profile Level [5.1/5.2] [5] for H.264/AVC with the following additional restrictions and requirements

- the maximum VCL Bit Rate is constrained to be 120Mbps with cpbBrVclFactor and cpbBrNalFactor being fixed to be 1250 and 1500, respectively.

- the bitstream does not contain more than 10 slices per picture

Note: High Profile for H.264/AVC excludes Flexible macro-block order (FMO), Arbitrary slice ordering (ASO), Redundant slices (RS), Data partition

Hence, for a Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation point, the following applies

-	The profile_idc shall be set to 100 indicating the High profile.

-	The constrain_set0_flag, constrain_set1_flag, constrain_set2_flag and constrain_set3_flag shall all be set to 0.

-	The value of level_idc shall not be greater than [51/52] (corresponding to the level [5.1/5.2]) and should indicate the lowest level to which the Bitstream conforms.

#### 5.1.5.3	Spatial resolutions

Due to the options provided in this operation point, additional original format may be considered that can then be decoded and rendered by a Receiver conforming to this operation point. Recommended formats in equirectangular projection (ERP) beyond those specified in clause 5.1.4.3 are:

- Mono formats: 6144 × 3072, 5880 × 2880

- Stereo formats with resolution for each eye: 4096 × 2048, 3840 × 1920, 3072 × 1536, 2880 × 1440, 2048 × 1024

The distribution formats are more flexible as additional VR metadata as defined in the remainder of clause 5.1.5 may be used.

According to H.264/AVC Progressive High Profile Level 5.2 [5], the maximum luminance width and height does not exceed 8688 pixels. For improved interoperability, for a Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation point, the decoded texture signal

- shall not exceed the luminance width of 8192 pixels, and

- shall not exceed the luminance height of 8192 pixels.

Beyond this, all requirements of H.264/AVC Progressive High Profile Level 5.2 [5] apply.

A Receiver conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall be capable of decoding and rendering Bitstreams with a decoded texture signal of maximum luminance width of 8192 pixels and maximum luminance height of 8192 pixels.

#### 5.1.5.4	Colour information

A Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall use Recommendation ITU-R BT.709 [3] colorimetry. Hence, in the VUI, the colour parameter information shall be present, i.e.

-	video_signal_type_present_flag value and colour_description_present_flag value shall be set to 1.

-	The colour_primaries value, the transfer_characteristics value and the matrix_coefficients value in the Video Usability Information shall all be set to 1.

A Receiver conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall be capable of decoding and rendering Bitstreams that use Recommendation ITU-R BT.709 [3] colorimetry according to the bitstream requirements documented above.

#### 5.1.5.5	Frame rates

A Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall have one of the following frame rates: 24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001, 90, 100, 120 Hz.

In the VUI, the timing information may be present.

-	If the timing information is present, i.e. the value of timing_info_present_flag is set to 1, then the values of num_units_in_tick and time_scale shall be set according to the frame rates allowed above. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level.

-	The frame rate shall not change between two RAPs. fixed_frame_rate_flag value shall be set to 1.

A Receiver conforming to the 3GPP VR Basic H.264/AVC Operation Point shall be capable of decoding Bitstreams that use frame rates according to the bitstream requirements documented above. Rendering shall be supported up to 60Hz and should be supported up to 120Hz.

#### 5.1.5.6	Random access point

For H.264/AVC random access point (RAP) definition refer to TS26.116 [X], clause 4.4.1.1.

RAPs shall be present in the Bitstream at least once every 5 seconds. It is recommended that RAPs occur in the video Bitstream on average at least every 2 seconds. The time interval between successive RAPs is measured as the difference between their respective decoding time values.

If viewport adaptation is offered, then RAPs should occur even more frequently to enable transitioning across these viewport-optimized bitstreams.

#### 5.1.5.7	Sequence parameter set

The following restrictions apply to the active Sequence Parameter Set (SPS):

-	gaps_in_frame_num_value_allowed_flag value shall be set to 0.

-	The Video Usability Information shall be present in the active Sequence Parameter Set. The vui_parameter_present_flag shall be set to 1.

-	The source video format shall be progressive. frame_mbs_only_flag shall be set to 1 for every picture of the Bitstream.

#### 5.1.5.8	Video usability information

In addition to the previous constraints on the VUI on colour information in clause 5.1.5.4 and on frame rates in clause 5.1.5.5, this clause contains further requirements.

The aspect ratio information shall be present, i.e.

-	The aspect_ratio_present_flag value shall be set to 1.

-	The aspect_ratio_idc value shall be set to 1 indicating a square pixel format.

There are no requirements on output timing conformance for H.264/AVC decoding (Annex C of [5]). The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

#### 5.1.5.9	Omni-directional Projection Format

This operation point uses equirectangular projection, such the video is automatically rendered in the 3GPP reference system. This is enabled by using the MPEG metadata on equirectangular projection.

A Bitstream conforming to the 3GPP VR Basic H.264/AVC Operation Point shall include the equirectangular projection SEI message (payloadType equal to 150) at every RAP. The erp_guard_band_flag shall be set to 0.

A Receiver conforming to the 3GPP VR Basic H.264/AVC Operation Point shall be able to process the information contained on equirectangular projection SEI message (payloadType equal to 150) with erp_guard_band_flag shall be set to 0.

Note: The functionality of the guard band signal may equivalently be provided by region-wise packing.

#### 5.1.5.10	Restricted Coverage

This operation point permits to distribute content with less than 360 degree coverage in an encoding optimized manner by the use of region-wise packing.

It is recommended that the number of pixels that are projected to non-covered regions are minimized in the decoded texture signal. If this is applied and not the full 360 video is encoded, the region-wise packing SEI message (payloadType equal to 155) shall be included in the bitstream to signal the encoded regions of the 360 video. If present, it shall be present in a H.264/AVC RAP.

Application or system-based signalling may support signalling the exact coverage region in the spherical coordinates.

#### 5.1.5.11	Viewport-Optimized Content

This operation point permits the use of region-wise packing, for example to optimize the spatial resolution of specific viewports. For some example usage and settings, refer to clause 5.1.5.15.

A Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation Point may include the region-wise packing SEI message (payloadType equal to 155). If present, it shall and shall only be present in a H.264/AVC RAP.

A Receiver conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall be able to process the information in the region-wise packing SEI message (payloadType equal to 155). 5.1.5.12	Frame packing arrangement

A Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation Point may include the frame packing arrange SEI message (payloadType equal to 45). If present, then the following settings shall apply

- The SEI message is present in a H.264/AVC RAP.

- The value of frame_packing_arrangement_cancel_flag is equal to 0.

- The value of frame_packing_arrangement_type is equal to 4.

- The value of quincunx_sampling_flag is equal to 0.

- The value of spatial_flipping_flag is equal to 0.

- The value of field_views_flag is equal to 0.

- The value of frame0_grid_position_x is equal to 0.

- The value of frame0_grid_position_y is equal to 0.

- The value of frame1_grid_position_x is equal to 0.

- The value of frame1_grid_position_y is equal to 0.

A Receiver conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall be able to process the information in the frame packing arrangement SEI (payloadType equal to 45) with settings restrictions as above. If processing is supported, then the Receiver shall render the viewport indicated by the message

#### 5.1.5.12	Other VR Metadata

For a Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation Point

- the sphere rotation SEI message (payloadType equal to 154) shall not be present.

- any frame-packing arrangement SEI message (payloadType equal to 45) that does not conform to an SEI message defined in clause 5.1.5.10 shall not be present.

#### 5.1.5.13	Receiver Compatibility

Receivers conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall support decoding and displaying 3GPP VR Basic H.264/AVC Operation Point Bitstreams and 3GPP VR Flexible H.264/AVC Operation Point Bitstreams.

Receivers conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall support all Receiver requirements in clause 5.1.5. Specifically, receivers conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall support decoding and rendering Bitstreams that include the following VR rendering metadata:

- the region-wise packing SEI message (for details see clause 5.1.5.10 and 5.1.5.11)

- the equirectangular projection SEI message (for details see clause 5.1.5.9)

- the frame-packing arrangement SEI message (for details see clause 5.1.5.12)

- any combinations of those

## 5.1.6	Basic H.265/HEVC

#### 5.1.6.1 	General

This operation point targets simple deployments and legacy receivers at basic quality. A full 360-degree video signal with equirectangular projection following the 3GPP reference system may be provided to the decoding and rendering system for immediate decoding and rendering. Note that this operation point enables to distribute 4k video up to 60fps.

Restricted coverage is supported as well, but only in a basic any backward-compatible fashion.

A Bitstream conforming to the 3GPP VR Basic H.265/HEVC Operation point shall conform to the requirements in the remainder of clause 5.1.6.

A receiver conforming to the 3GPP VR Basic H.265/HEVC Operation point shall support decoding and rendering a Bitstream conforming to the 3GPP VR Basic H.265/HEVC Operation point. Detailed receiver requirements are provided in the remainder of clause 5.1.6.

#### 5.1.6.2	Profile and level

A Bitstream conforming to the 3GPP VR Basic H.265/HEVC Operation point shall conform to H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [6].

Hence, for a Bitstream conforming to the 3GPP VR Basic H.265/HEVC Operation point shall comply with the following restrictions:

-	The general_profile_idc shall be set to 2 indicating the Main10 profile.

-	The general_tier_flag shall be set to 0 indicating the Main tier.

-	The value of level_idc shall not be greater than 153 (corresponding to the Level 5.1) and should indicate the lowest level to which the Bitstream conforms.

#### 5.1.6.3	Bit depth

Bitstreams conforming to the 3GPP VR Basic H.265/HEVC Operation point shall be encoded with either 8 or 10 bit precision.

-	bit_depth_luma_minus8 = 0 or 2 (8 or 10 bits respectively)

-	bit_depth_chroma_minus8 = bit_depth_luma_minus8

Receivers conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall support 8 bit and10 bit precision.

#### 5.1.6.4	Picture Aspect Ratios and Spatial Resolutions

Picture aspect ratio 2: 1 should be used for the encoded picture.

The spatial resolution of the original format in equirectangular projection (ERP) should be one of the following (expressed in luminance pixel width ×  luminance pixel height):

-	4096 × 2048, 3840 × 1920, 3072 × 1536, 2880 × 1440, 2048 × 1024.

The spatial resolution of the distribution format should be one of the following (expressed in luminance pixel width × luminance pixel height):

-	3840 × 1920, 2880 × 1440, 1920 × 960, 1440 × 720, 960 × 480, 720 × 360.

-	4096 × 2048, 3072 × 1536, 2048 × 1024, 1536 × 768, 1024 × 512.

NOTE 1:	Distribution formats do not exceed the native resolution of the Operation Point, but they may be subsampled in order to optimize distribution or adapt to the viewing conditions.

A Receiver conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall be capable of decoding and rendering Bitstreams that contain spatial resolutions as above.

#### 5.1.6.5	Colour information

A Bitstream conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall use either Recommendation ITU-R BT.709 [3] colorimetry or Recommendation ITU-R BT.2020 [4] colorimetry in non-constant luminance.

Specifically, in the VUI, the colour parameter information shall be present, i.e.

-	video_signal_type_present_flag value and colour_description_present_flag value shall be set to 1.

-	If BT.709 [3] is used, it shall be signalled by setting colour_primaries to the value 1, transfer_characteristics to the value 1 and matrix_coeffs to the value 1.

-	If BT.2020 [4] is used, it shall be signalled by setting colour_primaries to the value 9, transfer_characteristics to the value 14 and matrix_coeffs to the value 9.

A Receiver conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall be capable of decoding Bitstreams that use Recommendation ITU-R BT.709 [3] and ITU-R BT.2020 [4] colorimetry. Such a Receiver should support ITU-R BT.2020 [4] signalling and provide an appropriate mapping of the signal to the supported colour space of the device.

NOTE:	Colour spaces are not associated to any particular spatial resolution.

#### 5.1.6.6	Frame rates

A Bitstream conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall have one of the following frame rates: 24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001 Hz.

With the profile and level constraints of H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 all frame rates are supported at the recommended resolutions listed above.

In the VUI, the timing information may be present.

-	If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed in this clause. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level.

-	The frame rate shall not change between two RAPs. fixed_frame_rate_flag value, if present, shall be set to 1.

There are no requirements on output timing conformance for H.265/HEVC decoding (Annex C of [6]). The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

A Receiver conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall be capable of decoding and rendering Bitstreams that use frame rates according to the bitstream requirements documented above.

#### 5.1.6.7	Random access point

For H.265/HEVC random access point (RAP) definition refer to TS26.116 [X], clause 4.5.1.2.1.

RAPs shall be present in the Bitstream at least once every 5 seconds. It is recommended that RAPs occur in the video Bitstream on average at least every 2 seconds. The time interval between successive RAPs is measured as the difference between their respective decoding time values.

#### 5.1.6.8	Video and Sequence Parameter Sets

Receivers conforming to the 3GPP VR Basic H.265/HEVC Operation Point should ignore the content of all Video Parameter Sets (VPS) NAL units as defined in Recommendation ITU-T H.265 / ISO/IEC 23008-2 [6].

The following restrictions apply to the active Sequence Parameter Set (SPS):

-	The Video Usability Information (VUI) shall be present in the active Sequence Parameter Set. The vui_parameters_present_flag shall be set to 1.

-	The chroma sub-sampling shall be 4:2:0, chroma_format_idc value shall be set to 1.

-	The source video format shall be progressive, i.e.

-	The general_progressive_source_flag shall be set to 1,

-	The general_interlaced_source_flag shall be set to 0,

-	The general_frame_only_constraint_flag shall be set to 1.

-	Only 2D contents are required to be supported, i.e.

-	The general_non_packed_constraint_flag shall be set to 1.

Receivers conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall support Bitstreams with the restrictions on the SPS defined above.

#### 5.1.6.9	Video usability information

In addition to the previous constraints on the VUI on colour information in clauses 5.1.6.5 and 5.1.6.6, this clause contains further requirements.

The aspect ratio information shall be present, i.e.

-	The aspect_ratio_present_flag value shall be set to 1.

-	The aspect_ratio_idc value shall be set to 1 indicating a square pixel format.

There are no requirements on output timing conformance for H.265/HEVC decoding (Annex C of [6]). The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

#### 5.1.6.10	Omni-directional Projection Format

This operation point uses equirectangular projection, such the video is automatically rendered in the 3GPP reference system. This is enabled by using the MPEG metadata on equirectangular projection.

A Bitstream conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall include the equirectangular projection SEI message (payloadType equal to 150) at every RAP. The erp_guard_band_flag shall be set to 0.

A Receiver conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall be able to process the information contained on equirectangular projection SEI message (payloadType equal to 150) with erp_guard_band_flag shall be set to 0.

#### 5.1.6.11	Restricted Coverage

This operation point permits the decoding and rendering of restricted coverage video signals in a rudimentary way. In this case it is expected that pixels that are projected to a non-covered region are included in the full image, but are visually differentiated from the covered region, for example using black, grey or white color.

Application or system-based signalling may support signalling the coverage region.

#### 5.1.6.12	Other VR Metadata

For a Bitstream conforming to the 3GPP VR Basic H.265/HEVC Operation Point

- the equirectangular projection SEI message (payloadType equal to 150) with erp_guard_band_flag not set to 0 shall not be present.

- the sphere rotation SEI message (payloadType equal to 154) shall not be present.

- the region-wise packing SEI message (payloadType equal to 155) shall not be present.

- the frame-packing arrangement SEI message (payloadType equal to 45) shall not be present

#### 5.1.6.13	Receiver Compatibility

Receivers conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall support decoding and displaying 3GPP VR Basic H.265/HEVC Operation Point Bitstreams.

Receivers conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall support all Receiver requirements in clause 5.1.6.

## 5.1.7	Flexible H.265/HEVC

#### 5.1.7.1 	General

This operation targets enhanced 360 video decoding and rendering of H.265/HEVC video for VR applications. Among others, this operation point supports among others rendering of:

- 4K mono video at up to 120 Hz frame rates

- 4K stereoscopic video at up to 60 Hz frame rates

- Higher than 4K resolutions for restricted coverage

- Rendering of certain viewports in higher quality than others beyond 4K

- SDR and HDR transfer characteristics

- Equirectangular and Cubemap projections

A Bitstream conforming to the 3GPP VR Flexible H.265/HEVC Operation point shall conform to the requirements in the remainder of clause 5.1.7.

A Receiver conforming to the 3GPP VR Flexible H.265/HEVC Operation point shall support decoding and rendering a Bitstream conforming to the 3GPP VR Flexible H.265/HEVC Operation point. Detailed receiver requirements are provided in the remainder of clause 5.1.7.

#### 5.1.7.2	Profile and level

A Bitstream conforming to the 3GPP VR Flexible H.265/HEVC Operation point shall conform to H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [6].

Hence, for a Bitstream conforming to the 3GPP VR Flexible H.265/HEVC Operation point shall comply with the following restrictions:

-	The general_profile_idc shall be set to 2 indicating the Main10 profile.

-	The general_tier_flag shall be set to 0 indicating the Main tier.

-	The value of level_idc shall not be greater than 153 (corresponding to the Level 5.1) and should indicate the lowest level to which the Bitstream conforms.

#### 5.1.7.3	Bit depth

Bitstreams conforming to the 3GPP VR Flexible H.265/HEVC Operation point shall be encoded with either 8 or 10 bit precision.

-	bit_depth_luma_minus8 = 0 or 2 (8 or 10 bits respectively)

-	bit_depth_chroma_minus8 = bit_depth_luma_minus8

Receivers conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall support 8 bit and10 bit precision.

#### 5.1.7.4	Spatial Resolutions

Due to the options provided in this operation point, additional original format may be considered that can then be decoded and rendered by a Receiver conforming to this operation point. Recommended formats beyond those specified in clause 5.1.6.4 for equirectangular projection (ERP) are:

- Mono formats: 8192x4096, 6144 × 3072, 5880 × 2880

- Stereo formats with resolution for each eye: 8192x4096/2, 6144 × 3072/2, 5880 × 2880/2, 4096 × 2048/2, 3840 × 1920/2, 3072 × 1536/2, 2880 × 1440/2, 2048 × 1024/2

Recommended formats for cube-map (CMP) projection are:

- Mono Formats: 6144x4096, 4608x3072, 4320x2880, 3072x2048, 2880x1920, 2304x1536, 2160x1440

- Stereo Formats: 6144x4096/2, 4608x3072/2, 4320x2880/2, 3072x2048/2, 2880x1920/2, 2304x1536/2, 2160x1440/2

The distribution formats are more flexible as additional VR metadata as defined in the remainder of clause 5.1.6 may be used.

According to H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [6], the maximum luminance width and height does not exceed 8,444 pixels. For improved interoperability, for a Bitstream conforming to the 3GPP VR Flexible H.265/HEVC Operation point, the decoded texture signal

- shall not exceed the luminance width of 8192 pixels, and

- shall not exceed the luminance height of 8192 pixels.

Beyond this, all requirements of H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 [5] apply.

A Receiver conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall be capable of decoding and rendering Bitstreams with a decoded texture signal of maximum luminance width of 8192 pixels and maximum luminance height of 8192 pixels.

#### 5.1.7.5	Colour information and Transfer Characteristcs

A Bitstream conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall use either Recommendation ITU-R BT.709 [3] colorimetry or Recommendation ITU-R BT.2020 [4] colorimetry in non-constant luminance for standard dynamic range (SDR).   For High Dynamic Range (HDR), BT.2020 [4] colorimetry in non-constant luminance and Perceptual Quantization (PQ) electro-optical transfer function (EOTF) as defined in Recommendation ITU-R BT.2100 [11] are used.

Specifically, in the VUI, the colour parameter information shall be present, i.e.

-	video_signal_type_present_flag value and colour_description_present_flag value shall be set to 1.

-	If BT.709 [3] is used, it shall be signalled by setting colour_primaries to the value 1, transfer_characteristics to the value 1 and matrix_coeffs to the value 1.

-	If BT.2020 [4] and SDR is used, it shall be signalled by setting colour_primaries to the value 9, transfer_characteristics to the value 14 and matrix_coeffs to the value 9.

-	If BT.2020 [4] and ITU-R BT.2100 [11] are used in HDR, it shall be signalled by setting colour_primaries to the value 9, transfer_characteristics to the value 16 and matrix_coeffs to the value 9.

A Receiver conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall be capable of decoding and rendering according to any of the three above configurations.

SEI messages for HDR metadata signalling may be used. The requirements and recommendations for Bitstreams and Receivers as documented in TS26.116 [X], clause 4.5.5.7 also apply for the 3GPP VR Flexible H.265/HEVC Operation Point.

#### 5.1.7.6	Frame rates

A Bitstream conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall have one of the following frame rates: 24; 25; 30; 24/1.001; 30/1.001; 50; 60; 60/1.001, 90, 100, 120 Hz.

Selected combinations of frame rates with other source parameters are provided in Annex A.2.2.3.

In the VUI, the timing information may be present.

-	If the timing information is present, i.e. the value of vui_timing_info_present_flag is set to 1, then the values of vui_num_units_in_tick and vui_time_scale shall be set according to the frame rates allowed in this clause. The timing information present in the video Bitstream should be consistent with the timing information signalled at the system level.

-	The frame rate shall not change between two RAPs. fixed_frame_rate_flag value, if present, shall be set to 1.

There are no requirements on output timing conformance for H.265/HEVC decoding (Annex C of [6]). The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

A Receiver conforming to the 3GPP VR Basic H.265/HEVC Operation Point shall be capable of decoding and rendering Bitstreams that use frame rates according to the bitstream requirements documented above.

#### 5.1.7.7	Random access point

For H.265/HEVC random access point (RAP) definition refer to TS26.116 [X], clause 4.5.1.2.1.

RAPs shall be present in the Bitstream at least once every 5 seconds. It is recommended that RAPs occur in the video Bitstream on average at least every 2 seconds. The time interval between successive RAPs is measured as the difference between their respective decoding time values.

If viewport adaptation is offered, then RAPs should occur even more frequently to enable transitioning across these viewport-optimized bitstreams.

#### 5.1.7.8	Video and Sequence Parameter Sets

Receivers conforming to the 3GPP VR Flexible H.265/HEVC Operation Point should ignore the content of all Video Parameter Sets (VPS) NAL units as defined in Recommendation ITU-T H.265 / ISO/IEC 23008-2 [6].

The following restrictions apply to the active Sequence Parameter Set (SPS):

-	The Video Usability Information (VUI) shall be present in the active Sequence Parameter Set. The vui_parameters_present_flag shall be set to 1.

-	The chroma sub-sampling shall be 4:2:0, chroma_format_idc value shall be set to 1.

-	The source video format shall be progressive, i.e.

-	The general_progressive_source_flag shall be set to 1,

-	The general_interlaced_source_flag shall be set to 0,

-	The general_frame_only_constraint_flag shall be set to 1.

-	Only 2D contents are required to be supported, i.e.

-	The general_non_packed_constraint_flag shall be set to 1.

Receivers conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall support Bitstreams with the restrictions on the SPS defined above.

#### 5.1.7.9	Video usability information

In addition to the previous constraints on the VUI on colour information in clauses 5.1.7.5 and 5.1.7.6, this clause contains further requirements.

The aspect ratio information shall be present, i.e.

-	The aspect_ratio_present_flag value shall be set to 1.

-	The aspect_ratio_idc value shall be set to 1 indicating a square pixel format.

There are no requirements on output timing conformance for H.265/HEVC decoding (Annex C of [6]). The Hypothetical Reference Decoder (HRD) parameters, if present, should be ignored by the Receiver.

#### 5.1.7.10	Omni-directional Projection Formats

This operation point permits using either equirectangular projection or cubemap projection following the MPEG metadata specifications, such the video is automatically rendered in the 3GPP reference system.

A Bitstream conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall include at every RAP either

- the equirectangular projection SEI message (payloadType equal to 150) with the erp_guard_band_flag set to 0, or

- the cubemap projection SEI message (payloadType equal to 151).

A Receiver conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall be able to process the equirectangular projection SEI message (payloadType equal to 150) and the cubemap projection SEI message (payloadType equal to 151).

#### 5.1.7.11	Restricted Coverage

This operation point permits to distribute content with less than 360 degree coverage in an encoding optimized manner by the use of region-wise packing.

It is recommended that the number of pixels that are projected to non-covered regions are minimized in the decoded texture signal. If this is applied and not the full 360 video is encoded, the region-wise packing SEI message (payloadType equal to 155) shall be included in the bitstream to signal the encoded regions of the 360 video. If present, it shall be present in a H.265/HEVC RAP.

Application or system-based signalling may support signalling the exact coverage region in the spherical coordinates.

#### 5.1.7.12	Viewport-Optimized Content

This operation point permits the use of region-wise packing, for example to optimize the spatial resolution of specific viewports. For some example usage and settings, refer to Annex A.2.

A Bitstream conforming to the 3GPP VR Flexible H.265/HEVC Operation Point may include the region-wise packing SEI message (payloadType equal to 155). If present, it shall be present in a H.265/HEVC RAP.

A Receiver conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall be able to process the region-wise packing SEI message (payloadType equal to 155).

#### 5.1.7.13	Frame packing arrangement

A Bitstream conforming to the 3GPP VR Flexible H.265/HEVC Operation Point may include the frame packing arrange SEI message (payloadType equal to 45). If present, then the following settings shall apply

- The SEI message is present in a H.265/HEVC RAP.

- The value of frame_packing_arrangement_cancel_flag is equal to 0.

- The value of frame_packing_arrangement_type is equal to 4.

- The value of quincunx_sampling_flag is equal to 0.

- The value of spatial_flipping_flag is equal to 0.

- The value of field_views_flag is equal to 0.

- The value of frame0_grid_position_x is equal to 0.

- The value of frame0_grid_position_y is equal to 0.

- The value of frame1_grid_position_x is equal to 0.

- The value of frame1_grid_position_y is equal to 0.

A Receiver conforming to the 3GPP VR Flexible H.264/AVC Operation Point shall process the frame packing arrangement SEI (payloadType equal to 45) with settings restrictions as above. If processing is supported, then the Receiver shall render the viewport indicated by the message.

#### 5.1.7.14	Other VR Metadata

For a Bitstream conforming to the 3GPP VR Flexible H.265/HEVC Operation Point

- the sphere rotation SEI message (payloadType equal to 154) shall not be present.

- any frame-packing arrangement SEI message (payloadType equal to 45) that does not conform to an SEI message defined in clause 5.1.5.10 shall not be present.

#### 5.1.7.15	Receiver Compatibility

Receivers conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall support decoding and displaying 3GPP VR Basic H.265/HEVC Operation Point Bitstreams and 3GPP VR Flexible H.265/HEVC Operation Point Bitstreams.

Receivers conforming to the 3GPP VR Flexible H.265/HEVC Operation Point shall support all Receiver requirements in clause 5.1.7. Specifically, receivers conforming to the 3GPP VR Flexible H.265/HEVCVC Operation Point shall support decoding and rendering Bitstreams that include the following VR rendering metadata:

- the region-wise packing SEI message (for details see clause 5.1.7.11 and 5.1.7.12)

- the equirectangular projection SEI message (for details see clause 5.1.7.10)

- the cubemap projection SEI message (for details see clause 5.1.7.10)

- the frame-packing arrangement SEI message (for details see clause 5.1.7.13)

- any combinations of those

=====  CHANGE  =====

Annex A (informative): Content Generation Guidelines

<Editor’s Note: Adds guidelines on how content can be generated such that it conforms to the defined media and presentation profile>

## A.1	Introduction

tbd

## A.2	Video

### A.2.1	Overview

tbd

### A.2.2	Decode Signal Constraints

#### A.2.2.1	General

tbd

#### A.2.2.2	Decode Signal Constraints for Flexible H.264/AVC Operation Point

The profile and level constraints of H.264/AVC Progressive High Profile Level [5.1][5.2] require careful balance of the permitted frame rates, stereo modes, spatial resolutions, and usage of region wise packing for different resolutions and coverage restrictions. If the decoded texture signal is beyond the Profile and Level constraints, then a careful adaptation of the signal is recommended to fulfil the constraints.

This clause provides a brief overview of potential signal constraints and possible adjustments.

Table A.2-1 provides selected permitted combinations of spatial resolutions, frame rates and stereo modes assuming full coverage and no region-wise packing applied. Note that fractional frame rates are excluded for better readability.

Table A.2-1 Selected permitted combinations of spatial resolutions and frame rates

| Spatial resolution per eye | Stereo | Permitted Frame Rates in Hz |
| --- | --- | --- |
| 4096 × 2048 | None | 24; 25; 30[; 50; 60] |
| 3840 × 1920 | None | 24; 25; 30[; 50; 60] |
| 3072 × 1536 | None | 24; 25; 30; 50[; 60; 90; 100] |
| 2880 × 1440 | None | 24; 25; 30; 50; 60[; 90; 100; 120] |
| 2048 × 1024 | None | 24; 25; 30; 50; 60; 90; 100; 120 |
| 4096 × 2048 | TaB | [24; 25; 30] |
| 3840 × 1920 | TaB | [24; 25; 30] |
| 3072 × 1536 | TaB | 24; 25[; 30; 50] |
| 2880 × 1440 | TaB | 24; 25; 30[; 50; 60] |
| 2048 × 1024 | TaB | 24; 25; 30; 50; 60[; 90; 100; 120] |

Table A.2-2 provides the maximum percentage of high-resolution area that can be encoded assuming that the low-resolution area is encoded in 2k resolution, i.e. using 2048 × 1024 or 1920 × 960 and full coverage is provided for different frame rates. Note also that a viewport typically covers about 12-25% of a full 360 video. Note that fractional frame rates are excluded for better readability.

Table A.2-2 Maximum Percentage of high-resolution area when assuming that the low-resolution area is encoded in 2k resolution, i.e. using 2048 × 1024 or 1920 × 960 and full coverage is provided for different frame rates

[Level 5.1

| Spatial resolution per eye in VP | Spatial resolution per eye in non-VP | Stereo | Frame Rates in Hz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 24 | 25 | 30 | 50 | 60 | 90 | 100 | 120 |
| 6144 × 3072 | 2048 × 1024 | None | 10.0% | 10.0% | 8.6% | 4.0% | 2.9% | 1.0% | 0.6% | 0.0% |
| 4096 × 2048 | 2048 × 1024 | None | 23.3% | 23.3% | 20.0% | 9.3% | 6.7% | 2.2% | 1.3% | 0.0% |
| 3840 × 1920 | 1920 × 960 | None | 100.0% | 100.0% | 100.0% | 57.7% | 42.5% | 17.2% | 12.2% | 4.6% |
| 6144 × 3072 | 2048 × 1024 | TaB | 4.3% | 4.0% | 2.9% | 0.6% | 0.0% | 0.0% | 0.0% | 0.0% |
| 4096 × 2048 | 2048 × 1024 | TaB | 10.0% | 9.3% | 6.7% | 1.3% | 0.0% | 0.0% | 0.0% | 0.0% |
| 3840 × 1920 | 1920 × 960 | TaB | 61.5% | 57.7% | 42.5% | 12.2% | 4.6% | 0.0% | 0.0% | 0.0% |

]

[Level 5.2

| Spatial resolution per eye in VP | Spatial resolution per eye in non-VP | Stereo | Frame Rates in Hz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 24 | 25 | 30 | 50 | 60 | 90 | 100 | 120 |
| 6144 × 3072 | 2048 × 1024 | None | 10.0% | 10.0% | 10.0% | 10.0% | 9.2% | 5.2% | 4.4% | 3.2% |
| 4096 × 2048 | 2048 × 1024 | None | 23.3% | 23.3% | 23.3% | 23.3% | 21.5% | 12.1% | 10.2% | 7.4% |
| 3840 × 1920 | 1920 × 960 | None | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 73.3% | 62.7% | 46.7% |
| 6144 × 3072 | 2048 × 1024 | TaB | 10.0% | 10.0% | 9.2% | 4.4% | 3.2% | 1.2% | 0.8% | 0.2% |
| 4096 × 2048 | 2048 × 1024 | TaB | 23.3% | 23.3% | 21.5% | 10.2% | 7.4% | 2.7% | 1.8% | 0.4% |
| 3840 × 1920 | 1920 × 960 | TaB | 100.0% | 100.0% | 100.0% | 62.7% | 46.7% | 20.0% | 14.7% | 6.7% |

]

Table A.2-3 provides the maximum percentage of coverage area that can be encoded assuming that the remaining pixels are not encoded for different frame rates. Note that fractional frame rates are excluded for better readability.

Table A.2-3 Maximum Percentage of coverage area for different frame rates

[Level 5.1

| Spatial resolution per eye | Stereo | Frame Rates in Hz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24 | 25 | 30 | 50 | 60 | 90 | 100 | 120 |
| 6144 × 3072 | None | 50.0% | 50.0% | 44.4% | 26.7% | 22.2% | 14.8% | 13.3% | 11.1% |
| 4096 × 2048 | None | 100.0% | 100.0% | 100.0% | 60.0% | 50.0% | 33.3% | 30.0% | 25.0% |
| 3840 × 1920 | None | 100.0% | 100.0% | 100.0% | 68.3% | 56.9% | 37.9% | 34.1% | 28.4% |
| 6144 × 3072 | TaB | 27.8% | 26.7% | 22.2% | 13.3% | 11.1% | 7.4% | 6.7% | 5.6% |
| 4096 × 2048 | TaB | 62.5% | 60.0% | 50.0% | 30.0% | 25.0% | 16.7% | 15.0% | 12.5% |
| 3840 × 1920 | TaB | 71.1% | 68.3% | 56.9% | 34.1% | 28.4% | 19.0% | 17.1% | 14.2% |

]

[ Level 5.2

| Spatial resolution per eye | Stereo | Frame Rates in Hz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24 | 25 | 30 | 50 | 60 | 90 | 100 | 120 |
| 6144 × 3072 | None | 50.0% | 50.0% | 50.0% | 50.0% | 46.9% | 31.3% | 28.1% | 23.4% |
| 4096 × 2048 | None | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 70.3% | 63.3% | 52.7% |
| 3840 × 1920 | None | 100.0% | 100.0% | 100.0% | 100.0% | 100.0% | 80.0% | 72.0% | 60.0% |
| 6144 × 3072 | TaB | 50.0% | 50.0% | 46.9% | 28.1% | 23.4% | 15.6% | 14.1% | 11.7% |
| 4096 × 2048 | TaB | 100.0% | 100.0% | 100.0% | 63.3% | 52.7% | 35.2% | 31.6% | 26.4% |
| 3840 × 1920 | TaB | 100.0% | 100.0% | 100.0% | 72.0% | 60.0% | 40.0% | 36.0% | 30.0% |

]

#### A.2.2.3	Decode Signal Constraints for Flexible H.265/HEVC Operation Point

The profile and level constraints of H.265/HEVC Main-10 Profile Main Tier Profile Level 5.1 require careful balance of the permitted frame rates, stereo modes, spatial resolutions, and usage of region wise packing for different resolutions and coverage restrictions. If the decoded texture signal is beyond the Profile and Level constraints, then a careful adaptation of the signal is recommended to fulfil the constraints.

This clause provides a brief overview of potential signal constraints and possible adjustments.

Table A.2-4 provides selected permitted combinations of spatial resolutions, frame rates and stereo modes assuming full coverage and no region-wise packing applied. Note that fractional frame rates are excluded for better readability.

Table A.2-4 Selected permitted combinations of spatial resolutions and frame rates

| Spatial resolution per eye | Stereo | Permitted Frame Rates in Hz |
| --- | --- | --- |
| 4096 × 2048 | None | 24; 25; 30; 50; 60 |
| 3840 × 1920 | None | 24; 25; 30; 50; 60 |
| 3072 × 1536 | None | 24; 25; 30; 50; 60; 90; 100 |
| 2880 × 1440 | None | 24; 25; 30; 50; 60; 90; 100; 120 |
| 2048 × 1024 | None | 24; 25; 30; 50; 60; 90; 100; 120 |
| 4096 × 2048 | TaB | 24; 25; 30 |
| 3840 × 1920 | TaB | 24; 25; 30 |
| 3072 × 1536 | TaB | 24; 25; 30; 50 |
| 2880 × 1440 | TaB | 24; 25; 30; 50; 60 |
| 2048 × 1024 | TaB | 24; 25; 30; 50; 60; 90; 100; 120 |

Table A.2-5 provides the maximum percentage of high-resolution area that can be encoded assuming that the low-resolution area is encoded in 2k resolution, i.e. using 2048 × 1024 or 1920 × 960 and full coverage is provided for different frame rates. Note also that a viewport typically covers about 12-25% of a full 360 video. Note that fractional frame rates are excluded for better readability.

Table A.2-5 Maximum Percentage of high-resolution area when assuming that the low-resolution area is encoded in 2k resolution, i.e. using 2048 × 1024 or 1920 × 960 and full coverage is provided for different frame rates

| Spatial resolution per eye in VP | Spatial resolution per eye in non-VP | Stereo | Frame Rates in Hz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 24 | 25 | 30 | 50 | 60 | 90 | 100 | 120 |
| 6144 × 3072 | 2048 × 1024 | None | 9.29% | 9.29% | 9.29% | 9.29% | 9.29% | 5.24% | 4.43% | 3.21% |
| 4096 × 2048 | 2048 × 1024 | None | 21.67% | 21.67% | 21.67% | 21.67% | 21.67% | 12.22% | 10.33% | 7.50% |
| 3840 × 1920 | 1920 × 960 | None | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 74.12% | 63.38% | 47.26% |
| 6144 × 3072 | 2048 × 1024 | TaB | 9.29% | 9.29% | 9.29% | 4.43% | 3.21% | 1.19% | 0.79% | 0.18% |
| 4096 × 2048 | 2048 × 1024 | TaB | 21.67% | 21.67% | 21.67% | 10.33% | 7.50% | 2.78% | 1.83% | 0.42% |
| 3840 × 1920 | 1920 × 960 | TaB | 100.00% | 100.00% | 100.00% | 63.38% | 47.26% | 20.40% | 15.02% | 6.96% |

Table A.2-6 provides the maximum percentage of coverage area that can be encoded assuming that the remaining pixels are not encoded for different frame rates. Note that fractional frame rates are excluded for better readability.

Table A.2-6 Maximum Percentage of coverage area for different frame rates

| Spatial resolution per eye | Stereo | Frame Rates in Hz |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 24 | 25 | 30 | 50 | 60 | 90 | 100 | 120 |
| 6144 × 3072 | None | 47.22% | 47.22% | 47.22% | 47.22% | 47.22% | 31.48% | 28.33% | 23.61% |
| 4096 × 2048 | None | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 70.83% | 63.75% | 53.13% |
| 3840 × 1920 | None | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 80.59% | 72.53% | 60.44% |
| 6144 × 3072 | TaB | 47.22% | 47.22% | 47.22% | 28.33% | 23.61% | 15.74% | 14.17% | 11.81% |
| 4096 × 2048 | TaB | 100.00% | 100.00% | 100.00% | 63.75% | 53.13% | 35.42% | 31.88% | 26.56% |
| 3840 × 1920 | TaB | 100.00% | 100.00% | 100.00% | 72.53% | 60.44% | 40.30% | 36.27% | 30.22% |