3GPP TSG-SA4 Meeting 3GPPSA4-on VR Stream	AHVIC-163

14 June 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | xxx | rev | 0 | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR TS 26.118: VR Media Profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer HHI |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | VRStream |  |  |  |  |  |  | Date: |  | 2018-06-11 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Progressing on Media profiles for VR under consideration of the Video operation points. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Introduction

This contribution addresses the highlighted objectives and tasks of the VRStream objectives and time plan. It contains:

- an overview of the current standardization status for media profiles (Section 2)

- mapping of current operation points to OMAF media profiles (Section 3)

- proposed changes for media profiles for inclusion in TS 26.118 for support of the operation points in S4-180556, based on S4-180557 (Section 6)

During SA4#94 the New Work Item “3GPP Virtual Reality Profiles for Streaming Media (VRStream)” in S4-170751 was agreed and afterwards approved in by SA plenary #77 in SP-170612.

The objective of this work item is to define the relevant media and protocol enablers for the set of VR Streaming use cases related to UE consumption of managed and third-party VR content as documented in TR26.918. Specifically, the following topics are addressed: - Define the necessary presentation and media profile(s) to address the objectives of the use cases on VR Streaming in TR 26.918, clauses 5.3 and 5.4, taking into account the recommended assumptions and conclusions in TR 26.918. - Define the enablers for PSS-based download and streaming of a 3GPP VR Presentation - Define the enablers for MBMS streaming and MBMS-based download delivery of a 3GPP VR Presentation - Provide suitable optional extensions (such as metadata) to support an improved VR experience, - Document content generation and content consumption guidelines, taking into account a reference client architecture that provides: (a) the signalling and processing steps for download delivery, (b) PSS delivery, (c) interfaces between the VR service platform, the VR application (e.g. sensors), and the VR rendering system (displays, GPU, loudspeakers). There is a preference for solutions that enable harmonization with broader industry consensus, such as MPEG or VR-IF.

# Standardization status of media profiles for videos

MPEG OMAF has reached FDIS status after MPEG#120 in October in Macau [2]. The OMAF FDIS contains three media profiles for video (s. Table 1).

Table 1: Overview of OMAF media profiles for video

| Media Profile | Codec | Profile | Level |
| --- | --- | --- | --- |
| HEVC-based viewport-independent OMAF video profile | HEVC | Main 10 | 5.1 |
| HEVC-based viewport-dependent OMAF video profile | HEVC | Main 10 | 5.1 |
| AVC-based viewport-dependent OMAF video profile | AVC | Progressive High | 5.1 |

VR-IF adopted the MPEG HEVC-based viewport-independent OMAF video profile and the MPEG HEVC-based viewport-dependent OMAF video profile in its published draft guidelines [3] with detailed instructions for usage.

The HEVC-based viewport-dependent OMAF video profile has been implemented by independent parties such as Fraunhofer [4] and Nokia [5]. Interoperability and performance of the HEVC-based viewport-dependent OMAF video profile for live streaming was showcased during NABshow 2018 [18]. Similar concepts are implemented in proprietary systems on the market from Tiledmedia [6], Harmonic [7] and Bitmovin [8], and the benefits have been demonstrated at several trade-fairs with a multitude of partners [9][10].

Reference software [11] and test vectors [12] are available for the HEVC-based viewport-dependent OMAF video profile. Further software for testing and evaluation is under development in MPEG and VR-IF.

In 3GPP SA4, the HEVC-based viewport-dependent OMAF video profile has been thoroughly discussed in several contributions [13, 14] during the study item FS_VR as documented in the candidate solutions section (Clause 9.2.3) in TR 26.918 and during the work item VRStream [15][16][17].

# Mapping of operation points to media profiles

The contribution S4-180556 proposes the following video operation points for TS 26.118:

Table 2: Video operation points as described in S4-180556

| Operation Point name | Decoder | Bit depth | Typical Spatial Resolution | Frame Rate | Colour space format | Transfer Characteristics | Projection | Rotation | RWP | Stereo |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H.264/AVC Basic | H.264/AVC HP@L5.1 | 8 | Up to 4k | Up to 60 Hz | BT.709 | BT.709 | ERP w/o padding | No | No | No |
| H.264/AVC Flexible | H.264/AVC [HP@L5.1/ HP@L5.2] | 8 | Up to 6k in mono and 4k in stereo | Up to 120 Hz | BT.709 | BT.709 | ERP w/o padding | No | Full | TaB |
| H.265/HEVC Basic | H.265/HEVC MP10@L5.1 | 8, 10 | Up to 4k | Up to 60 Hz | BT.709 BT.2020 | BT.709, | ERP w/o padding | No | No | No |
| H.265/HEVC Flexible | H.265/HEVC MP10@L5.1 | 8, 10 | Up to 8k in mono and 4k in stereo | Up to 120 Hz | BT.709 BT.2020 | BT.709, BT.2100 PQ | ERP w/o padding CMP | No | Full | TaB |

Table 1 lists the video profiles as specified in MPEG OMAF. Table 2 summarizes the operation points from S4-180556.

Table 3 tries to map these operation points to the required OMAF media profile capabilities, based on the available media profiles in MPEG OMAF.

Table 3: Mapping of operation points to OMAF media profiles

| Operation Point name | Matching OMAF profile | Additional constraints |
| --- | --- | --- |
| H.264/AVC Basic | AVC-based viewport-dependent OMAF video profile | - Rotation not allowed - Stereo not allowed - RWPK not allowed |
| H.264/AVC Flexible | AVC-based viewport-dependent OMAF video profile | - Rotation not allowed - Stereo constrained to TaB only |
| H.265/HEVC Basic | HEVC-based viewport-independent OMAF video profile | - Rotation not allowed - Stereo not allowed |
| H.265/HEVC Flexible | HEVC-based viewport-dependent OMAF video profile | - Rotation not allowed - Stereo constrained to TaB only |

# Proposal

Section 6 provides changes for media profiles for inclusion in TS 26.118 for support of the operation points from S4-180556, based on S4-180557. We suggest agreeing to these changes and include them in TS 26.118.

# References

[1] 3GPP TR 26.918: "Virtual Reality (VR) media services over 3GPP".

[2] ISO/IEC FDIS 23090-2, “Information technology – Coded representation of immersive media (MPEG-I)– Part2: Omnidirectional media format (OMAF)”, w17235, Macau, China, October 2017.

[3] VR-IF draft Guidelines (online), http://www.vr-if.org/wp-content/uploads/VRIF_Draft_Guidelines-v0.0draft10-2017-09-12.pdf

[4] Skupin, Robert, Yago Sanchez, Dimitri Podborski, Cornelius Hellge, and Thomas Schierl. "HEVC tile based streaming to head mounted displays." In Consumer Communications & Networking Conference (CCNC), 2017 14th IEEE Annual, pp. 613-615. IEEE, 2017.

[5] Kammachi-Sreedhar, Kashyap, Igor D. D. Curcio, and Miska M. Hannuksela, "[OMAF] Crosscheck of the test vector of the HEVC viewport dependent baseline profile," MPEG M41629, October 2017.

[6] http://tiledmedia.com/index.php/2017/08/31/ibc-2017/

[7] Harmonic: https://www.cedmagazine.com/article/2017/08/vr-tiling-technology-changing-game-virtual-reality-video

[8] https://bitmovin.com/bitmovin-receives-excellence-dash-award-tile-based-streaming-vr-360-video/

[9] Akamai/Tiledmedia: IBC 2017, “CDN Optimization for VR Streaming”, R. van Brandenburg , R. Koenen , D. Sztykman.

[10] Ericsson/Tiledmedia: https://mediasolutions.ericsson.com/blog/2017/09/502/

[11] m41627, “[ISOBMFF] Extractors reference software”, Fraunhofer HHI, Macau, China, October 2017.

[12] m41619, “[OMAF] Additional test vectors for a viewport-dependent profile with tiles”, Fraunhofer HHI, Macau, China, October 2017.

[13] 3GPP SA4#92, Tdoc S4-170031, “VR Video Systems”, Fraunhofer HHI.

[14] 3GPP SA4#94, Tdoc S4-170589, “FS_VR: Viewport-dependent baseline media profile with tile streaming”, Fraunhofer HHI, Deutsche Telekom, Nokia Corporation.

[15] 3GPP SA4#95, Tdoc S4-170971, “VRStream: Towards an HEVC Viewport-dependent profile,” Nokia Corporation, Fraunhofer HHI, Deutsche Telekom, Huawei Technologies Co. Ltd, Belgrad, Serbia, October 2017.

[16] 3GPP SA4#95, Tdoc S4-170904, “Demonstrator for OMAF HEVC viewport dependent baseline media profile using tiles,” Fraunhofer HHI, Belgrad, Serbia, October 2017.

[17] 3GPP SA4#95, Tdoc S4-170979, “Draft CR on Additional information on viewport-dependent profile” Nokia Corporation, Fraunhofer HHI, Deutsche Telekom, Huawei Technologies Co. Ltd, Belgrad, Serbia, October 2017.

[18] VR-IF: https://www.vr-if.org/news/live-vr-showcased-at-nab-2018

- pCR to TS 26.118 based on S4-180557

## 5.3	Media Profiles

### 5.3.1	Introduction and Overview

This clause defines the media profiles for video. Media profiles include specification on the following:

- Elementary stream constraints based on the video operation points defined in clause 5.2

- File format encapsulation constraints and signalling including capability signalling. The defines to a 3GPP VR Track as defined above.

- DASH Adaptation Set constraints and signalling including capability signalling. This defines a DASH content format profile.

Table 5.3-1 provides an overview of the Media Profiles in defined in the remainder of clause 5.3.2.

## ================== CHANGE 1 ========================

Addressing the Note on merging of two flexible H.265/HEVC media profiles into one.

Table 5.3-1 Video Media Profiles

| Media Profile | Operation Point | Sample Entry | DASH Integration |
| --- | --- | --- | --- |
| Basic H.264/AVC | H.264/AVC Basic | resv avc1, avc3 | Single Adaptation Set Single Representation streaming |
| Flexible H.264/AVC | H.264/AVC Flexible | resv avc1, avc3 | Single or Multiple independent Adaptation Sets offered Single Representation streaming |
| Basic H.265/HEVC | H.265/HEVC Basic | resv hvc1 | Single Adaptation Set Single Representation streaming |
| Flexible H.265/HEVC | H.265/HEVC Flexible | resv hvc1, hvc2 | Single or Multiple independent or dependent Adaptation Sets offered Single or Multiple Representation streaming |

Note:

-	Flexibe H.264/AVC Receivers are expected to playback media profiles conforming to the Basic H.264/AVC Media Profiles.

-	Flexibe H.265/HEVC Receivers are expected to playback media profiles conforming to the Basic H.265/HEVC Media Profiles.

## ================== END OF CHANGE 1 ========================

## ================== CHANGE 2 ========================

5.3.3	Flexible H.264/AVC Media Profile

5.3.2.1 	Overview

This profile allows unconstrained use of rectangular region-wise packing and monoscopic and stereoscopic spherical video up to 360 degrees are supported. With the presence of region-wise packing, the resolution or quality of the omnidirectional video could be emphasized in certain regions, e.g., according to the user's viewing orientation. In addition, the untransformed sample entry type 'hvc2' is allowed, making it possible to use extractors and get a conforming HEVC bitstream when tile-based streaming is used.

5.3.3.2 	File Format Signaling and Encapsulation

When a track is the only track in a file, compatible_brands containing a brand equal to 'hevd' in FileTypeBox indicates that the track conforms to this media profile. When a file contains multiple tracks, compatible_brands containing a brand equal to 'hevd' in FileTypeBox indicates that at least one of the tracks conforms to this media profile.

compatible_brands containing a brand equal to 'hevd' in TrackTypeBox indicates that the track conforms to this media profile.

NOTE: Consider adding a brand for 3GPP that indicates there are additional constraints to 'hevd'(see frame packing arrangement SEI message)

A track of this media profile shall be indicated to conform to this media profile through one or both of FileTypeBox and TrackTypeBox.

At least one sample entry type of each sample entry of the track shall be equal to 'resv'.

NOTE: 	'resv' does not have to be the track sample entry type, when the track has undergone several transformations. Consequently, this media profile could also be used when the track is protected.

The scheme_type values of SchemeTypeBox in the RestrictedSchemeInfoBox and of all instances of CompatibleSchemeTypeBox in the same RestrictedSchemeInfoBox shall include 'podv' and at least one of 'erpv' and 'ercm'.

The untransformed sample entry type shall be equal to 'hvc1' or 'hvc2'.

When the untransformed sample entry type is 'hvc2', the track shall include one or more 'scal' track references.

LHEVCConfigurationBox shall not be present in VisualSampleEntry.

HEVCConfigurationBox in VisualSampleEntry shall indicate conformance to the elementary stream constraints of H.265/HEVC Flexible operation point.

The track_not_intended_for_presentation_alone flag of the TrackHeaderBox may be used to indicate that a track is not intended to be presented alone.

5.3.3.3 	Additional Restrictions for DASH Representations

To be done

5.3.3.4 	DASH Adaptation Set Constraints

To be done

## ================== END OF CHANGE 2 ========================