3GPP TSG-SA WG4 Meeting #Video SWG 	AHVIC-201

# Zoom, Telco, 22:00 - 23:59, 19 September 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.928 | CR | pseudo | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Proposed Updates to XR Device Categories |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GXR |  |  |  |  |  | Date: |  |  | 17/09/2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This pCR assumes adoption of the pCR in S4-190848 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 4.3.1	Device Types

Extended reality devices are of different form factors as shown in Figure 4.3-1. These form factors differ in processing capabilities, communication types and possibly power consumption.

A smartphone (XR5G-1) may be used both for AR as well as for VR (together with a card-board). In both cases, typically an XR engine/runtime is available to support processing of sensor data, viewport rendering as well as SLAM processing. In this case, the 5G modem and all media/XR processing is integrated in the device. Power consumption of such devices is important, but not ultimately critical.

For VR, the following device types are identified:

- XR5G-11 VR HMD standalone: Such device types are commonly available in 2019, except 5G connectivity. For such devices, the 5G modem, power supply as well as all media/XR processing is expected to be integrated in a single device.

- XR5G-12 Simple VR Viewer wired tethering: Such device types are commonly available in 2019. They only include sensors for tracking as well as a display. The remaining processing is done on a remote device, e.g. a puck or a smartphone. XR/Media Processing, connectivity and power supply are provided through  wired tethering.

- XR5G-13 Simple VR Viewer wireless tethering: Such device types are not yet available in 2019. They include sensors for tracking, a display, a wireless connection (which could be WiFi based or 5G Sidelink based), and a power supply. The remaining processing is done on a remote device, e.g. a puck or a smartphone.

- XR5G-14 Smart VR Viewer wireless tethering: Such device types are not yet available in 2019. They include sensors for tracking, a display, a wireless connection (which could be WiFi based or 5G Sidelink based), at least some XR processing, as well as a power supply. The remaining processing is done on a remote device, e.g. a puck or a smartphone.

- For AR Glasses,  design constraints are significantly more important. In particular, design constraits apply in terms of sleekness, weight and power. The processing power is expected to be low to avoid battery consumption and thermal dissipation. Wireless AR glasses are commercially compelling. The following device types are identified:

- XR5G-21 AR Wearable standalone: Such device types are not available in 2019 but are under consideration. For such devices, the 5G modem, power suppy, as well as all media/XR processing is expected to be integrated in a single device.

- XR5G-22 Simple AR Wearable wired tethering: Such device types are available in 2019. They include a minimum number of sensors, possibly cameras for AR localization, as well as a display. Power, XR processing and connectivity is supplied from an external source.

- XR5G-23 Simple AR Wearable wireless tethering: Such device types are not available in 2019 and are still far out. They would include a minimum number of sensors, possibly cameras for AR localization, power supply, as well as a wireless modem for connectivity. XR processing, AR localization as well as network connectivity is provided by external means, e.g. a puck or a smartphone.

- XR5G-24 Smart AR Wearable wireless tethering: Such device types are not available in 2019 and are still far out. In addition to an XR5G-23 device, such a device would include at least a certain amount of XR/Media processing capabilities such as encoders/decoders and XR processing.

Figure 4.3-1: XR Form Factors

A summary of the different device types is provided in Table 4.3-1 along with tethering examples, placement of 5G Uu modem, XR engine and localization support, power supply and typical maximum available power.

In all device types, the sensors are on the device. The table also addresses the options applicable for tethering between the device carrying the 5G Uu Modem, and the XR device, if applicable. The table also addresses options for the XR engine, where XR engine subsumes scene recognition and viewport rendering. The following definitions are used:

- External: the device only supports display and receives a fully rendered viewport data that can be displayed directly. Any scene recognition, if applicable, is not on the device.

- Split: the external device does a pre-rendering of the viewport based on sensor information and the device does the final rendering considering the latest sensor information. Different degrees of split exist, as discussed before. Similalrly, scene recognition can be subject to split computation. This option also subsumes the case of XR processing done entirely on the device, as defined under “XR device”.

- XR device: that device does the full rendering of the viewport in the device, sensor information is only processed locally. Any scene recognition, if applicable, is on the device.

Editor's Note: Need definition on 5G Uu, 5G Sidelink, etc, (Uu is the interface between User Equipement (UE) and Radio Access Network (RAN) as defined in TS38.300. Sidelink is a mode of communication whereby UEs can communicate with each other directly	as defined in TS38.300.)

Editor's Note: Also add the device types to the use cases. Not all use cases are supported on all device types.

One of the most important issues when considering form factors and processing is the ability of the device to dissipate power, especially when no external cooling is available. Figure 4.3-2 shows the temperature rise depending on the surface power density. As example, two points on the figure can be considered:

- At 5C rise over ambient, power density that can be dissipated is 0.023 W/square inch. A smart phone whould have a surface area from 20 to 30 square inch, i.e. the power that can be dissipated is 0.5 to 0.75 Watt.

- AT 25C rise over ambient, power density that can be dissipated is 0.18 W/square inch. For a smartphone this would allow around 4 to 5 W continuous power dissipation. However, for an AR glass, the surface area is much smaller and so much less power can be dissipated, somewhere in the range of 1W. As an example, in a 25C room, the device enclosure surface temp would be 50C, which is already on the higher end of what is generally considered acceptable.

Figure 4.3-2: Temperature rise vs. power density

Table 4.3-1: XR Device Types

| XR Type Number |  | XR Device Type Name |  |  | Tethering Examples |  |  | 5G Uu Modem |  |  | XR Engine Localization |  |  |  | Power Supply |  |  | Typical Max Avail Power |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XR5G-1 |  | Phone |  |  | n/a |  |  | XR device |  |  | XR device |  |  |  | Internal |  |  | 3-5 W |  |
| XR5G-11 |  | Simple VR Viewer wired tethering |  |  | USB-C |  |  | External |  |  | External |  |  |  | External |  |  | 2-5 W |  |
| XR5G-12 |  | Simple VR Viewer wireless tethering |  |  | 802.11ad/y, 5G sidelink, etc. |  |  | External |  |  | External |  |  |  | Internal |  |  | 2-3 W |  |
| XR5G-13 |  | Smart VR Viewer wireless tethering |  |  | 802.11ad/y, 5G sidelink, etc. |  |  | External |  |  | XR device or Split |  |  |  | Internal |  |  | 2-3 W |  |
| XR5G-14 |  | VR HMD Standalone |  |  | n/a |  |  | XR device |  |  | XR device or Split |  |  |  | Internal |  |  | 3-7 W |  |
| XR5G-21 | AR Wearable standalone |  | n/a |  | XR device |  | XR device or Split |  | Internal |  | 2 - 4 W |  |
| XR5G-22 |  | Simple AR Wearable wired tethering |  |  | USB-C |  |  | External |  |  | External |  |  |  | External |  |  | 1-3 W |  |
| XR5G-23 |  | Simple AR Wearable wireless tethering |  |  | 802.11ad/y, 5G sidelink. etc. |  |  | External |  |  | External |  |  |  | Internal |  |  | 0.5 – 2 W |  |
| XR5G-24 |  | Smart AR Wearable wireless tethering |  |  | 802.11ad/y, 5G sidelink. etc. |  |  | External |  |  | XR device or Split |  |  |  | Internal |  |  | 0.5 – 2 W |  |

### 4.3.2	Power Consumption

### This clause addresses the available power in different XR Device types as well as the power consumption of typical XR processing functions as identified in the context of XR services.

When designing media processing, XR functionality and 5G connectivity, it is important to understand the power consumption of different components that are possibly integrated in XR devices. The following should be considered:

-	Tracking and Sensing

-	3DoF tracking may be done with low power consumption, e.g., below 1 Watt

-	6DoF tracking involving for example, capturing cameras, LEDs for eye and hand tracking, etc. are more power-consumption intense

-	Display

-	Display power consumption is critical and depends on the device.

-	Display power consumption can be in the range of up to 1W

-	Render (GPU)

-	The power consumption of the GPU depends on frame rates, resolution, display technology

-	The power consumption can be from several mWatt to several Watt depending on use case

-	Compute and Media Processing (CPU)

-	Similar observation as for the GPU

-	If encoding is involved, power consumption is typically higher.

-	Connectivity

-	The power consumption of wireless contection such as 5G depends on several factors including bitrates, distance from radio access network, channel conditions, frequeny range, etc.

-	The power consumption can be from several mWatt to several Watt depending on use case.

It is expected that each of the components will undergo improvement to address power savings. It is important that in the development of technical specifications of XR devices, the power consumption of each component is considered.