3GPP TSG-SA4 Video SWG Conference Call                                                AHVIC-205
 October 1, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | <CR#> | rev | <Rev#> | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VR Metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Fraunhofer HHI, InterDigital Communications, Inc., Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRQoE |  |  |  |  |  | Date: |  |  | 2019-10-1 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With approval of the work item VRQoE, definition of new QoE metrics for VR services is necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal of new metrics, partly based on MPEG-I Part 6 WD metrics proposals. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Introduction

In TR 26.929, the 3GPP SA4 group has studied the importance of VR QoE metrics and their impact on the overall experience in VR services. At SA4#104, in S4-190781, a new VR Metrics clause was agreed to be added to the TS 26.118. Recently at SA#105, in S4-190994, a set of metrics based on the VR metrics defined in MPEG-I Part 6 Working Draft (WD) has been proposed, and it has been decided to consider the proposed metrics as baseline for further discussion. This document provides some further clarifications on top of S4-190994 and attempts to resolve the remaining issues which were documented in S4-190994 as editor’s notes.

This document is structured as follows. Section 2 discusses various aspects of VR metrics defined in ISO/IEC 23090-6, while Section 3 presents the proposed changes to TS 26.118 with the definition of VR metrics based on ISO/IEC 23090-6.

# Discussion

In TR 26.929, it was concluded that metrics defined in future normative work should ideally be coordinated with the other standardization bodies to achieve wide support. The adoption of VR metrics from ISO/IEC 23090-6 WD achieves that goal and is inline with the similarities between the defined observation points in the two specifications. As shown in Figure 1 the observation points defined in ISO/IEC 23090-6 align with observation points from TS 26.118.

Figure 1: Comparison of observation points defined in MPEG and 3GPP

The following VR metrics, defined in ISO/IEC 23090-6, are proposed with some additions and modifications some of which were already presented in S4-190994 during SA4#105:

- Display information set metric

- Rendered FoV set metric

- Rendered viewports metric

- Comparable quality viewport switching latency metric

In Section 3 we present the proposed changes to TS 26.118 in order to adopt the metrics from ISO/IEC 23090-6. Further details on each metric are provided in the remainder of this section.

Display information set metric

Display information set metric allows to gather a display characteristic of VR-capable devices used in a service. Display resolution, its pixel density and refresh rate are considered to be the most important ones. With this metric, a service provider can obtain information about the display properties of their customers and quickly adapt its content (or improve other parts of its service) accordingly. This information might also be helpful in assisting the content producer or distributor to optimise caching in the CDNs.

On a native implementation of a VR client application, it might be easy to obtain information about the display properties (e.g. through APIs such as DisplayMetrics [1] on Android, and UIScreen [2] on iOS). However, in a browser-based application, this might be more difficult. For example, a pixel (or reference pixel as defined in the W3C working draft [3]) is defined as the visual angle of one pixel on a device with a pixel density of 96 dpi and a distance of an arm’s length from the reader. Hence calculating the actual pixel density would be inconsistent.

Rendered FoV set metric

2D displays and programmable HMD devices have adjustable FoVs. The rendered FoV set metric includes the horizontal and vertical element of the rendered FoV in unit of degree. A record of rendered FoV may provide information about the device capabilities and the user’s most comfortable FoV during the VR playback. This information could be useful to optimise QoE for a service using the Advanced Video Media profile specified in TS 26.118. In a Javascript implementation using the ‘three.js’ library, the FoV can be set by adjusting the parameters of the perspective camera. This information might be helpful in assisting the content producer or distributor to determine the viewport granularity for viewport dependent streaming.

Rendered viewports metric

The rendered viewports metric reports a list of viewports that have been rendered. The time instance when a viewport is rendered and the duration of the rendered viewport playback are also included in the metric. For example, services using the Main Video Media profile may get the parameters for this metric to identify one or multiple preferred viewport to be carried in video adaptation sets; services using the Advanced Video Media profile may get the parameters for this metric which are available in the extractor track descriptor in an MPD file. The report might also include the time instance when the content corresponding to the selected extractor track was played and the duration of the playback. An analysis which aggregates rendered viewport metrics from multiple users can be used to identify the popular region(s) within the VR content. This information might be helpful in assisting the content distributor to optimise caching in the CDNs.

Comparable quality viewport switching latency metric

The comparable quality viewport switching latency metric reports the latency experienced by the user when switching from one viewport to another viewport until the presentation quality of the new viewport reaches a similar presentation quality as the first viewport. There are two options available to signal the presentation qualityof the viewport. One method is by identification of the effective resolution of the content covered by the viewport and the other method is by identification of the viewport quality. It is flexible to use either of the two or both. This metric includes the time instance when a viewport switching event is identified and assigns the time value to the ‘time’ field defined in the metric. The latency is measured until the presentation quality of the second viewport is comparable to that of the first viewport just before the time indicated by the parameter ‘time’. The metric also includes information on the lowest viewport presentation quality experienced during the latency measurement cycle. This metric can be used to assess the overall QoE for an immersive media streaming service. The comparable quality viewport switching latency can be affected by various factors such as bandwidth, segment duration, decoder buffer size, buffer fullness, the segment scheduling algorithm, etc. For example, in a service using the Advanced Video Media profile, the user can change the head position from one viewport position to another viewport position. A content request for the new viewport position is made. The system might experience some delay in achieving a similar presentation quality for the second viewport compared to that of the first viewport due to segment length, network delay, buffer fullness, etc.

The four ISO/IEC 23090-6 VR metrics listed above have been implemented in the JavaScript player [4]. The project includes an HTML5 player capable of playing content compliant to 3GPP TS 26.118 Advanced Media Profile. The player will allow gathering VR metrics as defined in in ISO/IEC 23090-6, which are useful for quality-related feedback and analysis. The intention of this new release is to show that the VR metrics which are defined in ISO/IEC 23090-6 are implementable and also valuable. At the moment of writing of this document the implementation is contained in a separate branch called ‘metrics’. After testing it will be included in the next software release.

Figure 2 shows a screenshot of the JavaScript player along with information on some of the VR metrics (note that by the time this document is presented, the implementation will look differently).

Figure 2. Screenshot of the Javascript player with VR metrics

# Proposed changes to TS 26.118

============================== CHANGE 1  ==============================

### 9.3.1	General

As the VR metrics functionality is based on the DASH QoE metrics, metrics already defined in TS 26.247 are valid also for a VR client. Therefore, the following sub-clauses only define additional VR-related metrics.

A new data type, ViewportDataType, is defined as shown in Table 1. ViewportDataType is an Object with six Integer keys that identify a viewport. The six keys are: viewpoint_id, centre_azimuth, centre_elevation, centre_tilt, azimuth_range, and elevation_range, respectively.

Table 1 ViewportDataType

| Key |  | Type | Description |
| --- | --- | --- | --- |
| ViewportDataType |  | Object |  |
|  | viewpoint_id | Integer | Specifies the identifier of the viewpoint to which the viewport belongs. |
|  | centre_azimuth | Integer | Specifies the azimuth of the centre of the viewport in units of 2−16 degrees. The value shall be in the range of −180 * 216 to 180 * 216 − 1, inclusive. |
|  | centre_elevation | Integer | Specifies the elevation of the centre of the viewport in units of 2−16 degrees. The value shall be in the range of −90 * 216 to 90 * 216, inclusive. |
|  | centre_tilt | Integer | Specifies the tilt angle of the viewport in units of 2−16 degrees. The value shall be in the range of −180 * 216 to 180 * 216 − 1, inclusive. |
|  | azimuth_range | Integer | Specifies the azimuth range of the viewport through the centre point of the viewport, in units of 2−16 degrees. |
|  | elevation_range | Integer | Specifies the elevation range of the viewport through the centre point of the viewport, in units of 2−16 degrees. |

## 9.3.2 Display information set metric

The display information set metric reports the characteristics of the display used by the client for rendering the VR content, including display resolution, pixel density, and refresh rate. The display information set metric is specified in Table 2.

Table 2 Display information set metric

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| DisplayInfoSet |  |  | Set | Set of display information |
|  | Entry |  | Object |  |
|  |  | start | Real-Time | Timestamp when the display information was logged. |
|  |  | mstart | Media-Time | The presentation time at which the display information was logged. |
|  |  | displayResolution | String | Display resolution, in units of pixels |
|  |  | displayPixelDensity | Integer | Display pixel density, in units of PPI |
|  |  | displayRefreshRate | Integer | Display refresh rate, in units of Hz |

This metric is a generic metric that also applies to other visual media applications than VR applications.

[ Editor's note: This metric partly overlaps with the existing device info metric in 26.247. ]

## 9.3.3 Rendered FOV set metric

The rendered FOV set metric reports a set of FOVs rendered by VR client devices and is specified in Table 3.

Table 3 Rendered FOV set metric

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| RenderedFovSet |  |  | Set | Set of rendered FOVs |
|  | Entry |  | Object |  |
|  |  | start | Real-Time | Timestamp when the FOV information was logged. |
|  |  | mstart | Media-Time | The presentation time at which the FOV information was logged. |
|  |  | renderedFovH | Integer | The horizontal element of the rendered FOV, in units of degrees |
|  |  | renderedFovV | Integer | The vertical element of the rendered FOV, in units of degrees |

[ Editor's note: This metric partly overlaps with the existing device info metric in 26.247.]

## Rendered viewports metric

[Editor's note: This text needs to be clarified/improved.

The rendered viewports metric reports a list of viewports that have been rendered longest time at particular intervals of media presentation times and is specified in Table 4.

Table 4 Rendered viewports metric

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| RenderedViewports |  |  | List | List of rendered viewports |
|  | Entry |  | Object |  |
|  |  | startTime | Media-Time | Specifies the media presentation time of the first played out media sample when the viewport indicated in the current entry is rendered starting from this media sample. |
|  |  | duration | Integer | The time duration, in units of milliseconds, of the continuously presented media samples when the viewport indicated in the current entry is rendered starting from the media sample indicated by startTime. "Continuously presented" means that the media clock continued to advance at the playout speed throughout the interval. |
|  |  | viewport | ViewportDataType | Indicates the region of the omnidirectional media corresponding to the viewport that is rendered longest time starting from the media sample indicated by startTime. |

]

## Comparable quality viewport switching latency

The comparable quality viewport switching latency metric reports the latency and the intermittent presentation quality degradation experienced by the user when switching from a first viewport to a second viewport. A switch event starts when the viewport moves into an area where the current content in viewport is in low-resolution/low-quality (e.g. background low-resolution content or low-quality content suddenly becomes visible), and the client initiates retrieval of high-resolution/high-quality content (i.e viewport movements which only occurs within the current high-resolution/high-qulaity area do not trigger a switch event). The switch event ends when the presenation quality of the viewport of the second viewport reaches a comparable presentation quality viewport resolution as the first viewport.

The comparable quality viewport switching latency metric is specified in Table 5.

Table 5 Comparable quality viewport switching latency metric

| Key |  |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- | --- |
| CQViewportSwitchingLatency |  |  |  |  | List | List of comparable quality viewport switching latencies |
|  | Entry |  |  |  | Object |  |
|  |  | firstViewport |  |  | ViewportDataType | Specifies the spherical region corresponding to the first viewport (i.e., before the switching). |
|  |  | secondViewport |  |  | ViewportDataType | Specifies the spherical region corresponding to the second viewport (i.e., after the switching). |
|  |  | firstViewportQuality |  |  | Float | Specifies the weighted average quality value of the first viewport. The average is calculated according to clause X.2. |
|  |  | secondViewportQuality |  |  | Float | Specifies the weighted average quality value of the second viewport. |
|  |  | worstViewportQuality |  |  | Float | Specifies the worst weighted average quality value experienced by the viewport during the switch duration. |
|  |  | effectiveResolution |  |  | Float | Specifies the weighted average resolution of the first viewport. The average is calculated according to clause X.3. |
|  |  | lowestEffectiveResolution |  |  | Float | Specifies the lowest weighted average resolution experienced by the viewport during the switch duration. |
|  |  | time |  |  | Real-Time | Specifies the measurement time of the viewport switching latency in wall-clock time. |
|  |  | Mtime |  |  | Media-Time | The presentation time at which the information was logged. |
|  |  | Latency |  |  | Integer | Specifies the delay in milliseconds between the time a user movement from first viewport to second viewport and the time when the presentation quality of the second viewport reaches a comparable presentation quality as the first viewport. |
|  |  | Reason |  |  | List | Specifies a list of possible causes for the latency. |
|  |  |  | Entry |  | Object |  |
|  |  |  |  | code | Enum | A possible cause for the latency. The value is equal to one of the following: - 0: Segment duration - 1: Buffer fullness - 2: Availability of comparable quality segment |

=========================== END OF CHANGE 1  ==========================

============================== CHANGE 2  ==============================

# X. Metric measurement process

## X.1 Introduction

This sub-clause specifies the immersive media metric measurement process to provide interoperability for consistent logging and monitoring of immersive media quality and experiences.

## X.2 Viewport quality

The quality of each region is determined with its respective quality ranking value. A viewport can be covered with multiple regions. A quality value for the viewport can be derived as weighted average of the quality ranking values of the regions covering the viewport. The weight of each region is defined as the percentage of the viewport area covered by the corresponding region. The viewport quality can be calculated by the following equation.

Qualityviewport=i=1NQualityRankingi×Coveragei

N: number of regions covering the viewport

QualityRanking: region-wise quality ranking value of ith quality ranking 2D region

Coverage: the percentage value of the viewport area being covered by i-th quality ranking 2D region.

Figure X.1 – An example of a viewport covered by four quality ranking regions

Figure X.1 is an example of a viewport covered by four quality ranking regions. The quality of the viewport is equal to the sum total of the quality ranking value and coverage percentage value of each quality ranking region.

## X.3 Viewport Resolution

The resolution of each region is determined with its respective width and height values in pixel which are available in the SphereRegionQualityRanking box. The effective resolution for the viewport can be derived as the weighted average of the resolution of each region covering the viewport. The weight of each region is defined by the percentage of the viewport area covered by the corresponding region. The viewport resolution can be calculated by the following equation.

Resolutionviewport=i=1Nwidthi×heighti×Coveragei

N: number of regions covering the viewport

width: width in pixels of ith quality ranking 2D region

height: height in pixels of ith quality ranking 2D region

Coverage: the percentage value of the viewport area being covered by i-th quality ranking 2D region.

Region [1]

Region [2]

Region [4]

Region [3]

Viewport

35%

15 %

40 %

10%

Resolution [1]

Resolution [2]

Resolution [4]

Resolution [3]

Coverage [1]

Coverage [2]

Coverage [3]

Coverage [4]

width[2]

width[1]

height[1]

height[2]

height [3]

height[4]

width[3]

width[4]

Figure X.2 – An example of a viewport covered by four resolution regions

Figure X.2 is an example of a viewport covered by four resolution regions. The effective resolution of the viewport is equal to the sum total of the resolution value and coverage percentage value of each quality ranking region.

## X.4. Comparable quality viewport switching latency measurement

A viewport is covered by a subset of the projected frame. A picture may be divided into multiple sub-pictures, and each sub-picture may be independently encoded as a sub-bitstream. A viewport switching event can be identified when at least one new sub-picture not included in last viewport is being requested and rendered. The comparable quality viewport switching latency metric is measured when a viewport switching event is identified. The viewport switching time is assigned to the field time. The client may log the viewport, viewport quality, and wall clock time based on a constant sampling time, and/or a configurable viewport movement threshold.

There could a number of quality regions covering the viewport. It is difficult to determine which presentation quality of viewport before the viewport switching time can be used for reporting the first viewport. It is critical in a scenario where the user keeps moving their viewing orientation before the viewport switching time. Therefore, two measurement time intervals are used to set the measurement range. The first-time interval has a duration of M milliseconds that stretches back in time from the viewport switching time. The second time interval has a duration of N milliseconds starting from the viewport switching time. The highest weighted average viewport resolution logged within M milliseconds before the switching time is assigned to the field effectiveResolution. Similarly, the highest viewport quality within the M milliseconds is assigned to firstViewportQuality. Depending on which flavor of presentation quality is choosen for identification, the associated viewport at the time of identification is assigned to the field firstViewport. The viewport quality and effective viewport resolution can be calculated as described in clause X.2 and clause X.3. respectively.

When the logged viewport presentation quality value of a rendered viewport within N milliseconds is comparable to firstViewportQuality/effectiveResolution, then the associated rendered viewport is assigned to the field secondViewport. The latency is calculated as the time interval between the logged times for firstViewport and secondViewport. In case of identification by viewport quality, the viewport quality at second viewport is assigned to field secondViewportQuality. During viewport switching latency measurement cycle, the worst logged viewport quality is assigned to the field worstViewportQuality. Similarly, the worst logged effective viewport resolution is assigned to the field lowestEffectiveResolution.  The occurrence of lowestEffectiveResolution and worstViewportQuality may or may not be the align. 

During a comparable-quality viewport switching latency measurement cycle, if the viewport presentation quality value cannot match firstViewportQuality/ effectiveResolution during the next N milliseconds after the viewport switching time, the measurement cycle is interrupted and the metric is not reported. This is also the case if a new viewport switching event occurrs before a comparable viewport quality can be identified within N milliseconds after the viewport switching time.

Figure X.3 – Comparable quality viewport switching latency measurement example

Figure X.3 presents an example of the metric measurement operation. The services identify viewport switching based on effective resolution. The media playback module renders the high-quality sub-picture #1 at time t1. The user viewing orientation is gradually changing from sub-pic#1 to sub-pic#2 as the time progresses. At time t2, the media playback module starts to render the buffered low-resolution representation of sub-pic#2 as the viewport moves into sub-picture #2. A viewport switching event is then identified at time t2. The viewport quality value associated with the highest weighted average viewport resolution logged within the past M milliseconds is set as effectiveResolution and the associated viewport is set as firstViewport. A viewport resolution value for the new viewport that is comparable to effectiveResolution after viewport switching time is logged at time t4. The least weighted average value of viewport resolution during the switch is assigned to the field lowestEffectiveResolution. The comparable-quality viewport switching latency is measured as the time interval between the logged times for firstViewport (t1 in this example) and secondViewport (t4 in this example).

=========================== END OF CHANGE 2  ==========================

# References

[1] Android API reference (DisplayMetrics class), https://developer.android.com/reference/android/util/DisplayMetrics.html

[2] UIKit Framework (UIScreen class), https://developer.apple.com/documentation/uikit/uiscreen

[3] W3C Working Draft, CSS Values and Units Module Level 4, 31 January 2019

[4] HTML5 MSE Playback of MPEG 360 VR Tiled Streaming, https://github.com/fraunhoferhhi/omaf.js