3GPP SA4 Video SWG Telco on FS_VRQOE #1                                              AHVIC-213

18th November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | <CR#> | rev | <Rev#> | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VRQoE quality level and effective resolution calculation correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRQoE |  |  |  |  |  | Date: |  |  | 2019-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | defects on quality level and effective resolution equation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal of correction on the equation to calculate QualityLevel and EffectiveResolution |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This document proposes editorial correction on S4-191282 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex D (informative):
Examples for comparable quality viewport switching latency

# D.1 Introduction

This sub-clause illustrates how the viewport quality and effective resolution are calculated.

# D.2 Viewport quality

The quality level of each region is determined with its respective quality ranking (QR) value. A viewport can be covered with multiple regions. A quality level value for the viewport can be derived as weighted average of the QR values of the regions covering the viewport. The weight of each region is defined as the percentage of the viewport area covered by the corresponding region. The viewport quality level can be calculated by the following equation.

QualityLevelviewport=i=1NQRi×Coveragei/100

N: Number of regions covering the viewport

QR[i]: QR value of i-th quality ranking region

Coverage[i]: The viewport coverage value (in percent) of i-th quality ranking region

Figure D.1 – An example of a viewport covered by four quality ranking 2D regions

Figure D.1 is an example of a viewport covered by four quality ranking regions. The quality of the viewport is equal to the weighted sum of the quality ranking value and the coverage percentage value of each quality ranking region.

# D.3 Effective Viewport Resolution

The resolution of each region is determined by its respective width and height values in pixel which are available in the quality ranking box under the name orig_width and orig_height. Note that these values are already normalized to represent the full-sphere resolution you would get if the resolution of this region would be used for the full sphere.

The effective resolution (i.e. the total number of original pixels) for the content visible in the viewport can be derived as the weighted average of the resolution of each region covering the viewport. The weight of each region is defined by the percentage of the viewport area covered by the corresponding region. The effective viewport resolution can be calculated by the following equation.

EffectiveResolutionviewport=i=1Nwidthi×heighti×Coveragei/100

N: Number of regions covering the viewport

width[i]: The width component of the original source pixel resolution for the i-th quality ranking region

height[i]: The height component of the original source pixel resolution resolution for the i-th quality ranking region

Coverage[i]: The viewport coverage value (in percent) of i-th quality ranking region

Figure D.2 – An example of a source packed image with four quality ranking 2D regions with different resolutions

Resolution [3]

Resolution [2]

Resolution [4]

Viewport

35%

15 %

40 %

10%

Coverage [1]

Coverage [2]

Coverage [3]

Coverage [4]

Resolution [1]

Figure D.3 – An example of a viewport covered by four different quality ranking 2D regions

Figure D.2 is an example of a source with four regions with different resolution. Figure D.3 represents an example of a viewport which is covered by the four quality ranking 2D regions. The effective viewport resolution is equal to the weighted sum of the resolution for each quality-ranking 2D region and its corresponding viewport coverage percentage value.