3GPP TSG-S4 Meeting #107	S4-200083

# Wroclaw, Poland,  20th Jan.  - 24th Jan. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | <CR#> | rev | <Rev#> | Current version: | 16.3 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FLUS Stage 3 for Remote Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 10.1.2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The procedures (Stage 2) for Remote Controlling a FLUS Source from a Remote Controller has been agreed. This CR is defining the stage 3 procedures for the Remote Control Procedures. The CR is also correcting a could of issues in the configuration procedures for the FLUS Source and aligning the Stage 3 terminology to the stage 2 procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No stage 3 for the already agreed stage 2 procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 7	Uplink Streaming Control Interface

## 7.1	General

### This section specifies REST API procedures for FLUS sink discovery, capability discovery, FLUS Sink configuration management and FLUS Sink Configuration termination between the Control source and the Control sink.

### 7.1.1	Resources

F-C defines a set of resources managed at the FLUS sink that are controlled by the Control source. The set of resources are defined in the following table:

Table 7.1.1-1: Resources for managing FLUS Sink configuration at FLUS sink

| Resource Name | Resource Type | Description |
| --- | --- | --- |
| Sink | Instance resource | Represents a single FLUS sink as described in section 7.1.1.1. The Control source can query the configuration properties of that sink. However, a Control source cannot create, update, or terminate a configuration at a FLUS sink. |
| Session | Instance resource | Represents a single FLUS Sink configuration resource as described in section 7.1.1.2. The Control source can provision or modify a single configuration at the FLUS sink. |
| Sessions | Collection Resource | Represents a collection of FLUS Sink configuration resources. |

Table 7.1.1-2: Resources for managing FLUS Source configuration

| Resource Name | Resource Type | Description |
| --- | --- | --- |
| Capabilities | Instance resource | Represents a single FLUS Source as described in section 7.1.1.3. The Remote Controller can query the capabilityproperties of that source. |
| Configuration | Instance resource | Represents a single FLUS Source configuration resource as described in section 7.1.1.4. The Remote Controller can provision or modify a single configuration at the FLUS source. |
| Configurations | Collection Resource | Represents a collection of FLUS source configuration resources. |

#### 7.1.1.1	Sink Resource

A Sink resource provides a representation of the capabilities of a FLUS sink. It also contains a sessions property that represents a collection of FLUS sessions. A FLUS source can query sink resources using REST API methods to examine the properties of the sink resource. Different properties of sink resources represent the capabilities of the corresponding FLUS sink.

Each sink resource has the set of properties described in Table 7.1.1.1-1.

Table 7.1.1.1-1: Properties of Sink Resource

| Property Name | Description | Example Values |
| --- | --- | --- |
| capabilities | List of supported features and instantiations by the FLUS sink. Each capability is to be expressed using an URN. | urn:vnd:xzy:capability-name |

As described in Table 7.1.1.1-1 above, each sink resource describes the capabilities of the corresponding FLUS sink. A Control source can retrieve the resource description of a sink resource and find out its capabilities and make a decision if it wants to use the corresponding FLUS sink as described using the capability exchange procedure in section 7.3.

#### 7.1.1.2	Session Resource

A Session resource provides the representation of a FLUS Sink configuration. A Control source can create a configuration at a FLUS sink using REST API methods operating on the session resource described in this section.

Each session resource has the set of properties described in Table 5.2.2.7-1.

Control sources use the above session resource representation to create and manage sessions at the selected FLUS sink.

#### 7.1.1.3	Capabilities Resource

A Capabilities resource provides a representation of the capabilities of a FLUS Source.

Each source resource has the set of properties described in Table 7.1.1.3-1.

Table 7.1.1.3-1: Properties of Source Resource

| Property Name | Description | Example Values |
| --- | --- | --- |
| capabilities | List of supported features and instantiations by the FLUS source. Each capability is to be expressed using an URN. | urn:vnd:xzy:capability-name |

As described in Table 7.1.1.3-1 above, each source resource describes the capabilities. A Remote Controller can retrieve the resource description of a source resource and find out its capabilities and make a decision if it wants to use the corresponding FLUS source as described using the capability exchange procedure in section 7.7.

#### 7.1.1.2	Configuration Resource

A configuration resource provides the representation of a FLUS Source configuration. A Remote Controller can create a configuration at a FLUS source using REST API methods operating on the session resource described in this section.

Each configuration resource has the set of properties described in Table 5.2.2.7-1.

Remote Controllers use the above configuration resource representation to create and manage configurations at the selected FLUS source.

### 7.1.2	Supported Methods

### The F-C API follows the RESTful design principles. All operations shall be performed using HTTP 1.1 (IETF RFC 7231 [6]) over TLS (3GPP TS 33.246 [7]).

| HTTP Method | CRUD | Resource | PATH |
| --- | --- | --- | --- |
| POST | Create | Session | /flus/v1.0/sessions |
| GET | Read | Session | /flus/v1.0/sessions/{session-res-id} |
|  |  | Sink | /flus/v1.0/sinks/{sink-res-id}/ |
| PUT | Replace | Session | /flus/v1.0/sessions/{session-res-id} |
| PATCH | Modify | Session | /flus/v1.0/sessions/{session-res-id} |
| DELETE | Delete | Session | /flus/v1.0/session/{session-res-id} |

### 7.1.3	Error Handling

The Uplink Streaming Interface API shall use the HTTP status codes to indicate any errors that might occur in the processing of operations on FLUS resources. Unless defined otherwise, the HTTP status codes shall be interpreted as specified in IETF RFC 7231 [6]. API operations that are not successfully handled shall not leave the resource at an undefined state. The response should provide sufficient information for a human operator to understand and locate the error.

## 7.2	FLUS Sink Discovery

### The FLUS sink discovery procedure is used by the FLUS source to discover available FLUS sinks that are provided by the operator.

### To query the list of available FLUS sinks, the FLUS REST client shall use one of the following procedures:

-	The operator provides one or more FLUS sink URLs to the FLUS source.

-	DNS discovery of the FLUS sink

-	The FLUS Sources uses the pre-defined URL: http://flus.mnc<MNC>.mcc<MCC>.pub.3gppnetwork.org/flus/v1.0/sinks/ in an HTTP request. The response shall be a JSON document that represents an array of objects, each of which contains a URL to the entry point to the FLUS Sink.

## 7.3	 FLUS Sink Capability retrieval

GET /flus/v1.0/capabilities

### Using this capability exchange procedure, the Control source enquires about the supported features and instantiations of the FLUS sink.

### To retrieve the capabilities of a FLUS sink, the Control source shall use the HTTP GET method on the "sink" instance resource as follows:

-	the request URI with the "path" is set to "/flus/v1.0/capabilities"

-	the Host field is set to the FQDN of the FLUS sink

### The URL in the request is set according to the information retrieved using the Discovery procedure described in section 7.2.

The sink resource information provides in detail the capabilities of the FLUS sink. The response from the FLUS sink to the FLUS source shall contain the following:

-	The Content-Type field specifying the MIME type (JSON) using which the sink resource information is encoded.

-	The Content-Length is the length of the content body.

The above header fields are followed by the content body in the format indicated by the Content-Type field. The content body includes the detailed representation of the sink resource as described in section 7.1.1.1 from which the capabilities of that FLUS sink can be inferred.

The possible response messages from the FLUS sink to the FLUS source are shown in Table 7.3-1.

Table 7.3-1: Response status code, message, and contents for sink resource retrieval using HTTP GET

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS sink provides the sink capabilities to the FLUS source. |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| Note: In addition to the above response codes, the FLUS sink can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

## 7.4	Manage a FLUS Sink configuration

### 7.4.1	FLUS Sink configuration fetch procedure

GET /flus/v1.0/sessions/{session-res-id}

Configurations can be read by the Control source when it wishes to know the latest representation of the session resource at the FLUS sink. To fetch the properties of a FLUS Sink Configuration, the Control source sends a HTTP GET request to the FLUS sink as follows:

-	the request URI with the "path" set to "/flus/v1.0/sessions/{session-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

### The {session-res-id} in the request URI is the FLUS Sink configuration resource identifier previously created between the FLUS source and the FLUS sink.

### Upon receiving HTTP GET request from the Control source, the FLUS sink checks to see if such a configuration exists that matches the given session resource identifier. If such a session resource exists, the FLUS sink shall respond to the Control source with a 200 OK message along with the complete representation of the session resource. The response from the FLUS sink to the Control source shall contain the following:

-	The Content-Type field specifying the MIME type (JSON) using which the session resource information is encoded.

-	The Content-Length is the length of the content body.

### The content body of this response message shall be the representation of the session resource as described in sub clause 7.1.1.2

Alternatively, if such a session resource cannot be found by the FLUS sink, it shall send a 404 Not Found message to the FLUS sink. If the request cannot be fulfilled, the FLUS sink shall send a 403 Forbiddent message to the Control source.

The possible response messages from the FLUS sink, depending on whether the GET request is successful or unsuccessful, are shown in Table 7.4.1-1.

Table 7.4.1-1: Response status code, message, and contents for service modification using HTTPS GET

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS sink provides session properties of the session resource to the Control source |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the FLUS sink can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

### 7.4.2	FLUS Sink configuration update procedure

The session resource at the FLUS sink can be either partially or completely modified by using the procedures as specified in clauses 7.4.2.1 and 7.4.2.2, respectively.

#### 7.4.2.1	Partial modification of FLUS Sink configuration

PATCH /flus/v1.0/sessions/{session-res-id}

To update some of the FLUS Sink configuration parameters at the sink, the Control source sends an HTTP PATCH request as follows:

-	the request URI with the "path" is set to "/flus/v1.0/sessions/{session-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

-	The Content-Type field specifying the MIME type (JSON) using which the session resource information is encoded.

-	The Content-Length is the length of the content body.

### The {session-res-id} in the request URI is the session resource identifier of the configuration whose modification is sought.

The content body of the HTTP PATCH message shall contain the updated partial representation of the session resource.

Upon receiving the HTTP PATCH request from the Control source, the FLUS sink checks to see if such a session resource exists at the sink. If such a session resource exists, FLUS sink shall update the configuration properties based on the values from the incoming request. Upon successful update of the requested configuration, the FLUS sink shall respond to the Control source with a 200 OK success message indicating that the session resource was successfully updated. The FLUS sink shall also include the session resource identifier of the configuration that is updated. As alternative to the 200 OK message, FLUS sink may send a 204 No Content success message without any message content to the Control source. If the session resource cannot be updated, the FLUS sink shall send a 403 message. If the session is not found, the FLUS sink shall send a 404 message.

The possible response messages from the FLUS sink, depending on whether the PATCH request is successful or unsuccessful, are shown in Table 7.4.2.1-1.

Table 7.4.2.1-1: Response status code, message, and contents for session modification using HTTP PATCH

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS sink provides the session resource identifier of the session that is modified |
| 204 No Content | The request has succeeded | None |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the FLUS sink can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

#### 7.4.2.2	Full modification of FLUS Sink configuration

PUT /flus/v1.0/sessions/{session-res-id}

For complete update of the configuration parameters at the FLUS sink, the Control source sends an HTTP PUT request as follows:

-	the request URI with the "path" is set to "/flus/v1.0/sessions/{session-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

-	The Content-Type field specifying the MIME type (JSON) using which the session resource information is encoded.

-	The Content-Length is the length of the content body.

### The {session-res-id} in the request URI is the session resource identifier of the configuration whose modification is sought.

The content body of the HTTP PUT message shall contain the updated complete representation of the session resource.

Upon receiving the HTTP PUT request from the Control source, the FLUS sink checks to see if such a session resource exists at the sink. If such a session resource exists, FLUS sink shall update the configuration properties based on the values from the incoming request. Upon successful update of the requested session, the FLUS sink shall respond to the Control source with a 200 OK success message indicating that the session resource was successfully updated. The FLUS sink shall also include the session resource identifier of the configuration that is updated. As alternative to the 200 OK message, FLUS sink may send a 204 No Content success message without any message content to the Control source. If the session cannot be updated, the FLUS sink shall send a 403 message. If the session is not found, the FLUS sink shall send a 404 message.

The possible response messages from the FLUS sink, depending on whether the PATCH request is successful or unsuccessful, are shown in Table 7.4.2.2-1.

Table 7.4.2.2-1: Response status code, message, and contents for session modification using HTTP PUT

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS sink shall send the session resource identifier of the session that is modified |
| 204 No Content | The request has succeeded | None |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the FLUS sink can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

## 7.5	FLUS Sink configuration creation

POST /flus/v1.0/sessions

To create a configuration at the FLUS sink, the Control source shall use the HTTP GET method on the "sessions" collection resource as follows:

-	the request URI with the "path" is set to "/flus/v1.0/sessions"

-	the Host field is set to the FQDN of the FLUS sink

-	the Content-Type field specifying the MIME type (JSON) using which the session resource information is encoded.

-	the Content-Length is the length of the content body.

The content body of the POST request shall contain the representation of the session resource. The session resource representation includes the details such as the media streams, their formats and codecs, the relationship between them, and the selected user plane instantiation and user plane control protocol.

Upon receipt of HTTP POST to create a sessions resource, the FLUS sink shall create a configuration. Upon successful creation of session resource, the FLUS sink shall respond back to the Control source with a 201 success message indicating that the session is successfully created along with the session resource of the created session. The session information returned to the Control source along with the 201 message includes the parameters applied for the session, thus confirming the parameters, and any default values assigned to session parameters which were not supplied by the Control source in the HTTP POST message.

The session resource identifier is the identifier that uniquely identifies the session within the list of all sessions at that sink. When the Control source receives the session resource identifier, it shall use this identifier in subsequent requests to the FLUS sink to refer to this session.

Alternatively, if the creation of session is failed, the FLUS sink shall send a 403 message.

The possible response messages from the FLUS sink, depending on whether the POST request is successful or unsuccessful, are shown in Table 7.5-1.

Table 7.5-1: Response status code, message, and contents for session creation

| Status Code | Message | Contents |
| --- | --- | --- |
| 201 Created | Session created successfully | The FLUS sink provides the session resource identifier of the created session and detailed session information |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| Note: In addition to the above response codes, the FLUS sink can also send appropriate response codes described in IETF RFC 7231 [11] as applicable |  |  |

## 7.6	Session termination

DELETE /flus/v1.0/sessions/{session-res-id}

To terminate a FLUS session (incl. the FLUS Sink configuration), the Control source shall use the HTTP DELETE method on the "session" instance resource as follows:

-	the request URL with the "path" is set to "/flus/v1.0/sessions/{session-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

The {session-res-id} is the session resource identifier for which the session termination is sought.

Upon reception of a HTTP DELETE request, the FLUS sink will check to see if the configuration with the given session resource identifier exists at the given sink. If such a configuration exists, the FLUS sink shall delete the configuration associated with the given session resource identifier. Further, the FLUS sink responds back to the Control source with a 200 success message along with the session resource identifier indicating that the configuration was successfully deleted. Alternately, if the configuration cannot be deleted, the FLUS sink shall send a 403 Forbidden message. If the session resource was not found, the FLUS sink returns back with a 404 message.

The possible response messages from the FLUS sink, depending on whether the DELETE request is successful or unsuccessful, are shown in Table 5.2.2.2.4-1.

Table 7.6-1: Response status code, message, and contents for session deletion

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS sink shall send the session resource identifier of the configuration that is deleted |
| 204 No Content | The request has succeeded | None |
| 403 Forbidden | Request cannot be fulfilled | The FLUS sink may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the FLUS sink can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

## 7.7	FLUS Source capability discovery

GET /flus/v1.0/capabilities

### Using this capability exchange procedure, the Remote Controller enquires about the supported features and instantiations of the FLUS source.

### To retrieve the capabilities of a FLUS source, the Remote Controller shall use the HTTP GET method as follows:

-	the request URI with the "path" is set to "/flus/v1.0/capabilities"

-	the Host field is set to the FQDN of the FLUS sink

The capabilities resource information provides in detail the capabilities of the FLUS source. The response from the FLUS source to the Remote Controller shall contain the following:

-	The Content-Type field specifying the MIME type (JSON) using which the resource information is encoded.

-	The Content-Length is the length of the content body.

The above header fields are followed by the content body in the format indicated by the Content-Type field. The content body includes the detailed representation of the resource as described in section 7.1.1.1 from which the capabilities of that FLUS source can be inferred.

The possible response messages from the FLUS source to the Remote Controller are shown in Table 7.7-1.

Table 7.7-1: Response status code, message, and contents using HTTP GET

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS source provides the source capabilities to the Remote Controller. |
| 403 Forbidden | Request cannot be fulfilled | The FLUS source may include optional text to indicate why the request could not be fulfilled |
| Note: In addition to the above response codes, the FLUS source can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

## 7.8	Remote FLUS Source configuration creation

POST /flus/v1.0/configurations

To create a configuration at the FLUS source, the Remote Controller shall use the HTTP GET method on the "configurations" collection resource as follows:

-	the request URI with the "path" is set to "/flus/v1.0/configurations"

-	the Host field is set to the FQDN of the FLUS source

-	the Content-Type field specifying the MIME type (JSON) using which the session resource information is encoded.

-	the Content-Length is the length of the content body.

The content body of the POST request shall contain the representation of the configuration resource.

Upon receipt of HTTP POST to create a configuration resource, the FLUS source shall create a configuration. Upon successful creation of configuration resource, the FLUS source shall respond back to the Remote Controller with a 201 success message indicating that the configuration is successfully created along with the resource identifier of the created configuration.

The configuration resource identifier is the identifier that uniquely identifies the configuiration within the list of all configurations at that source. When the Remote Controller receives the configuration resource identifier, it shall use this identifier in subsequent requests to the FLUS source to refer to this configuration.

Alternatively, if the creation of session is failed, the FLUS source shall send a 403 message.

The possible response messages from the FLUS source, depending on whether the POST request is successful or unsuccessful, are shown in Table 7.8-1.

Table 7.8-1: Response status code, message, and contents for session creation

| Status Code | Message | Contents |
| --- | --- | --- |
| 201 Created | Configuration created successfully | The FLUS source provides the configuration resource identifier of the created configuration |
| 403 Forbidden | Request cannot be fulfilled | The FLUS source may include optional text to indicate why the request could not be fulfilled |
| Note: In addition to the above response codes, the FLUS source can also send appropriate response codes described in IETF RFC 7231 [11] as applicable |  |  |

## 7.9	Remote Manage a FLUS Source configuration

### 7.9.1	FLUS Source configuration fetch procedure

GET /flus/v1.0/configurations/{config-res-id}

Configurations can be read by the Control source when it wishes to know the latest representation of the session resource at the FLUS sink. To fetch the properties of a FLUS Sink Configuration, the Control source sends a HTTP GET request to the FLUS sink as follows:

-	the request URI with the "path" set to "/flus/v1.0/ configurations/{config-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

### The {config-res-id} in the request URI is the configuration resource identifier previously created by the Remote Controller.

### Upon receiving HTTP GET request from the Remote Controller, the FLUS source checks to see if such a configuration exists that matches the given configuration resource identifier. If such a configuration resource exists, the FLUS source shall respond to the Remote Controller with a 200 OK message along with the complete representation of the configuration resource. The response from the FLUS source to the Remote Controller shall contain the following:

-	The Content-Type field specifying the MIME type (JSON) using which the configuration resource information is encoded.

-	The Content-Length is the length of the content body.

### The content body of this response message shall be the representation of the configuration resource as described in sub clause 7.1.1.2

Alternatively, if such a configuration resource cannot be found by the FLUS source, it shall send a 404 Not Found message to the Remote Controller. If the request cannot be fulfilled, the FLUS source shall send a 403 Forbiddent message to the Remote Controller.

The possible response messages from the FLUS source, depending on whether the GET request is successful or unsuccessful, are shown in Table 7.9.1-1.

Table 7.9.1-1: Response status code, message, and contents for modification using HTTPS GET

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS source provides session properties of the configuration resource |
| 403 Forbidden | Request cannot be fulfilled | The FLUS source may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the FLUS source can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

### 7.9.2 	FLUS Source configuration update procedure

The configuration resource at the FLUS source can be either partially or completely modified by using the procedures as specified in clauses 7.9.2.1 and 7.9.2.2, respectively.

#### 7.9.2.1	Partial modification of FLUS Source configuration

PATCH /flus/v1.0/configurations/{config-res-id}

To update some of the FLUS Source configuration parameters at the sink, the Remote Controller sends an HTTP PATCH request as follows:

-	the request URI with the "path" is set to "/flus/v1.0/configurations/{config-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

-	The Content-Type field specifying the MIME type (JSON) using which the configuration resource information is encoded.

-	The Content-Length is the length of the content body.

### The {config-res-id} in the request URI is the configuration resource identifier of the configuration whose modification is sought.

The content body of the HTTP PATCH message shall contain the updated partial representation of the configuration resource.

Upon receiving the HTTP PATCH request from the Remote Controller, the FLUS source checks to see if such a configuration resource exists at the sink. If such a configuration resource exists, FLUS source shall update the configuration properties based on the values from the incoming request. Upon successful update of the requested configuration, the FLUS source shall respond to the Remote Controller with a 200 OK success message indicating that the configuration resource was successfully updated. The FLUS source shall also include the configuration resource identifier of the configuration that is updated. As alternative to the 200 OK message, FLUS source may send a 204 No Content success message without any message content to the Remote Controller. If the configuration resource cannot be updated, the FLUS source shall send a 403 message. If the configuration is not found, the FLUS source shall send a 404 message.

The possible response messages from the FLUS source, depending on whether the PATCH request is successful or unsuccessful, are shown in Table 7.9.2.1-1.

Table 7.9.2.1-1: Response status code, message, and contents for configuration modification using HTTP PATCH

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS source provides the configuration resource identifier of the configuration that is modified |
| 204 No Content | The request has succeeded | None |
| 403 Forbidden | Request cannot be fulfilled | The FLUS source may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the FLUS source can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

#### 7.9.2.2	Full modification of FLUS Source configuration

PUT /flus/v1.0/configurations/{config-res-id}

For complete update of the configuration parameters at the FLUS source, the Remote Controller sends an HTTP PUT request as follows:

-	the request URI with the "path" is set to "/flus/v1.0/configurations/{config-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

-	The Content-Type field specifying the MIME type (JSON) using which the session resource information is encoded.

-	The Content-Length is the length of the content body.

### The {config-res-id} in the request URI is the configuration resource identifier of the configuration whose modification is sought.

The content body of the HTTP PUT message shall contain the updated complete representation of the configuration resource.

Upon receiving the HTTP PUT request from the Remote Controller, the FLUS source checks to see if such a configuration resource exists at the source. If such a configuration resource exists, FLUS source shall update the configuration properties based on the values from the incoming request. Upon successful update of the requested configuration, the FLUS source shall respond to the Remote Controller with a 200 OK success message indicating that the configuration resource was successfully updated. The FLUS source shall also include the configuration resource identifier of the configuration that is updated. As alternative to the 200 OK message, FLUS source may send a 204 No Content success message without any message content to the Remote Controller. If the configuration cannot be updated, the FLUS source shall send a 403 message. If the configuration is not found, the FLUS source shall send a 404 message.

The possible response messages from the FLUS source, depending on whether the PUT request is successful or unsuccessful, are shown in Table 7.9.2.2-1.

Table 7.9.2.2-1: Response status code, message, and contents for session modification using HTTP PATCH

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS source shall send the configuration resource identifier of the configuration that is modified |
| 204 No Content | The request has succeeded | None |
| 403 Forbidden | Request cannot be fulfilled | The FLUS source may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the FLUS source can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

## 7.10	FLUS Source configuration deletion

DELETE /flus/v1.0/configurations/{config-res-id}

To delete a FLUS Source configuration (and terminate on-going FLUS sessions), the Remote Controller shall use the HTTP DELETE method on the configuration resource as follows:

-	the request URL with the "path" is set to "/flus/v1.0/ configurations/{configuration-res-id}"

-	the Host field is set to the FQDN of the FLUS sink

The {config-res-id} is the configuration resource identifier for which the session termination is sought.

Upon reception of a HTTP DELETE request, the FLUS source will check to see if the configuration with the given configuration resource identifier exists at the given source. If such a configuration exists, the FLUS source shall delete the configuration associated with the given configuration resource identifier. Further, the FLUS source responds back to the Remote Controller with a 200 success message along with the configuration resource identifier indicating that the configuration was successfully deleted. Alternately, if the configuration cannot be deleted, the FLUS source shall send a 403 Forbidden message. If the configuration resource was not found, the FLUS source returns back with a 404 message.

The possible response messages from the FLUS source, depending on whether the DELETE request is successful or unsuccessful, are shown in Table 7.10-1.

Table 7.10-1: Response status code, message, and contents for configuration deletion

| Status Code | Message | Contents |
| --- | --- | --- |
| 200 OK | The request has succeeded | The FLUS source shall send the configuration resource identifier of the configuration that is deleted |
| 204 No Content | The request has succeeded | None |
| 403 Forbidden | Request cannot be fulfilled | The FLUS source may include optional text to indicate why the request could not be fulfilled |
| 404 Not Found | Requested resource not found | None |
| Note: In addition to the above response codes, the FLUS source can also send appropriate response codes described in IETF RFC 7231 [6] as applicable. |  |  |

## 7.11	Session establishment for Remote Control

### 7.10.1	General

The FLUS Source should support delivery of Remote Control commands using WebSockets (IETF RFC 6455 [x]).

The FLUS Source may exposes an HTTPS server to receive Remote Control messages. However, security features like firewalls may prevent reaching the UE.

### 7.10.2	Remote Control using WebSockets

This procedure in the present subclause only apply, when the Remote Control Target and the Remote Controller supports WebSockets and when the Remote Control feature is activated. In this case the Remote Control Target is configured with a WebSocket URL and establish the websocket connection at FLUS Source startup.

The following framing for the request and response shall be used when delivering a request and its response through Websockets.

The Remote Controll session towards the Remote Control Target, the Remote Controller shall embed the following json structure in a separate Websocket data frame with 0x1 (Text) opcode in the following way:

-	The json object contains two key / value pairs: the first object for the HTTP headers and the second object for the HTTP body.

- 	The key value of the first object is “header”. The value of the object is a json array of json object.

-	Each json object of the array contains the key and the value of a single HTTP header.

-	The Host HTTP header may be omitted, since the WebSocket Connection is already established.

-	The key value of the second object is “body”. The value of the object is formatted according to the Content-Type header field. The content type of the body object is defined by the individual API.

**** Last Change ****