3GPP TSG-SA4 Meeting #107 	S4-200085

# Wroclaw, Poland, Jan 20 - 24, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.139 | CR | xxx | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding RTP Tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | RTCPVer |  |  |  |  |  | Date: |  |  | 2020-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No RTP tests have any descriptive text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding RTP test descriptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RTP tests will not be described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-200085 Initial draft |  |  |  |  |  |  |  |  |

First Change

## 6.3	RTP Tests

### 6.3.1	General

This clause describes tests that are applicable to all RTP and SRTP packets. If not explicitly stated otherwise, any reference to RTP is equally applicable to SRTP. Tests that are only applicable to SRTP and not to RTP are specified in clause 6.5.

### [6.3.2	Basic RTP Tests

#### 6.3.2.1	Receive RTP Padding Test

Purpose:	Test if RTP packets with padding are correctly received.

FFS.

#### 6.3.2.2	Send RTP Padding Test

Purpose:	Test if RTP packets with padding are correctly generated.

FFS.

#### 6.3.2.3	SSRC Value Test

Purpose:	Test if the SSRC value is chosen randomly for every new RTP stream.

FFS.

#### 6.3.2.4	Initial Sequence Number Test

Purpose:	Test if the Sequence Number start value is chosen randomly for every new RTP stream.

FFS.

#### 6.3.2.5	Wrapped Sequence Number Test

Purpose:	Test if RTP packets are correctly received even when Sequence Number wraps around.

FFS.

#### 6.3.2.6	Initial Time Stamp Test

Purpose:	Test if the Time Stamp start value is chosen randomly for every new RTP stream.

FFS.

#### 6.3.2.7	Wrapped Time Stamp Test

Purpose:	Test if RTP packets are correctly received even when Time Stamp wraps around.

FFS.

#### 6.3.2.7	Unknown Payload Type Test

Purpose:	Test if RTP packets with unknown (non-negotiated) payload type are correctly ignored.

FFS.

]

### 6.3.3	RTP Header Extension Tests

#### 6.3.3.1	Ignore Unknown Header Extension Test

Purpose:	Test if receiving RTP packets with an unknown header extension correctly ignores the header extension but accepts the RTP payload.

FFS.

### 6.3.4	RTP Contributing Source Tests

#### 6.3.4.1	Ignore Contributing Source Test

Purpose:	Test if receiving RTP packets with a non-zero CC field and CSRC list correctly ignores the CSRC list but accepts the RTP payload.

FFS.

End of document