3GPP TSG-SA4 Meeting #107 	S4-200086

# Wroclaw, Poland, Jan 20 - 24, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.139 | CR | xxx | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Conformance Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | RTCPVer |  |  |  |  |  | Date: |  |  | 2020-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The work item objectives include a method to announce conformance to this specification "on the network", i.e., in call setup signalling and in RTP/RTCP information, but this is currently missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a method to announce conformance in call setup signalling and in RTP/RTCP information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible for an RTP/RTCP endpoint to announce conformance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 (new), 7.1 (new), 7.2 (new), 7.2.1 (new), 7.2.2 (new), 7.2.3 (new), 7.2.4 (new), 7.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-200086 Initial draft |  |  |  |  |  |  |  |  |

First Change

# 7	Conformance Indication

## 7.1	General

An RTP/RTCP implementation conforming to this specification may announce its compliance during call setup and within the RTP/RTCP protocol information. If announcing compliance to this specification within RTP/RTCP, an "3GPPRTP" RTCP SDES item (see clause 7.3) must be used. If SIP/SDP is used for call signalling, announcing compliance to this specification must both use an SDP attribute "a=3gpp-rtp" (see clause 7.2) and through the above mentioned "3GPPRTP" RTCP SDES item. The reason to require information both in SDP and in RTCP is to ensure that call signalling and RTP/RTCP stack implementations are aligned.

## 7.2	The a=3gpp-rtp SDP attribute

### 7.2.1	General

The a=3gpp-rtp attribute may be provided on SDP session level, SDP media level or both. It is not defined for use on source or dcsa level. If used on SDP session level, it must be taken as applicable to all all m= lines in the SDP using any RTP profile, e.g. RTP/AVP, RTP/AVPF, RTP/SAVP, or RTP/SAVPF. If used on SDP media level using any RTP profile, it must be taken as applicable to that media description, overriding any a=3gpp-rtp attribute on session level. It is not defined for use on non-RTP SDP media level.

### 7.2.2	ABNF syntax and semantics

The 3gpp-rtp ABNF syntax is:

3gpp-rtp-value = spec-version [SP 3gpp-rtp-extension]

spec-version = 1*2DIGIT "." 1*2DIGIT "." 1*2DIGIT

3gpp-rtp-extension = byte-string

; SP and byte-string as defined by IETF RFC 4566

Example:

a=3gpp-rtp:16.5.0

### 7.2.3	SDP offer/answer considerations

This attribute is a unilateral announcement from the party creating the SDP offer or answer, and there are thus no SDP offer/answer considerations. When announcing compliance to this specification, the a=3gpp-rtp attribute shall be included in the SDP offer, and shall be included in the SDP answer regardless if it was also present in the corresponding SDP offer.

### 7.2.4	IANA registration information

Contact name, email address, and telephone number:

3GPP Specifications Manager

3gppContact@etsi.org

+33 (0)492944200

Attribute Name (as it will appear in SDP):

3gpp-rtp

Long-form Attribute Name in English:

3GPP-conformant RTP/RTCP implementation

Type of Attribute:

Session level and media level

Is Attribute Value subject to the Charset Attribute?

This attribute is not dependent on charset.

Purpose of the attribute:

This attribute is used to indicate the RTP/RTCP stack conformance to 3GPP TS 26.139.

Appropriate Attribute Values for this Attribute:

See 3GPP TS 26.139 clause 7.2.2 for ABNF and detailed usage.

MUX Category for this Attribute:

IDENTICAL

## 7.3	The 3GPPRTP RTCP SDES item

This SDES item shall be included in at least the three first compound RTCP packets after RTP session setup and in at least the three first compound RTCP packets after RTP session re-negotiation. It may but should not be provided in every compound RTCP packet. It uses the format of Private Extensions SDES Item (PRIV) in section 6.5.8 of RFC 3550 [2] (copied below for convenience), using "3GPPRTP" as prefix string with prefix length=7.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     PRIV=8    |     length    | prefix length |prefix string...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

...             |                  value string               ...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 7.3-1 RTCP SDES PRIV format

The value string shall be identical to the content of 3gpp-rtp-value from clause 7.2.2, with the additional restriction that it shall not be zero-terminated (as required for any SDES item) and any characters at the end beyond 247 (255 minus the 7-byte prefix string and the prefix length byte) shall be truncated before being put into the value string.

Using the same example 3gpp-rtp-value as in clause 7.2.2, this would be the SDES item content, in total 16 bytes, separating bytes with "|", using decimal byte values, and enclosing characters in single quotes (e.g. 'A' is equivalent to decimal 65):

|8|14|7|'3'|'G'|'P'|'P'|'R'|'T'|'P'|'1'|'6'|'.'|'5'|'.'|'0'|

Figure 7.3-2 Example 3GPPRTP RTCP SDES item content

No IANA registration is required for SDES PRIV prefixes.

End of document