3GPP TSG-WG SA4 Meeting #107	S4-200096

# Wroclaw, PL, January 2020

| CR-For v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR |  | rev | 0 | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Client Provisioning Parameters over M5d |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In order for a media streaming session to benefit from the 5G features offered to them by the 5G system, the Session Handler needs to exchange information with the Network Assistance Media AF regarding the starting/ongoing session. This pCR specifies a set of parameters that are exchanged over the M5d interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR defines a set of parameters that are exchanged between the Session Handler and the Assistance Media AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 solution will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 4.X	Network Assitance Configuration (NAC) Object

The 5GS offers network assistance to multimedia sessions for which provisioning has been performed by the content provider with the Provisioning Media AF. A new media streaming session is detected by the Session Handler, which in turn will intiate a network assistance session with the Assistance Media AF over the M5d interface.

A Network Assistance Configuration (NAC) Object contains all information relative to an established media streaming session. The Network Assistance Configuration Object is hosted in the Assistance Media AF and is controlled by the Session Handler.

The NAC Object is defined in the following table:

| Parameter | type | Description |
| --- | --- | --- |
| SessionId | Integer | Identifier of the session that is being managed. |
| QoS | Object | This object provides information about possible operation points and their related QoS requirements. This information may be received from the Provisioning or from the Session Handler, which in turn may get it from the Media Player. |
| selectedOperationPointId | Integer | Indicates the current operation point that is selected for the ongoing session. |
| OperationPoints | Array of Objects |  |
| OPId | Integer | Provides an indentifier for the current operation point |
| MaxGBR | Integer | The maximum value for guaranteed bitrate. Note that this implies that GBR = MBR for the operation point that is provisioned for this session. Traffic that exceeds the MaxGBR will be shaped, which may result in excessive delays and packet drops. |
| AveragingWindow | Integer | The averaging window over which the MaxGBR value is calculated. The value is provided in milliseconds. |
| MaxBurst | Integer | The maximum burst that is allowed for the session at this operation point. The MaxBurst is provided in bytes. |
| BurstAveragingWindow | Integer | The averaging window over which the burst is calculated. This value is typically an order of magnitude smaller than the AveragingWindow and is meant to allow for short-lived bursts in the traffic. |
| RouteSeletions | Array of Objects | Provides information to the SessionHandler to assist with the selection of the Route at the UE. |
| OperationPointId | Integer | Points to the operation point to which this selection applies. |
| NSSAI | Integer | Identifier of the network slice to use for this operation point. |
| DNN | String | Name of the Data Network that should be used for this session at this operation point. |
| TrafficDescriptor | Object | Contains information about the traffic descriptors that can be used to identify and filter the traffic. The traffic descriptor shall be formatted as described in 6.6.2.1-2 of TS 23.503. |
| Reporting | Object | Provides information about reporting from the Session Handler to the network. |
| ConsumptionReporting | Object | Contains information about the requested consumption reporting. The object shall be formatted as defined in X. |
| QoEReporting | Object | Contains information about the requested QoE reporting. The object shall be formatted as defined in the Metrics object of TS 26.247. |