Source:	Nokia Corporation

Title:	On recording stereo audio test material

Document for:	Discussion

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

Some aspects relating to IVAS testing are under discussion. For example, discussion has begun on test material including background noise. Some general guidelines are available in the IVAS WID [1], and first proposals are included in the draft IVAS performance requirements (IVAS-3) [2]. Furthermore, the processing plan for the IVAS qualification phase (IVAS-7a) was initiated at SA4#106 [3].

In [4], Orange provides for discussion prior work that could be useful for IVAS standardization. This includes among other points the following items to discuss and investigate:

- whether speech experiments should rely on two-talker combinations with some overlap with short test samples,

- whether the reverberation/positioning approach based on impulses responses in ITU-T STL should be reused and would cover the relevant scenarios for IVAS, and

- details of noisy speech processing (noise database definition, noise types, SNR definition, etc.) which may not be directly taken/easily reused from ITU-T.

In this document, we discuss some aspects relating to the speech experiments, consider the applicability of the reverberation/positioning approach for specific recording scenarios for IVAS, and describe those recording scenarios (including background noise) based on relevant use cases. This discussion covers mainly stereo and binaural audio inputs, but some further spatial audio input formats and configurations are of relevance for some of the recording use cases. That topic will remain for further study.

# 2. Discussion

Subjective testing plays a significant part in successful audio codec standardization. For successful subjective testing, it is important to consider also realistic and representative test material providing a good coverage of targeted use cases.

With its immersive capabilities, the IVAS codec will cover a multitude of relevant use cases. The same is true even when limiting the discussion to stereo (and binaural audio). Some of the most common use cases should ideally be identified, and their requirements reflected in the subjective listening testing to provide better understanding of the codec capability in repeatable and well-understood real-world conditions. Here, we discuss some use cases in broad categories. The discussion is not an exhaustive listing of use cases. Rather the idea is to consider different types of audio capture scenarios that can be considered for realistic test material generation.

Scenarios as discussed here may be, e.g., recorded in a real background environment for assessing real-world signals. Alternatively, for controlled background noise levels, it may be used clean speech recordings mixed with typical background noises recorded using the same capture configuration and in the same originating format.

## 2.1 Stereo and binaural telephony

Telephony is an extremely important high-level use case that is of relevance in several scenarios. For telephony, we may naturally consider both stereo and immersive use cases [5]. However, the current discussion is limited to two channels: stereo and binaural audio. Two-channel telephony use cases can be further divided into several capture scenarios. We identify here three that we may call (near-)binaural, handset, and hands-free. These three capture scenarios are described below.

Binaural or near-binaural telephony

Use of headphones and earbuds is growing significantly. Firstly, it should of course be noted the importance of these device categories as presentation devices. The IVAS WID also states that the predominant audio rendering instrument for IVAS is headphones [6]. However, increasingly often the headphone or earbud device is also an audio capture device. Depending on the exact microphone positions, the type of headphones, and various DSP processing, the audio captured can be considered to be either stereo or binaural audio. We may call this scenario, e.g., near-binaural. This type of audio capture is thus relevant both for IVAS stereo and IVAS binaural audio input testing.

In a practical test material generation setup, it can be considered microphones placed close to or directly on each of a talker’s ears. The (main) talker is here naturally in the middle of the audio scene and surroundings are logically oriented on the left-right plane. Typical environments include relatively quiet home or office environments, or the person talking may be on the move, e.g., on street or in a shopping mall or sitting in a car. In most cases, there is only a single talker present. However, a second talker may be talking, e.g., somewhere in the front sector of the main talker. This can be a secondary talker for the call or an interfering talker.

Most relevant presentation is likely via headphones.

Stereo telephony in handset mode

The second capture scenario may be called handset. Here, the user is operating a UE on the ear, however the device is capable of (at least) stereo capture. (The same capture scenario can be highly relevant for spatial audio targeting MASA input generation; however, this particular setup may be difficult to record using an Eigenmike [7].) The stereo capture is relevant for IVAS stereo input testing.

In a practical test material generation setup, it can be considered two microphones placed such that one is close to the talker’s mouth and one is close to either one of the talker’s ear. Left and right channel denomination may be switched freely but should remain constant for each recording or sample. The (main) talker is here predominantly in one of the channels with ambient audio predominantly in the other channel. Typical environments include relatively quiet home or office environments, or the person talking may be on the move, e.g., on street or in a shopping mall or sitting in a car. In most cases, there is only a single talker present. However, a second talker may be talking, e.g., somewhere in the front sector of the main talker. This can be a secondary talker for the call or an interfering talker.

Most relevant presentation is likely via headphones.

Stereo telephony in handsfree mode

The third capture scenario may be called handsfree. This telephony use case may be understood with or without accompanying video, i.e., this could be a video call with user holding their smartphone/UE in front of themselves or having placed it, e.g., on a table. (This general use case is a typical mobile spatial capture scenario suitable also, e.g., for MASA input generation.) The stereo capture is relevant for IVAS stereo input testing.

In a practical test material generation setup, it can be considered two microphones in front of the (main) talker or, e.g., on a table about 30 cm from the edge and 20 cm above the table with the (main) talker sitting at the table. This corresponds to a position where a person would typically hold their phone when talking in hands free mode and also how the microphones may be located in case of tablet or laptop capture. Typical environments include relatively quiet home or office environments. For example, a café could also be a relevant environment, or the person could be sitting in a car. The main talker is typically in the middle of the audio scene, and potential secondary talkers may appear on either side of the main talker.

Relevant presentation options are headphones and stereo speakers.

We note that for all of these two-channel audio capture scenarios, it is possible to derive either real-world signals with real background noise, or separate clean speech and background noise signals can be recorded that may then be mixed together in a controlled way for testing with background noises. Therefore, test material generation may be based on real stereo with natural characteristics instead of using artificially generated stereo signals based on mono recordings.

## 2.2 Stereo and binaural teleconferencing

Teleconferencing is another important high-level use case that covers various scenarios. Here, we consider some two-channel teleconferencing scenarios. While stereo is likely much more relevant, it may be useful to discuss also binaural audio in this context.

For teleconferencing scenarios, we consider both direct real-world captures and artificial mixes that may be performed, e.g., by a teleconferencing equipment or service. In both cases, initial capture may be UE-originating. In the former case it is typically a stereo capture, and in the latter case a mono capture.

Ad-hoc multi-person meeting call

In this use case, spatial audio capture may be preferable. However, stereo capture still provides improvement over mono capture by better differentiating the talker positions. According to the recording scenario, a meeting call is quickly set up using a device that is capable of (at least) stereo capture. Alternatively, it could be considered that there is more than one device or device with some accessory that are able to connect to each other and provide a joint stereo audio input. This is similar to a regular meeting room setup. The various configurations mean that many different microphone distances can be considered. If we have a single device (e.g., mobile phone), the microphone distance is fairly small, e.g., 10-15 cm. On the other hand, the microphones based on joint capture scenario or a specific meeting room installation can be, e.g., 1 or 2 m apart. In some meeting room systems, the microphones could be coincident. The microphone placement is typically on a table and local call participants are placed around the table.

This scenario may correspond to a one-to-one call where there is more than one talker at one end. Alternatively, this may be, e.g., a conference call where a stereo signal is accepted as input and forwarded to receiving users. The stereo capture is therefore directly relevant for IVAS stereo input testing.

In a practical test material generation setup, it can be considered a spaced stereo pair on a table with several talkers sitting at the table. Various microphone distances (as discussed above) may be considered. In case of meeting room setups, a relatively quiet office environment is most typical. However, to consider the ad-hoc meeting call aspect, other background noises, such as cafeteria, train station, or airport could be considered. The talkers (two or more) can be placed freely around the microphones. Left and right channel denomination may be switched freely but should remain constant for each recording or sample.

Most relevant presentation is likely via headphones.

We note that for this two-channel audio capture scenario, it is possible to derive either real-world signals with real background noise, or separate clean speech and background noise signals can be recorded that may then be mixed together in a controlled way for testing with background noises. Therefore, test material generation may be based on real stereo with natural characteristics instead of using artificially generated stereo signals based on mono recordings.

Telco with artificially generated stereo or binaural downstream

In this use case, the audio capture is typically a mono capture. A stereo or pre-binauralized signal is created by a conferencing server. For example, the conferencing server may pan each talker to an arbitrary position (left-right), where the position may be fixed or adaptive. Alternatively, a full spatial representation may be used in this type of service, e.g., using Ambisonics or audio objects. The pre-binauralized signal is a binaural audio representation of the full spatial scene. The artificially generated stereo or binaural audio signal is relevant for IVAS stereo or IVAS binaural audio testing.

In a practical test material generation setup, clean mono speech is recorded. A suitable reverberation/positioning approach may then be used to derive the required stereo signal. In case of binaural audio generation, various strategies could be utilized. If there is interest to test binaural audio input in this scenario, a common processing should probably be agreed.

Most relevant presentation is likely via headphones.

We note that for artificially generated stereo or binaural audio teleconferencing scenarios, mono clean speech recordings are likely sufficient. Some low-level background noise (e.g., office noise) could be additionally considered. However, the mixing can be carried out for mono signals.

# 3. Summary

We presented discussion on several stereo (and some binaural audio) input generation scenarios. It is to be discussed which types of recording scenarios are relevant for IVAS and will provide good coverage for foreseen use cases.

It is the general view of the source that real stereo recordings should be used wherever possible instead of artificially generated stereo inputs based on mono recordings. The exception to this may be a teleconferencing scenario where mono audio inputs are mixed into a stereo or binaural downstream for the recipients. In either case, it is possible to mix clean speech and separately recorded background noises together in order to derive controlled noise levels for noisy speech testing.

# References

[1]	SP-170611: “New WID on EVS Codec Extension for Immersive Voice and Audio Services”

[2]	S4-191064: “IVAS Performance Requirements (IVAS-3)”, v0.0.4

[3]	S4-191308: “IVAS-7a: Processing plan for qualification phase”, v0.1.0

[4]	AHEVS-509: “IVAS processing functions for stereo and binaural audio”, Orange

[5]	S4-191063: “IVAS Usage Scenarios (IVAS-9)”, v0.0.3

[6]	SP-170611: “New WID on EVS Codec Extension for Immersive Voice and Audio Services”

[7]	S4-191167: “Description of the IVAS MASA C Reference Software”, Nokia Corporation