## Source:	Fraunhofer IIS

## Title:	On the Combination of Input Formats

## Document for:	Discussion & Agreement

## Agenda Item:	7.5

# 1	Introduction

The IVAS Design Constraints document IVAS-4 [1] still has open issues with respect to the Encoder Input Formats. This contribution addresses the issue of combinations of input formats.

# 2	Background

The current design constraints on the input formats read as follows [1]:

| Encoder Input Formats | The encoder shall support the following input formats: Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] Binaural audio Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Note: ACN component ordering and SN3D normalization. [Spatial audio, [N] channels and spatial metadata defined by [TBD].] [Editor’s Note FFS: Spatial metadata definition for the spatial audio format will require further input.] Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. [In addition, the IVAS codec shall support combinations of the above, totalling to no more than [TBD] audio streams. Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.] Editor’s note: Input is invited for defining the combinations of input formats. The inclusion of Binaural Audio for the combinations may matter to the rendered output formats. |
| --- | --- |

Combinations of input formats are one of the key features of next generation audio codecs. For broadcast-codecs, such as MPEG-H, this becomes manifest for example in the combination of a channel-bed plus one or several objects. By means of the channel-bed typically the non-interactive components of a scene are coded (e.g. a capture of the sports event, movie score, scene background, …). By means of the objects the interactive components of a scene are coded. This could be the commentator of the sport scene (increase/decrease volume, switch to different language, …), the movie characters (increase/decrease volume, switch to different language, …) or individual sound components where interactivity is seen beneficial (increase/decrease volume, switch to different language, move, …).

It could of course be argued that non-interactive content could also be transmitted and composed by means of individual audio objects. This is in general true, however there are important reasons not to do so:

- Availability: In many cases object-based content is not available. The most important reason for this is the higher effort of doing a pure object-based capturing/processing.

- Preservation of Artistic Intent: Mixing is often carried out as part of the artistic process and with the intention that this is not changed and the content is played back exactly as intended. Alteration of this mix is not desired by the artist or producer.

- Complexity: Processing, Coding and Mixing of many objects is in general more complex than processing of a pre-produced channel-bed

For communication use-cases a combination of objects plus channel bed or scene-based audio is in many cases crucial, too. Use-cases for this are for example the interaction of multiple users in a real or artificial scene: Multiple users communicate in a scene which is either a real capture or an artificially created. The users are represented as individual objects, the scene either as channel-bed or scene-based audio scene. It’s either coming as a real capture from one of the users (e.g. represents the conference room with possibly other participants in the background) or could also be an artificial scenario (e.g. beach, urban environment, space-station, game scene…) set up by the bridge or an MCU.

User-generated content can be seen in a similar way as broadcast-content: A scene is captured either by means of a channel-bed or scene-based audio. In addition, there might be a commentator which e.g. describes what he is currently doing/seeing/feeling or simply talks to the audience – an incredible amount of such content is currently produced for social media platforms. In many cases it’s advantageous to have the possibility of interactivity for the commentator.

Further on, for a number of reasons and application scenarios it is crucial that this combination of formats is treated jointly by the codec and not transmitted via several codec-instances:

- Guarantee of full user-experience: A single bitstream for a complete scene is the simplest and most effective way to fully ensure the intended user experience: No separate instances could get lost or interact in a non-intended way.

- Joint optimization of bitrate assignment: The bitrate assignment to individual components can be jointly optimized. In such a way e.g. the bitrate assignment to inactive components can be reduced for the benefit of active components.

- Complexity: A single complexity limit will apply to the jointly coded combination, making sure that the decoder will be able to decode the complete scene and not only parts of it.

It is known that the overall complexity typically scales with the number of audio-streams (i.e. number of channels for channel-based audio, number of objects for object-based audio, number of coefficients for scene-based audio). Since the IVAS codec targets mobile devices, a certain limitation of the number of audio-streams is required to also limit the overall codec-complexity. The source proposes to limit the number of streams to at maximum 16 simultaneous streams. Since the complexity of an LFE is usually negligible, it is proposed to not take this into account. This would for example allow for:

- 16 objects

- 5.1 channels

- 7.1 channels + 9 objects

- [7.1+4 + 5 objects]

- FOA SBA + 12 objects

- HOA2 SBA + 8 objects

- HOA3 SBA

# 3	Proposal

| Encoder Input Formats | The encoder shall support the following input formats: Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] Binaural audio Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Note: ACN component ordering and SN3D normalization. [Spatial audio, [N] channels and spatial metadata defined by [TBD].] [Editor’s Note FFS: Spatial metadata definition for the spatial audio format will require further input.] Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. In addition, the IVAS codec shall support combinations of the above, totalling to no more than 16 audio streams (plus one potential LFE stream). [Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.] Editor’s note: Input is invited for defining the combinations of input formats. The inclusion of Binaural Audio for the combinations may matter to the rendered output formats. |
| --- | --- |

# 4	Summary

The source has provided evidence for the necessity of combinations of input formats. It is proposed to reflect this in [1] as outlined in Section 3.

References:

[1]	3GPP S4-191307, IVAS Design Constraints (IVAS-4), v0.2.0