Source:	Orange

Title:	Processing functions for binaural and stereo testing

## Document for:	Agreement

## Agenda Item:	7.5

## Introduction

The present Tdoc is a continuation of AHEVS-509 [1]. We discuss some aspects to progress IVAS-7a.

## Proposed processing steps

Based on experience (from ITU-T activities and internal exploration), we propose the following steps for inclusion in IVAS-7a, with the working assumption that the P.800 DCR methodology is used in qualification to evaluate binaural and stereo cases over headphones:

- For speech experiments:

  - define categories with artificial binaural/stereo (AB, MS) capture

  - use two-talker combinations with no talker overlap over time, using 4s single sentences

  - use the reverb tool from ITU-T STL with pre-selected sets of impulse responses to position talkers in space and simulate anechoic or reverberant conditions – note: alternative impulses responses may be considered

Note: natural capture may be considered, however requirements on recording conditions shall be set sufficiently in advance for test labs to prepare the necessary database(s).

- Adjust the binaural/stereo level based on ITU-R BS.1770-4 with a reference level defined in dB LUFS. Note: inputs are invited to provide a reference implementation.

- Define calibration anchors, including at least dual P.50 MNRU in SWB – note: stereo anchors such as SDRU or ESDRU from ITU-T P.811 [4] may be also relevant for IVAS.

We propose to capture the above proposals in IVAS-7a [2].

The source may provide further details (command lines and figures illustrating the processing steps) if found necessary.

## References

- 3GPP Tdoc AHEVS-509, IVAS processing functions for stereo and binaural audio, Source: Orange

- 3GPP Tdoc S4-191308, IVAS-7a: Processing plan for qualification phase, v0.1.0, Source: Editor (Ericsson LM)

- TD 178 (WP 3/16), Processing test plan for the ITU-T Joint G.722/G.711.1 SWB/Stereo extension Optimisation/Characterization phase, V0.5, March 2011

- TD 378 (GEN/16), Optimisation/characterization Quality Assessment Test Plan for the Superwideband stereo/mono extension to ITU-T G.711.1 and G.722, V1.2, March 2011

- ITU-T P.811 Rec. (01/2019), Subjective test methodology for evaluating Speech oriented stereo communication systems over headphones