3GPP TSG-S4 Meeting #107	S4-200247

# Wroclaw, Poland,  20th Jan.  - 24th Jan. 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0007 | rev | - | Current version: | 16.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Provisioning procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 10.1.2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document corrects the baseline M1d provisioning procedure and clarifies the domain model for the M1d resources. Depending on the collaboration scenario, one or more 5GMSA features (like Content Hosting / Ingest, Consumption Reporting, QoE reporting, Dynamic Policy or other Network Assistance Services) are activated and provisioned. The results are communicated (directly or indirectly) to the Media Session Handler. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A domain model for different configuration properties is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to provision different features for 5G Media Streaming. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 5.3	Provisioning Session for Media Streaming

### 5.3.1	Domain Model

The M1d baseline domain model is depicted in Figure 5.3.1-1. It consists of an Application Service Configuration, which contains at least one of the following

- 	An Ingest Session Configuration,

-	A Consumption Reporting Configuration,

-	A Policy Template, or

-	QoE reporting (not depicted).

Each Application Service Configuration is uniquely identified within the 5GMSA system by an Application Service Configuration identifier.

Figure 5.3.1-1: M1d Domain Model

When a certain 5GMSA feature is selected, the 5GMSd AF compiles the resulting service access information so that the 5GMSd Client can access the services via M4d and / or M5d.

### 5.3.2	Baseline provisioning procedure

The present clause describes the baseline procedure to provision the features using the 5GMS System. The 5GMS Application provider has already negotiated SLAs with the 5GMSA System provider.

Figure 5.3-1: High Level Procedure for provisioning the 5GMS System for downlink streaming sessions

Steps

1.	The 5GMSd Application Provider discovers the address (URL) of the 5GMSd AF (M1d) for Session Provisioning. The address maybe be the result of a manual SLA negotiation procedure or a self-on-boarding procedure.

2.	The 5GMSd Application Provider authenticates itself with the system.

3.	The 5GMSd Application Provider creates a provisioning session, providing its 5GMSd Application Provider ID as input. 5GMSd Application Provider queries the capabilities and authorized features.

4. 	The 5GMSd Application Provider provisions one or more 5GMSA features. A set of authorized features are activated, such as QoE Reporting, Consumption Reporting, Dynamic Policy, Network Assistance and Content Hosting (incl. Ingest). When Content Hosting is provided by the 5GMS System, the 5GMS Application provider selects the ingest format.

5.	When content hosting is desired, the 5GMSd AF interacts with the 5GMSd AS to allocate M2d resources and configure the ingest format. Then the 5GMSd AS responds with the M2d address.

6.	The 5GMSd AF compiles the service access information. The Service Access Information contain access details and options such as M5d (Media Session Handling) Addresses for Consumption Reporting, QoE Reporting, Dynamic policy, Network Assistance, etc. When content hosting is provisioned, then also M4d (Media Streaming) information such as the DASH MPD is included.

7.	The 5GMSd AF provides the results to the 5GMSd Application Provider.

- When the 5GMSd Application provider selected the full service access information, then the results are provided in form or addresses and configurations for M2d (Ingest), M5d (Media Session Handling) and M4d (Media Streaming)

- When the 5GMSd Application Provider delegated the service access information handling to the 5GMS System, then a reference to the service access information (e.g. an URL) is provided. The Media Session Handler fetches the full service access information later from the 5GMSd AF.

8.	When Content Hosting is activated, the 5GMSd Application provider can start ingesting content, using the M2d API address.

9.	The 5GMSd Application Provider executes Service Announcement and updates the UEs.

Optional (during lifetime of the provisioning session)

10.	The 5GMSd Application Provider may update the provisioning session.

According Provisioning

11.	The 5GMSd AF may send event related or periodic notifications to the 5GMS Application Provider.

According to Schedule or upon request

12. 	The 5GMSd Application Provider may manually terminate the Provisioning Session (at any time). All associated resources are released. Content may be removed from the 5GMSd AS. The 5GMSd Provider may configure a schedule for Provisioning Session termination.

13.	The 5GMSd AF sends a notification upon Provisioning Session terminations.

The 5GMSd AF may request the creation or reuse of one or more network slices for the distribution of the content of the provisioned session.

If more than one network slices are provisioned for the distribution of the content of a session, the list of allowed S-NSSAIs shall be conveyed to the target UEs (e.g. through URSP or through M5d or M8d).

NOTE: The 5GMSd AS(s) serving the content are only accessible through the DNN(s) used by the network slice(s) provisioned for the distribution of that content.

**** Last Change ****