3GPP TSG-SA4 Meeting #107	S4-200286

# Wroclaw, Poland, 2020-01-20 - 2020-01-24

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0011 | rev | 0 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for Unicast Downlink Streaming with 5GMSd AS interactions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2020-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The 5GMSA architecture enables multiple collaboration models, where the Media Player may interact with multiple instances of the 5GMSd AS, enabling a distributed CDN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds a note clarifying that the Media Player can interact with multiple 5GMSd AS instances. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion about the cardinality of the 5GMSd AS remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

## 5.2	Baseline procedure for Unicast Media Downlink Streaming Session establishment

### 5.2.1	General

This clause describes the baseline procedure for establishment of a unicast media downlink streaming session. This baseline procedure assumes that the 5GMSd AF and the 5GMSd AS resides in the external DN and that there are no interactions between the 5GMSd AF and the 5G System.

### 5.2.2	Progressive Download of On-Demand Content

This baseline procedure describes the establishment of a unicast downlink streaming session. A streaming session may use 3GP File Format (Progressive Download), 3GP Time Text or other (potentially non-3GPP defined) formats.

Figure 5.2-1: High Level Procedure for progressive download for on-demand media

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMS Application Provider.

Steps:

1: The 5GMSd Aware Application triggers the Service Announcement and Service and Content Discovery procedure. The service announcement includes either the whole service access information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the service access information.

The Service and Content Discovery procedure only involves the 5GMSd Aware Application and the 5GMSd Application Provider and hence is out of scope of the present document.

2: A Media Player Entry is selected.

3: The 5GMSd Aware Application triggers the Media Session handler to start the playback. The media player entry is provided to the Media Session handler.

4: When the 5GMS Aware Application has received a reference to the service access information (see step 1), the Media Session Handler interacts with the Media AF to acquire the whole service access information.

5: The Media Session Handler triggers the 5GMS Player to start the session.

6: The Media Player establishes the transport session.

7: The Media Player sends the request for the progressive download content.

8: The Media Player receives the initialization information of the progressive download content. The initialization information contains configuration parameters for reception of the media, and optionally also DRM information.

9: The Media Player configures the rendering pipeline for media playback.

10: The Media Player notifies the media session handler, providing the transport session information and some media content related information.

11: Optional: Media Player acquires a DRM License from an External AS.

12: The Media Player receives media content and puts it into the rendering pipeline.

13: The Media Player continuously receives and plays back the media content.

### 5.2.3	DASH Streaming

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.2-2: High Level Procedure for DASH content

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMS Application Provider.

Steps:

1: The 5GMSd Aware Application triggers the Service Announcement and Service and Content Discovery procedure. The Service and Content Discovery procedure only involves the App and the external Application Server. The service announcement includes either the whole service access information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the service access information.

2: A media content item is selected.

3: The 5GMSd Aware Application triggers the 5GMSd client to start media playback. The Media Player Entry is provided to the 5GMSd Client.

4: When the 5GMS Aware Application has received a reference to the service access information (See Step 1), the Media Session Handler interacts with the 5GMSd AF to acquire the whole service access information.

5: In parallel, the Media Player is invoked to start media access and playback.

6: The Media Player establishes the transport session for acquiring the MPD referenced by the Media Player Entry.

7: The Media Player requests the MPD

8: The media access client receives the MPD

9: The Media Player processes the MPD. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the MPD information to initialize the media pipelines for each media stream. The MPD should also contain information to initialize the DRM client, when DRM is used.

10: The Media Player notifies the media session handler about the MPD. The notification may contain parameters from the MPD

11: Optional: the Media Player acquires the necessary DRM information, for example a DRM License.

12: The Media Player configures the media playback pipeline.

13: The Media Player establishes the necessary transport sessions for the content. For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

14: The Media Player notifies the media session handler is notified that it is ready to playback and optionally provides transport session parameters. .

15: The Media Player requests initialization information. The Media Player repeats this step for each required initialization segment.

16: The Media Player receives the initialization information.

17: The Media Player requests media segments according to the MPD.

18: the Media Player receives media segments and puts the information into the according media rendering pipeline.

19: Previous steps are repeated according to the MPD information.

NOTE: The Media Player can interact with multiple instances of the 5GMSd AS. The Media Player can be made aware of the 5GMSd AS instances via the manifest.

*** End of first change ***