3GPP TSG-SA4 Meeting #107 	S4-200305

# Wroclaw, Poland, 20 January 2020 - 24 January 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.132 | CR | 0102 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alternative noise field simulation method for terminal testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HEAD acoustics GmbH, Sony Mobile Communications, Qualcomm Incorporated, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 24 January 2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Results of the round robin test conducted in FS_ANTeM indicated that the alternative noise field simulation provides equivalent results to the currently specified method, while having a more efficient equalization procedure and less variance across labs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of test setup description, reference to TR 26.921 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The advantages of the alternative noise field simulation is not available for testing purposes and higher variance across labs may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The work item code ANTeM will be added to or replace TEI16 after the approval of the corresponding WID by SA#87 plenary. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- ------------------------- START OF CHANGE 1 -------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[…]

[52]	3GPP TR 26.921: "Investigations on ambient noise reproduction systems for acoustic testing of terminals".

- ------------------------- END OF CHANGE 1 -------------------------

- ------------------------- START OF CHANGE 2 -------------------------

5.1.5.1	Handset

The setup for simulating realistic ambient noises and the positioning of the HATS in a lab-type environment is described in ETSI ES 202 396-1 [35].

ETSI ES 202 396-1 [35] contains a description of the recording arrangement for realistic ambient noises, a description of the setup for a loudspeaker arrangement suitable to simulate an ambient noise field in a lab-type environment and a database of realistic ambient noises, part of which is used for testing the terminal performance with a variety of conditions.

Two different methods (Method 1 and Method 2) for equalization and calibration of the loudspeakers are allowed and specified below. The method selected for the test setup (Method 1 or Method 2) shall be reported. Method 2 is preferred over Method 1 due to the improved cross-lab reproduction accuracy reported in clause 5 of TR 26.921 [52].

Method 1:

The equalization and calibration procedure for this test setup given in ETSI ES 202 396-1 [35] shall be used.

This method works semi-automated and requires manual adjustment steps during the equalization procedure. The equalization is carried out in the frequency range between 50 Hz and 10 kHz.

Method 2:

The equalization and calibration procedure for this test setup given in Clause 7 of ETSI TS 103 224 [43] shall be used. During the equalization procedure, a so-called “reference sound” is needed for verification purposes. In this case, the noise type “TrainStation_bin” of Clause 8 of ETSI TS 103 224 [43] (time range 15.0 to 22.5s) shall be used.

NOTE:	Clause 7 of ETSI TS 103 224 [43] describes recording and reproduction arrangement for an (almost) arbitrary number of microphones and loudspeakers. In order to produce ambient noise field compatible to Method 1, a flexible setup with two recording/playback channels (binaural noise signals) and a 4.0 (or 4.1) loudspeaker configuration is used.

In contrast to the Method 1 according to ETSI ES 202 396-1 [35], a fully automated equalization procedure including reduction of cross-talk is provided by Method 2. The equalization is carried out in the frequency range between 50 Hz and 20 kHz.

- ------------------------- END OF CHANGES -------------------------