3GPP TSG-SA4 Meeting #108-e	S4-200577

# 2-9 April, 2020, Electronic Meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | 1.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API for Service Access information aquisition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 26.501 defines the Service Access information concept in Clause 4.2.3 and the usage in clause 5.1 (e.g. step 6). This pCR introduces the data model and the resource for fetching the service access information. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.x (new), 5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.6.x.	Procedures for aquiring Service Access Information

This procedure shall be used by the Media Session Handling to aquire the Service Access Information from the 5GMSd AF. The Service Access Information is the set of parameters and addresses, which are needed by the 5GMSd Client to activate the reception of a downlink streaming session.

The downlink streaming session, for which the Media Session Handler is requesting data, is identified by a unique reference, which is contained in the path of the URL.

**** Next Change ****

## 5.x	Aquiring Service Access Information

## 5.x.1	General

### 5.x.2	Data model

#### 5.x.2.1	Structured data types

##### 5.x.2.1.1	Type: ServiceAccessInformation

Table 5.x.2.1.2-1: Definition of ServiceAccessInformation Data Type

| Property name | Data type | Cardinality | Description | Applicability (NOTE) |
| --- | --- | --- | --- | --- |
| provisioningSessionId | string | 1..1 | Unique identification of the M1d Provisioning Session. |  |
| StreamingAccess | object | 0..1 |  |  |
| mediaPlayerEntry | url | 1..1 | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |  |
| URL signing information |  |  | Editor’s Note: For further study |  |
| ConsumptionReporting | object | 0..1 |  |  |
| reportInterval | integer | 1..1 | Identify the interval when the consumption report messages are sent by the 5GMS Media Session Handler. |  |
| serverAddress | [url] | 1..n | A list of 5GMSd AF addresses where the consumption reporting messages are sent by the 5GMS Media Session Handler. |  |
| locationReporting | Boolean | 1..1 | Identify whether the 5GMS Media Session Handler provides the location data to the Media AF (in case of MNO or trusted third parties) |  |
| DynamicPolicyInvocation | Object | 0..1 |  |  |
| serverAddress | [url] | 1..n | A list of 5GMSd AF addresses which offer the APIs for dynamic policy invocation sent by the 5GMS Media Session Handler. . (Opaque URL, following the 5GMSA URL format) |  |
| policyTemplateId | [String] | 1..n | A list of Policy Template Ids, which the 5GMSd Client is authorized to use. |  |
| Metrics | Object | 0..1 |  |  |
| serverAddress | [url] | 1..n | A list of 5GMSd AF addresses to which metric reports shall be sent. |  |
| dNN | String |  | The DNN which shall be used when sending metrics report. If not specified, the default DN shall be used. |  |
| reportingInterval | Integer |  | The sending interval between metrics reports. If not specified, a single final report shall be sent after the streaming session has ended. |  |
| samplePercentage | Float |  | The percentage of streaming sessions that shall report metrics. If not specified, reports shall be sent for all sessions |  |
| streamingSourceFilter | [string] |  | A list of URL patterns for which metrics reporting shall be done. If not specified, reporting shall be done for all sessions. |  |
| metrics | [string] |  | A list of metrics which shall be reported. |  |

### 5.x.3 Resources

The aquiring Service Access Information API is accessible through this URL paths:

{apiRoot}/3gpp-m5d/v1/serviceAccessInformation/{provisioning_session_id}

The following operations and the corresponding HTTP methods are supported. In each case, the sub-resource path specified in the second column shall be substituted into {provisioning_session_id } in the above URI template:

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Fetch Service Access Information |  | GET | This operation is used to aquire a Service Access Information. |

### 5.x.4 Operations

This clause defines the behaviour that is expected from the 5GMSd AF when a Service Access Information is aquired from the Media Session Handler. The main operation that is performed is to look-up or compile the Service Access Information.

**** Last Change ****