3GPP TSG-SA4 Meeting #90-2	S4-200746

# E-Meeting,  <Start_Date> - <End_Date>

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR |  | rev |  | Current version: | 1.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF-based Network Assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 18-05-2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This pCR proposes the REST API for AF-based network assistance. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 11.6 Network Assistance

### 11.6.1	General

The Network Assistance (NA) feature enables a UE that is receiving a downlink media stream or sending an uplink stream to improve the QoE of the media streaming session through the usage of bitrate requests and recommendations.

Bitrate recommendations allow the network to inform the UE about a recommended bitrate that can be used for a specific media streaming session.

Bitrate requests allow the 5GMS client to request a certain QoS level for the media streaming session either at the start of the session or during the lifetime of that session. The request maybe based on the bitrate recommendations that are provided by the network.

Network assistance may be offered to the UE in one of two ways:

- Based on interaction between the UE and the 5GMSd AF, with a subsequent interaction between the AF and the PCF;

- Based on interaction between the UE and the RAN, re-using the ANBR RAN-layer signalling.

### 11.6.2	AF-Based Network Assistance API

### 11.6.2.1	General

The 5GMS AF shall offer bitrate recommendations and bitrate request API based on existing policy templates that match the filtering criteria for a media streaming session and through the usage of the Npcf_PolicyAuthorization API either over N5 or N33 interfaces to the PCF.

When serving a media streaming session that belongs to the AF application session context, the AF shall subscribe to the following notifications with the PCF:

- Service Data Flow QoS notification control

- Service Data Flow Deactivation

- resources allocation outcome

If no corresponding AF application session context already exists, the AF shall use the Npcf_PolicyAuthorization_Create method with the appropriate service information to create and provision an application session context. The information in the AppSessionContextReqData shall be derived from the policy template.

When requesting QoS provisioning for a media streaming session, the 5GMS AF shall use the configured operation points in the corresponding policy template to determine the list of the QoS references within the “altSerReqs”. The lowest priority index shall be assigned to the operation points with the lowest QoS requirement and the highest priority shall be assigned to the requested operation point by the UE (if the UE is allowed to use that operation point).

Media streaming sessions shall use exactly one component per session. It is assumed that a signle sub-component is used, unless otherwise indicated.

### 11.6.2.2	Data Model

### 11.6.2.2.1	StreamingSessionContext resource

The data model for the StreamingSessionContext resource is specified in the following table:

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| applicaitonContext | Object | 1 | This property holds the information to identify the application context to which this media streaming session belongs. |
| aspId | Object | 0..1 | See definition in 5.6.2.2 of 29.514. |
| dnn | Object | 0..1 | See definition in 5.6.2.2 of 29.514. |
| sliceInfo | Object | 0..1 | See definition in 5.6.2.2 of 29.514. |
| ipDomain | Object | 0..1 | See definition in 5.6.2.2 of 29.514. |
| ueIpv4 | Object | 0..1 | See definition in 5.6.2.2 of 29.514. |
| ueIpv6 | Object | 0..1 | See definition in 5.6.2.2 of 29.514. |
| allowedOperationPoints | [Object] | 1 | A read-only array of operation points that this individual streaming session is allowed to use. Each operation point contains a description of the corresponding downlink and uplink QoS parameters. |
| recommendedOperationPoint | Object | 0..1 | If available, this attribute will contain the currently recommended operation point based on the QoS provisioning information with the PCF for that individual streaming session. This attribute is read-only. |
| requestedOperationPointId | integer | 0..1 | Indicates the latest requested operation point identifier for the associated media streaming session |
| requestedOperationPoint | Object | 0..1 | is used to request a custom operation point for which the AF does not have a pre-assigned operation point id (e.g. with a provisioned policy template). |
| subscriptionURL | string | 0..1 | If available, this property will hold a URL for the session handler to use to subscribe to events notified by the AF and relating to the streaming session. |

### 11.6.2.2.2	Operations

#### Create a StreamingSessionContext

This procedure is used by the media session handler to create a streaming session context with the 5GMS AF. If a matching policy template is provisioned, then the AF will apply policies configured by the policy template. Otherwise, the session is assumed to be a best effort session. The AF uses the information in the applicationContext to match and authorize the UE to use a provisioned policy template.

If the procedure is successful, the 5GMSd AF shall generate a resource id representing the new streaming session context resource.

#### Read the StreamingSessionConext Properties

This procedure is used by the media session handler to read the current status of the streaming session context resource. In particular, it is used to query the currently recommended operation point information.

#### Request an Operation Point

This procedure is used by the media session handler to request the AF to apply a specific operation point, identified through its operation point identifier, or a custom operation point with indicated QoS paramters.

If the request is accepted and the media session handler has subscribed for notifications, the media session handler will be notified about the outcome of the request. The media session handler may query the streaming session context resource to find the latest recommended operation point.

#### Subscribe to Notifications

The media session handler uses this procedure to detect the URL to use to subscribe/unsubscribe for events related to this streaming session context.