3GPP TSG-WG SA4 #90-e	S4-200751

# 20th May – 3rd  June 2020

| CR-For v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR |  | rev | 0 | Current version: | 1.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Dynamic Policy Configuration Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The external application provider configures a set of dyamic policy templates that may be applied to the streaming sessions offered by it. It defines these as a set of operation points for the service, each of which with corresponding QoS requirements and charging configurations. These policies are also shared with the Session Handler on the UE to assist the client with the selection of the proper operation point. This CR defines the Policy Template resource and the operations that are performed over the M1d/M1u. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR defines the policy template configuration API that is offered for the provisioning of downlink and uplink streaming session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 solution will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# First Change

### 4.2.4	Policy Template Configuration procedures

### 4.2.4.1	General

These procedures are used by the 5GMS Application Provider to configure the Policy Templates for streaming sessions of a particular Provisioning Session.

Since Policy Templates require 5GMSd System operator verification, a Policy Template that is newly created cannot be used immediately. Upon creation, a Policy Template shall be in the develop state. It transitions into the state verfication pending when the policy template is activated by the application provider. Once it has been validated by the 5GMS System operator, it shall take the ready state, indicating that it may be applied to streaming sessions. Finally, a Policy Template may be suspended by the 5GMS System operator, e.g. in case of a violation of the usage terms or for some other reasons, which renders it unusable. The 5GMSd/5GMSu AF shall verify the status of a Policy Template prior to using it.

Figure 4.2.4.1-1 below is a state diagram showing the life-cycle of a Policy Template:

Figure 4.2.4.1-1: Policy Template State Diagram

A media streaming session may be offered in more than one Operation Point. An Operation Point identifies a level of QoS that is required to consume that service at that operation point. Each operation point may be associated with a separate charging policy as well.

### 4.2.4.2	Create Policy Template

This procedure is used by the 5GMS Application Provider to create a new Policy Template. The HTTP POST method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall generate a resource identifier to uniquely identify the newly created Policy Template. In that case, it shall respond with a 201 (Created) HTTP response message and provide the URL to the newly created resource in the Location header field.

The default state of a Policy Template is “develop”.

### 4.2.4.3	Read Policy Template

This procedure is used by the 5GMS Application Provider and other 5GMSd/5GMSu AFs to query the properties of an existing Policy Template resource from the 5GMSd/5GMSu AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response that includes the Policy Template in the response message body.

### 4.2.4.4	Update Policy Template

The update operation is invoked by the 5GMS Application Provider to modify the properties of an existing Policy Template. All available properties except state may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

Any update to the policy template will change the state temporarily into develop state, which makes it temporarily unusable. This is decided by the 5GMS System operator. In such case, media streaming sessions that use that policy template shall not be affected by the updates, i.e. no interruptions to existing streamings sessions shall be incurred.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message that includes the Policy Template in the response message body. Modifications to read-only properties, such as changes to the state of a policy template, shall be rejected with a 403 (Forbidden) HTTP response.

### 4.2.4.5	Delete Policy Template

This operation is used by the 5GMS Application Provider to destroy a Policy Template resource. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd/5GMSu AF will remove the Policy Template from any Provisioning Sessions that reference it.

Currently active streaming sessions using the destroyed Policy Template, if any exists, shall not be affected by the removal of the Policy Template.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message.

# Second Change

### 5.5	Policy Template API

### 5.5.1 Overview

The policy template API over M1 allows the application provider to configure policy templates that can then be applied to the media streaming sessions that belong to that application provider. A Policy Template is used to identify in-scope traffic flows, and to specify the traffic shaping and charging policies to be applied to them. A new media streaming session is detected by the Media Session Handler, which in turn will intiate a Network Assistance session with the 5GMSd AF over the M5d interface.

A Policy Template provides a description of a set of alternative levels at which the service can be consumed, so called Operation Points. Each Operation Point has its respective QoS and charging parameters and requirements. For example, a service may offer two different operation points, an HD operation point and a 4K operation point, each of which with different QoS and charging policies. The Policy Template is configured as part of the Provisioning procedures with the 5GMS AF and is then used by the 5GMS AF to request specific QoS and charging policies for that session from the PCF.

### 5.5.2	Data model

The Policy Template resource is defined according Table 5.5.2-1 below:

Table 5.5.2-1: Definition of PolicyTemplate resource

| Property | Type | Cardinality | Description |
| --- | --- | --- | --- |
| policyTemplateId | Integer | 1..1 | Unique identifier of this Policy Template within the scope of the Provisioning Session. |
| state | Enumeration of Strings | 1..1 | A Policy Template may be in the develop, ready, verification pending, or suspended state. Only a Policy Template in the ready state may be applied to streaming sessions. |
| OperationPoints | [Object] | 1..n |  |
| operationPointId | Integer | 1..1 | Unique identifier for the current Operation Point with the policy template. The operationPointId may be used over M5 to indicate which desired operation point will be used by the session. |
| QoSSpecification | Object | 0..1 | Specifies the network quality of service to be applied to streaming sessions at this Operation Point. |
| marBwUl | BitRate | 0..1 | As defined in 8.6.2.7 of TS 29.514. |
| marBwDl | BitRate | 0..1 | As defined in 8.6.2.7 of TS 29.514. |
| maxPacketLossRateDl | PacketLossRateRm | 0..1 | As defined in 8.6.2.7 of TS 29.514. |
| maxPacketLossRateUl | PacketLossRateRm | 0..1 | As defined in 8.6.2.7 of TS 29.514. |
| maxSuppBwDl | BitRate | 0..1 | As defined in 8.6.2.7 of TS 29.514. |
| maxSuppBwUl | BitRate | 0..1 | As defined in 8.6.2.7 of TS 29.514. |
| minDesBwDl | BitRate | 0..1 | As defined in 8.6.2.7 of TS 29.514. |
| minDesBwUl | BitRate | 0..1 | As defined in 8.6.2.7 of TS 29.514. |
| mirBwUl | BitRate | 0..1 | As defined in 8.6.2.7 of TS 29.514. |
| mirBwDl | BitRate | 0..1 | As defined in 8.6.2.7 of TS 29.514. |
| ApplicationSessionContext | Object | 1..1 | Specifies information about the application session context to which this policy template can be applied. |
| afAppId | Object | 0..1 | As defined in section 5.6.2.3 of TS 29.514. |
| sliceInfo | Object | 0..1 | As defined in section 5.6.2.3 of TS 29.514. |
| dnn | Object | 0..1 | As defined in section 5.6.2.3 of TS 29.514. |
| aspId | Object | 0..1 | As defined in section 5.6.2.3 of TS 29.514. |
| Charging | Object | 0..1 | Provides information about the charging policy to be used for this Operation Point. |
| sponId | Object | 0..1 | As defined in section 5.6.2.3 of TS 29.514. |
| sponStatus | Object | 0..1 | As defined in section 5.6.2.3 of TS 29.514. |
| gpsi | [Object] | 0..1 | List of UEs that are allowed to use this operation point. |

### 5.5.3	Resource structure

The Policy Template API is accessible through this URL paths:

{apiRoot}/3gpp-m1d/v1/provisioning/{provisioning-subresource}

The following operations and the corresponding HTTP methods are supported. In each case, the sub-resource path specified in the second column shall be substituted into {provisioning-subresource} in the above URI template:

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Policy Template | policy-templates | POST | This is used to create a new Policy Template resource. |
| Fetch a Policy Template | policy-templates/{policy-template-id} | GET | This operation is used to retrieve an existing Policy Template resource. |
| Update a Policy Template | policy-templates/{policy-template-id} | PUT, PATCH | This operation is used to modify the configuration of an existing Policy Template. |
| Delete a Policy Template | policy-templates/{policy-template-id} | DELETE | This operation is used to delete an existing Policy Template resource. |

# Third Change

# Annex X:	AF QoS Mapping

### X.1	Overview

This annex defines the mapping from media streaming description into Policy Template properties for different types of media streaming descriptions.

### X.2	DASH mapping

If the service is distributed using DASH, the 5GMS Application Provider may use the ServiceDescription element in the MPD to convey the operation points and their corresponding identifiers. For this purpose, a dedicated scope is identified as follows:

- @schemeIdUri: “urn:org:3gpp:5g:dash:policy:op”.

- @value: indicates the operation point identifier that corresponds to this ServiceDescription. The value shall be a number between 0 and 255.

The ServiceDescription shall include at least one OperatingBandwidth child element and may contain a Latency child element.