3GPP TSG SA WG4#109-e meeting	S4-200757

20th May – 3rd June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev | - | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Announcement API for enTV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In enTV for ROM services, service announcement may be configured through a TV Service configuration. There is exists a way to do this through OMA DM according to TS 24.117, but OMA DM may not be always be available. If a service provider wants to provide TV Service Configuration information to the MBMS client through an app API, this would be lacking In Rel-16 for MCData, the addSA functionality was added, but only for file delivery. It is believed that a generic way for any API should be available to initialize the MBMS client with Service information or to update SA information through the app. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide the add SA also to any other service scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Add SA would not be supported and service configuration cannot be done by the app. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.22, 6.2.3.23, 6.3.3.1, 6.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

#### 6.2.3.22	Add SA file

##### 6.2.3.22.1	Overview

This clause defines addSA() interface.

An MAA calls the addSA() interface to provide SA file to the MBMS Client.

Figure 6.2.3.22.1-1 shows a call flow and the usage of the addSA() interface for the file delivery service.

Figure 6.2.3.22.1-1: Add SA file sequence diagram

##### 6.2.3.22.2	Parameters

The parameter for the addSA() API is:

-	string saFileLocation – identifies the location where the MBMS Client can find the SA file, following the profile 1c (Annex L.3 of 3GPP 26.346 [2]).

-	string mimeType – identifies the mimeType and profile of the SA file. If not present, the SA file shall conform to the profile 1c (Annex L.3 of 3GPP 26.346 [2]).

##### 6.2.3.22.3	Pre-Conditions

The application is registered with the MBMS client to consume Application Services.

##### 6.2.3.22.4	Usage of Method for Application

The application uses the method addSA() to provide additionnal service announcement information to the MBMS Client.

##### 6.2.3.22.5	Expected MBMS Client Actions

When this method is received, the MBMS client reads the SA file from the location given by the saFileLocation parameter and parses it.

The MBMS Client

-	adds the new MBMS user services announced by the SA file to the internal _service[] list for the invoking MAA;

-	updates the MBMS user services announced by the SA file whose service ID is already included with the _service[] list for the invoking MAA.

Note:	the internal _service[] lists for the other MAA are unchanged.

##### 6.2.3.22.6	Post-Conditions

The application expects a addSAResponse() as defined in clause 6.2.3.23.

=====  CHANGE  =====

#### 6.2.3.23	Add SA file Response

##### 6.2.3.23.1	Overview

This subclause defines the addSAResponse() call.

As illustrated in Figure 6.2.3.22.1-1, the MBMS client responds to an MAA call to the addSA() API with a addSAResponse() call back providing the result of addition of the SA file.

##### 6.2.3.23.2	Parameters

The parameters for the addSAResponse() API are:

-	EmbmsCommonTypes::AddSAresponseCode value – provides a result code on the registration request. The allowed values are:

-	SUCCESS – indicates that the SA file has been parsed and the announced MBMS user service have been added to the internal _service[] list.

-	SA_FILE_INVALID – indicates that the SA file can not be parsed according the announced profile

-	string message – provides an associated text description of the error message. The message may be empty.

##### 6.2.3.23.3	Pre-Conditions

The MBMS client has received a call via the addSA() API with the parameters documented in subclause 6.2.3.23.2.

##### 6.2.3.23.4	Usage of Method for Application

If the MBMS-aware application receives such a notification with the response code set to SA_FILE_INVALID, the application may inform the application user or the content provider that the SA file could not be imported by the MBMS client.

##### 6.2.3.23.5	Expected MBMS Client Actions

The parameters of the MBMS client are updated. The MBMS client may issue an fdServiceListUpdate() notification as defined in subclause 6.2.3.17 or any other service list update depending on the application.

##### 6.2.3.23.6	Post-Conditions

There is no change in the client state.

=====  CHANGE  =====

#### 6.3.3.1	Overview

Table 6.3.3.1-1 provides an overview over the methods defined for the Streaming Delivery Application Service API. Different types are differentiated, namely state changes triggered by the MAA, status query of the MAA to the client, parameter updates as well as notifications from the client. The direction of the main communication flow between MAA (A) and MBMS Client (C) is provided.

Table 6.3.3.1-1: Methods defined for Streaming Delivery Application Service API

| Method | Type | Direction | Brief Description | Section |
| --- | --- | --- | --- | --- |
| registerStreamingApp | State change | A -> C | MAA registers a callback listener with the MBMS client | 6.3.3.2 |
| deregisterStreamingApp | State change | A -> C | MAA deregisters with the MBMS client | 6.3.3.10 |
| startStreamingService | State change | A -> C | Starts streaming service | 6.3.3.7 |
| stopStreamingService | State change | A -> C | Stop streaming service | 6.3.3.9 |
| getStreamingServices | Status query | C <-> A | Get list of currently active services | 6.3.3.4 |
| getVersion | Status query | C <-> A | Retrieves the list of files previously captured for the MAA | 6.3.3.13 |
| setStreamingServiceClassFilter | Update to parameter list | A -> C | MAA sets a filter on file delivery services in which it is interested | 6.3.3.5 |
| registerStreamingResponse | Update to parameter list | C-> A | The response to the MAA streaming service register API | 6.3.3.3 |
| serviceStarted | Notification | C -> A | Notification to MAA when the streaming service started. | 6.3.3.8 |
| streamingServiceListUpdate | Notification | C -> A | Notification to MAA on an update of the available for Media streaming delivery services | 6.3.3.6 |
| streamingServiceError | Notification | C -> A | Notification to MAA when there is an error with the download of service | 6.3.3.12 |
| serviceStalled | Notification | C -> A | Notification to MAA that download Media segments failed | 6.3.3.11 |
| addSA | Update to parameter list | A -> C | Adds service announcement to the MBMS client on streaming services | 6.2.3.22 |
| addSAResponse | Notification | C -> A | Call back to addSA() | 6.2.3.23 |

=====  CHANGE  =====

#### 6.4.3.1	Overview

Table 6.4-2 provides an overview over the methods defined for the MBMS Packet Delivery Service API. Different types are provided, namely state changes triggered by the MAA, status query of the MAA to the client, parameter updates as well as notifications from the client. The direction of the main communication flow is provided between the MAA (A) and the MBMS Client (C).

Table 6.4-2: Methods defined for MBMS Packet Delivery Service API

| Method | Type | Direction | Brief Description | Section |
| --- | --- | --- | --- | --- |
| registerPacketApp | State change | A -> C | MAA registers a callback listener with the MBMS client | 6.4.3.2 |
| deregisterPacketApp | State change | A -> C | MAA deregisters with the MBMS client | 6.4.3.10 |
| startPacketService | State change | A -> C | Starts Packet service | 6.4.3.7 |
| stopPacketService | State change | A -> C | Stop Packet service | 6.4.3.9 |
| getPacketServices | Status query | C <-> A | Get list of currently active services | 6.4.3.4 |
| getVersion | Status query | C <-> A | Get the API version | 6.4.3.13 |
| setPacketServiceClassFilter | Update to parameter list | A -> C | MAA sets a filter on MBMS Packet Delivery services in which it is interested | 6.4.3.5 |
| registerPacketResponse | Update to parameter list | C -> A | The response to the MAA Packet service register API | 6.4.3.3 |
| serviceStarted | Notification | C -> A | Notification to MAA when the MBMS Packet Delivery service started | 6.4.3.8 |
| rtpServiceListUpdate | Notification | C -> A | Notification to MAA on an update of the available for MBMS Packet Delivery services | 6.4.3.6 |
| rtpServiceError | Notification | C -> A | Notification to MAA when there is an error with the reception download of service | 6.4.3.12 |
| serviceStalled | Notification | C -> A | Notification to MAA that MBMS Packet Delivery service failed | 6.4.3.11 |
| addSA | Update to parameter list | A -> C | Adds service announcement to the MBMS client on MBMS packet delivery services | 6.2.3.22 |
| addSAResponse | Notification | C -> A | Call back to addSA() | 6.2.3.23 |