3GPP TSG SA WG4#109-e meeting	S4-200759

# 20th May – 3rd June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | - | Current version: | 1.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DASH and CMAF in 5GMSd |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | DASH and CMAF not mentioned, no relation to TS26.511 |  |  |  |  |  |  |  |  |
| Summary of change: |  | First attempt to connect the dots |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | We have control for nothing to stream |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.6., 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[3]		DASH Industry Forum, "Specification of Live Media Ingest", 
https://dashif-documents.azurewebsites.net/Ingest/master/DASH-IF-Ingest.pdf

[4]	3GPP TS 26.247: "Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[5]	Standard ECMA-262, 5.1 Edition, "ECMAScript Language Specification", June 2011.

[6]	IETF RFC 6234: "US Secure Hash Algorithms (SHA and SHA-based HMAC and HKDF)".

[7]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[8]	ITU-T Recommendation X.509 (2005) | ISO/IEC 9594-8:2005: "Information Technology – Open Systems Interconnection – The Directory: Public-key and attribute certificate frameworks".

[9]	IETF RFC 7230: "Hypertext-Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[10]	IETF RFC 4648: "The Base16, Base32, and Base64 Data Encodings".

[11]	IEEE Standard 1003.1, Issue 7: "The Open Group Base Specifications", 2018.
https://pubs.opengroup.org/onlinepubs/9699919799/

[12]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[13]	ISO/IEC 23009-1: "Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats".

=====  CHANGE  =====

## 4.6	Procedures of the M4d (Media Streaming) interface

### 4.6.1	Procedures for DASH Session

This procedure is used by a 5GMSd client establish a DASH session via the M4d interface. In order to establish such a session, the Media AS shall host an MPD and the MPD URL shall be known to the Media Player, in this case a DASH player.

The Media Player receives an MPD URL from the Media Session Handler or the 5GMSd-Aware Application. The Media Player shall send an HTTP GET message including the URI of the MPD resource to the 5GMSd AS to fetch the an MPD. The 5GMSd AS shall respond with a 200 (OK) message that includes the MPD.

Additional procedures for reactions to different HTTP status codes are provided in TS26.247 [X] and ISO/IEC 23009-1 [Y], clause A.7.

Additional procedures for handling partial file responses are provided in TS26.247 [X], clause A.9.

### 4.6.2	Procedures for Progressive Download Session

This procedure is used by a 5GMSd client establish a Progressive Download session via the M5d interface.

Details are ffs.

=====  CHANGE  =====

# 10	Media Streaming (M4) APIs

## 10.1	DASH Distribution

### 10.1.1	General

The DASH Media Streaming API is a RESTful API that allows the Media Player to access media on a 5GMSd AS. The API defines data models, resources and the related procedures for the creation and management of DASH-based media streaming.

In the case of DASH distribution, M4d is relevant for the distribution as shown in Figure 10.1-1. For DASH-based distribution according to TS26.247 [X] and ISO/IEC 23009-1 [Y], two main formats are of relevance:

- The Media Presentation Description (MPD) that terminates in the DASH Access Client. Some information may be proviudedonly partially using xlink.

- The Segment formats that terminate in the Media Playback and Content Decryption Platform.

Other resources may be referenced in the MPD, for example DRM related information.

The Segment formats for DASH Streaming in the context of 5G Media Streaming are defined in TS 26.511 [12] based on the CMAF encapsulation. The DASH access client downloads the Segments from the 5GMd AS bas based on the instructions in the MPD and the instructions from the 5GMSd-aware application through M7d (see clause 13 for details). The interface between the DASH access client and the Media Playback Platform is defined based on the playback requirements in TS 26.511 [12].

Figure 10.1-1 M4d usage for DASH distribution

### 10.1.2	Data model

This clause defines data structures to be used in DASH streaming procedure.

Table 10.1.2.1-1 specifies data types re-used by the DASH Streaming API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the 5GMS DASH streaming API.

Table 10.1.2.1-1: 5GMS DASH Streaming API re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| MPD | ISO/IEC 23009-1 [13] TS 26.247 [X] | Media Presentation Description as defined in ISO/IEC 23009-1 or TS 26.247 |
| Xlink Expansion | ISO/IEC 23009-1 [13] | A part of the MPD provide in an xlink |
| CMAF Addressable Resource | ISO/IEC 23000-19 [13] TS 26.511 [12] | A CMAF Addressable Resource that is referenced in the MPD. |
| DASH Segment | ISO/IEC 23009-1 [13] TS 26.247 [X] | A DASH Segment that is referenced in the MPD. |
| Other DASH Resource | ISO/IEC 23009-1 [13] | Other information that may be included in the MPD, for example DRM related information. |

This specification does not define any resources that are shared across M4d for DASH streaming.

### 10.1.3	Resource structure

All resource URIs of this API shall have the following root:

{apiRoot}/5g-dash/v1/

"apiRoot" is set as described in subclause X. "apiName" shall be set to "5g-dash" and "apiVersion" shall be set to "v1" for the version defined in the present document.

All resource URIs in the subclauses below are defined relative to the above root URI.

The following resources and HTTP methods initiated by the 5GMS Client are supported for this API:

Table 11.4.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Meaning | Reference |
| --- | --- | --- | --- | --- |
| MPD | 5g-dash/v1/{aspId}/session/ | GET | Acquire the MPD. | 10.1.4 |
|  |  | PUT | Forbidden |  |
| Xlink | 5g-dash/v1/{aspId}/session/ | GET | Acquire an xlink part of the MPD | 10.1.5 |
|  |  | POST | Forbidden. |  |
| CMAF Addressable Resource | 5g-dash/v1/{aspId}/session/ | GET | Acquire the CMAF Resource. | 10.1.6 |
|  |  | POST | Forbidden |  |
| DASH Segment | 5g-dash/v1/{aspId}/session/ | GET | Acquire the DASH Segment. | 10.1.7 |
|  |  | POST | Forbidden |  |
| Other DASH Resource | 5g-dash/v1/{aspId}/session/ | GET | Acquire other resource | 10.1.8 |
|  |  | POST | Forbidden |  |

### 10.1.4	MPD

#### 10.1.4.1	Introduction

This method allows the Media Player to acquire the MPD when the DASH Streaming is used for a Downlink Streaming session.

#### 10.1.4.2	Resource Definition

Resource URI: {apiRoot}/5g-dash/v1/{aspId}/session/{sessionId}/acquire

This method shall support the resource URI variables defined in table 10.1.4.2-1.

Table 10.1.4.2: Resource URI variables for resource "MPD Acquisition"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS Downlink Streaming session. |

#### 10.1.4.3	Resource methods

##### 10.1.4.3.1	GET

The GET method allows the Media Player to acquire DASH Media Streaming resources. It is initiated by the Media Player and answered by the 5GMSd AS.

This method shall support request and response data structures, and response codes, as specified in the Table 10.1.4.3.1-1.

Table 10.1.4.3.1-1: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | MPD | 1..1 | 200 OK | A valid MPD shall be returned |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

##### 10.1.4.3.2	PUT

This HTTP method is not supported for these resources.

##### 10.1.4.3	PATCH

This HTTP method is not supported for these resources.

##### 10.1.4.4	POST

This HTTP method is not supported for these resources.

##### 10.1.4.5	DELETE

This HTTP method is not supported for these resources.

### 10.1.5	Xlink Resource

#### 10.1.5.1	Introduction

This method allows the Media Player to acquire parts of an MPD when the DASH Streaming is used for a Downlink Streaming session.

#### 10.1.5.2	Resource Definition

Resource URI: {apiRoot}/5g-dash/v1/{aspId}/session/{sessionId}/acquire

This method shall support the resource URI variables defined in table 10.1.5.2-1.

Table 10.1.4.2: Resource URI variables for resource "MPD Acquisition"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS Downlink Streaming session. |

#### 10.1.5.3	Resource methods

##### 10.1.5.3.1	GET

The GET method allows the Media Player to acquire DASH Media Streaming resources. It is initiated by the Media Player and answered by the 5GMSd AS.

This method shall support request and response data structures, and response codes, as specified in the Table 10.1.5.3.1-1.

Table 10.1.4.3.1-1: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | XLINK Resource | 1..1 | 200 OK | A valid XML snippet shall be returned |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

##### 10.1.5.3.2	PUT

This HTTP method is not supported for these resources.

##### 10.1.5.3	PATCH

This HTTP method is not supported for these resources.

##### 10.1.5.4	POST

This HTTP method is not supported for these resources.

##### 10.1.5.5	DELETE

This HTTP method is not supported for these resources.

### 10.1.6	CMAF Addressable Resource

#### 10.1.6.1	Introduction

This method allows the Media Player to acquire a CMAF Addressable Resource when the DASH Streaming is used for a Downlink Streaming session.

#### 10.1.6.2	Resource Definition

Resource URI: {apiRoot}/5g-dash/v1/{aspId}/session/{sessionId}/acquire

This method shall support the resource URI variables defined in table 10.1.6.2-1.

Table 10.1.6.2-1: Resource URI variables for resource "MPD Acquisition"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS Downlink Streaming session. |

#### 10.1.6.3	Resource methods

##### 10.1.6.3.1	GET

The GET method allows the Media Player to acquire DASH Media Streaming resources. It is initiated by the Media Player and answered by the 5GMSd AS.

This method shall support request and response data structures, and response codes, as specified in the Table 10.1.6.3.1-1.

Table 10.1.6.3.1-1: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | Addressable Resource | 1..1 | 200 OK | A valid CMAF Addressable shall be returned |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

<also specify partial GETs>

##### 10.1.6.3.2	PUT

This HTTP method is not supported for these resources.

##### 10.1.6.3	PATCH

This HTTP method is not supported for these resources.

##### 10.1.6.4	POST

This HTTP method is not supported for these resources.

##### 10.1.6.5	DELETE

This HTTP method is not supported for these resources.

### 10.1.7	DASH Segment

#### 10.1.7.1	Introduction

This method allows the Media Player to acquire a DASH Segment when the DASH Streaming is used for a Downlink Streaming session.

#### 10.1.7.2	Resource Definition

Resource URI: {apiRoot}/5g-dash/v1/{aspId}/session/{sessionId}/acquire

This method shall support the resource URI variables defined in table 10.1.7.2-1.

Table 10.1.7.2-1: Resource URI variables for resource "MPD Acquisition"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS Downlink Streaming session. |

#### 10.1.7.3	Resource methods

##### 10.1.7.3.1	GET

The GET method allows the Media Player to acquire DASH Media Streaming resources. It is initiated by the Media Player and answered by the 5GMSd AS.

This method shall support request and response data structures, and response codes, as specified in the Table 10.1.7.3.1-1.

Table 10.1.7.3.1-1: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | DASH Segment | 1..1 | 200 OK | A DASH Segment shall be returned |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

<also specify partial GETs>

<availability times should be added>

##### 10.1.7.3.2	PUT

This HTTP method is not supported for these resources.

##### 10.1.7.3.3	PATCH

This HTTP method is not supported for these resources.

##### 10.1.7.3.4	POST

This HTTP method is not supported for these resources.

##### 10.1.7.3.5	DELETE

This HTTP method is not supported for these resources.

### 10.1.8	Other DASH Resources

#### 10.1.8.1	Introduction

This method allows the Media Player to acquire a DASH Resource other than defined in clause 10.1.4, 10.1.5, 10.1.6 and 10.1.7 when the DASH Streaming is used for a Downlink Streaming session.

#### 10.1.8.2	Resource Definition

Resource URI: {apiRoot}/5g-dash/v1/{aspId}/session/{sessionId}/acquire

This method shall support the resource URI variables defined in table 10.1.8.2-1.

Table 10.1.8.2-1: Resource URI variables for resource "MPD Acquisition"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122 [x3]. |
| aspId | Identifier of the 5GMS Application Provider. |
| sessionId | Identifier of the 5GMS Downlink Streaming session. |

#### 10.1.8.3	Resource methods

##### 10.1.8.3.1	GET

The GET method allows the Media Player to acquire DASH Media Streaming resources. It is initiated by the Media Player and answered by the 5GMSd AS.

This method shall support request and response data structures, and response codes, as specified in the Table 10.1.8.3.1-1.

Table 10.1.8.3.1-1: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | DASH Segment | 1..1 | 200 OK | Other DASH resource |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 [x3] also apply. |  |  |  |  |

##### 10.1.8.3.2	PUT

This HTTP method is not supported for these resources.

##### 10.1.8.3.3	PATCH

This HTTP method is not supported for these resources.

##### 10.1.8.3.4	POST

This HTTP method is not supported for these resources.

##### 10.1.8.3.5	DELETE

This HTTP method is not supported for these resources.