3GPP TSG SA WG4#109-e meeting	S4-200764

20th May – 3rd June 2020	revision of AHVIC-245

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev | 3 | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Messaging and Social Sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Orange, Tencent (Sony, BBC) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2020-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Until now, only a single scenario was looked at |  |  |  |  |  |  |  |  |
| Summary of change: |  | Messaging and Social Sharing is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An important category with very different properties would be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.3, 4.6, 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Presenter: Thomas Stockhammer (Qualcomm) Discussion: - Paul: not yet time to check this - Gilles: not yet time to check this, but likely to be able to support this - Richard: Generally remain supportive - Iraj: We are also interested in supporting this - Gilles: What type of sequences do you imagine? - Thomas: user generated content, like me skiing - Richard: that is likely good - Thomas: maybe, but it looks more professional than user generated Decision: - Content is agreed. Still waiting for more companies to co-sign AHVIC-245 is agreed. This document adds test sequences. |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

Add at the end

[A]	Nick Zarzycki, "The Complete Guide to Social Media Video Specs in 2018", July 23, 2018, https://blog.hootsuite.com/social-media-video-specs/

[B]		YouTube Help, "Recommended upload encoding settings", https://support.google.com/youtube/answer/1722171?hl=en

[C]	Facebook Help Center, "What are the video format guidelines for live streaming on Facebook?", https://www.facebook.com/help/1534561009906955

[D]	Deepak Kumar, "All about TikTok video size [ Full Guide]-2020", February 2020, https://tiktoktip.com/tiktok-size/

[E]	Snapchat business center, "Single Image or Video Specifications", https://businesshelp.snapchat.com/en-US/article/top-snap-specs

[F]	ISO/IEC 23000-19: "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

[G]	GSMA White Paper, "Chinese operators make major RCS commitment: Whitepaper", April 9, 2020, https://www.gsma.com/futurenetworks/latest-news/china-operators-make-major-rcs-commitment-whitepaper/

[H]	3GPP TS 26.140, "Multimedia Messaging Service (MMS); Media formats and codecs".

[I]	GSMA RCC.71, "RCS Universal Profile Service Definition Document", version 2.4, October 2019.

[J]	GSMA NG.114, "IMS Profile for Voice, Video and Messaging over 5GS", February 2020.

=====  CHANGE  =====

## 4.3	5G Media Streaming

### 4.3.1	Downlink Streaming

Ffs

### 4.3.2	Uplink Streaming

The detailed requirements for uplink streaming for 5G Media Streaming and the codec requirements are defined in 3GPP TS 26.511 [13]. For video encoding the support for HEVC-FullHD-Enc as defined in clause 4.2.2.2 of TS26.511 [13] is required together with the the sender requirements for HEVC-FullHD-Enc Operation Point, i.e. real-time encoding. Encapsulation is based on the Common Media Application Format (CMAF) as defined in ISO/IEC 23000-19 [F].

-

=====  CHANGE  =====

## 4.6	Messaging Services

3GPP TS 26.140 [H] specifies the media types, formats and codecs for the MMS within the 3GPP system. The document extends to codecs for speech, audio, video, still images, bitmap graphics, and other media in general, as well as scene description, multimedia integration and synchronization schemes.

Specifically, for video, the following capabilities are defined

H.264 (AVC) Constrained Baseline Profile (CBP) Level 1.3 is required to be supported.

-	H.264 (AVC) High Profile Level 3.1 with frame_mbs_only_flag=1 is recommended to be supported by MMS clients supporting HDTV video content at a resolution of 1280x720 (720p) with progressive scan at 30 frames per second.

-	H.265 (HEVC) Main Profile, Main Tier, Level 3.1 decoder is recommended to be supported

The specification has not been updated since 2014.

=====  CHANGE  =====

## 6.X	Scenario X: Messaging and Social Sharing

### 6.X.1	Motivation

According to the 2020 Mobile Internet Phenomena Report from Sandvine [9] Video traffic continues to grow worldwide, and the increasing popularity of mobile consumers sharing video has not only caused growth in downstream traffic, but also in upstream traffic as well. Instagram ™ grew in the upstream as more consumers share images and videos. TikTok ™, Snapchat ™ (video), FaceTime ™, and even Facebook ™ Live were all in the top 50 applications worldwide on the upstream that are video-sharing-centric. Messaging applications, especially on the upstream, continue to become a critical part of the mobile experience, replacing old style text messaging, and increasingly are video-based. Four of the top 20 applications on the upstream are messaging apps.

Some typical examples and restrictions in April 2020 are provided in the following:

- WhatApp™ [A]

  - The maximum size of the video that you can share is 16 MB.

  - Various container formats that are supported by include MP4, MKV, AVI, 3GP, and MOV. H.264/AVC video codec and AAC audio codec are needed today.

- YouTube™ [B]

  - H.264/AVC is the recommended codec with the following settings

    - Progressive scan

    - High Profile

    - 2 consecutive B frames

    - Closed GOP. GOP of half the frame rate.

    - CABAC

    - Variable bitrate. No bitrate limit required, though we offer recommended bit rates below for reference

    - Chroma subsampling: 4:2:0

  - Resolution Formats: 360p, 480p, 720p, 1080p, 1440p, 2160p

  - Both SDR and HDR are possible

  - The standard aspect ratio is 16:9

- Facebook Live ™ [C]. To live stream on Facebook™, these video format guidelines are provided:

  - Recommended max bit rate is 4000 Kbps (4 mbps).

  - Max: 1080p (1920x1080) resolution, at 60 frames per second.

  - An I-frame (keyframe) must be sent at least every 2 seconds throughout the stream.

  - H264 encoded video.

- TikTok™ [D], some video restrictions

  - Upload from Android, up to 72 MB at most. upload videos from iOS up to 287.6 MB.

  - Video dimensions should be 1080 x 1920

  - MP4 or MOV file format. Video should be H.264/AVC encoded

- Snapchat™ [A][E], The latest information from 2018

  - Recommended size: 1080 by 1920 pixels (9:16 aspect ratio)

  - Recommended specs: .MP4 or MOV, H.264 encoded, maximum file size 1GB

According to Sandvine's report [9], sharing and uploading content as part of social sharing is predominantly pictures and videos that uploaded directly into a cloud and uploaded to one or many social networks, and then discussed (or shared again) over messaging networks. The relevant quality-of-experience factors include the quality of shared content, the time it takes to upload, the costs associated with the upload and also the processing and battery consumption requirements on the device to prepare the content for upload.

In another activity, three large operators released a 5G messaging white paper [G] to promote enhanced messaging services based on Rich Communication Services (RCS). This also shows the relevance of operator-based messaging services. Generally, uplink resources are even more precious and costly in 5G network operation and hence efficient technologies are vital for mass-scale services. This aspect is also considered by GSMA RCS Universal Profile specification, promoted as the industry standard for RCS Business Messaging, ensuring the telecoms industry remains at the centre of digital communications [J].

### 6.X.2	Description of the Anticipated Application

In the context of 3GPP services, 5G Media Streaming [13] provides the following encoding benchmark capabilities:

-	HEVC-FullHD-Enc: the capability to encode a video signal with

-	up to 133,693,440 luma samples per second, and

-	up to a luma picture size of 2,228,224 samples, and

-	up to 240 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being either 8 or 10 bit,

to a bitstream that is decodable by a decoder that is HEVC-FullHD-Dec capable as defined in clause 4.2.2.1 of TS26.511 and defined as the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 4.1 [3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

Based on the considerations in clause 6.X.1, it is also recommened to take into account the AVC-FullHD-Enc capabilities as defined in TS26.511 [13]:

AVC-FullHD-Enc: the capability to encode a video signal with

-	up to 245,760 macroblocks per second, and

-	up to a frame size of 8,192 macroblocks, and

-	up to 240 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being 8 bit,

to a bitstream that is decodable by a decoder that is AVC-HD-Dec capable as defined in clause 4.2.1.1 of TS26.511 and defined as the capability to decode H.264 (AVC) Progressive High Profile Level 4.0 [2] bitstreams.

Based on future expectations of higher quality uploads, it is also recommened to take into account the HEVC-UHD-Enc capabilities as defined in TS26.511 [13]:

HEVC-UHD-Enc: the capability to encode a video signal with

-	up to 534,773,760 luma samples per second, and

-	up to a luma picture size of 8,912,896 samples, and

-	up to 480 frames per second, and

-	the Chroma format being 4:2:0, and

-	the bit depth being either 8 or 10 bit,

to a bitstream that is decodable by a decoder that is HEVC-UHD-Dec capable as defined in clause 4.2.2.1 of TS26.511 and defined as the capability the capability to decode H.265 (HEVC) Main10 Profile, Main Tier, Level 5.1[3] bitstreams that have general_progressive_source_flag equal to 1, general interlaced_source_flag equal to 0, general_non_packed_constraint_flag equal to 1, and general_frame_only_constraint_flag equal to 1.

The considered scenario is the uploading and uplink streaming into the ISO/BMFF and CMAF container formats. Important aspects that are expected to be considered when evaluating a codec in the context of this:

-	Quality and Coding Efficiency:

-	The ability to compress a video sequence targeting the maximum file size and maintaining high quality.

-	The ability to compress a video stream in real time to the available uplink streaming resources.

- 	Considered settings for encoding:

-	Regular random access at least every 2 seconds, preferably more often

-	No specific encoding latency constraints are applicable

-	Encoding in this scenario is typically done as

-	Real-time encoding for social sharing

-	Offline encoding for messaging

- 	UE-based Encoding

### 6.X.3	Source Format Properties

Table 6.X-1 provides an overview of the different source signal properties for Social Sharing and Messaging. This information is used to select proper test sequences.

Table 6.X-1 Social sharing and source format properties

| Source format properties | Social Sharing |
| --- | --- |
| Spatial resolution | 1920 x 1080 (Permitted distribution formats: 1920 × 1080, 1280 × 720, 854 × 480) |
| Chroma format | Y’CbCr |
| Chroma subsampling | 4:2:0 |
| Picture aspec ratio | 16:9 |
| Frame rates | 25, 30, 50, 60 Hz |
| Bit depth | 8, 10 |
| Colour space formats | BT.709, BT.2020 |
| Transfer characteristics | BT.709, BT.2100 (HDR) |

### 6.X.4	Encoding and Decoding Constraints

Table 6.X-2 provides an overview of encoding and decoding constraints for Social sharing and messaging category using AVC and HEVC codecs. This will support the definition of detailed test conditions.

Table 6.X-2 Encoding and Decoding Configurations for Social sharing and messaging

| Encoding and Decoding Constraints | AVC HD | HEVC HD |
| --- | --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.264/AVC Main Profile Level 4.0 [X] | H.265/HEVC Main-10 Profile Level 4.0 [8] |
| RAP period | 1 and 5 seconds | 1 and 5 seconds |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | B = {0.5, 1, 2, 5} Mbps Capped-VBR (social sharing) and VBR (messaging) Fixed QP | B = {0.5, 1, 2, 5} Mbps Capped-VBR (social sharing) and VBR (messaging) Fixed QP |
| Latency requirements and specific encoding settings | No latency requirements beyond RAP so picture reordering allowed | No latency requirements beyond RAP so picture reordering allowed |
| Encoding complexity context | real-time encoding (social sharing), offline encoding (messaging) | real-time encoding (social sharing), offline encoding (messaging). |
| Required decoding capabilities | H.264/AVC Main Profile Level 4.0 [X] | H.265/HEVC Main-10 Profile Level 4.0 [8] |

### 6.X.5	Performance Metrics

tbd

### 6.X.6	Interoperability Considerations

tbd

### 6.X.7	Test Sequences

Tbd

https://photos.app.goo.gl/6QrmTTMoizVtxAoQA

### 6.X.8	Detailed Test Conditions

tbd

### 6.X.9	External Performance Data

tbd

### 6.X.10	Additional Information

tbd