3GPP TSG-SA4 Meeting #109-e	S4-200820

# Electronic, The Internet,  20.5. - 3.6.2020									revision of S4-AHI986

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GMSd based Network Assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 26.501 defines two methods for providing network assistance, ANBR based and 5GMSd based. This pCR proposes the stage 3 for 5GMSd based network assistance. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.6.7.3	5GMSd AF-based Network Assistance

This procedure is used by the 5GMSd Client to request Network Assistance from the 5GMSd AF.

The 5GMSd Client first creates a Network Assistance Session resource and provides information such as a nominal time period (recommendation window) for which a bit rate recommendation is sought, an optional set of usable bit rate values and optional flow identifications. The information is used by the Network Assistance function to recommend a bit rate to the 5GMSd Client.

In the case of DASH, the 5GMSd Client determines the values for the usable bit rates from the DASH MPD by creating reasonable representation combinations with media components from different adaptation sets.

After the Network Assistance Session resource is provisioned, the 5GMSd Client uses the Network Assistance Session identifier when requesting a bit rate recommendation.

In order to terminate a Network Assistance Session, the 5GMSd Client deletes the Network Assistance session resource.

**** Next Change ****

## 10.x	5GMSd AF-based Network Assistance API

### 10.x.1	Overview

### 10.x.2	Data model

#### 10.x.2.1 	Network Assistance Session resource

The structure of the NetworkAssistanceSession resource is specified in Table 10.x.2.2-1 below:

Table 10.x.2.1-1: Definition of Network Assistance Session resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| windowDuration | Integer | 1..1 | The nominal time period, expressed in seconds. |
| usableBitRates | [Integer] | 0..1 | Set of available bit rates in the downlink media session, e.g. total representation bit rates derived from the DASH MPD. |
| flowDescriptions | [FlowDescription] | 0..1 | Identification of application flows, e.g. 2-tuple (IP addresses) or 5-tuple (IP Addresses, protocol and ports). |

#### 10.x.2.2	Bit Rate Recommendation resource

The structure of the BitRateRecommendation resource is specified in Table 10.x.2.2-1 below:

Table 10.x.2.2-1: Definition of Bit Rate Recommendation resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| bitRateRecommendation | Integer | 1..1 | An estimation of throughput, or the recommendation of a bit rate, for the ensuing nominal time period (i.e. value of windowDuration). The 5GMSd client should not exceed the estimated throughput over the nominal time period. |
| windowDuration | Integer | 1..1 | Nominal time window. |

### 10.x.3	Resource structure

The Network Assistance API is accessible through this URL path:

{apiRoot}/3gpp-m5d-network-assistance/v1/

The following operations and the corresponding HTTP methods are supported:

| Operation | {apiSpecificResourceUriPart} | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Network Assistance Session resource | / | POST | Used to provision a new Network Assistance Session. |
| Fetch a Network Assistance Session resource | {naSessionId} | GET | Fetch the properties of an existing Network Asssistance Session. |
| Update a Network Assistance Session resource | { naSessionId } | PUT, PATCH | Update the properties of an existing Network Assistance Session. |
| Delete a Network Assistance Session resource | { naSessionId } | DELETE | Destroy a Network Assistance Session. |
| Request a bit rate recommendation | { naSessionId }/recommendation | GET | Obtain a bit rate recommendation for the next recommendation window. |

### 10.x.4	Operations

**** Next Change ****