3GPP TSG-SA4 Meeting #109-e	S4-200821

# Electronic, The Internet,  20.5. - 3.6.2020				revision of S4-200574 and AHI987

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | <Spec#> | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | M1d Procedure for Dynamic Policy Provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed is tdoc <>, the usage of N5 and N33 APIs require a certain set of parameters, which should not be discloded to a 5GMSd client. The 5GMSd AF stores parameters which identifies the 5GMSd Application Provider as authorized entity towards the 5G System. Additional parameters with MNO defined values are stored in the policy template, which are needed for the operations (like the QoS reference parameters). In addition, a set of bitrate parameters are needed when activating a QoS bearer via the N5 interface. Similar parameters for N33 are not yet completely identified. It is suggested to enable the 5GMSd client to determine the set of bitrate parameters. According Stage 2 alignment is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Discussion ****

The Policy Templates and the N5 / N33 APIs can be used for different dynamic PCC related use-cases, which are

- Dynamic QoS: The 5GMSd client desires to use a QoS bearer for the application flows.

- Background Data: The 5GMSd client desires to put a VoD asset download flow onto a background data policy

- Conditional Zero Rating (Dynamic Charging): The data of the traffic flow should not be charged (or differently charged), since the bitrate remains below a certain bitrate threshold. (also called Binge-on)

- Dynamic Charging: The data flow should be charged differently.

**** First Change ****

### 4.2.4	Dynamic Policy Configuration procedures

#### 4.2.4.1	General

These procedures are used by the 5GMSd Application Provider and the 5GMSd AF on M1d to configure different policy templates, which are used in combination of the dynamic policy invocation API. They are further elaborated in clause 5.X.

#### 4.2.3.2	Create a new Policy Template resource

This procedure is used by the 5GMSd Application Provider to create a new Policy Template resource. The 5GMSd Application Provider shall use the HTTP POST method for this purpose.

If the procedure is successful, the 5GMSd AF shall generate a resource identifier representing the new Policy Template Configuration. In this case, the 5GMSd AF shall respond with a 201 (Created) HTTP response message, and provide the URL to the newly created resource in the Location header field.

#### 4.2.3.3	Enumerate Policy Templates

This procedure is used by the 5GMSd Application Provider to obtain a list of existing Policy Templates resource identifiers from the 5GMSd AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message that includes the Policy Template resource identifiers in the response message body.

#### 4.2.3.4	Read a Policy Template resource

This procedure is used by the 5GMSd Application Provider to obtain the properties of an existing Policy Template resource from the 5GMSd AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message that includes the Policy Templates in the response message body.

#### 4.2.3.5	Update properties of a Policy Template resource

The update operation is invoked by the 5GMSd Application Provider to modify/overwrite the properties of an existing Policy Templates resource. All available parameters may be updated at once using the HTTP PUT method (overwrite). Selected properties are modified using the HTTP PATCH method.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) and provide the content of the resource in the response, reflecting the successful update operation.

#### 4.2.3.6	Delete a Policy Template resource

This operation is used by the 5GMSd Application Provider to delete a Policy Template resource. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd AF will release any associated resources.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message.

Editor’s Note: Check the HTTP Status Codes.

**** Next Change ****

## 5.2a	Policy Templates Provisioning API

### 5.2a.1	Overview

This clause specifies the API that a 5GMSd Application Provider uses at interface M1d to provision and manage 5GMSd AS Policy Template configurations by interacting with a 5GMSd AF. Each Policy Template configuration is represented by a PolicyTemplate, the data model for which is specified in clause 5.2a.2 below. The RESTful resources for managing Policy Templates are specified in clause 5.2a.3 and the operations on these resources are further elaborated in clause 5.2a.4.

### 5.2a.2	Data model

#### 5.2a.2.1 PolicyTemplate data type

The data model for the PolicyTemplate resource is specified in Table 5.2a.2.2-1 below:

Table 5.2a.2.2-1: Definition of PolicyTemplate resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| policyTemplateId | String | 1..1 | Unique identification (key) of this Policy Template. The Media Session Handler uses the policyTemplateId on M5d, when activating a dynamic policy. |
| api | String | 1..1 | Indicates which API (in form of an URL) should be invoked when activating a dynamic policy based on this Policy Template. N5: Npcf Policy Authorization Service, N33: AsSessionWithQoS or CHargableParty. |
| state | Enum | 1..1 | Defines the provisioning state of the Policy Template. |
| afAppId | AfAppId | 1..1 | See Npcf Policy Authorzation Service. |
| aspId | AspId | 1..1 | See Npcf Policy Authorzation Service. |
| qosReference | String | 0..1 | See Npcf Policy Authorzation Service and AsSessionWithQoS. |
| afChargId | String | 0..1 | See Npcf Policy Authorzation Service. |
| bdtRefId | String | 0..1 | See Npcf Policy Authorzation Service and ApplingBdtPolicy. |
| maximumDownlinkBitRate | BitRate | 0..1 | Defines a maximum authorized downlink bit rate for this Policy Template, |
| maximumUplinkBitRate | BitRate | 0..1 | Defines a maximum authorized uplink bit rate for the policy template |

### 5.2a.3	Resource structure

The policy templates provisioning API is accessible through this URL paths:

{apiRoot}/3gpp-m1d/v1/application-service-providers/{application-Service-Provider-Id}/{provisioningSubresource}

The following operations and the corresponding HTTP methods are supported. In each case, the sub-resource path specified in the second column shall be substituted into {provisioning-subresource} in the above URI template:

| Operation | {apiSpecificResourceUriPart}- | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Policy Template resource | policyTemplates | POST | This is used to create a new Policy Template resource. |
| Enumerate Policy Templates | policyTemplates | GET | This is used to retrieve a list of all existing Policy Templates resource identifiers. |
| Fetch a Policy Template resource | policyTemplates/{policy-template-id} | GET | This is used to retrieve an existing Policy Template resource. |
| Update a Policy Template resource | policyTemplates/{policy-template-id} | PUT, PATCH | This operation is used to modify the properties of an existing Policy Template resource. |
| Delete a Policy Templates resource | policyTemplates/{policy-template-id} | DELETE | This operation is used to delete an existing Policy Template resource. |

### 5.2a.4	Operations

The 5GMSd Application Provider creates a Policy Template resource using the HTTP POST method. The URL for the newly created Policy Template resource is returned within the Location header field of the HTTP response.

The 5GMSd Application Provider enumerates a list of Policy Template resource identifiers using the HTTP GET method. The response contains a list of resource identifiers.

The 5GMSd Application Provider replaces a Policy Template resource using the HTTP PUT method. The request body shall contain the replacement PolicyTemplate object.

The 5GMSd Application Provider modifies a PolicyTemplate objects using the HTTP PATCH method. The request body shall contain the PolicyTemplate object.

The state of a newly created Policy Template is “devel”. The 5GMS Application Provider may modify the Policy Template resources, while in state “devel”.

**** Next Change ****

#### 5.3.2.1	DynamicPolicy resource type

Table 5.3.2.1-1: Definition of Dynamic Policy resource

| Property name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| policyTemplateId | String | 1..1 | Identifies the Policy Template which should be applied to the application flow(s). |
| flowDescription | FlowDescription | 1..1 | Data type specified in TS 29.214 clause 5.3.8. |
| provisioningSessionId | String | 1..1 | Uniquely Identifies 5GMSd Provisioning Session that represents the 5GMSd Application Provider. |
| maximumRequestedDonwlinkBitRate | BitRate | 1..1 | Maximum requested bit rate for the Downlink. |
| maximumRequestedUplinkBitRate | BitRate | 1..1 | Maximum requested bit rate for the Uplink. |
|  |  | 1..1 |  |
|  |  | 1..1 |  |
| minimumDesiredDownlinkBitRate | BitRate | 1..1 | Minimum desired bandwidth for the Downlink. |
| minimumDesiredUplinkBitRate | BitRate | 1..1 | Minimum desired bandwidth for the Uplink. |
| minimumRequestedDownlinkBitRate | BitRate | 1..1 | Minimum requested bandwidth for the Downlink. |
| l | BitRate | 1..1 | Minimum requested bandwidth for the Uplink. |
| enforcementMethod | String | 1..1 | Description of the Policy Enforcement Method. The parameter is set by the 5GMSd AF. |