3GPP TSG-SA WG4 Meeting #109-e	S4-200833

# Online, 20 May – 3 June

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.447 | CR | 0014 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the algorithmic description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVS_Codec |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Following a review of the text, errors have been identified in the main body of the text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of errors in the text and clarification of confusing descriptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies, innaccuracies and possible ambiguities will be present in the text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.2, 5.4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-200833 Initial version |  |  |  |  |  |  |  |  |

#### 5.4.3.2	PLC method selection

In case the last good frame prior to a loss was coded with HQ MDCT a range of different specifically optimized PLC methods is available that are selected based on second level criteria described in this subclause.

#### (…)

Figure 2 shows the block diagram for packet loss concealment of NB signals for the MDCT mode. A frequency-domain approach operates on the frequency domain signal such as the input to the IMDCT block in the figure. A time-domain approach operates on the time domain signal after the IMDCT block. When a frame erasure occurs, the spectral coefficients of the current frame are estimated. To accomplish this using the frequency-domain approach, the synthesized spectral coefficients of the last good frame are repeated for the current frame with signal modification such as a gain scaling and a random sign changing. In the time-domain approach, an additional PLC operation is added to enhance the performance of the frequency-domain approach depending on the input signal characteristics. For this additional operation, the appropriate packet loss concealment tool, either the phase matching tool or the repetition and smoothing tool is selected.

Figure 2: Block diagram for NB PLC for MDCT mode

#### 5.4.3.4	MDCT frame repetition with sign prediction

An analysis of the sign change of the MDCT coefficients in the received frames is continuously performed. The analysis of and is performed on 4-dimensional bands up to 1.6 kHz ( MDCT coefficients divided into  bands).

Two 16-dimensional state variables, used to determine the sign of the reconstructed MDCT vector, and hold the number of sign switches between consecutive frames. The analysis takes also into account signal dynamics (measured by a transient detector), to decide on the reliability of past data. Updates for both state variables are done only for, if the values are set to zero.

Within a sub- band, first state variable is incremented whenever the sign of the corresponding MDCT coefficients switches:

(174)

The second state variable accumulates number of sign switches over consecutive frames:

(175)

When frame is lost, the missing MDCT vector is reconstructed by copying the last available coefficients. The sign of the reconstructed vector can be preserved or changed on a sub-band basis (every 4 coefficients). Inside a band the decision whether to change the sign or not is based on comparing the second state variable to a pre-determined threshold as follows (wherein a sign flip or reversal is indicated by -1 and preservation of the sign is indicated by +1):

fork∈bsignXM'mk=-1ifΔCm-1b≥T+1ifΔCm-1b<T	(176)

The threshold is adjusted to the past decision of the transient detector. The sequential decision logic is illustrated in Table 10.

Table 10: Sign extrapolation decision logic

| - If any of frames or contains transient | Apply random sign to the copied coefficients |
| --- | --- |
| - If frames or are good, but frame contains transient | Apply sign extrapolation with |
| - If frames ,, and are good | Apply sign extrapolation with |

*** End of changes ***