3GPP TSG SA WG4#109-e meeting	S4-200854

20th May – 3rd June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | <CR#> | rev | - | Current version: | 16.5.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing H.263 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-05-22 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | H.263 was a state-of-the art codec in the last millennium and made mobile video possible and an actual reality. Many 3GPP specs adopted H.263 and H.263 was the format of choice for the first mobile video deployments. However, more than 20 years later, this format has done its duty and 3GPP should feel good about sending this codec to retirement as part of their Rel-16 specs. Actually, several specifications already removed any status around H.263 from their specifications, but have some leftover H.263 related statements. Why is it relevant to retire older codecs? Supporting codecs on hardware is a significant amount effort and cost, including area size, design and testing. Even if the codec is supported in SW only (which may well be ok for H.263), it still requires a significant amount of unnecessary and costly testing efforts. Supporting such codecs on newly shipping 5G device will just reduce space for new codecs and technologies to be potentially added. One important reason is, that despite on Android there is SW codec for these formats, there are more and more devices such as watches which which do not use Android and hence would require custom H.263 integration. The same applies for MPEG-4 video codec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove any reference to H.263 and MPEG-4 Video |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary costs for testing and implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 15.1, 15.2, 15.3, 17.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[3]	3GPP TS 26.235: "Packet switched conversational multimedia applications; Default codecs".

[4]	3GPP TS 26.236: "Packet switched conversational multimedia applications; Transport protocols".

[5]	3GPP TR 26.914: "Multimedia telephony over IP Multimedia Subsystem (IMS); Optimization opportunities".

[6]	3GPP TR 22.973: "IMS Multimedia Telephony service; and supplementary services".

[7]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[8]	IETF RFC 4566 (2006): "SDP: Session Description Protocol", M. Handley, V. Jacobson and C. Perkins.

[9]	IETF RFC 3550 (2003): "RTP: A Transport Protocol for Real-Time Applications", H. Schulzrinne, S. Casner, R. Frederick and V. Jacobson.

[10]	IETF RFC 3551 (2003): "RTP Profile for Audio and Video Conferences with Minimal Control", H. Schulzrinne and S. Casner.

[11]	3GPP TS 26.071: "Mandatory Speech Codec speech processing functions; AMR Speech CODEC; General description".

[12]	3GPP TS 26.090: "Mandatory Speech Codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Transcoding functions".

[13]	3GPP TS 26.073: "ANSI C code for the Adaptive Multi Rate (AMR) speech codec".

[14]	3GPP TS 26.104: "ANSI-C code for the floating-point Adaptive Multi Rate (AMR) speech codec".

[15]	3GPP TS 26.093: "Mandatory speech codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Source controlled rate operation".

[16]	3GPP TS 26.103: "Speech codec list for GSM and UMTS".

[17]	3GPP TS 26.171: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; General description".

[18]	3GPP TS 26.190: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Transcoding functions".

[19]	3GPP TS 26.173: "ANCI-C code for the Adaptive Multi Rate - Wideband (AMR-WB) speech codec".

[20]	3GPP TS 26.204: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; ANSI-C code".

[21]	3GPP TS 26.193: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Source controlled rate operation".

[22]	(void).

[23]	(void).

[24]	ITU-T Recommendation H.264 (04/2013): "Advanced video coding for generic audiovisual services".

[25]	IETF RFC 6184 (2011): "RTP Payload Format for H.264 Video", Y.-K. Wang, R. Even, T. Kristensen, R. Jesup.

[26]	ITU-T Recommendation T.140 (02/1998): "Protocol for multimedia application text conversation".

[27]	ITU-T Recommendation T.140 (02/2000): "Protocol for multimedia application text conversation - Addendum 1".

[28]	IETF RFC 4867 (2007): "RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs", J. Sjoberg, M. Westerlund, A. Lakaniemi and Q. Xie.

[29]	(void).

[30]	Void.

[31]	IETF RFC 4103 (2005): "RTP Payload for Text Conversation", G. Hellstrom and P. Jones.

[32]	IETF RFC 3555 (2003): "MIME Type Registration of RTP Payload Formats", S. Casner and P. Hoschka.

[33]	3GPP TR 25.993: "Typical examples of Radio Access Bearers (RABs) and Radio Bearers (RBs) supported by Universal Terrestrial Radio Access (UTRA)".

[34]	3GPP TS 22.105: "Services and service capabilities".

[35]	3GPP TS 26.131: "Terminal acoustic characteristics for telephony; Requirements".

[36]	3GPP TS 26.132: "Speech and video telephony terminal acoustic test specification".

[37]	3GPP TS 28.062: "Inband Tandem Free Operation (TFO) of speech codecs; Service description; Stage 3".

[38]	3GPP TS 23.153: "Out of band transcoder control; Stage 2".

[39]	IETF RFC 0768 (1980): "User Datagram Protocol", J. Postel.

[40]	IETF RFC 4585 (2006): "Extended RTP Profile for Real-time Transport Control Protocol (RTCP) - Based Feedback (RTP/AVPF)", J. Ott, S. Wenger, N. Sato, C. Burmeister and J. Rey.

[41]	RTP Tools: http://www.cs.columbia.edu/IRT/software/rtptools/.

[42]	IETF RFC 3556 (2003): "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth", S. Casner.

[43]	IETF RFC 5104 (2008): "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)", S. Wenger, U. Chandra, M. Westerlund and B. Burman.

[44]	Void.

[45]	3GPP TS 26.111: "Codec for circuit switched multimedia telephony service; Modifications to H.324".

[46]	3GPP TS 23.172: "Technical realization of Circuit Switched (CS) multimedia service; UDI/RDI fallback and service modification; Stage 2".

[47]	3GPP TS 23.002: "Network Architecture".

[48]	IETF RFC 3388 (2002): "Grouping of Media Lines in the Session Description Protocol (SDP)", G. Camarillo, G. Eriksson, J. Holler and H. Schulzrinne.

[49]	IETF RFC 4102 (2005): "Registration of the text/red MIME Sub-Type", P. Jones.

[50]	ITU-T H.248 (06/2000): "Packages for text conversation, fax and call discrimination".

[51]	ETSI EG 202 320, v1.2.1 (2005-10): "Human Factors (HF); Duplex Universal Speech and Text (DUST) communications".

[52]	3GPP TS 26.226: "Cellular text telephone modem; General description".

[53]	IETF RFC 4504 (2006): "SIP Telephony Device Requirements and Configuration", H. Sinnreich, Ed., S. Lass and C. Stredicke.

[54]	ITU-T Recommendation V.151 (05/2006): "Procedures for end-to-end connection of analogue PSTN text telephones over an IP network utilizing text relay".

[55]	ITU-T Recommendation V.152 (09/2010): "Procedures for supporting Voice Band Data over IP networks".

[56]	IETF RFC 3448 (2003): "TCP Friendly Rate Control (TFRC): Protocol Specification", M. Handley, S. Floyd, J. Padhye and J. Widmer.

[57]	3GPP TS 24.173: "IMS Multimedia Telephony Communication Service and Supplementary Services".

[58]	IETF RFC 3264 (2002): "An Offer/Answer Model with the Session Description Protocol (SDP)", J. Rosenberg and H. Schulzrinne.

[59]	3GPP TS 26.141: "IP Multimedia System (IMS) Messaging and Presence; Media formats and codecs".

[60]	3GPP TS 26.234: "Transparent end-to-end Packet-switched Streaming Service; Protocols and codecs".

[61]	IETF RFC 4733 (2006): "RTP Payload for DTMF Digits, Telephony Tones, and Telephony Signals", H. Schulzrinne and T.Taylor.

[62]	3GPP TS 23.014: "Support of Dual Tone Multi-Frequency (DTMF) signalling".

[63]	ETSI ES 201 235-2, v1.2.1: "Specification of Dual Tone Multi-Frequency (DTMF); Transmitters and Receivers; Part 2: Transmitters".

[64]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[65]	3GPP TS 29.163: "Interworking between the IP Multimedia (IM) Core Network (CN) subsystem and Circuit Switched (CS) networks".

[66]	Void.

[67]	OMA-ERELD-DM-V1_2-20070209-A: "Enabler Release Definition for OMA Device Management, Approved Version 1.2".

[68]	Void.

[69]	IETF RFC 5939 (2010): "Session Description Protocol (SDP) Capability Negotiation", F. Andreasen.

[70]	3GPP TS 22.042: "Network Identity and Time Zone (NITZ), Stage 1".

[71]	IETF RFC 1952 (May 1996): "GZIP file format specification version 4.3", P. Deutsch.

[72]	IETF RFC 2326 (1998): "Real Time Streaming Protocol (RTSP)".

[73]	IETF RFC 2616 (June 1999): "Hypertext Transfer Protocol -- HTTP/1.1".

[74]	3GPP TS 26.346 "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[75]	3GPP TS 23.040: "Technical realization of Short Message Service (SMS)".

[76]	IETF RFC 6236 (2011): "Negotiation of Generic Image Attributes in the Session Description Protocol (SDP)", I. Johansson and K. Jung.

[77]	ITU-T G.711 (11/1988): "Pulse code modulation (PCM) of voice frequencies".

[78]	ITU-T G.722 (09/2012): "7 kHz audio-coding within 64 kbit/s".

[79]	IETF RFC 4821 (2007): "Packetization Layer Path MTU Discovery".

[80]	3GPP TS 23.003: "Numbering, addressing and identification".

[81]	IETF RFC 4796 (2007): "The session description protocol (SDP) content attribute", J. Hautakorpi and G. Camarillo.

[82]	3GPP TS 24.247: "Messaging service using the IP Multimedia (IM) Core Network (CN) subsystem".

[83]	IETF RFC 3168 (2001): "The Addition of Explicit Congestion Notification (ECN) to IP", K. Ramakrishnan, S. Floyd and D. Black.

[84]	IETF RFC 6679 (2012): "Explicit Congestion Notification (ECN) for RTP over UDP", M. Westerlund, et. al.

[85]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description".

[86]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for  Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[87]	IETF RFC 5506 (2009):  "Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences".

[88]	IETF RFC 3611 (2003): "RTP Control Protocol Extended Reports (RTCP XR) ", T. Friedman, R. Caceres and A. Clark.

[89]	3GPP TS 25.401: "UTRAN overall description".

[90]	3GPP TS 23.203: "Policy and charging control architecture".

[91]	ITU-T Recommendation T.4 (07/2003): "Standardization of Group 3 facsimile terminals for document transmission".

[92]	ITU-T Recommendation T.30 (09/2005): "Procedures for document facsimile transmission in the general switched telephone network".

[93]	ITU-T Recommendation T.38 (09/2010): "Procedures for real-time Group 3 facsimile communication over IP networks".

[94]	IETF RFC 3362 (2002): "Real-time Facsimile (T.38) - image/t38 MIME Sub-type Registration".

[95]	IETF RFC 5285 (2008): "A General Mechanism for RTP Header Extensions", D. Singer, H. Desineni.

[96]	IETF RFC 5168 (2008): "XML Schema for Media Control", O. Levin, R. Even and P. Hagendorf.

[97]	3GPP2 C.S0055-A, version 1.0: "Packet Switched Video Telephony Service (PSVT/MCS)".

[98]	ETSI TS 181 005, v3.3.1: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Service and Capability Requirements".

[99]	3GPP2 C.S0014-E, version 1.0: "Enhanced Variable Rate Codec (EVRC)".

[100]	ITU-T Recommendation G.729 (06/2012): "Coding of speech at 8 kbit/s using conjugate-structure algebraic-code-excited linear prediction (CS-ACELP)".

[101]	ITU-T Recommendation G.729.1 (05/2006): "G.729-based embedded variable bit-rate coder: An 8-32 kbit/s scalable wideband coder bitstream interoperable with G.729".

[102]	3GPP2 C.S0076, version 1.0: "Discontinuous Transmission (DTX) of Speech in cdma2000 Systems".

[103]	IETF RFC 5188 (2008):"RTP Payload Format for the Enhanced Variable Rate Wideband Codec (EVRC-WB) and the Media Subtype Updates for EVRC-B Codec".

[104]	IETF RFC 4749 (2006): "RTP Payload Format for the G.729.1 Audio Codec".

[105]	IETF RFC 5459 (2009): "G.729.1 RTP Payload Format Update: Discontinuous Transmission (DTX) Support".

[106]	IETF RFC 4788 (2007): "Enhancements to RTP Payload Formats for EVRC Family Codecs".

[107]	IETF RFC 4855 (2007): "Media Type Registration of RTP Payload Formats".

[108]	ITU-T Recommendation P.10 (07/2006): "Vocabulary and effects of transmission parameters on customer opinion of transmission quality".

[109]	ETSI TS 103 737, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband wireless terminals (handset and headset) from a QoS perspective as perceived by the user".

[110]	ETSI TS 103 738, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband wireless terminals (handsfree) from a QoS perspective as perceived by the user".

[111]	ETSI TS 103 739, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband wireless terminals (handset and headset) from a QoS perspective as perceived by the user".

[112]	ETSI TS 103 740, v1.1.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband wireless terminals (handsfree) from a QoS perspective as perceived by the user".

[113]	ETSI TS 202 737, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband VoIP terminals (handset and headset) from a QoS perspective as perceived by the user".

[114]	ETSI TS 202 738, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for narrowband VoIP loudspeaking and handsfree terminals from a QoS perspective as perceived by the user".

[115]	ETSI TS 202 739, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband VoIP terminals (handset and headset) from a QoS perspective as perceived by the user ".

[116]	ETSI TS 202 740, v1.3.2: "Speech and multimedia Transmission Quality (STQ); Transmission requirements for wideband VoIP loudspeaking and handsfree terminals from a QoS perspective as perceived by the user ".

[117]	ETSI EN 300 175-8, v2.5.1: "Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 8: Speech and audio coding and transmission".

[118]	ETSI TS 300 176-2, v2.2.1: "Digital Enhanced Cordless Telecommunications (DECT); Test specification; Part 2: Audio and speech".

[119]	ITU-T Recommendation H.265 (04/2013): "High efficiency video coding".

[120]	IETF RFC 7798 (2016): "RTP Payload Format for High Efficiency Video Coding (HEVC)", Y.-K. Wang, Y. Sanchez, T. Schierl, S. Wenger, M. M. Hannuksela.

[121]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General Overview".

[122]	3GPP TS 26.442: "Codec for Enhanced Voice Services (EVS); ANSI C code (fixed-point)".

[123]	3GPP TS 26.443: "Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)".

[124]	3GPP TS 26.444: "Codec for Enhanced Voice Services (EVS); Test Sequences".

[125]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[126]	3GPP TS 26.446: "Codec for Enhanced Voice Services (EVS); AMR-WB Backward Compatible Functions".

[127]	3GPP TS 26.447: "Codec for Enhanced Voice Services (EVS); Error Concealment of Lost Packets".

[128]	3GPP TS 26.448: "Codec for Enhanced Voice Services (EVS); Jitter Buffer Management".

[129]	3GPP TS 26.449: "Codec for Enhanced Voice Services (EVS); Comfort Noise Generation (CNG) Aspects".

[130]	3GPP TS 26.450: "Codec for Enhanced Voice Services (EVS); Discontinuous Transmission (DTX)".

[131]	3GPP TS 26.451: "Codec for Enhanced Voice Services (EVS); Voice Activity Detection (VAD)".

[132]	3GPP TS 45.003: "Radio Access Network; Channel coding".

[133]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage2".

[134]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage2".

[135]	ITU-T Recommendation H.224 (01/05): "A real time control protocol for simplex applications using the H.221 LSD/HSD/MLP channels ".

[136]	ITU-T Recommendation H.224 (2005): Corrigendum 1 (08/07).

[137]	ITU-T Recommendation H.281 (11/94): Transmission of non-telephone signals "A far end camera control protocol for videoconferences using H.224".

[138]	ITU-T Recommendation H.323 (12/2009): "Packet-based multimedia communications systems".

[139]	IETF RFC 4573 (2006): "MIME Type Registration for RTP Payload Format for H.224".

[140]	IETF RFC 4588 (2006): "RTP Retransmission Payload Format", J. Rey, D. Leon, A. Miyazaki, V. Varsa and R. Hakenberg.

[141]	IETF RFC 8627 (2019): "RTP Payload Format for Flexible Forward Error Correction (FEC)".

[142]	TR 26.922:  "Video Telephony Robustness Improvements Extensions (VTRI_EXT): Performance Evaluation".

[143]	IETF RFC 5956 (2010): "Forward Error Correction Grouping Semantics in the Session Description Protocol", A. Cengiz.

[144]	3GPP TR 26.924: "Multimedia telephony over IP Multimedia Subsystem (IMS); Study on improved end-to-end Quality of Service (QoS) handling for Multimedia Telephony Service for IMS (MTSI)".

[145]	IETF RFC 4796 (2007): "The Session Description Protocol (SDP) Content Attribute".

[146]	Void.

[147]	3GPP TS 24.147: "Conferencing Using IP Multimedia Core Network; Stage 3".

[148]	IETF RFC 4575 (2006): "A Session Initiation Protocol (SIP) Event Package for Conference State".

[149]	IETF RFC 4582 (2006): "The Binary Floor Control Protocol (BFCP)".

[150]	IETF RFC 4583 (2006): "Session Description Protocol (SDP) Format for Binary Floor Control (BFCP) Streams".

[151]	Void.

[152]	3GPP TR 26.980: "Multimedia telephony over IP Multimedia Subsystem (IMS); Media handling aspects of multi-stream multiparty conferencing for Multimedia Telephony Service for IMS (MTSI)".

[153]	IETF RFC 5234 (2008): "Augmented BNF for Syntax Specifications: ABNF", D. Crocker and P. Overell.

[154]	IETF Internet Draft, draft-ietf-mmusic-sdp-simulcast-05 (2016): "Using Simulcast in SDP and RTP Sessions" (WORK IN PROGRESS).

[155]	IETF Internet Draft, draft-ietf-mmusic-rid-07 (2016): "RTP Payload Format Constraints" (WORK IN PROGRESS).

[156]	IETF RFC 7728 (2016): "RTP Stream Pause and Resume".

[157]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[158]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification".

[159]	"Mobile Location Protocol (MLP)", Open Mobile Alliance, OMA-LIF-MLP-V3_1, Approved Version 3.1 – 20 Sep 2011.

[160]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[161]	3GPP TS 27.007: " Technical Specification Group Core Network and Terminals; AT command set for User Equipment (UE)".

[162]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[163]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[164]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[165]	3GPP TS 26.452: "Codec for Enhanced Voice Services (EVS); ANSI C code; Alternative fixed-point using updated basic operators".

[166]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[167]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[168]	3GPP TR 26.952: "Codec for Enhanced Voice Services (EVS); Performance characterization".

[169]	3GPP TR 26.959: "Study on enhanced Voice over LTE (VoLTE) performance".

[170]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[171]	3GPP TS 37.324: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Service Data Adaptation Protocol (SDAP) specification".

[172]	IETF Internet Draft, draft-ietf-mmusic-data-channel-sdpneg-28 (2019): "SDP-based Data Channel Negotiation" (WORK IN PROGRESS)

[173]	IETF RFC 4960 (2007): "Stream Control Transmission Protocol"

[174]	IETF RFC 8261 (2017): "Datagram Transport Layer Security (DTLS) Encapsulation of SCTP Packets"

[175]	IETF Internet Draft, draft-ietf-rtcweb-data-channel-13 (2015): "WebRTC Data Channels" (WORK IN PROGRESS)

[176]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

=====  CHANGE  =====

## 15.1	General

The MTSI client in the terminal may use the OMA-DM solution specified in this clause for enhancing the SDP negotiation and resource reservation process. If a MTSI client in the terminal uses this feature, it is mandatory for the MTSI client in the terminal to implement the Management Object (MO) as described in this clause.

The 3GPP MTSINP (MTSI Network Preference) MO defined in this clause may be used to manage the QoS profile settings which express the network preference for the MTSI client in the terminal. The MO covers parameters that the MTSI client in the terminal could make use of in SDP negotiation and resource reservation process.  If a MTSI client in the terminal supports the feature, the usage of the MO includes:

1.	During SDP negotiation process, MTSI client in the terminal should start SDP negotiation based on the MO parameters.

2.	During resource reservation process, MTSI client in the terminal should start QoS negotiation based on the MO parameters.

The following parameters in MTSI should be included in the Management Object (MO):

Speech	codec (AMR, AMR-WB, EVS) and bearer QoS parameters

Video	codec (H.264 (AVC), H.265 (HEVC)) and bearer QoS parameters

Real Time text	bearer QoS parameters

Indication of the priority when there are more than one alternative for a media type is included. Version numbering is included for possible extending of MO.

The Management Object Identifier shall be: urn:oma:mo:ext-3gpp-mtsinp:1.0.

Protocol compatibility:  The MO is compatible with OMA Device Management protocol specifications, version 1.2 and upwards, and is defined using the OMA DM Device Description Framework as described in the Enabler Release Definition OMA-ERELD _DM-V1_2[67].

=====  CHANGE  =====

## 15.2	Nodes Definition

The following nodes and leaf objects in figure 15.1 shall be contained under the 3GPP_MTSINP node if a MTSI client in the terminal support the feature described in this clause (information of DDF for this MO is given in Annex H):

Figure 15.1: MTSI network preference management object tree

Node: /<X>

This interior node specifies the unique object id of a MTSI network preferences management object. The purpose of this interior node is to group together the parameters of a single object.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

The following interior nodes shall be contained if the MTSI client in the terminal supports the "MTSI network preferences Management Object".

/<X>/Speech

The Speech node is the starting point of the speech codec definitions (if any speech codec are available)

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Speech/<X>

This interior node is used to allow a reference to a list of speech codec objects.

-	Occurrence: OneOrMore

-	Format: node

-	Minimum Access Types: Get

/<X>/Speech/<X>/ID

This leaf node represents the identification number of a set of parameters for speech session.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/TAG

This leaf node represents the identification tag of a set of parameters for speech session. It is recommended to have at least a node, for example, ID, TAG, or implementation-specific ones, for the identification purpose such that each set of parameters can be distinguished and accessed.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/Priority

This leaf represents the priority of a set of parameters for speech session. Lower value means higher priority and the value is used in the terminal for client initiated QoS handling. The priority uses a 16 bit unsigned integer.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

-	Values: Zero or higher

/<X>/Speech/<X>/IPver

This leaf represents the version of the Internet Protocol used in the session.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

-	Values: "IPv4", "IPv6"

/<X>/Speech/<X>/Codec

This leaf gives the MIME subtype name of speech codec. This leaf is preferably pre-configured by the device.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

-	Values: MIME subtype name of speech codec, e.g., "AMR", "AMR-WB", "EVS".

The value "AMR" refers to the AMR speech codec as defined in 3GPP. The value "AMR-WB" refers to the AMR-WB speech codec as defined in 3GPP. The value "EVS" refers to the EVS speech codec as defined in 3GPP.

/<X>/Speech/<X>/Bandwidth

This interior node is used to allow a reference to a list of parameters related to speech bandwidth assignment.

-	Occurrence: One

-	Format: node

-	Minimum Access Types: Get

-	Values: positive integer

/<X>/Speech/<X>/Bandwidth/AS

This leaf gives the preferred speech codec bandwidth by the network for the bearer set-up, including RTP/UDP/IP headers. It provides the value for "b=AS" line for speech part used in the end-to-end SDP negotiation process, which represents the bit rate in kbits/sec.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/Bandwidth/RS

This leaf provides the value for "b=RS" line for speech part used in the end-to-end SDP negotiation process, which represents the bit rate in bits/sec.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/Bandwidth/RR

This leaf provides the value for "b=RR" line for speech part used in the end-to-end SDP negotiation process, which represents the bit rate in bits/sec.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/RateSet

This leaf node represents a list of bit rates used by speech codec. Depending on the codec, each value can be understood as either the highest rate or the average rate. The entries in the list may either be generic, i.e., usable for any codec, but can also be codec-specific. The default usage is the generic list where the bit rates in bits/sec are included, e.g., "5000, 6000, 7500, 12500". A codec-specific list may indicate the desired modes. For example, in the case of AMR, the list could be "0, 2, 4, 7".

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/EVS

This interior node is used to allow a reference to a list of parameters related to the configuration of EVS speech codec.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Speech/<X>/EVS/Br

This leaf gives the value of br, a parameter representing the range or value of bit-rate for EVS speech codec defined in [125].

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/EVS/Bw

This leaf gives the value of bw, a parameter representing the range or value of bandwidth for EVS speech codec defined in [125].

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/ConRef

This node specifies a reference to QoS parameters Management Object. The interior node’s leaf nodes specify the network preferred QoS parameters as defined in 3GPP TS 24.008 and they should be used in the bearer request when client initiated QoS happen. Implementation specific MO may be referenced.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/Ext

The Ext is an interior node where the vendor specific information can be placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video

The Video node is the starting point of the video codec definitions (if any video codec are available)

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>

This interior node is used to allow a reference to a list of video codec objects.

-	Occurrence: OneOrMore

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/ID

This leaf node represents the identification number of a set of parameters for video session.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/TAG

This leaf node represents the identification tag of a set of parameters for video session. It is recommended to have at least a node, for example, ID, TAG, or implementation-specific ones, for the identification purpose such that each set of parameters can be distinguished and accessed.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Video/<X>/Priority

This leaf represents the priority of a set of parameters for speech session. Lower value means higher priority and the value is used in the terminal for client initiated QoS handling. The priority uses a 16 bit unsigned integer.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

-	Values: Zero or higher

/<X>/Video/<X>/IPver

This leaf represents the version of the Internet Protocol used in the session.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

-	Values: "IPv4", "IPv6"

/<X>/Video/<X>/Codec

This leaf gives the MIME subtype name of video codec. This leaf is preferably pre-configured by the device.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

-	Values: MIME subtype name of video codec, e.g., "H264", "H265".

The values "H264" and "H265" refer to the H.264 (AVC) and H.265 (HEVC) codecs as defined by MPEG and ITU respectively. The usage of H.264 (AVC) and H.265 (HEVC) codecs (profiles, levels etc) is described in the document TS 26.114 Chapter 5.5.2.

/<X>/Video/<X>/Bandwidth

This interior node is used to allow a reference to a list of parameters related to video bandwidth assignment.

-	Occurrence: One

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/Bandwidth/AS

This leaf gives the preferred video codec bandwidth by the network for the bearer set-up, including RTP/UDP/IP headers. It provides the value for "b=AS" line for video part used in the end-to-end SDP negotiation process, which represents the bit rate in kbits/sec.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/Bandwidth/RS

This leaf provides the value for "b=RS" line for video part used in the end-to-end SDP negotiation process, which represents the bit rate in bits/sec.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/Bandwidth/RR

This leaf provides the value for "b=RR" line for video part used in the end-to-end SDP negotiation process, which represents the bit rate in bits/sec.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/Bandwidth/Source

This leaf gives the preferred video encoding bandwidth in kbits/sec.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

/<X>/Video/<X>/Bandwidth/PayloadSize

This leaf gives the preferred payload size for video, excluding payload header, which represents the amount of encoded video data in bytes transported over a RTP packet.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/ProfileLevel

This interior node is used to allow a reference to a list of parameters related to the profile and level of video codec.

-	Occurrence: One

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/ProfileLevel/H264

This leaf gives the profile-level-id of H.264 (AVC) video codec, which indicates the profile that the codec supports and the highest level supported for the signaled profile [24], [25].

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Video/<X>/ProfileLevel/H265

This interior node is used to allow a reference to a list of parameters related to the profile and level of H.265 (HEVC) video codec.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/ProfileLevel/H265/Profile

This leaf gives the value of profile-id, a parameter representing the profile of H.265 (HEVC) video codec defined in [119], [120].

-	Occurrence: One

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/ProfileLevel/H265/Level

This leaf gives the value of level-id, a parameter representing the level of H.265 (HEVC) video codec defined in [119], [120]. Level indicates the maximum computational complexity supported by the offerer in performing decoding for the given profile.

-	Occurrence: One

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/ImageAttr

This interior node is used to allow a reference to a list of parameters related to the image sizes supported or preferred, specified with the "imageattr" attribute. (see clause A.4)

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/ImageAttr/Send

This leaf gives the supported image sizes for the send direction. The value is a string such as "176, 144, 224, 176, 272, 224, 320, 240" which means four image sizes, 176x144, 224x176, 272x224, and 320x240 are supported for the send direction. The maximum image size in this leaf shall not exceed the maximum size limited by the offered codec level.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

/<X>/Video/<X>/ImageAttr/Recv

This leaf gives the supported image sizes and their preferences for the receive direction. The value is a string such as "176, 144, 0.5, 224, 176, 0.5, 272, 224, 0.6, 320, 240, 0.5" which means four image sizes, 176x144, 224x176, 272x224, and 320x240 are supported for the receive direction but 272x224 is preferred since it might fit the available space on the display of the receiver better than the other image sizes. The maximum image size in this leaf shall not exceed the maximum size limited by the offered codec level. The value representing the level of preference by the offerer, defined in [76], is between 0 and 1 inclusive and 0.5 by default.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

/<X>/Video/<X>/ConRef

This node specifies a reference to QoS parameters Management Object. The interior node’s leaf nodes specify the network preferred QoS parameters as defined in 3GPP TS 24.008 and they should be used in the bearer request when client initiated QoS happen.  Implementation specific MO may be referenced.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Video/<X>/Ext

The Ext is an interior node where the vendor specific information can be placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Text

The Text node is the starting point of the real time text codec definitions (if the real time text codec is available).

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Text/<X>

This interior node is used to allow a reference to the real time text codec objects.

-	Occurrence: OneOrMore

-	Format: node

-	Minimum Access Types: Get

/<X>/Text/<X>/ID

This leaf node represents the identification number of a set of parameters for text session.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Text/<X>/TAG

This leaf node represents the identification tag of a set of parameters for text session. It is recommended to have at least a node, for example, ID, TAG, or implementation-specific ones, for the identification purpose such that each set of parameters can be distinguished and accessed.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Text/<X>/Priority

This leaf represents the priority of a set of parameters for text session. Lower value means higher priority and the value is used in the terminal for client initiated QoS handling. The priority uses a 16 bit unsigned integer.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

-	Values: Zero or higher

/<X>/Text/<X>/IPver

This leaf represents the version of the Internet Protocol used in the session.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

-	Values: "IPv4", "IPv6"

/<X>/Text/<X>/TextFormat

This leaf node represents the MIME subtype name of text conversation protocol. The value "t140" refers to T.140 defined in ITU-T [26], [27].

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

-	Values: MIME subtype name of the text conversation protocol, e.g., "t140"

/<X>/Text/<X>/Bandwidth

This interior node is used to allow a reference to a list of parameters related to text bandwidth assignment.

-	Occurrence: One

-	Format: node

-	Minimum Access Types: Get

/<X>/Text/<X>/Bandwidth/AS

This leaf provides the value for "b=AS" line for text part used in the end-to-end SDP negotiation process, which represents the bit rate in kbits/sec.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/Bandwidth/RS

This leaf provides the value for "b=RS" line for text part used in the end-to-end SDP negotiation process, which represents the bit rate in bits/sec.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/Bandwidth/RR

This leaf provides the value for "b=RR" line for text part used in the end-to-end SDP negotiation process, which represents the bit rate in bits/sec.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Text/<X>/RedundancyLevel

This leaf node represents the level of redundancy when redundancy is used with T.140 text.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

-	Values: 0, 100, 200, 300

/<X>/Text/<X>/SamplingTime

This leaf node, defined in clause 9.4, represents the period for which text may be buffered before transmission. Buffering time, defined in [31], has an identical meaning as this node, i.e., the shortest period between text transmissions. Default value is 300 ms.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Text/<X>/ConRef

This node specifies a reference to QoS parameters Management Object.  The interior node’s leaf nodes specify the network preferred QoS parameters as defined in 3GPP TS 24.008 and they should be used in the bearer request when client initiated QoS happen.  Implementation specific MO may be referenced.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Text/<X>/Ext

The Ext is an interior node where the vendor specific information can be placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Ext

The Ext is an interior node where the vendor specific information can be placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

=====  CHANGE  =====

## 15.3	Example Configuration of 3GPP MTSINP MO

The examples below are configurations of 3GPP MTSINP MO for selected speech, text, and video sessions in Annex A. An example of SDP offer for speech session is shown in Table A.6.1, which includes two RTP payload types for AMR-NB. Parameter values in Table 15.1 may apply to both payload types and additional SDP parameters such as max-red may be included under the Ext node as vendor extensions. Depending on the implementation, two sets of session parameters may be defined for the two payload types respectively.

Table 15.1: Example configuration of MTSINP for speech session

| Speech | ID |  | 4 |
| --- | --- | --- | --- |
|  | TAG |  | Undefined |
|  | Priority |  | 2 |
|  | IPver |  | IPv4 |
|  | Codec |  | "AMR" |
|  | Bandwidth | AS | 30 |
|  |  | RS | 0 |
|  |  | RR | 2000 |
|  | RateSet |  | Undefined |
|  | ConRef |  | Undefined |

An example configuration of MTSINP for video session is shown in Table 15.3, which includes the RTP payload type for H.264. Although the "b=AS" value can also be computed with the Source and PayloadSize nodes, a different value with appropriate implementation margin can be directly assigned to the AS node. If the AS, Source, and PayloadSize nodes are defined together, the AS node value should be used for setting "b=AS". In Table 15.3, the "b=AS" value of 315for H.264is computed assuming IPv4 addressing. Note that the Priority node of H.264 is assigned values of 5, which shows that depending on service policy, parameters sets of lower priority may be preferred in the construction of SDP offer. If the ImageAttr node is to be defined, the maximum image size in either the Send or Recv node shall not exceed the maximum size limited by the offered codec level, which is 352x288 for Baseline profile at level 1.1.

Table 15.2: Example configuration of MTSINP for text session

| Text | ID |  | 3 |
| --- | --- | --- | --- |
|  | TAG |  | Undefined |
|  | Priority |  | 1 |
|  | IPver |  | IPv4 |
|  | TextFormat |  | "t140" |
|  | Bandwidth | AS | 2 |
|  |  | RS | 0 |
|  |  | RR | 500 |
|  | RedundancyLevel |  | 200 |
|  | ConRef |  | Undefined |
| Text | ID |  | 4 |
|  | TAG |  | Undefined |
|  | Priority |  | 2 |
|  | IPver |  | IPv4 |
|  | TextFormat |  | "t140" |
|  | Bandwidth | AS | 2 |
|  |  | RS | 0 |
|  |  | RR | 500 |
|  | RedundancyLevel |  | 0 |
|  | ConRef |  | Undefined |

An example of SDP offer for video session is shown in Table A.4.4b, which includes a RTP payload type for H.264. Although the "b=AS" value can also be computed with the Source and PayloadSize nodes, a different value with appropriate implementation margin can be directly assigned to the AS node. If the AS, Source, and PayloadSize nodes are defined together, the AS node value should be used for setting "b=AS". In Table 15.3, the "b=AS" value of 315, for H.264, is computed assuming IPv4 addressing. Note that the Priority node of H.264 is assigned value 3, which shows that depending on service policy, parameters sets of lower priority may be preferred in the construction of SDP offer. If the ImageAttr node is to be defined, as for H.264 in Table A.4.10a, the maximum image size in either the Send or Recv node shall not exceed the maximum size limited by the offered codec level, which is 352x288 for Baseline profile at level 1.1.

Table 15.3: Example configuration of MTSINP for video session

| Video | ID |  |  | 4 |
| --- | --- | --- | --- | --- |
|  | TAG |  |  | Undefined |
|  | Priority |  |  | 5 |
|  | IPver |  |  | IPv4 |
|  | Codec |  |  | "H264" |
|  | Bandwidth | AS |  | 315 |
|  |  | RS |  | 0 |
|  |  | RR |  | 2500 |
|  |  | Source |  | 300 |
|  |  | PayloadSize |  | 1250 |
|  | ProfileLevel | H263 | Profile | Undefined |
|  |  |  | Level | Undefined |
|  |  | MPEG4 |  | Undefined |
|  |  | H264 |  | "42e00c" |
|  | ImageAttr | Send |  | "176, 144, 224, 176, 272, 224, 320, 240" |
|  |  | Receive |  | "176, 144, 0.5, 224, 176, 0.5, 272, 224, 0.6, 320, 240, 0.5" |
|  | ConRef |  |  | Undefined |
| Video | ID |  |  | 1 |
|  | TAG |  |  | Undefined |
|  | Priority |  |  | 3 |
|  | IPver |  |  | IPv4 |
|  | Codec |  |  | "H263-2000" |
|  | Bandwidth | AS |  | 57 |
|  |  | RS |  | 0 |
|  |  | RR |  | 2500 |
|  |  | Source |  | 48 |
|  |  | PayloadSize |  | 250 |
|  | ProfileLevel | H263 | Profile | 0 |
|  |  |  | Level | 10 |
|  |  | MPEG4 |  | Undefined |
|  |  | H264 |  | Undefined |
|  | ImageAttr | Send |  | Undefined |
|  |  | Receive |  | Undefined |
|  | ConRef |  |  | Undefined |

=====  CHANGE  =====

## 17.2	Media adaptation management object

The following nodes and leaf objects in figure 17.1 shall be contained under the 3GPP_MTSIMA node if the MTSI client in terminal supports the feature described in this clause. Information of DDF for this MO is given in Annex J.

Figure 17.1: MTSI media adaptation management object tree

Node: /<X>

This interior node specifies the unique object id of a MTSI media adaptation management object. The purpose of this interior node is to group together the parameters of a single object.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

The following interior nodes shall be contained if the MTSI client in terminal supports the "MTSI media adaptation management object".

/<X>/Speech

The Speech node is the starting point of parameters related to speech adaptation if any speech codec are available.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Speech/<X>

This interior node is used to allow a reference to a list of speech adaptation parameters.

-	Occurrence: OneOrMore

-	Format: node

-	Minimum Access Types: Get

/<X>/Speech/<X>/ID

This leaf node represents the identification number of a set of parameters related to speech adaptation.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/TAG

This leaf node represents the identification tag of a set of parameters for speech adaptation. It is recommended to have at least a node, for example, ID, TAG, or implementation-specific ones, for the identification purpose such that each set of parameters can be distinguished and accessed.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/PLR

This interior node is used to allow a reference to a list of parameters related to packet loss rate (PLR).

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Speech/<X>/PLR/MAX

This leaf node represents the maximum PLR tolerated when redundancy is not used, before the receiver signals the sender to attempt adaptation that reduces PLR or operate at modes more robust to packet loss.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

-	Values: 0 ~ 100 %

/<X>/Speech/<X>/PLR/LOW

This leaf node represents the minimum PLR tolerated, before the receiver signals the sender to probe for higher bit rate, increase the packet rate, reduce redundancy, or perform other procedures that could improve speech quality under such favorable conditions.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

-	Values: 0 ~ 100 %

/<X>/Speech/<X>/PLR/STATE_REVERSION

This leaf node represents the maximum PLR tolerated after adaptation state machine has taken actions, based on the measured PLR lower than LOW. Once PLR exceeds this threshold, the receiver decides that the actions taken to improve speech quality were not successful.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

-	Values: 0 ~ 100 %

/<X>/Speech/<X>/PLR/RED_INEFFECTIVE

This leaf node represents the maximum PLR tolerated, after adaptation state machine has taken actions to increase redundancy. Once PLR exceeds this threshold, the receiver decides that the situation was not improved but degraded.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

-	Values: 0 ~ 100 %

/<X>/Speech/<X>/PLR/DURATION_MAX

This leaf node represents the duration (ms) of sliding window over which PLR is observed and computed. The computed value is compared with the MAX threshold.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/PLR/DURATION_LOW

This leaf node represents the duration (ms) of sliding window over which PLR is observed and computed. The computed value is compared with the LOW threshold.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/PLR/DURATION_STATE_REVERSION

This leaf node represents the duration (ms) of sliding window over which PLR is observed and computed. The computed value is compared with the STATE_REVERSION threshold.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/PLR/DURATION_RED_INEFFECTIVE

This leaf node represents the duration (ms) of sliding window over which PLR is observed and computed. The computed value is compared with the RED_INEFFECTIVE threshold.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/PLR/DURATION

This leaf node represents the duration (ms) of sliding window over which PLR is observed and computed. The computed value is compared with the PLR thresholds.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/PLB

This interior node is used to allow a reference to a list of parameters related to an event, packet loss burst (PLB), in which a large number of packets are lost during a limited period.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Speech/<X>/PLB/LOST_PACKET

This leaf node represents the number of packets lost during a period of PLB/DURATION.

-	Occurrence: One

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/PLB/DURATION

This leaf node represents the period (ms) for which LOST_PACKET is counted.

-	Occurrence: One

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/ECN

This interior node is used to allow a reference to a list of parameters related to Explicit Congestion Notification (ECN) to IP.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Speech/<X>/ECN/USAGE

This leaf node represents a Boolean parameter that enables or disables ECN-based adaptation.

-	Occurrence: ZeroOrOne

-	Format: bool

-	Minimum Access Types: Get

/<X>/Speech/<X>/ECN/MIN_RATE

This leaf node represents the minimum bit rate (bps, excluding IP, UDP, RTP and payload overhead) that speech encoder should use during ECN-based adaptation.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/ECN/STEPWISE_DOWNSWITCH

This leaf node represents a Boolean parameter that selects which down-switch method to use, i.e., direct or step-wise, for ECN-triggered adaptation.

-	Occurrence: ZeroOrOne

-	Format: bool

-	Minimum Access Types: Get

/<X>/Speech/<X>/ECN/RATE_LIST

This leaf node represents the list of bit rates to use during stepwise down-switch. This parameter is only applicable when stepwise down-switch is used.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/ECN/INIT_WAIT

This leaf node represents the time (ms) that the sender should wait before an up-switch is attempted in the beginning of the session if no rate control information or reception quality feedback information is received.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/ECN/INIT_UPSWITCH_WAIT

This leaf node represents the time (ms) that the sender should wait at each step during up-switch in the beginning of the session.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/ECN/CONGESTION_WAIT

This leaf node represents the minimum interval (ms) between detection of ECN-CE and up-switch from the reduced rate.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/ECN/CONGESTION_UPSWITCH_WAIT

This leaf node represents the waiting time (ms) at each step during up-switch after a congestion event, except for the initial up-switch which uses the ECN/CONGESTION_WAIT time.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/ICM

This interior node is used to allow a reference to a list of parameters related to Initial Codec Mode (ICM).

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Speech/<X>/ICM/INITIAL_CODEC_RATE

This leaf node represents the bit rate (bps, excluding IP, UDP, RTP and payload overhead) that the speech encoder should use when starting the encoding in the beginning of the session.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/ICM/INITIAL_CODEC_BANDWIDTH

This leaf node represents the audio bandwidth that the EVS speech encoder should use when starting the encoding in the beginning of the session, unless specified by bw, bw-send, or bw-recv parameter.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

-	Values: nb, wb, swb, fb

/<X>/Speech/<X>/ICM/INIT_WAIT

This leaf node represents the time (ms) that the sender should wait before an up-switch is attempted in the beginning of the session if no rate control information or reception quality feedback information is received.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/ICM/INIT_UPSWITCH_WAIT

This leaf node represents the time (ms) that the sender should wait at each step during up-switch in the beginning of the session.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/ ICM/INIT_PARTIAL_REDUNDANCY_OFFSET_SEND

This leaf node represents the initial partial redundancy offset (-1, 0, 2, 3, 5, or 7) that the EVS speech encoder should use when starting the encoding in the beginning of the session that uses channel aware mode, unless asked otherwise by the far-end MTSI client in terminal with the ch-aw-recv parameter .

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/ ICM/INIT_PARTIAL_REDUNDANCY_OFFSET_RECV

This leaf node represents the initial partial redundancy offset (-1, 0, 2, 3, 5, or 7) that the MTSI client in terminal should ask the far-end MTSI client in terminal with the ch-aw-recv parameter to use when starting the encoding in the beginning of the session that uses channel aware mode.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/MEDIA_ROBUSTNESS

This interior node is used to allow a reference to a list of parameters related to Media Robustness Adaptation that can be used for the CHEM feature.  Each unique codec type is identified by the CODEC_ID under a corresponding instance of the  MEDIA_ROBUSTNESS node which groups the parameters associated with the codec type/CODEC_ID.

-	Occurrence: ZeroOrMore

-	Format: node

-	Minimum Access Types: Get

/<X>/Speech/<X>/MEDIA_ROBUSTNESS/CODEC_ID

This leaf node represents the codec MIME type.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/MEDIA_ROBUSTNESS/TAG

This leaf node represents the identification tag of a set of parameters for speech robustness adaptation of a codec type identified by the CODEC_ID. It is recommended to have at least a node, for example, TAG, or implementation-specific ones, for the identification purpose such that each set of parameters can be distinguished and accessed.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/MEDIA_ROBUSTNESS/CFG_BIT_RATE_LIST

This interior node is used to provide a list of the bit rates of the configurations of the codec type (CODEC_ID) listed from the bit rate of the least robust configuration first to the bit rate of the most robust listed last.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/MEDIA_ROBUSTNESS/CFG_RED_LIST

This interior node is used to provide a list of the redundancy levels of the configurations of the codec type (CODEC_ID) listed from the redundancy level of the least robust configuration first to the redundancy level of the most robust listed last.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/MEDIA_ROBUSTNESS/HIGH_PLR_THRESH_LIST

This interior node is used to provide a list of the high PLR thresholds for each codec configuration except for the most robust configuration. A high PLR threshold for a given codec configuration is the highest tolerable PLR at that codec configuration before the MTSI client requests a more robust codec configuration that will yield lower PLR.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/MEDIA_ROBUSTNESS/LOW_PLR_THRESH_LIST

This interior node is used to provide a list of the low PLR thresholds for each codec configuration except for the least robust configuration. A low PLR threshold for a given codec configuration is the lowest tolerable PLR at that codec configuration before the MTSI client requests a less robust codec configuration that will yield better quality.

-	Occurrence: One

-	Format: chr

-	Minimum Access Types: Get

/<X>/Speech/<X>/MEDIA_ROBUSTNESS/DJB_PLR

This interior node indicates whether the estimated PLR is measured before or after de-jitter buffering.

-	Occurrence: One

-	Format: boolean

-	Minimum Access Types: Get

/<X>/Speech/<X>/MEDIA_ROBUSTNESS/PLR_AVG_WINDOW

This interior node indicates the duration of the sliding window used by the media receiver to estimate the received PLR.

-	Occurrence: One

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/N_INHIBIT

This leaf node represents the period (number of speech frames) for which adaptation is disabled to avoid the ping-pong effects, when adaptation state machine transitions from one state to another then back to the original state.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/N_HOLD

This leaf node represents the period (proportion of PLR/DURATION) that can substitute other periods such as DURATION_LOW or DURATION_RED_INEFFECTIVE, when they are not available.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/T_RESPONSE

This leaf node represents the expected response time (ms) for a request to be fulfilled. If a request transmitted to the far-end is not granted within a period of T_RESPONSE, the request can be considered lost during transmission or the far-end MTSI client in terminal might have decided not to grant it.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Speech/<X>/Ext

The Ext is an interior node where the vendor specific information can be placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video

The Video node is the starting point of parameters related to video adaptation if any video codec are available.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>

This interior node is used to allow a reference to a list of video adaptation parameters.

-	Occurrence: OneOrMore

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/ID

This leaf node represents the identification number of a set of parameters related to video adaptation.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/TAG

This leaf node represents the identification tag of a set of parameters for video adaptation. It is recommended to have at least a node, for example, ID, TAG, or implementation-specific ones, for the identification purpose such that each set of parameters can be distinguished and accessed.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Video/<X>/PLR

This interior node is used to allow a reference to a list of parameters related to PLR.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/PLR/MAX

This leaf node represents the maximum PLR tolerated, before the receiver signals the sender to reduce the bit rate such that PLR is reduced.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

-	Values: 0 ~ 100 %

/<X>/Video/<X>/PLR/LOW

This leaf node represents the minimum PLR tolerated, before the receiver signals the sender to increase the bit rate.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

-	Values: 0 ~ 100 %

/<X>/Video/<X>/PLR/DURATION_MAX

This leaf node represents the duration (ms) of sliding window over which PLR is observed and computed. The computed value is compared with the MAX threshold.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/PLR/DURATION_LOW

This leaf node represents the duration (ms) of sliding window over which PLR is observed and computed. The computed value is compared with the LOW threshold.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/PLB

This interior node is used to allow a reference to a list of parameters related to PLB.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/PLB/LOST_PACKET

This leaf node represents the number of packets lost during a period of PLB/DURATION.

-	Occurrence: One

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/PLB/DURATION

This leaf node represents the period (ms) for which LOST_PACKET is counted.

-	Occurrence: One

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/MIN_QUALITY

This interior node is used to allow a reference to a list of parameters related to the minimum video quality.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/MIN_QUALITY/BIT_RATE

This interior node is used to allow a reference to a list of parameters related to the minimum bit rate.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/MIN_QUALITY/BIT_RATE/ABSOLUTE

This leaf node represents the minimum bit rate (kbps) that video encoder should use.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

/<X>/Video/<X>/MIN_QUALITY/BIT_RATE/RELATIVE

This leaf node represents the minimum bit rate (proportion of the bit rate negotiated for the video session) that video encoder should use.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

-	Values: 0 ~ 100 %

/<X>/Video/<X>/MIN_QUALITY/FRAME_RATE

This interior node is used to allow a reference to a list of parameters related to the minimum frame rate.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/MIN_QUALITY/FRAME_RATE/ABSOLUTE

This leaf node represents the minimum frame rate (fps, frames per second) that video encoder should use.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

/<X>/Video/<X>/MIN_QUALITY/FRAME_RATE/RELATIVE

This leaf node represents the minimum frame rate (proportion of the maximum frame rate limited by the codec profile/level negotiated for the video session) that video encoder should use.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

-	Values: 0 ~ 100 %

/<X>/Video/<X>/MIN_QUALITY/QP

This interior node is used to allow a reference to a list of parameters related to video quantisation.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/MIN_QUALITY/QP/H264

This leaf node represents the maximum value of luminance quantization parameter QPY that video encoder should use if H.264 is negotiated for the video session.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

-	Values: 0 ~ 51

/<X>/Video/<X>/ECN

This interior node is used to allow a reference to a list of parameters related to Explicit Congestion Notification (ECN) to IP.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/ECN/STEP_UP

This leaf node represents the proportion of current encoding rate estimated by video receiver, which is used to ask video sender to increase the rate by this value.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/ECN/STEP_DOWN

This leaf node represents the decrease in the requested maximum encoding rate over current rate, when a down-switch is requested by the receiver.

-	Occurrence: ZeroOrOne

-	Format: chr

-	Minimum Access Types: Get

/<X>/Video/<X>/ECN/INIT_WAIT

This leaf node represents the minimum waiting time (ms) before up-switch is attempted in the initial phase of the session.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/ECN/INIT_UPSWITCH_WAIT

This leaf node represents the waiting time (ms) at each step during up-switch in the beginning of the session.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/ECN/CONGESTION_WAIT

This leaf node represents the minimum interval (ms) between detection of ECN-CE and up-switch from the reduced rate.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/ECN/CONGESTION_UPSWITCH_WAIT

This leaf node represents the waiting time (ms) at each step during up-switch after a congestion event, except for the initial up-switch which uses the ECN/CONGESTION_WAIT time.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/ECN/MIN_RATE

This interior node is used to allow a reference to a list of parameters related to the minimum bit rate during ECN-based adaptation.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Video/<X>/ECN/MIN_RATE/ABSOLUTE

This leaf node represents the minimum bit rate (kbps, excluding IP, UDP, RTP and payload overhead) that video encoder should use during ECN-based adaptation.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

/<X>/Video/<X>/ECN/MIN_RATE/RELATIVE

This leaf node represents the minimum bit rate (proportion of the bit rate negotiated for the video session) that video encoder should use during ECN-based adaptation.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

/<X>/Video/<X>/RTP_GAP

This leaf node represents the maximum interval between packets (proportion of the estimated frame period) tolerated, before the receiver declares bursty packet loss or severe congestion condition.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

/<X>/Video/INC_FBACK_MIN_INTERVAL

This leaf node represents the minimum interval (ms) at which rate adaptation feedback such as TMMBR should be sent from the receiver to the sender, when the bit rate is being increased.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/DEC_FBACK_MIN_INTERVAL

This leaf node represents the minimum interval (ms) at which rate adaptation feedback such as TMMBR should be sent from the receiver to the sender, when the bit rate is being decreased.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/TP_DURATION_HI

This leaf node represents the duration (ms) of sliding window over which the interval between packet arrival and playout is observed. The computed value is compared with TARGET_PLAYOUT_MARGIN_HI.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/TP_DURATION_MIN

This leaf node represents the duration (ms) of sliding window over which the interval between packet arrival and playout is observed. The computed value is compared with TARGET_PLAYOUT_MARGIN_MIN.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/TARGET_PLAYOUT_MARGIN_HI

This leaf node represents the upper threshold of the interval (ms) between packet arrival and its properly scheduled playout.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/TARGET_PLAYOUT_MARGIN_MIN

This leaf node represents the lower threshold of the interval (ms) between packet arrival and its properly scheduled playout.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/RAMP_UP_RATE

This leaf node represents the rate (kbps/s) at which video encoder should increase its maximum bit rate from current value to the value indicated in the most recently received TMMBR message.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

/<X>/Video/<X>/RAMP_DOWN_RATE

This leaf node represents the rate (kbps/s) at which video encoder should decrease its maximum bit rate from current value to the value indicated in the most recently received TMMBR message.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

/<X>/Video/<X>/DECONGEST_TIME

This leaf node represents the time (ms) the receiver should command the sender to spend in decongesting the transmission path, before attempting to transmit at the sustainable rate of the path.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

/<X>/Video/<X>/HOLD_DROP_END

This leaf node represents a tri-valued parameter that controls how the sender should behave in case video quality cannot meet the requirements set in BIT_RATE, FRAME_RATE, or QP.

-	Occurrence: ZeroOrOne

-	Format: int

-	Minimum Access Types: Get

-	Values: 0, 1, 2

/<X>/Video/<X>/INITIAL_CODEC_RATE

This leaf node represents the initial bit rate (proportion of the bit rate negotiated for the video session) that the sender should begin encoding video at.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

-	Values: 0 ~ 100 %

/<X>/Video/<X>/X_PERCENTILE

This leaf node represents the percentile point of packet arrival distribution used with the TARGET_PLAYOUT_MARGIN parameters.

-	Occurrence: ZeroOrOne

-	Format: float

-	Minimum Access Types: Get

-	Values: 0 ~ 100 %

/<X>/Video/<X>/Ext

The Ext is an interior node where the vendor specific information can be placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

/<X>/Ext

The Ext is an interior node where the vendor specific information can be placed (vendor meaning application vendor, device vendor etc.). Usually the vendor extension is identified by vendor specific name under the ext node. The tree structure under the vendor identified is not defined and can therefore include one or more un-standardized sub-trees.

-	Occurrence: ZeroOrOne

-	Format: node

-	Minimum Access Types: Get

Table 17.1: Speech adaptation parameters of 3GPP MTSIMA MO

| Parameter (Unit) | Usage |
| --- | --- |
| PLR/MAX (%) | Packet loss rate (PLR) above this threshold, when redundancy is not used, indicates that performance is not satisfactory. Adaptation state machine at the receiver should signal the sender to attempt adaptation that reduces PLR or operate at modes more robust to packet loss. When using the example adaptation state machines of Annex C, this parameter corresponds to PLR_1. |
| PLR/LOW (%) | PLR below this threshold indicates that conditions are favorable and better quality can be supported. Adaptation state machine at the receiver should signal the sender to probe for higher bit rate, increase the packet rate, reduce redundancy, or perform other procedures that could improve speech quality under such favorable conditions. When in the probing state, if PLR falls below this threshold, then the sender should adapt to a higher bit rate. When using the example adaptation state machines of Annex C, this parameter corresponds to PLR_2. |
| PLR/STATE_REVERSION (%) | PLR above this threshold, after adaptation state machine has taken actions based on PLR lower than LOW, indicates that the actions taken to improve speech quality were not successful. Adaptation state machine at the receiver should signal the sender to return to the previous state where it stayed before attempting to improve speech quality. When using the example adaptation state machines of Annex C, this parameter corresponds to PLR_3. |
| PLR/RED_INEFFECTIVE (%) | PLR above this threshold, after adaptation state machine has taken actions to increase redundancy, indicates that situation was not improved but degraded. Adaptation state machine at the receiver should signal the sender to use a lower bit rate and no redundancy. When using the example adaptation state machines of Annex C, this parameter corresponds to PLR_4. |
| PLR/DURATION_MAX (ms) | Duration of sliding window over which PLR is observed and computed. The computed value is compared with the MAX threshold. |
| PLR/DURATION_LOW (ms) | Duration of sliding window over which PLR is observed and computed. The computed value is compared with the LOW threshold. |
| PLR/DURATION_STATE_REVERSION (ms) | Duration of sliding window over which PLR is observed and computed. The computed value is compared with the STATE_REVERSION threshold. |
| PLR/DURATION_RED_INEFFECTIVE (ms) | Duration of sliding window over which PLR is observed and computed. The computed value is compared with the RED_INEFFECTIVE threshold. |
| PLR/DURATION (ms) | Duration of sliding window over which PLR is observed and computed. The computed value is compared with the PLR thresholds. This applies as the default duration in case no specific DURATION is specified. |
| PLB/LOST_PACKET (integer) | When loss of LOST_PACKET or more packets is detected in the latest period of PLB/DURATION, this event is categorized as a packet loss burst (PLB) and adaptation state machine should take appropriate actions to reduce the impact on speech quality. |
| PLB/DURATION (ms) | Duration of sliding window over which lost packets are counted. |
| ECN/USAGE (Boolean) | Switch to enable or disable ECN-based adaptation. This parameter should be translated as follows: "0" = OFF, "1" = ON. |
| ECN/MIN_RATE (bps) | Lower boundary for the media bit-rate adaptation in response to ECN-CE marking. The media bit-rate shall not be reduced below this value as a reaction to the received ECN-CE. The value of this parameter is assigned to the ECN_min_rate parameter defined in Clause 10.2.0. The ECN_min_rate should be selected to maintain an acceptable service quality while reducing the resource utilization. Default value: Same as ICM/INITIAL_CODEC_RATE if defined, otherwise same as Initial Codec Mode (ICM), see Clause 7.5.2.1.6. |
| ECN/STEPWISE_DOWNSWITCH (Boolean) | Switch to select down-switch method. This parameter should be translated as follows: "0" = direct down-switch to ECN/MIN_RATE; "1" = stepwise down-switch according to ECN/RATE_LIST (one step per congestion event). |
| ECN/RATE_LIST (character set) | List of bit rates (e.g. codec modes) to use during stepwise down-switch. This parameter is only applicable when stepwise down-switch is used. If the codec does not support exactly the rate which is indicated then the highest rate supported by the codec below the indicated value should be used. Depending on the codec, the values can be understood as either the highest rate or the average rate.The entries in the list may either be generic, i.e. usable for any codec, but can also be codec-specific. The default usage is the generic list where the bit rates [in bps] are included, e.g. (5000, 6000, 7500, 12500). A codec-specific list may indicate desired modes, e.g. for AMR the list could be (0,2,4,7). The use of certain rates in this list may be prevented by the results of session negotiation involving SDP attributes such as the "mode-set" parameter. The SDP parameter "mode-change-neighbor" may lead to using intermediate modes when transitioning between rates in this list. If this parameter is not defined or contains bit rates not negotiated in the session, then the mode-set included in SDP is used. If no mode-set is defined in SDP, then "4750, 5900, 7400, 12200" is used for AMR, which corresponds to the "0, 2, 4, 7" modes. |
| ECN/INIT_WAIT (ms) | The waiting time before the first up-switch is attempted in the beginning of the session, to avoid premature up-switch. This parameter shall be used instead of the ICM/INIT_WAIT parameter if ECN is used in the session. Default value is defined in Clause 7.5.2.1.6. |
| ECN/INIT_UPSWITCH_WAIT (ms) | This parameter is used in up-switches in the beginning of the session. Note that the first up-switch in the beginning of the session uses the ECN/INIT_WAIT time. Only the subsequent up-switches use the ECN/INIT_UPSWITCH_WAIT time. This parameter shall be used instead of the ICM/INIT_UPSWITCH_WAIT parameter if ECN is used in the session. Default value: is defined in Clause 7.5.2.1.6. |
| ECN/CONGESTION_WAIT (ms) | The waiting time after an ECN-CE marking for which an up-switch shall not be attempted. The value of this parameter is assigned to the ECN_congestion_wait parameter defined in Clause 10.2.0. A negative value indicates an infinite waiting time, i.e. to prevent up-switch for the whole remaining session. Default value: Same as the ECN_congestion_wait parameter defined in Clause 10.2.0. |
| ECN/CONGESTION_UPSWITCH_ WAIT (ms) | This parameter is used in up-switches after a congestion event. Note that the first up-switch after a congestion event uses the ECN/CONGESTION_WAIT time. Only the subsequent up-switches use the ECN/CONGESTION_UPSWITCH_WAIT time. Default value is 5000 ms. |
| ICM/INITIAL_CODEC_RATE (bps) | The bit rate that the speech encoder should use for the encoding of the speech at the start of the RTP stream. |
| ICM/INITIAL_CODEC_BANDWIDTH (character set) | The audio bandwidth that the EVS speech encoder in EVS Primary mode should use for the encoding of the speech at the start of the RTP stream. |
| ICM/INIT_WAIT (ms) | To avoid premature up-switch when ECN is not used in the session, this parameter defines the waiting time before the first up-switch is attempted in the beginning of the session. Default value: Same as Initial Waiting Time as defined in Clause 7.5.2.1.6. |
| ICM/INIT_UPSWITCH_WAIT (ms) | When ECN is not used in the session, this parameter is used in up-switches in the beginning of the session until the first down-switch occurs. Note that the first up-switch in the beginning uses the INIT_WAIT time. Only the subsequent up-switches use the INIT_UPSWITCH_WAIT time. Default value: Same as Initial Upswitch Waiting Time as defined in Clause 7.5.2.1.6. |
| ICM/INIT_PARTIAL_REDUNDANCY_OFFSET_SEND (integer) | The initial partial redundancy offset (-1, 0, 2, 3, 5, or 7) that the EVS speech encoder should use when starting the encoding in the beginning of the session that uses channel aware mode, unless asked otherwise by the far-end MTSI client in terminal with the ch-aw-recv parameter. |
| ICM/INIT_PARTIAL_REDUNDANCY_OFFSET_RECV (integer) | The initial partial redundancy offset (-1, 0, 2, 3, 5, or 7) that the MTSI client in terminal should ask the far-end MTSI client in terminal to use with the ch-aw-recv parameter when starting the encoding in the beginning of the session that uses channel aware mode. |
| N_INHIBIT (integer) | If adaptation state machine transitions from one state to another then back to the original state, adaptation state machine should not return to the other state in less than N_INHIBIT speech frames, to avoid the ping-pong effects. |
| N_HOLD (integer) | N_HOLD x PLR/DURATION can be used as the period for which PLR is observed and computed. For example, the computed value can be compared with the LOW threshold when DURATION_LOW is not defined. |
| T_RESPONSE (ms) | If the receiver does not detect expected responses from the sender within a period of T_RESPONSE after having sent a request, the receiver should consider this request as not fulfilled and take appropriate actions. |
| CODEC_ID (character set) | MIME Type of the codec for which the media robustness adaptation PLR thresholds are configured. |
| CFG_BIT_RATE_LIST (character set) | List of bit rates (or codec modes) describing the codec configurations to use during media robustness adaptation. The entries are listed in order of the bit rate of the least robust configuration first to the bit rate of the most robust configuration listed last. If there are multiple codec configurations with the same bit rate but different loss robustness (e.g., EVS 13.2 channel aware and non-channel aware modes, or a codec mode with different levels of application layer redundancy), the same bit rate is listed multiple times in the list. If the codec does not support exactly the rate which is indicated, then the highest rate supported by the codec below the indicated value should be used. Depending on the codec, the values can be understood as either the highest rate or the average rate. The entries in the list may either be generic, i.e. usable for any codec, but can also be codec-specific. The default usage is the generic list where the bit rates [in bps] are included, e.g. (5000, 6000, 7500, 12500). A codec-specific list may indicate desired modes, e.g. for AMR the list could be (0,2,4,7). The use of certain rates in this list may be prevented by the results of session negotiation involving SDP attributes such as the "mode-set" parameter or “b=AS” attribute. The SDP parameter "mode-change-neighbor" may lead to using intermediate modes when transitioning between rates in this list. If this parameter is not defined or contains bit rates not negotiated in the session, then the rates or mode-set included in SDP is used. |
| CFG_RED_LIST (character set) | List of redundancy levels describing the codec configurations to use during media robustness adaptation. The redundancy levels are listed in order to correspond to respective CFG_BIT_RATE_LIST entries and describe the redundancy levels from the least robust configuration first to the most robust last. The redundancy level is described using one of the values below: Value; Description. 0; No redundancy. P; Partial Redundancy. 1; 100% repetition application layer redundancy. 2; 200% repetition application layer redundancy. 3; 300% repetition application layer redundancy. Commas are used to separate redundancy levels of each codec configuration in the list. If the codec configuration does not support the redundancy level, then the codec configuration shall not be requested by the media receiver for media robustness adaptation. |
| HIGH_PLR_THRESH_LIST (character set) | List of high PLR thresholds for each codec configuration in the order described for the CFG_BIT_RATE_LIST with the exception of not having a high PLR threshold for the most robust codec configuration, i.e., the last entry in the HIGH_PLR_THRESH_LIST corresponds to the threshold for requesting the most robust configuration when using the second most, or a less, robust configuration. When the estimated PLR exceeds a PLR threshold in this list corresponding to a given codec configuration, the media receiver shall request the next more robust codec configuration. E.g., if the first high PLR threshold in the list, which corresponds to the least robust codec configuration is exceeded, then the media receiver requests the second least robust codec configuration. The PLR values are represented as a percent (e.g., 2.5 is 2.5% PLR) and separated by commas for each codec configuration. |
| LOW_PLR_THRESH_LIST (character set) | List of low PLR thresholds for each codec configuration in the order described for the CFG_BIT_RATE_LIST with the exception of not having a low PLR threshold for the least robust codec configuration, i.e., the first entry in the LOW_PLR_THRESH_LIST corresponds to the threshold for requesting the least robust configuration when using the second least, or a more, robust configuration. When the estimated PLR drops below a PLR threshold in this list corresponding to a given codec configuration, the media receiver shall request the next less robust codec configuration. E.g., if the estimated PLR drops below the last low PLR threshold in the list, which corresponding to the most robust codec configuration, then the media receiver requests the second most robust codec configuration. The PLR values are represented as a percent (e.g., 2.5 is 2.5% PLR) and separated by commas for each codec configuration. |
| DJB_PLR (Boolean) | Indicates whether the PLR estimate for comparing against the PLR thresholds is measured before or after de-jitter buffer processing. Value; Description. 0; Measure PLR pre-DJB. 1; Measure PLR post-DJB. |
| PLR_AVG_WINDOW (ms) | Indicates the duration of the sliding window (in ms) over which the PLR is observed and computed. |

Table 17.2: Video adaptation parameters of 3GPP MTSIMA MO

| Parameter (Unit) | Usage |
| --- | --- |
| PLR/MAX (%) | Upper threshold of PLR above which adaptation state machine at the receiver should signal the sender to reduce the bit rate. PLR is measured per RTP packet and in addition to packets that do not arrive at the receiver ever, packets that arrive but do not make it in time for their properly scheduled playout are considered as lost. |
| PLR/LOW (%) | Lower threshold of PLR below which adaptation state machine at the receiver may signal the sender to increase the bit rate. |
| PLR/DURATION_MAX (ms) | Duration of sliding window over which PLR is observed and computed. The computed value is compared with the MAX threshold. |
| PLR/DURATION_LOW (ms) | Duration of sliding window over which PLR is observed and computed. The computed value is compared with the LOW threshold. |
| PLB/LOST_PACKET (integer) | When loss of LOST_PACKET or more packets is detected in the last period of PLB/DURATION, this event is categorized as a packet loss burst (PLB) and adaptation state machine should take appropriate actions to reduce the impact on video quality. |
| PLB/DURATION (ms) | Duration of sliding window over which lost packets are counted. |
| MIN_QUALITY/BIT_RATE /ABSOLUTE (kbps) | Minimum bit rate that video encoder should use. If the MTSI client in terminal is unable to maintain this minimum bit rate, it should drop the video stream component or put it on hold. If both MIN_QUALITY/BIT_RATE/ABSOLUTE and MIN_QUALITY/BIT_RATE/RELATIVE are set, the larger of these two shall be used as the minimum bit rate. |
| MIN_QUALITY/BIT_RATE /RELATIVE (%) | Minimum bit rate (as a proportion of the bit rate negotiated for the video session) that the video encoder should use. If the MTSI client in terminal is unable to maintain this minimum bit rate, it should drop the video stream component or put it on hold. If both MIN_QUALITY/BIT_RATE/ABSOLUTE and MIN_QUALITY/BIT_RATE/RELATIVE are set, the larger of these two shall be used as the minimum bit rate. |
| MIN_QUALITY/FRAME_RATE /ABSOLUTE (fps) | Minimum frame rate that video encoder should use. If the MTSI client in terminal is unable to maintain this minimum frame rate, it should drop the video stream component or put it on hold. The minimum frame rate is considered unmet if the interval between encoding times of video frames is larger than the reciprocal of the minimum frame rate. If both MIN_QUALITY/FRAME_RATE/ABSOLUTE and MIN_QUALITY/FRAME_RATE/RELATIVE are set, the larger of these two shall be used as the minimum frame rate. |
| MIN_QUALITY/FRAME_RATE /RELATIVE (%) | Minimum frame rate (as a proportion of the maximum frame rate supported as specified by the video codec profile/level negotiated for the session) that video encoder should use. If the MTSI client in terminal is unable to maintain this minimum frame rate, it should drop the video stream component or put it on hold. The minimum frame rate is considered unmet if the interval between encoding times of video frames is larger than the reciprocal of the minimum frame rate. If both MIN_QUALITY/FRAME_RATE/ABSOLUTE and MIN_QUALITY/FRAME_RATE/RELATIVE are set, the larger of these two shall be used as the minimum frame rate. |
| MIN_QUALITY/QP/H264 (integer) | Maximum value of QPY that video encoder should use if H.264 is negotiated for the video session. The encoder should generate video stream such that QPY does not exceed H264. If the MTSI client in terminal is unable to maintain this maximum QPY value, it should drop the video stream component or put it on hold. |
| ECN/STEP_UP (%) | When an up-switch is requested by the receiver, this parameter defines the proportion of the session media bandwidth (b=AS) that is used to increment the requested maximum encoding rate over the currently used rate. The receiver estimates the currently used rate over an implementation dependent time period. Default value: 10. |
| ECN/STEP_DOWN (character set) | List of proportions (%) by which video receiver requests that the encoder rate be reduced relative to the currently used rate in response to each congestion event. The receiver estimates the currently used rate over an implementation dependent time period. The receiver uses the first value in the list for the first congestion event, the second value for the second congestion event etc. The list may consist of only one value. If there are more congestion events than there are values in the list, then the last value is used for each additional congestion event. The receiver resets to use the first value in the list after an up-switch has started i.e. after the CONGESTION_WAIT time. Default Value: "30, 20, 10". |
| ECN/INIT_WAIT (ms) | The waiting time before the first up-switch is attempted in the initial phase of the session, to avoid premature up-switch. Default value is 500 ms. The initial phase starts at the beginning of the session and ends when the first congestion event is detected. |
| ECN/INIT_UPSWITCH_WAIT (ms) | This parameter is the waiting time used before attempting up-switches in the initial phase of the session. Note that the first up-switch in the initial phase uses the INIT_WAIT time. Only the subsequent up-switches use the INIT_UPSWITCH_WAIT time. Default value: 500 ms. |
| ECN/CONGESTION_WAIT (ms) | The waiting time after an ECN-CE marking for which an up-switch shall not be attempted. A negative value indicates an infinite waiting time, i.e. to prevent up-switch for the whole remaining session. Default value: 5000 ms. |
| ECN/CONGESTION_UPSWITCH_ WAIT (ms) | This parameter is the waiting time used before attempting up-switches after a congestion event. Note that the first up-switch after a congestion event uses the CONGESTION_WAIT time. Only the subsequent up-switches use the CONGESTION_UPSWITCH_WAIT time. Default value is 5000 ms. |
| ECN/MIN_RATE/ABSOLUTE (kbps) | Lower boundary for the media bit-rate adaptation in response to ECN-CE marking. The media bit-rate shall not be reduced below this value as a reaction to the received ECN-CE. The ECN/MIN_RATE/ABSOLUTE should be selected to maintain an acceptable service quality while reducing the resource utilization. If the GBR is known to the client to be lower than the ECN/MIN_RATE then the GBR value shall be used instead of the ECN/MIN_RATE value. Default value: 48 kbps. If both ECN/MIN_RATE/ABSOLUTE and ECN/MIN_RATE/RELATIVE are set, the larger of these two shall be used as the lower boundary for the media bit-rate adaptation in response to ECN-CE marking. |
| ECN/MIN_RATE/RELATIVE (%) | Lower boundary (as a proportion of the bit rate negotiated for the video session) for the media bit-rate adaptation in response to ECN-CE marking. The media bit-rate shall not be reduced below this value as a reaction to the received ECN-CE. The ECN/MIN_RATE/RELATIVE should be selected to maintain an acceptable service quality while reducing the resource utilization. If the GBR is known to the client to be lower than the ECN/MIN_RATE then the GBR value shall be used instead of the ECN/MIN_RATE value. Default value: Same as INITIAL_CODEC_RATE for video. If both ECN/MIN_RATE/ABSOLUTE and ECN/MIN_RATE/RELATIVE are set, the larger of these two shall be used as the lower boundary for the media bit-rate adaptation in response to ECN-CE marking. |
| RTP_GAP (float) | If no RTP packets are received for longer than this period (proportion of the estimated frame period), the receiver should declare bursty packet loss or severe congestion condition. Packet loss gap can be detected as follows: based on the reception history of video packets and their time-stamps, the receiver keeps a running estimate of the frame period, T_FRAME_EST. If the receiver does not receive any RTP packets for a duration of RTP_GAP x T_FRAME_EST, then it should react accordingly. Typical RTP_GAP values can range from 0.5 to 5.0. |
| INC_FBACK_MIN_INTERVAL (ms) | Minimum interval between transmitting TMMBR messages that increase the maximum rate limit. |
| DEC_FBACK_MIN_INTERVAL (ms) | Minimum interval between transmitting TMMBR messages that decrease the maximum rate limit. |
| TP_DURATION_HI (ms) | Duration of sliding window over which the interval between packet arrival and playout is observed and computed. The computed value is compared with the TARGET_PLAYOUT_MARGIN_HI threshold. |
| TP_DURATION_MIN (ms) | Duration of sliding window over which the interval between packet arrival and playout is observed and computed. The computed value is compared with the TARGET_PLAYOUT_MARGIN_MIN threshold. |
| TARGET_PLAYOUT_MARGIN_HI (ms) | Upper threshold of the interval between packet arrival and its properly scheduled playout. The interval is measured from playout time to the X percentile point (X_PERCENTILE) of the packet arrival distribution. When this upper threshold is exceeded, the receiver may signal the sender to increase the bit rate. |
| TARGET_PLAYOUT_MARGIN_MIN (ms) | Lower threshold of the interval between packet arrival and its properly scheduled playout. The interval is measured from playout time to the X percentile point (X_PERCENTILE) of the packet arrival distribution. When this lower threshold is exceeded, the receiver should signal the sender to decrease the bit rate. |
| RAMP_UP_RATE (kbps/s) | Rate at which video encoder should increase its target bit rate to a higher max rate limit. |
| RAMP_DOWN_RATE (kbps/s) | Rate at which video encoder should decrease its target bit rate to a lower max rate limit. |
| DECONGEST_TIME (ms) | Minimum time the receiver should command the sender to spend in decongesting the transmission path, before attempting to transmit at the sustainable rate of the path. The receiver can achieve decongestion by first sending a TMMBR message with a value below the sustainable rate of the path. Once the receiver concludes that congestion has been cleared, it can send a TMMBR message with a value closer to the sustainable rate of the path. If the receiver concludes that congestion has not been cleared yet, it may attempt to clear the remaining congestion for another period of DECONGEST_TIME. A short DECONGEST_TIME results in a quick and aggressive decongestion by reducing the bit rate radically while a long DECONGEST_TIME results in a long and conservative decongestion. A value of 0 indicates that the receiver should not attempt to perform any decongestion at all. |
| HOLD_DROP_END (integer) | Tri-valued parameter that controls how the sender should behave in case video quality cannot meet the requirements set in BIT_RATE, FRAME_RATE, or QP. This parameter indicates whether the sender should put the video stream on hold while maintaining QoS reservations, drop the video stream and release QoS reservations, or end the session. Allowed values of this parameter are defined as follows: "0" = HOLD, "1" = DROP, "2" = END. |
| INITIAL_CODEC_RATE (%) | Initial bit rate (proportion of the bit rate negotiated for the video session) that the sender should begin encoding video at. |
| X_PERCENTILE (%) | X percentile point of the packet arrival distribution used with TARGET_PLAYOUT_MARGIN parameters. |