3GPP TSG-SA4 Meeting #109-e 	S4-200861

# Online, May 20 - June 3, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.247 | CR | - | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoE Measurement Collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | QOED |  |  |  |  |  | Date: |  |  | 2020-05-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In LS S4-200721 SA5 requested SA4 to implement certain QoE measurement functionalities: - QoE Reference Id - Recording Session Id - Recording Session Indication - Within-area Indication - Temporary stop of reporting |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR implements: - QoE Reference Id - Recording Session Id - Recording Session Indication - Within-area Indication Comments regarding the other requested functionality: - Temporary stop of reporting: SA4 assumes that this functionality will be handled on RAN level, and not involve the application. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficult for the management system to correlate and use the QoE reports. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 10.6.2, Annex L |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | . |  |  |  |  |  |  |  |  |

========================= CHANGE ==============================

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 22.233: "Transparent End-to-End Packet-switched Streaming Service; Stage 1".

[2]	3GPP TS 26.233: "Transparent end-to-end Packet-switched Streaming service (PSS); General description".

<---- snipped ---->

[61]	3GPP TS 27.007: " Technical Specification Group Core Network and Terminals; AT command set for User Equipment (UE)".

[62]	DASH Industry Forum: "DASH Player’s Application Events and Timed Metadata Processing Model and APIs", see under https://dashif.org/guidelines/.

[63]	3GPP TS 28.405; "Management of Quality of Experience (QoE) measurement collection; Control and configuration"

========================= CHANGE ==============================

### 10.6.2	Report Format

The QoE report is formatted as an XML document that complies with the following XML schema:

<?xml version="1.0"?> <xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:metadata:2011:HSD:receptionreport" xmlns:sup="urn:3gpp:metadata:2016:PSS:SupplementQoEMetric" xmlns:sv="urn:3gpp:metadata:2016:PSS:schemaVersion" xmlns="urn:3gpp:metadata:2011:HSD:receptionreport" elementFormDefault="qualified"> <xs:element name="ReceptionReport" type="ReceptionReportType"/> <xs:complexType name="ReceptionReportType"> <xs:choice> <xs:element name="QoeReport" type="QoeReportType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:choice> <xs:attribute name="contentURI" type="xs:anyURI" use="required"/> <xs:attribute name="clientID" type="xs:string" use="optional"/> </xs:complexType> <xs:complexType name="QoeReportType"> <xs:sequence> <xs:element name="QoeMetric" type="QoeMetricType" minOccurs="1" maxOccurs="unbounded"/> <xs:element ref="sup:supplementQoEMetric" minOccurs="0" maxOccurs="1"/> <xs:element ref="sv:delimiter”/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="periodID" type="xs:string" use="required"/> <xs:attribute name="reportTime" type="xs:dateTime" use="required"/> <xs:attribute name="reportPeriod" type="xs:unsignedInt" use="required"/> <xs:attribute name="qoeReferenceId" type="xs:hexBinary" use="optional"/> <xs:attribute name="recordingSessionId" type="xs:hexBinary" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="QoeMetricType"> <xs:choice> <xs:element name="HttpList" type="HttpListType"/> <xs:element name="RepSwitchList" type="RepSwitchListType"/> <xs:element name="AvgThroughput" type="AvgThroughputType" maxOccurs="unbounded"/> <xs:element name="InitialPlayoutDelay" type="xs:unsignedInt"/> <xs:element name="BufferLevel" type="BufferLevelType"/> <xs:element name="PlayList" type="PlayListType"/> <xs:element name="MPDInformation" type="MpdInformationType" maxOccurs="unbounded"/> <xs:element name="PlayoutDelayforMediaStartup" type="xs:unsignedInt"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="HttpListType"> <xs:choice> <xs:element name="HttpListEntry" type="HttpListEntryType" maxOccurs="unbounded"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="HttpListEntryType"> <xs:choice> <xs:element name="Trace" type="HttpThroughputTraceType" maxOccurs="unbounded"/> </xs:choice> <xs:attribute name="tcpid" type="xs:unsignedInt" use="optional"/> <xs:attribute name="type" type="ExtensibleHttpEntryResourceType" use="optional"/> <xs:attribute name="url" type="xs:string" use="required"/> <xs:attribute name="actualUrl" type="xs:string" use="optional"/> <xs:attribute name="range" type="xs:string" use="optional"/> <xs:attribute name="trequest" type="xs:dateTime" use="required"/> <xs:attribute name="tresponse" type="xs:dateTime" use="required"/> <xs:attribute name="responsecode" type="xs:unsignedInt" use="optional"/> <xs:attribute name="interval" type="xs:unsignedInt" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="HttpEntryResourceType"> <xs:restriction base="xs:string"> <xs:enumeration value="MPD"/> <xs:enumeration value="MPDDeltaFile"/> <xs:enumeration value="XLinkExpansion"/> <xs:enumeration value="InitializationSegment"/> <xs:enumeration value="IndexSegment"/> <xs:enumeration value="MediaSegment"/> </xs:restriction> </xs:simpleType> <xs:simpleType name="StringPatternType"> <xs:restriction base="xs:string"> <xs:pattern value="x:\S.*"/> </xs:restriction> </xs:simpleType> <xs:simpleType name="ExtensibleHttpEntryResourceType"> <xs:union memberTypes="HttpEntryResourceType StringPatternType"/> </xs:simpleType> <xs:complexType name="HttpThroughputTraceType"> <xs:attribute name="s" type="xs:dateTime" use="required"/> <xs:attribute name="d" type="xs:unsignedInt" use="required"/> <xs:attribute name="b" type="UnsignedIntVectorType" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RepSwitchListType"> <xs:choice> <xs:element name="RepSwitchEvent" type="RepSwitchEventType" maxOccurs="unbounded"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RepSwitchEventType"> <xs:attribute name="to" type="xs:string" use="required"/> <xs:attribute name="mt" type="xs:duration" use="optional"/> <xs:attribute name="t" type="xs:dateTime" use="optional"/> <xs:attribute name="lto" type="xs:unsignedInt" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="AvgThroughputType"> <xs:attribute name="numBytes" type="xs:unsignedInt" use="required"/> <xs:attribute name="activityTime" type="xs:unsignedInt" use="required"/> <xs:attribute name="t" type="xs:dateTime" use="required"/> <xs:attribute name="duration" type="xs:unsignedInt" use="required"/> <xs:attribute name="accessbearer" type="xs:string" use="optional"/> <xs:attribute name="inactivityType" type="InactivityType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="InactivityType"> <xs:restriction base="xs:string"> <xs:enumeration value="Pause"/> <xs:enumeration value="BufferControl"/> <xs:enumeration value="Error"/> </xs:restriction> </xs:simpleType> <xs:complexType name="BufferLevelType"> <xs:choice> <xs:element name="BufferLevelEntry" type="BufferLevelEntryType" maxOccurs="unbounded"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="BufferLevelEntryType"> <xs:attribute name="t" type="xs:dateTime" use="required"/> <xs:attribute name="level" type="xs:unsignedInt" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="PlayListType"> <xs:choice> <xs:element name="Trace" type="PlayListEntryType" maxOccurs="unbounded"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="PlayListEntryType"> <xs:choice> <xs:element name="TraceEntry" type="PlayListTraceEntryType" maxOccurs="unbounded"/> </xs:choice> <xs:attribute name="start" type="xs:dateTime" use="required"/> <xs:attribute name="mstart" type="xs:duration" use="required"/> <xs:attribute name="startType" type="StartType" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="PlayListTraceEntryType"> <xs:attribute name="representationId" type="xs:string" use="optional"/> <xs:attribute name="subrepLevel" type="xs:unsignedInt" use="optional"/> <xs:attribute name="start" type="xs:dateTime" use="required"/> <xs:attribute name="sstart" type="xs:duration" use="required"/> <xs:attribute name="duration" type="xs:unsignedInt" use="required"/> <xs:attribute name="playbackSpeed" type="xs:double" use="optional"/> <xs:attribute name="stopReason" type="StopReasonType" use="optional"/> <xs:attribute name="stopReasonOther" type="xs:string" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="StartType"> <xs:restriction base="xs:string"> <xs:enumeration value="NewPlayoutRequest"/> <xs:enumeration value="Resume"/> <xs:enumeration value="OtherUserRequest"/> <xs:enumeration value="StartOfMetricsCollectionPeriod"/> </xs:restriction> </xs:simpleType> <xs:simpleType name="StopReasonType"> <xs:restriction base="xs:string"> <xs:enumeration value="RepresentationSwitch"/> <xs:enumeration value="Rebuffering"/> <xs:enumeration value="UserRequest"/> <xs:enumeration value="EndOfPeriod"/> <xs:enumeration value="EndOfContent"/> <xs:enumeration value="EndOfMetricsCollectionPeriod"/> <xs:enumeration value="Failure"/> <xs:enumeration value="Other"/> </xs:restriction> </xs:simpleType> <xs:complexType name="MpdInformationType"> <xs:choice> <xs:element name="Mpdinfo" type="RepresentationType" maxOccurs="unbounded"/> </xs:choice> <xs:attribute name="representationId" type="xs:string" use="required"/> <xs:attribute name="subrepLevel" type="xs:unsignedInt" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RepresentationType"> <xs:attribute name="codecs" type="xs:string" use="required"/> <xs:attribute name="bandwidth" type="xs:unsignedInt" use="required"/> <xs:attribute name="qualityRanking" type="xs:unsignedInt" use="optional"/> <xs:attribute name="frameRate" type="xs:double" use="optional"/> <xs:attribute name="width" type="xs:unsignedInt" use="optional"/> <xs:attribute name="height" type="xs:unsignedInt" use="optional"/> <xs:attribute name="mimeType" type="xs:string" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="DoubleVectorType"> <xs:list itemType="xs:double"/> </xs:simpleType> <xs:simpleType name="StringVectorType"> <xs:list itemType="xs:string"/> </xs:simpleType> <xs:simpleType name="UnsignedIntVectorType"> <xs:list itemType="xs:unsignedInt"/> </xs:simpleType> </xs:schema>

The following schema is an extension to allow additional QoE metrics.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns="urn:3gpp:metadata:2016:PSS:SupplementQoEMetric"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:metadata:2016:PSS:SupplementQoEMetric"

elementFormDefault="qualified">

<xs:element name="supplementQoEMetric" type="SupplementQoEMetricType"/>

<xs:complexType name="SupplementQoEMetricType">

<xs:sequence>

<xs:element name="deviceinformation" type="DeviceInformationType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>
    </xs:complexType>

<xs:complexType name="DeviceInformationType">
        <xs:choice>
            <xs:element name="Entry" type="DeviceInformationEntryType" maxOccurs="unbounded"/>
        </xs:choice>
        <xs:anyAttribute processContents="skip"/>
    </xs:complexType>

<xs:complexType name="DeviceInformationEntryType">

<xs:attribute name="start" type="xs:dateTime" use="required"/>
        <xs:attribute name="mstart" type="xs:duration" use="required"/>
        <xs:attribute name="videoWidth" type="xs:unsignedInt" use="required"/>
        <xs:attribute name="videoHeight" type="xs:unsignedInt" use="required"/>
        <xs:attribute name="screenWidth" type="xs:unsignedInt" use="required"/>
        <xs:attribute name="screenHeight" type="xs:unsignedInt" use="required"/>

<xs:attribute name="pixelWidth" type="xs:double" use="required"/>
        <xs:attribute name="pixelHeight" type="xs:double" use="required"/>

<xs:attribute name="fieldOfView" type="xs:double" use="required"/>
        <xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:schema>

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema 	xmlns="urn:3gpp:metadata:2016:PSS:schemaVersion"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:metadata:2016:PSS:schemaVersion"

elementFormDefault="qualified">

<xs:element name="schemaVersion" type="xs:unsignedInt"/>

<xs:element name="delimiter" type="xs:byte"/>

</xs:schema>

Note: If a supplementQoEMetric needs to be sent when no ordinar QoEMetric are due, a dummy MPDInformation metric shall be sent with codecs="none", bandwidth=0, mimeType="none", representationId="none".

Note: If the attribute qoeReferenceId was defined in the QMC configuration (see clause L.2), the value shall be copied into each QoE report, to facilitate network-side correlation (see [63]). If this attribute was defined the attribute recordingSessionId shall also be returned for each QoE report. The recordingSessionId is a two-byte octet defined by the client. It shall remain the same for all QoE reports belonging to the same streaming session, and it should be different for QoE reports belonging to different streaming sessions.

========================= CHANGE ==============================

Annex L (normative): 
QoE Measurement Collection Functionalities

# L.1	Configuration and reporting

As an alternative to configuration via MPD or OMA-DM, the QoE configuration can optionally be specified by the QoE Measurement Collection (QMC) functionality. In this case the QoE configuration is received via specific RRC [53] messages for UMTS and RRC [59] messages for LTE over the control plane, and the QoE reporting is also sent back via RRC messages over the control plane.

If QMC is supported, the UE shall support the following QMC functionalities:

-	QoE Configuration: The QoE configuration will be delivered via RRC to the UE as a container according to "Application Layer Measurement Configuration" (see [53]) for UMTS, and "measConfigApplicationLayer" (see [59]) for LTE. The container is an octet string with a maximum length of 1000 bytes, with gzip-encoded data (see [18]) stored in network byte order. The container shall be uncompressed, and is then expected to conform to XML-formatted QoE configuration data according to clause L.2 in the current specification. This QoE Configuration shall be forwarded to the DASH client. The interface towards the RRC signalling is handled by the AT command +CAPPLEVMC [61].

-	QoE Metrics: QoE Metrics from the DASH client shall be XML-formatted according to clause 10.6 in the current specification. The XML data shall be compressed with gzip (see [18]) and stored in network byte order into an octet string container with a maximum length of 8000 bytes. The container shall be delivered via RRC to the RNC according to "Application Layer Measurement Reporting" (see [53]) for UMTS, and to the eNB according to "measReportApplicationLayer" (see [59]) for LTE. The behaviour if the compressed data is larger than 8000 bytes is unspecified in this version of the specification. The interface towards the RRC signalling is handled by the AT command +CAPPLEVMR [61].

-	The UE shall also set the QMC capability "QoE Measurement Collection for streaming services" (see [53]) to TRUE for UMTS, and include the QMC capability "qoe-MeasReport" (see [59]) for LTE.

-	The QoE configuration AT command +CAPPLEVMC [61] may also indicate with an Within-area Indication if the UE is inside or outside a wanted geographic area. Such an indication may arrive with or without any QoE configuration container attached. If the DASH client is informed that it is not inside the area, it shall not start any new QoE measurements even if it has received a valid QoE configuration container, but shall continue measuring for already started sessions.

-	When a new session is started, the QoE reporting AT command +CAPPLEVMR [61] shall be used to send a Recording Session Indication. Such an indication does not contain any QoE report, but indicates that QoE recording has started for a session.

The exact implementation is not specified here, but example signalling diagrams for UMTS and LTE below show the QMC functionality with a hypothetical "QMC Handler" entity.

Figure L-1: Example signalling diagram for UMTS

Figure L-2: Example signalling diagram for LTE

Note that the QMC Handler is only shown here as one possible implementation, and it need not be implemented as such. The corresponding QMC functionality could be built into the DASH client or into other UE entities. In this version of the specification the detailed implementation of the above functionalities is left to the UE vendor.

# L.2	XML configuration

When QoE reporting is configured via the QMC functionality, the configuration is done according to the XML schema below. The interpretation of the different elements and attributes are the same as described in clauses 10.4, 10.5 and Annex F in the current specification.

Note that if geographical filtering is handled on the network side (i.e. QoE reporting is turned on/off  by the network depending on the UE location), no LocationFilter should be specified in the QoE Configuration, as this would mean two consecutive filterings.

Also note that the optional attribute qoeReferenceId is a reference set by the network side (see [63]), which is not directly used by the client. However, if this attribute is defined, it shall be copied into each QoE report, to facilitate network-side correlation.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema targetNamespace="urn:3GPP:ns:PSS:DASH:QMC14"

elementFormDefault="qualified"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:sv="urn:3gpp:metadata:2016:PSS:schemaVersion"

xmlns="urn:3GPP:ns:PSS:DASH:QMC14">

<xs:element name="QmcConfiguration" type="QmcConfigurationType"/>

<xs:complexType name="QmcConfigurationType">

<xs:sequence>
            <xs:element name="Range" type="RangeType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="LocationFilter" type="LocationFilterType" minOccurs="0"/>

<xs:element name="StreamingSourceFilter" type="StreamingSourceFilterType" minOccurs="0" maxOccurs="unbounded"/>
            <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>
        </xs:sequence>
        <xs:attribute name="metrics" type="xs:string" use="required"/>

<xs:attribute name="samplePercentage" type="xs:double" use="optional"/>

<xs:attribute name="reportingInterval" type="xs:unsignedInt" use="optional"/>

<xs:attribute name="qoeReferenceId" type="xs:hexBinary" use="optional"/>
        <xs:anyAttribute namespace="##other" processContents="lax"/>
    </xs:complexType>

<xs:complexType name="RangeType">
        <xs:sequence>
            <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>
        </xs:sequence>
        <xs:attribute name="startTime" type="xs:unsignedInt" use="optional"/>
        <xs:attribute name="duration" type="xs:duration" use="required"/>
        <xs:anyAttribute namespace="##other" processContents="lax"/>
    </xs:complexType>

<xs:complexType name="LocationFilterType">
        <xs:sequence>
            <xs:element name="cellID" type="xs:unsignedLong" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="shape" type="ShapeType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>
        </xs:sequence>

<xs:anyAttribute namespace="##other" processContents="lax"/>

</xs:complexType>

<xs:complexType name="ShapeType">
        <xs:sequence>
            <xs:element name="PolygonList" type="PolygonListType" minOccurs="0"/>

<xs:element name="CircularAreaList" type="CircularAreaListType" minOccurs="0"/>
            <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>
        </xs:sequence>

<xs:anyAttribute namespace="##other" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PolygonListType">
        <xs:annotation>

<xs:documentation> see [OMA MLP] </xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="Polygon" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="ConfLevel" type="xs:unsignedInt" use="optional"/>

<xs:anyAttribute namespace="##other" processContents="lax"/>

</xs:complexType>

<xs:complexType name="CircularAreaListType">
        <xs:annotation>

<xs:documentation> see [OMA MLP] </xs:documentation>

</xs:annotation>

<xs:sequence>

<xs:element name="CircularArea" minOccurs="0" maxOccurs="unbounded"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="ConfLevel" type="xs:unsignedInt" use="optional"/>

<xs:anyAttribute namespace="##other" processContents="lax"/>

</xs:complexType>

<xs:complexType name="StreamingSourceFilterType">
        <xs:sequence>
            <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>
        </xs:sequence>

<xs:attribute name="streamingSource" type="xs:string" use="required"/>
        <xs:anyAttribute namespace="##other" processContents="lax"/>
    </xs:complexType>

</xs:schema>