3GPP TSG-SA4 Meeting #109-e 	S4-200873

# Electronic meeting, Telco, May 25-29 and June 2-3, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.347 | CR | 0010 | rev | 1 | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Partial File Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Enensys |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eDASH |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clause 7.3, “HTTP Interface” describes how an MBMS client may provide a local HTTP server function such that an MBMS Aware Application (MAA) is able to access file content delivered over the MBMS user service by the use of regular HTTP methods (e.g., HTTP GET, and 200 OK or 4xx response codes). Such file request/delivery between the MAA and MBMS client may include partial file delivery request/response mechanisms whereby the application signals its willingness to receive a portion of the requested file resource should the entire file not be available at the MBMS client. Such partial file handling mechanisms are defined in clause 7.3 of this specification which are aligned with the MBMS partial file handling as defined in TS 26.346, and with related mechanisms in the case of DASH-over-MBMS services as described in TS 26.247. CRs against Rel-15 TS 26.346 in Tdoc S4-200712 and against Rel-15 TS 26.247 in Tdoc S4-200713 are submitted to the SA4#109e meeting to propose correction of the existing partial file handling functinality – namely by adding support for independent signaling of partial file handling capability by the HTTP server (e.g., the MBMS client) to the client application (e.g. an MBMS application or 3GP-DASH client). This contribution represents a companion and auxiliary CR to those documents by providing corresponding corrective text regarding partial file handling functionality in TS 26.347, to maintain alignment with the proposed changes to TS 26.346 and TS 26.247. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Spec text change to amend the clauses in TS 26.346 and TS 26.247 to be referenced by TS 26.347 with regards to partial file handling mechanisms. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inability to leverage partial file delivery to a compatible application to the fullest possible extent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.346 (Rel-15), CR S4-200712 |  |  |
|  |  | X |  | Other core specifications |  |  |  | TS 26.247 (Rel-15), CR S4-200713 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S4-200717 |  |  |  |  |  |  |  |  |

CHANGE: Modifications to clause 5.3.2

## 7.3	HTTP Interface

The MBMS client may provide an HTTP server such that the MAA can access the files delivered over the MBMS User services by using regular HTTP Methods. The MBMS client may act as an HTTP cache for the resources delivered through the MBMS system.

The MBMS client offering delivered files and DASH Segments through HTTP Interface should comply with a server as specified in RFC 2616 [15]. MBMS Clients providing resources through an HTTP interface should implement relevant HTTP server functionalities to support HTTP GET methods as required by the APIs.

MAAs communicating with the MBMS client over HTTP should comply with a client as specified in RFC 2616 [15]. MAAs should use the HTTP GET method or the HTTP partial GET method, as specified in RFC 2616 [15] to access files offered at HTTP-URLs.

MAAs communicating with the MBMS client over HTTP should support partial-file-accept requests and partial file responses as defined in TS 26.346 [5], clauses 7.9.1, 7.9.2.1 and 7.9.2.2.

Without excluding other response options, as a response to a partial-file-accept request using a regular HTTP GET request an MAA may typically receive one of the following responses:

1)	200 OK with Content-Type set to the Media Type of the requested object

2)	200 OK with the Content-Type set to application/3gpp-partial and the message format according to the definition in clause 7.9.2.2 of TS 26.346 [5].

3)	416 Requested Range Not Satisfiable with the additional information according to the definition in clause 7.9.2.2 of TS 26.346 [5].

4)	404 Not Found.

Case 1 is the regular response.

Guidelines for handling request responses according to case 4 from above are provided in clauses 7.3.8 and A.7 of TS 26.247 [6].

Guidelines for handling request responses 2 and 3 from above are provided in clause A.9 of TS 26.247 [6].

END OF CHANGE