3GPP TSG SA WG4 #112e	S4-210056

# E-meeting, 1st – 10th February 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.8xx | CR | <CR#> | rev | - | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GMS-EXT] Key Topic Per-application-authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS-EXT |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The study item description identifes the key topic “Per-application-authorization” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds the structure and description for this key topic |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Key topic not addressed |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: " 5G Media Streaming (5GMS); General description and architecture".

[3]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[4]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

=====  CHANGE  =====

# 4	Introduction to 5G Media Streaming

## 4.1	Introduction

## 4.2	Collaboration Scenarios

## 4.3	Architectures

## 4.4	Summary of Stage-3 enablers

=====  CHANGE  =====

# 5	Key Topics

## 5.1	Introduction

## 5.9	Per-application-authorization

### 5.9.1	Description

[Operation of certain 5GMSA and 5G System enabled services include an SLA between the Application Provider and the 5GMS System provider. Different solutions to enable per-application authorization should be studied.]

Editor’s Note: Document the above key topics in more detail, in particular how they relate to the 5GMS Architecture and protocols.

### 5.9.2	Collaboration Scenarios

Editor’s Note: Study collaboration scenarios between the 5G System and Application Provider for each of the key topics.

### 5.9.3	Deployment Architectures

Editor’s Note: Based on the 5GMS Architecture, develop one or more deployment architectures that address the key topics and the collaboration models.

### 5.9.4	Mapping to 5G Media Streaming and High-Level Call Flows

Editor’s Note: Map the key topics to basic functions and develop high-level call flows.

### 5.9.5	Potential open issues

Editor’s Note: Identify the issues that need to be solved.

### 5.9.6	Candidate Solutions

Editor’s Note: Provide candidate solutions (including call flows) for each of the identified issues.