3GPP TSG-SA WG4 Meeting #112-e	S4-210146

# Online,  Feb 1 - 10, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | 0508 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IETF Reference Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | QOSE2EMTSI |  |  |  |  |  | Date: |  |  | 2021-01-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | IETF Internet Drafts are referenced and IETF has now published those documents with RFC status |  |  |  |  |  |  |  |  |
| Summary of change: |  | A set of referenced document identifications are changed from Internet Drafts to RFCs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IETF references will be obsolete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-210146 Initial version |  |  |  |  |  |  |  |  |

## *** Start change 1 ***

### 19.3.2	SDP grammar

The ABNF RFC 5234 [153] for this attribute is the following:

bw-attrib		= "a=bw-info:" pt-def SP direction SP bw-def *(";" [SP] bw-def)

pt-def		= "*" / pt-val *("," pt-val)

pt-val		= 1*3DIGIT

direction		= "send" / "recv" / "sendrecv" / direction-ext

direction-ext	= 1*VCHAR

bw-def		= bw-name "=" bw-val-def

bw-name		= 1*VCHAR			; Label defining the bandwitdh property

bw-val-def	= zero-based-int-or-real / bw-val-def-ext	; Bandwidth value for the bandwidth property

bw-val-def-ext	= zero-based-int-or-real *(":" zero-based-int-or-real)

; Extension possibility

; DIGIT as defined by IETF RFC 4566

; zero-based-int-or-real as defined by IETF RFC 8866

The 'a=bw-info' attribute defines the following possible directionalities for the bandwidth. Three directionalities are defined here:

-	send: In the send direction for SDP Offer/Answer agent providing the SDP or in case of declarative use in relation to the device that is being configured by the SDP.

-	recv: In the receiving direction for the SDP Offer/Answer agent providing the SDP or in case of declarative use in relation to the device that is being configured by the SDP.

-	sendrecv: In both the send and receiving directions for the SDP Offer/Answer agent providing the SDP or in case of declarative use in relation to the device that is being configured by the SDP.

Additional directionalities may be defined in the future, if needed. If an ‘a=bw-info’ line is received with an unknown directionality then the entire ‘a=bw-info’ line is ignored.

The directionality shall be specified when the ‘a=bw-info’ attribute is used. Only one directionality can be specified on each ‘a=bw-info’ line.

It is allowed to define different bandwidth properties on different attribute lines for the same payload type and direction. However, special care should be taken to avoid conflicting definitions. Therefore, a single bandwidth property shall not be included in several attribute lines applicable to the same payload type, direction and IP version. For example, if bandwidth property ‘MaxSupBw’ is defined for payload number 96 on one ‘a=bw-info’ line for direction ‘send’ and IPv6, then ‘MaxSupBw’ cannot be defined on another 'a=bw-info' line for the same payload type number, direction and IP version. However, for example, ‘MaxSupBw’ and ‘MinSupBw’ may be defined in different ‘a=bw-info’ lines, even for the same payload type, direction and IP version. This applies also when a wild card is used for the payload type number.

The ‘bw-name’ is a character string describing the name (or label) used for the bandwidth property that is defined. Four bandwidth property names are defined here for indicating bandwidth needs:

-	MaxSupBw: The Maximum Supported Bandwidth, see clause 19.2.2. The Maximum Supported Bandwidth may be a real value.

-	MaxDesBw: The Maximum Desired Bandwidth, see clause 19.2.3. The Maximum Desired Bandwidth may be a real value.

-	MinDesBw: The Minimum Desired Bandwidth, see clause 19.2.4. The Maximum Desired Bandwidth may be a real value.

-	MinSupBw: The Minimum Supported Bandwidth, see clause 19.2.5. The Maximum Supported Bandwidth may be a real value.

These bandwidth properties include IP/UDP/RTP overhead but not RTCP bandwidth, similar to b=AS. This means that the Maximum Supported Bandwidth for the receiving direction corresponds to the b=AS value used in an SDP offer-answer negotiation.

The following bandwidth property names are also defined:

-	IpVer: The IP version, see clause 19.2.6. Allowed values are 4 and 6 for IPv4 and IPv6, respectively. If the IP version is not indicated then IPv6 is assumed.

-	MaxPRate: The Maximum Packet Rate, see clause 19.2.7. The maximum packet rate may be a real value.

-	MinPRate: The Minimum Packet Rate, see clause 19.2.8. The minimum packet rate may be a real value.

Unknown bandwidth property names shall be ignored, and shall not be included in an answer if received in an offer. A received SDP may include ‘a=bw-info’ lines containing both known and unknown bandwidth properties. Ignoring unknown bandwidth properties shall not cause known bandwidth properties to be ignored.

## *** End change 1 ***

## *** End of changes ***