3GPP TSG-SA4 Meeting #112-e	S4-210174

# Online, 1st – 10th Februay 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0007 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on OpenAPI Implementation of m1 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 27th January 2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR is with the accordance with the new 3GPP procedures for documenting OpenAPI interfaces on 3GPP Forge. It contains the implementation of the m1 interface in the branch m1-dev |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR documents the implementation of the m1 interface by providing the corresponding YAML files. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# C.3	OpenAPI representation of the M1 APIs

## C.3.1	Common Types

openapi: 3.0.0 servers: # Added by API Auto Mocking Plugin - description: 3GPP SA4 5G Media Streaming Interfaces url: https://virtserver.swaggerhub.com/ibouazizi/3gpp-m1d/1.0.0 - url: 'https://www.3gpp.org/3gpp-m1d/v1' description: 5G Media Provisioning Interface variables: {} info: description: Documentation of the 3GPP M1d Provisioning interface. version: 1.0.0 title: 3GPP M1d interface contact: email: bouazizi@qti.qualcomm.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1d externalDocs: description: Find out more url: 'http://www.3gpp.org' externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26346.htm' components: schemas: PathRewriteRule: type: object properties: originPathPattern: type: string distributionPath: type: string Geo: type: object properties: locationType: type: string enum: - cell - city locations: type: array items: type: string Pattern: type: object properties: pattern: type: string QoSSpecification: type: object properties: marBwUl: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/BitRate' marBwDl: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/BitRate' maxPacketLossRateDl: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/PacketLossRateRm' maxPacketLossRateUl: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/PacketLossRateRm' maxSuppBwDl: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/BitRate' maxSuppBwUl: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/BitRate' minDesBwDl: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/BitRate' minDesBwUl: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/BitRate' mirBwUl: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/BitRate' mirBwDl: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/BitRate' ApplicationSessionContext: type: object properties: afAppId: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AfAppId' sliceInfo: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/Snssai' dnn: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/Dnn' aspId: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' ChargingSpecification: type: object properties: sponId: type: string sponStatus: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/SponsoringStatus' gpsi: type: array items: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29571_CommonData.yaml#/components/schemas/Gpsi' securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []

## C.3.2	Provisioning Sessions API

openapi: 3.0.0 servers: # Added by API Auto Mocking Plugin - description: 3GPP SA4 5G Media Streaming Interfaces url: https://virtserver.swaggerhub.com/ibouazizi/3gpp-m1d/1.0.0 - url: 'https://www.3gpp.org/3gpp-m1d/v1' description: 5G Media Provisioning Interface variables: {} info: description: Documentation of the 3GPP M1d Provisioning interface. version: 1.0.0 title: 3GPP M1d interface provisionings contact: email: bouazizi@qti.qualcomm.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1d Provisionings externalDocs: description: Find out more url: 'http://www.3gpp.org' paths: /provisioning-sessions: get: summary: 'List of all Provisionings' description: '' parameters: - name: provisioningSessionId in: query required: false schema: type: string description: A unique identifier of the Provisioning operationId: 'listProvisionings' responses: '200': description: list of Provisioning Sessions content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' post: summary: 'Create a new Provisioning Session' description: '' requestBody: description: initial configuration of new Provisioning Session required: true content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' parameters: [] operationId: 'Create a new Provisioning Session' responses: '201': description: Created /provisioning-sessions/{provisioningSessionId}: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: A unique identifier of the Provisioning Session get: summary: 'Get Provisioning Session' description: '' parameters: [] operationId: 'getProvisioningById' responses: '200': description: list of Provisioning sessions content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' put: summary: 'Update a Provisioning Session' description: '' parameters: [] operationId: '' responses: default: description: Default response delete: summary: '' description: '' parameters: [] operationId: '' responses: default: description: Default response externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26346.htm' components: schemas: ProvisioningSession: type: object required: - id properties: id: type: integer format: int64 securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []

## C.3.3	Server Certificates Provisioning API

openapi: 3.0.0 servers: # Added by API Auto Mocking Plugin - description: 3GPP SA4 5G Media Streaming Interfaces url: https://virtserver.swaggerhub.com/ibouazizi/3gpp-m1d/1.0.0 - url: 'https://www.3gpp.org/3gpp-m1d/v1' description: 5G Media Provisioning Interface variables: {} info: description: Documentation of the 3GPP M1d Provisioning interface Certificate version: 1.0.0 title: 3GPP M1d interface contact: email: bouazizi@qti.qualcomm.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1d Certificate externalDocs: description: Find out more url: 'http://www.3gpp.org' paths: /provisioning-sessions/{provisioningSessionId}/certificates: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: 'A unique identifier of the the Provisioning' post: summary: 'Create a new security certificate' description: '' responses: '200': description: 'The generated certificate' headers: Location: description: 'Location of the newly created server certificate' schema: type: string content: 'application/x-pem-file': schema: type: string get: summary: 'retrieve previously uploaded server certificate' parameters: - name: certificateId in: path required: true schema: type: string description: 'A unisque identifier of the server certificate' responses: '200': description: 'the server certificate' content: 'application/x-pem-file': schema: type: string externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26346.htm' components: schemas: securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []

## C.3.4	Content Preparation Templates Provisioning API

## C.3.5	Content Protocols Discovery API

## C.3.6	Content Hosting Configuration API

openapi: 3.0.0 servers: # Added by API Auto Mocking Plugin - description: 3GPP SA4 5G Media Streaming Interfaces url: https://virtserver.swaggerhub.com/ibouazizi/3gpp-m1d/1.0.0 - url: 'https://www.3gpp.org/3gpp-m1d/v1' description: 5G Media Provisioning Interface variables: {} info: description: Documentation of the 3GPP M1d Provisioning interface. version: 1.0.0 title: 3GPP M1d interface contact: email: bouazizi@qti.qualcomm.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1d Content Hosting Configuration externalDocs: description: Find out more url: 'http://www.3gpp.org' paths: /provisioning-sessions/{provisioningSessionId}/content-hosting-configurations: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: 'A unique identifier of the Provisioning' get: summary: 'retrieve an existing Content Hosting Configuration' description: 'retrieve an existing Content Hosting Configuration' parameters: - name: contentHostingConfigurationId in: query required: false schema: type: string description: A unique identifier of the Content Hosting Configuration responses: '200': description: The Content Hosting Configuration content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' post: summary: 'create a Content Hosting Configuration' description: 'Used to retrieve an existing Content Hosting Configuration' responses: '201': description: Created headers: Location: description: 'Location of the newly created Content Hosting Configuration' schema: type: string content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration/{contentHostingConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: A unique identifier of the Provisioning Session - name: contentHostingConfigurationId in: path required: true schema: type: string description: A unique identifier of the Content Hosting Configuration get: summary: 'retrieve an existing Content Hosting Configuration' description: 'retrieve an existing Content Hosting Configuration' responses: '200': description: The identified Provisioning content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' put: summary: '' description: '' responses: default: description: '' delete: summary: '' description: '' responses: default: description: Default response /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration/{contentHostingConfigurationId}/purge: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: A unique identifier of the Provisioning - name: contentHostingConfigurationId in: path required: true schema: type: string description: A unique identifier of the Content Hosting Configuration post: summary: 'Purge the content of the cache' description: '' requestBody: description: 'The RegEx pattern for resources to purge from the cache' required: true content: application/json: schema: $ref: '#/components/schemas/Pattern' responses: '200': description: 'Content was purged successfully' externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26346.htm' components: schemas: ContentHostingConfiguration: type: object required: - id - origin properties: id: type: integer format: int64 ingestConfiguration: $ref: '#/components/schemas/IngestConfiguration' distributionConfiguration: type: array items: $ref: '#/components/schemas/DistributionConfiguration' IngestConfiguration: type: object properties: name: type: string path: type: string pull: type: boolean protocol: type: string enum: - dashifpush entryPoint: type: string xml: name: IngestConfiguration DistributionConfiguration: type: object properties: contentPreparationTemplateId: type: string targetDomain: type: string originDomain: type: string pathRewriteRules: type: array items: $ref: '#/components/schemas/PathRewriteRule' cacheConfiguration: type: array items: $ref: '#/components/schemas/CachingConfiguration' geoFencing: type: array items: $ref: '#/components/schemas/Geo' urlSignature: type: object properties: urlPattern: type: string tokenName: type: string passphraseName: type: string passphrase: type: string tokenExpiryName: type: string useIPAddress: type: boolean ipAddressName: type: string certificateId: type: integer xml: name: DistributionConfiguration PathRewriteRule: type: object properties: originPathPattern: type: string distributionPath: type: string CachingConfiguration: type: object properties: urlPatternFilter: type: string cachingDirectives: type: object properties: statusCodeFilters: type: array items: type: integer noCache: type: boolean maxAge: type: integer format: int32 securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []

## C.3.7	Consumption Reporting Provisioning API

openapi: 3.0.0 servers: # Added by API Auto Mocking Plugin - description: 3GPP SA4 5G Media Streaming Interfaces url: https://virtserver.swaggerhub.com/ibouazizi/3gpp-m1d/1.0.0 - url: 'https://www.3gpp.org/3gpp-m1d/v1' description: 5G Media Provisioning Interface variables: {} info: description: Documentation of the 3GPP M1d Provisioning interface Consumption Reporting. version: 1.0.0 title: 3GPP M1d interface contact: email: bouazizi@qti.qualcomm.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1d Consumption Reporting externalDocs: description: Find out more url: 'http://www.3gpp.org' paths: /provisioning-sessions/{provisioningSessionId}/consumption-reporting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: 'A unique identifier of the Provisioning' post: summary: 'Activate the consumption reporting procedure' description: 'Activate the consumption reporting procedure' responses: '200': description: 'the consumption reporting is successfully activate' get: summary: 'Retrieve the consumption reporting configuration' description: 'retrieve the consumption reporting configuration' responses: '200': description: 'a description of the consumption reporting configuration' content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26346.htm' components: schemas: ConsumptionReportingConfiguration: type: object properties: reportingInterval: $ref: 'https://forge.etsi.org/rep/3GPP/5G_APIs/blob/master/TS29122_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: type: number format: float minimum: 0.0 maximum: 100.0 locationType: $ref: '#/components/schemas/Geo' securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []

## C.3.8	Metrics Reporting Provisioning API

## C.3.9	Policy Templates Provisioning API

openapi: 3.0.0 servers: - description: 3GPP SA4 5G Media Streaming Interfaces url: https://virtserver.swaggerhub.com/ibouazizi/3gpp-m1d/1.0.0 - url: 'https://www.3gpp.org/3gpp-m1d/v1' description: 5G Media Provisioning Interface variables: {} info: description: Documentation of the 3GPP M1d Policy Template version: 1.0.0 title: 3GPP M1d Policy Template interface contact: email: bouazizi@qti.qualcomm.com license: name: Apache 2.0 url: 'http://www.apache.org/licenses/LICENSE-2.0.html' tags: - name: 3GPP TS 26.512 description: 5G Media Provisioning Interface M1d Provisioning Interface externalDocs: description: Find out more url: 'http://www.3gpp.org' paths: /provisioning-sessions/{provisioningSessionId}/policy-templates: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: 'A unique identifier of the Provisioning Session' post: summary: 'Create a new Policy Template' description: 'create a new Policy Template' responses: '201': description: 'the newly created Policy Template' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' /provisioning-sessions/{provisioningSessionId}/policy-templates/{policyTemplateId}: parameters: - name: provisioningSessionId in: path required: true schema: type: string description: 'A unique identifier of the Provisioning Session' - name: policyTemplateId in: path required: true schema: type: string description: 'A unique identifier of the Policy Template' get: summary: 'retrieve a description of the Policy Template' description: 'retrieve a description of the Policy Template' responses: '200': description: 'a description of the Policy Template' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' put: summary: 'modify the configuration of a Policy Template' description: '' requestBody: content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '200': description: 'update a policy template' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' delete: summary: 'delete a Policy Template' description: 'delete a Policy Template' responses: '200': description: 'a Policy Template is deleted' externalDocs: description: TS 26.512 url: 'http://www.3gpp.org/DynaReport/26512.htm' components: securitySchemes: m1d_auth: type: oauth2 flows: authorizationCode: authorizationUrl: https://www.3gpp.org/3gpp-m1d/v1/authorize tokenUrl: https://www.3gpp.org/3gpp-m1d/v1/token scopes: read: Grants read access write: Grants write access admin: Grants access to admin operations links: {} callbacks: {} security: - m1d_auth: []