Agenda item: 	16.3

Source: 	Qualcomm Inc., BBC, Tencent

Title: 	EMSA Architecture

Document for	Discussion and Agreement

# Introduction

In this contribution, we propose a solution for the mapping of the 5G edge architecture as developed by SA6 and the management architecture for edge as developed by SA5 onto the 5GMS architecture. This is a follow-up on an earlier contribution that offered 2 options for the mapping. The goal is to avoid the definition of any new interfaces between the 5GMS elements and the other architecture elements.

# EMSA Architecture

The EMSA architecture is an integration of the 5GMS architecture, the SA6 Edge architecture and the SA5 management architecture. The EMSA architecture is depicted in the following figure:

Figure X1: The reference edge-enabled 5GMS media architecture

The above architecture demonstrates how the edge-enabled 5GMS media architecture leverages the edge functionality and architectural elements specified in [TS 23.558]. In this illustration, the EEC, EES, and EAS are shown as parts of the MSH, 5GMS AF, and 5GMS AS, respectively.

This architecture implies the following:

- An Media Session Handler (MSH) that is edge-enabled is required to implement the EDGE-5 API.

- An 5GMS AF that is edge-enabled is required to support EES functionality including:

  - EDGE-3 API towards the EAS function of 5GMS AS instances.

  - EDGE-6 API for registering with an ECS function.

  - EDGE-9 API for media session relocation.

  - EDGE-1 API for supporting registration and provisioning of EEC functions, and discovery by them of EAS instances.

- An 5GMS AF that is edge-enabled may perform compute resource allocation using the MnS-C interface.

- An 5GMS AS that is edge-enabled is required to support EAS functionality including EDGE-3 API.

In general, in order to deploy edge support for the 5GMS application, each 5GMS function has to become edge-enabled and a union of each 5GMS function with its corresponding edge function (MSH + EEC, AF + EES, AS + EAS) would satisfy the requirements. In this regard, various deployment scenarios including the standalone existence of the edge functions can be considered.

Editor’s Note: Examples of the different deployment scenarios will be shown in Annex XYY (TBD).

This edge-enabled 5G media streaming architecture supports both client-driven as well as AP-driven management of the edge processing session.

In the client-driven approach, the 5GMS-Aware Application is aware of the support of edge processing in the network and takes steps, such as using the EDGE-5 APIs, to discover and locate an application server in the edge DN.

In Application Provider (AP)-driven approach, the 5GMS Application Provider provisions 5GMS features as normal within the scope of an M1 Provisioning Session, but additionally requests the 5GMS System to accommodate the provisioned features using edge computational resources, where appropriate. The 5GSMS AF acts on behalf of the 5GMS Application Provider to allocate processing resources based on the application’s needs. In order for this transparent resolution to work, SA2 is expected to define a solution to allow the AF to influence the DNS resolution for the domain name(s) related to the media distribution by the Application Provider as described in clause X.

# Generic Call Flow for Session Establishment

Two generic call flow for edge-enabled 5G media streaming session establishment are demonstrated below.

# Client-driven edge discovery

Figure X2 outlines a detailed call flow for client-driven session establishment.

Figure X2: Client-driven session establishment

The steps are:

- Edge Computing Provisioning phase:

  - Spawn ECS

  - Spawn 5GMS AF

  - EES Configuration

  - EES Registration with ECS

- 5GMS Application Provider Provisioning phase:

- Create Provisioning Session

- Provision 5GMS features

- UE Edge Computing Discovery phase:

- Application Initialization

- Locate EAS/5GMS AS

- Locate local EES

- Locate EAS/5GMS AS

Optional subflow to provision an additional 5GMS AS instance (if needed):

- Check resource template

- Instantiate new EAS/5MGS AS

- Spawn 5GMS AS instance

- EAS configuration

- Register EAS with EES

- Configure provisioned features

Completion of UE Edge Computing Discovery phase:

- 5GMS AS location response

- 5GMS AS location response

- 5GMS Session phase

- Start session

- Session starting event

- Retrieve service access information

- Media transfer

- Method calls and notifications

- Reporting, network assistance, and dynamic policy

- End session

- Session ending event

- Final reporting

# AP-driven management of 5GMS edge processing

Figure X3 outlines a detailed call flow for the AP-driven management of edge processing. In the previous sequence, the optional provisioning of an additional 5GMS AS instance occurs in response to an explicit call from the Application Client to an on-board EEC, whereas in this sequence it occurs (if needed) in response to Application Provider provisioning. It may also be repeated at any time during the lifetime of the 5GMS Provisioning Session in response to satisfy demand levels (not illustrated for brevity).

Figure X3: AP-driven management of 5GMS edge processing

The steps are:

- Edge Computing Provisioning phase (same as before):

  - Spawn ECS

  - Spawn 5GMS AF

  - EES Configuration

  - EES Registration with ECS

B.	5GMS Application Provider Provisioning phase:

  - Create Provisioning Session

  - Provision 5GMS features

Optional sub-flow to provision an additional 5GMS AS instance (if needed, same as before):

  - Check resource template

  - Instantiate new EAS/5MGS AS

  - Spawn 5GMS AS instance

  - EAS configuration

  - Register EAS with EES

  - Configure provisioned features

D:	5GMS Session phase (same as before):

  - Start session

  - Session starting event

  - Retrieve service access information

  - Media transfer

  - Method calls and notifications

  - Reporting, network assistance, and dynamic policy

  - End session

  - Session ending event

  - Final reporting

# Proposal

We propose to agree the proposed architecture mapping and to add the content of sections 2 and 3 to the EMSA TR 26.803.