| 3GPP TR 26.803 V0.5.1 (2021-02) |  |
| --- | --- |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on 5G Media Streaming Extensions for Edge Processing (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Overview of Relevant Architectures	7

4.1	General	7

4.2	SA6 Edge Architecture	7

4.3	SA2 Edge Support	8

4.3.1	Overview	8

4.3.2	Process of Discovering EAS	11

4.3.2.1	General	11

4.3.2.2	DNS based solutions for Multiple PDU Sessions	11

4.3.2.3	DNS-based solutions for Distributed Anchors	12

4.3.2.4	DNS-based solutions for Session Breakout	12

4.3.3.5	Summary of DNS based EAS Discovery	12

4.3.3.6	Discussion	13

4.3.4	Summary of EAS relocation solutions	13

4.4	 SA5 Edge Management	13

4.5	5GMS Architecture	14

5	Use Cases for Edge Media Processing	14

5.1 	General	14

5.2	Downlink Streaming Use Cases	14

5.2.1	Caching Downlink Streaming Content	14

5.2.2	Split Rendering	16

5.2.3	User-generated live streaming	18

5.2.4	Augmented Video Streaming	22

5.2.5	Generalized Split and Cloud Rendering and Processing	24

5.2.6	Photo-realistic AR Rendering in Network	26

5.2.7	Media Services in the Edge	27

5.2.8	Partial delivery of 3D content (point cloud, mesh) for AR/MR device	29

5.3	Uplink Streaming Use Cases	30

5.3.1	Multi-Camera Uplink Stream Processing	30

5.3.2	Cloud/Split Rendering of Immersive Live Events	33

5.3.3	Pandemic Stadium	35

6	Potential 5GMS Architecture Extensions	37

6.1	General	37

6.2	EMSA Architecture	37

6.3	Generic Call Flow for Session Establishment	38

6.3.1	General	38

6.3.2	Client-driven edge discovery	39

6.3.3	AP-driven management of 5GMS edge processing	41

Annex <A> (normative): Deployment Scenarios	43

Annex <X> (informative): Change history	44

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

In this Technical Report, we study the architectural enablers and extensions to the 5G Media Streaming Architecture for edge media processing. It covers aspects such as discovery, configuration, execution, and management of media processing in 5G to address the needs identified for example by the XR5G and FLUS items.

# 1	Scope

The present document is a study of use cases for multimedia processing in the edge and the potential 5G media streaming architecture extensions to enable them.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[3]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)"

[4]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in 5G Core network (5GC)".

[5]		3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2"

[6]	3GPP TR 26.928: "Extended Reality (XR) in 5G"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Overview of Relevant Architectures

## 4.1	General

In this section, we provide an overview of ongoing SA activities on edge computing architectures.

## 4.2	SA6 Edge Architecture

SA6 has taken significant steps towards the definition of normative edge computing architecture for 5GC in [3]. Starting from common scenarios, described in the Annex, a set of requirements is defined, and the following architecture is proposed:

Figure 1 SA6 Edge data network architecture

The architecture defines the key nodes and functions as well as the interfaces between them.

The identified functions with a brief description is given here:

- Edge Enabler Server (EES): provides supporting functions needed for Edge Application Servers and Edge Enabler Client.

- Edge Enabler Client (EEC): provides supporting functions needed for Application Client(s).

- Edge Configuration Server (ECS): provides supporting functions needed for the Edge Enabler Client to connect with an Edge Enabler Server.

- Edge Application Server (EAS): the application server resident in the Edge Data Network, performing the server functions. The Application Client connects to the Edge Application Server in order to avail the services of the application with the benefits of Edge Computing.

- Application Client (AC): application resident in the UE performing the client function. Details of the Application Client are out of scope of this specification.

A typical sequence of steps to use edge computing services is as follows:

- EESs register with the ECS to publish their edge configuration capabilities

- The EEC is provisioned with a list of EESs, e.g. from the ECS

- The EEC registers with a selected EES.

- EASs register with EESs to publish their edge capabilities

- The EEC queries the EES to discover specific EASs. Different types of filtering can be used during this discovery phase

- The EAS may register for location and other notifications with the EES related to a specific session

- The AC is now able to connect to the EAS to consume edge computing services

Figure 2 represents the SA6 edge server architecture as defined in [3].

The XR server capabilities needed for the split rendering use-case if run on an edge server, is supported by an Edge Application Server (EAS) in the above figure.

TS23.558 [3] provides Edge Application Server KPI discovery as shown in Table 1:

Table 1: Edge Application Server Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the Application Client supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the Application Client's service requests. |
| Availability | O | Advertised percentage of time the server is available for the Application Client's use. |
| Available Compute | O | The maximum compute resource available for the Application Client. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the Application Client. |
| Available Memory | O | The maximum memory resource available for the Application Client. |
| Available Storage | O | The maximum storage resource available for the Application Client. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the Application Client's use. |

## 4.3	SA2 Edge Support

### 4.3.1	Overview

Edge Computing has been identified as a key feature of 5G early on and has been specified as part of the 5G System Architecture in clause 5.13 of [2]. It describes the selection of a close UPF for non-roaming or LBO-connected UEs. The specification lists different options to enable access to edge computing:

- User plane (re)selection: the 5G Core Network (re)selects UPF to route the user traffic to the local Data Network

- Local Routing and Traffic Steering: the 5G Core Network selects the traffic to be routed to the applications in the local Data Network

- Session and service continuity to enable UE and application mobility

- An Application Function may influence UPF (re)selection and traffic routing via PCF or NEF

- Network capability exposure: 5G Core Network and Application Function to provide information to each other via NEF or directly

- QoS and Charging: PCF provides rules for QoS Control and Charging for the traffic routed to the local Data Network;

- Support of Local Area Data Network: 5G Core Network provides support to connect to the LADN in a certain area where the applications are deployed

In addition, SA2 has started a new release 17 study item on enhancement of support for edge computing in 5GC [4] to address identified gaps to enable edge computing. The findings will be documented in TR 23.748 [4].

So far, the identified key issues are:

- Discovery of Edge Application Server:

The UE needs to find the optimal Edge Application Server (EAS) so that traffic is locally routed to the respective EAS. Furthermore, replacing the old EAS with new EAS when the old EAS is no more optimal is also an important aspect to address.  This key issue deals with such EAS discovery.

- Edge relocation:

As the UE moves across the 5G system, the UE location may change and require the network and the EAS to deal with the change. Furthermore, Edge Application Server (EAS) may become congested or may even become unavailable due to outage, which also need to be dealt by changing the EAS. This key issue deals with addressing such EAS relocations.

- Network Information Provisioning to Locate Applications with Low Latency:

There might be a need to deliver certain timely QoS information to the Edge Application Server (EAS) that is deployed locally. The usual way to do this is via the NEF, however it will not be possible to do this with low latency because the NEF is usually located in the central location to avoid frequent switching between NEFs. This key issue deals with being able to provision such QoS information to the local applications that run in the EAS with low latency

- (Consecutive traffic steering in different N6-LAN)

Traffic steering might be needed consecutively first to Local Application Server and then to Central Application Server for further processing. Also for downlink, there might be a traffic first that moves to Central Application Server and then brought to the Local Application Server (to get it closer to the UE) and then finally to the UE.

- Activating the traffic routing towards local data network per AF request

In order to activate the traffic routing towards Local (access to) Data Network, the SMF should be configured with the required DNAI. For ETSUN case, either SMF or I-SMF should be configured with the requested DNAI. For some of edge computing use case scenarios, the SMF or I-SMF of the PDU session may not be configured with the requested DNAI. This key issue deals with the mechanism to activate the traffic routing towards the Local Data Network in such cases.

Section 6 of 23.748 documents a set of solutions for the previously identified key issues.

The following architecture figures show 5GS and Edge Application Servers hosted in Edge Hosting Environment.

Figure 2: SA2 Architecture for Accessing Edge Application Server with UL CL/BP

Figure 4: SA2 Architecture for Accessing Edge Application Server without UL CL/BP

NOTE:	These figures show the relationship between the EAS and 5GC defined in TS 23.501 [2]. The application layer architecture for enabling edge computing is out of the scope of the SA2 study.

5GC supports at least following three connectivity models to enable Edge Computing:

-	Distributed Anchor Point: the PDU Session anchor is moved far out in the network, to the local sites. It is the same for all the user PDU session traffic. Re-anchoring (SSC#2 and SSC#3) is used to optimize traffic routing for all applications when moving long distances.

-	Session Breakout: The PDU session has a PDU Session anchor in a central site and a PDU Session anchor in the local site. Only one of them provides the IP anchor point. The Edge Computing application traffic is selectively diverted to the local PDU Session anchor using UL Classifier or multihoming BP technology. Re-anchoring of the local PDU Session anchor is used to optimize traffic routing for locally diverted traffic as the user moves.

-	Multiple PDU sessions: Edge Computing applications use a specific PDU session with the PDU Session anchor in the local site. The rest of applications use a PDU Session with a central PDU Session anchor. The mapping between applications and PDU sessions is steered by the URSP rules. Re-anchoring (SSC#2 and SSC#3) is used to optimize traffic routing for Edge Computing applications as the user moves.

These three connectivity models are illustrated in the figure below:

Figure 5: SA2 5GC Connectivity Models for Edge Computing

### 4.3.2	Process of Discovering EAS

#### 4.3.2.1	General

DNS based solutions have been concluded as the final solution for discovering the Edge Application Server for the above three connectivity models.

#### 4.3.2.2	DNS based solutions for Multiple PDU Sessions

This solution proposes to enhance the NEF service(s) to allow the AF to influence PCF decisions for URSP rules so that Domain Descriptors are used to steer the DNS and the Application traffic into specific PDU sessions with the PDU Session Anchor UPF that is servicing the EAS.

The following clarifications are therefore required in normative specification(s) on how the Domain Descriptors in URSP are used by the UE in clause 7.1.4 of [5]:

-	When a UE attempts to resolve an FQDN, the PDU Session through which to send the DNS query message is controlled by URSP rules.

-	The UE should be able to use URSP procedures to set up the PDU session prior to sending a DNS query.-	The IP address of the DNS server for the PDU Session is configured on the UE by the SMF using one of the methods already specified.

The solution is not guaranteed to work when:

a)	The UE doesn't support URSP rules provisioned from the PCF.

b)	The DNS Server configuration at the UE has been overridden by the user.

c)	The Application Client deliberately circumvents the Operating System’s default DNS libraries, for example by using DNS-over-TCP (DoT), DNS-over-HTTPS (DoH) or any other over-the-top mechanism.

NOTE:	This can correspond to devices doing tethering, or to devices deployed for specific corporate purposes. Whether URSP rules work in the OS level is up to implementations. How the cooperation between the OS level and the modem works depends on the further studies.

These limitations and informative guidelines will be captured in a technical specification to cover scenarios where the Operating System, user or applications may override the operator-provided URSP or DNS settings. The guidelines should assume no restriction on the Operating System, user or application.

#### 4.3.2.3	DNS-based solutions for Distributed Anchors

The decision for anchoring the UE in the distributed anchor point scenario for Edge Computing will be described in a technical specification as:

a)	Using subscription policy information to set proper UE policy (e.g. URSP via usage of dedicated DNN); and/or

b)	Applying proper policies at session (SMF) level to apply PCC rules based on the Nnef_TrafficInfluence service and based on the user subscription.

#### 4.3.2.4	DNS-based solutions for Session Breakout

For Session Breakout connectivity mode, the LDNSR, a new stand-alone 5GC network function is proposed. It allows coordination of EAS Discovery using DNS and 5GC connectivity. The LDNSR facilitates selection of an EAS closer to the edge, and it allows Dynamic ULCL/BP/Local PSA insertion.

1.	The LDNSR is dynamically configured with address records of the EAS instances it is to handle so that it can respond appropriately when a local UE attempts to resolve one of these FQDNs.

2.	The SMF selects the LDNSR serving the PDU session and configures the UE to use the LDNSR as its DNS Server for that PDU Session.

3.	The LDNSR maintains a PDU session context during the lifetime of the PDU session and needs to be made aware of the release of the PDU Session.

4.	The LDNSR is able to inform SMF with the IP address of an EAS resolved by DNS, which that may trigger the SMF to perform local UPF insertion/relocation if needed.

Editor’s Note: The name of “LDNSR” would be up to SA2’s final WID outcome.

#### 4.3.3.5	Summary of DNS based EAS Discovery

The DNS-based EAS Discovery solution depends on DNS message parsing. The Core Network can decide the location of the local PDU Session Anchor and can deliver the DNS query message to DNS with UE’s location indicated. Then the DNS can return the IP address of EAS. Finally, the UE connects to the Edge Application Server via the PDU Session.

When the UE moves, the 5GC (SMF) may change the user plane path according to the UE’s new location, i.e. TA granularity. Then the User Plane path change notification may be sent to the AF and AF decides whether to trigger the EAS relocation procedure or not.

NOTE:	Currently, the DNS-based EAS discovery solution is only useful with Operator-provided DNS Setting. If the DNS server is overwritten by a third party, or if the application or application library uses DNS over TCP (DoT) or DNS over HTTPS (DoH), the solution is still under development in SA2. The Operated-provided DNS can also be the recursive DNS.

Editor’s Note: Alternative solutions for the cases where enhanced/controlled DNS resolution won’t work will be up to SA2’s WID conclusion.

#### 4.3.3.6	Discussion

With the concluded solutions in SA2, the application client can directly discover and connect to the EAS before any application layer exchanges between the client and EAS. There will be less impact for SA4 aspects without frequent exchanges at the application layer.

However, without the exchanges between clients and application servers, which capability does this application need? Especially when the capability of EAS need to be determined based on the capability and requirements of clients. The SA2 solutions may need to be enhanced for discovering EAS with specific capabilities to avoid additional re-discovering the appropriate EAS.

### 4.3.4	Summary of EAS relocation solutions

Currently the concluded solutions in SA2 mainly focus the following parts:

- EAS Rediscovery:

  - The DNS-based EAS rediscovery procedure is triggered by UE to find the new EAS. The triggers can be a new IP address assigned to the UE or the EAS relocation indicator within the NAS message from SMF. Then UE can establish the new connection to the new EAS.

  - The UE is informed via the application layer signaling, like HTTP REDIRECT, which is out of 3GPP SA2 scope.

- For less packet loss, buffering the uplink data in the target PSA-UPF is proposed when the EAS relocation begins.

- The User Plane latency can be taken into consideration by 5GC during the user plane path change/selection. The AF can give a UP latency request during the EAS relocation and 5GC can follow that requirement during the change/selection of UP path.

- The Coordination indication can be provided by AF to let both the connections to the source EAS and target EAS can coexist for a time period.

- EAS IP replacement in the PSA UPF can be used to keep UE unaware of the EAS change. However, this conclusion assumes the TCP/TLS/QUIC context transfer between EAS can be handled by other solutions.

During the EAS relocation, how to transfer the EAS context including application layer context and transport layer context is out the scope of the SA2 study. Hence, the main concluded solutions seem like preparation work, e.g. EAS rediscovery triggers different considerations during the EAS relocation. How to define the context transfer is not defined yet.

Transfer of context between EAS instances is application- and deployment-specific, but a 3GPP-defined API to initiate context transfer would enable the 5GMS AF to interface with different virtual infrastructure management systems.

NOTE: Certain applications may be sensitive to the speed of context transfer, so it would be useful for a 5GMS AF to be able to obtain an estimate in advance from the 5G System of how long a particular transfer might take.

## 4.4		SA5 Edge Management

SA5 has recently agreed a new study item [4] on the management aspects of edge computing. The focus of the study item will be on the following scenarios:

- How 3GPP management solutions support the 3rd party service provider to deploy and manage Edge Configuration Server.

- How 3GPP management solutions support Edge Computing Service Provider(s) to deploy and manage Edge Enable Server(s).

- How 3GPP management solutions support Edge Application Provider(s) to deploy and manage Edge Application Server(s).

As can be seen, SA5 adopts the SA6 architecture and will study ways of enabling it through providing the necessary management functions.

The scope has been set to the following items:

  - The lifecycle management of EAS, EES and ECS needed to support edge computing, by considering the various deployment scenarios.

  - Deployment and provisioning of 5GC network functions needed to support the edge computing.

  - Provisioning of EES, and ECS needed to support edge computing.

  - Performance assurance of EES, ECS and relevant 5GC functions needed to support edge computing.

  - Fault supervision of EES, ECS and relevant 5GC functions needed to support edge computing.

  - Mechanism(s) to enable and support EAS deployment on a particular edge data network.

  - Mechanism(s) to enable and support

    - ECSP to deploy and manage EES and ECS.

    - ASP to deploy and manage EAS.

  - Studying the need of providing management provisions to create and manage communication service(s) at a particular edge data network.

## 4.5	5GMS Architecture

# 5	Use Cases for Edge Media Processing

## 5.1 	General

Editor’s Note: only relevant use cases should be documented.

## 5.2	Downlink Streaming Use Cases

### 5.2.1	Caching Downlink Streaming Content

Use Case Name

Caching downlink streaming content

Description

A Mobile Network Operator that deploys a downlink streaming service or supports the delivery of media content from a third-party service wants to offer that content in the highest possible quality to all of its users. The MNO also notices that video streaming already accounts for a large part of the traffic on the backhaul network. For these reasons, the MNO wants to offload (part of the) content hosting from the CDN to caches near or within its network. Users of the service may access the content from the edge, allowing them to select higher quality renditions of the content (e.g., DASH representations) and play it back without interruptions. The MNO may improve the hit ratios of the cache by employing intelligent caching. Furthermore, to ensure that clients access the content from the optimal edge, the network operator may want to direct clients to this edge.

Categorization

Type: CDN Delivery: Download, Live Streaming, On Demand Streaming Device: Phone, tablet, HMD, TV

Preconditions

End user devices should be able to stream, decode, and display the video streams. Modern smartphones already have these capabilities.

Requirements in terms of Capabilities and QoS/QoE Considerations

The capabilities of edge nodes are similar to regular CDN nodes distributing video content, although at smaller scale. This means that edge nodes should have storage and HTTP serving capabilities, and UEs should have high-bandwidth connectivity to edge nodes. Higher video quality, less playback interruptions, and shorter loading times improve the QoE.

Feasibility and Industry Practices

CDNs are a well-known and well-established technology. Mobile CDN is receiving the attention of various third-party CDN operators due to the increase of mobile users and the increased prevalence of applications and services that target mobile users. A recent report "Mobile CDN Market" (https://www.transparencymarketresearch.com/mobile-cdn-market.html) lists different CDN operators operating a Mobile CDN, including: Cloudflare, Fastly, Akamai Technologies, Amazon Web Services and Ericsson. Cloudflare explains its reasons for supporting Mobile CDN: https://www.cloudflare.com/performance/accelerate-mobile-experiences/. This use case allows an MNO to offer services to third-party CDN providers or to improve the performance of its own Operator CDN (if the MNO offers CDN service itself) by enabling distributed CDN cache instances to run as Edge Application Servers (EAS). This use case relies on the network being able to select an edge cache that is closest to the UE client, as well as performing cache re-selection during mobility of the UE client. The architecture to support this is being worked on by SA2 in the Release 17 study item on enhancement of support for edge computing in 5GC under key issue 1 (‘Discovery of Edge Application Server’) and key issue 2 (‘Edge relocation’).

Nominal Cost Analysis

Using edge computing for video content caching is the next step in distributing video delivery. It will allow MNOs and streaming services to further scale up and serve more users, while reducing load on the backhaul network. As in a regular CDN node, a node at the edge can be used by many users at the same time and servers scale horizontally. MNOs can use existing facilities at PoPs or points further in the network with serving capabilities.

Benefits and Impact

The major benefit is expected for MNOs and service providers, who are able to serve more users with high quality video while significantly reducing the load on the backhaul network, thus improving the efficiency of the network infrastructure. End-users are expected to benefit as it will increase the access to content in a high video quality, also enabling demanding streaming applications including VR, and delivering those applications with shorter loading times and with fewer interruptions.

Potential Technical Requirements

- It should be possible for edge caches to be operated either by the MNO or by a third-party service such as a 5GMSd Application Provider. - It should be possible for the network to steer clients to a certain edge or CDN. - It should be possible for (third-party) services to specify caching directives. - It should be possible for DASH clients to send hints (e.g., about anticipated upcoming requests) to the network enabling intelligent caching on the edge.

Potential Standardization Status and Needs

TBD

### 5.2.2	Split Rendering

Use Case Name

Split Rendering

Description

The system design for split rendering follows the discussion and requirements from TR26.928, clause 6.2.5. The architecture us shown in Figure 1. Figure 6 Split Rendering with Asynchronous Time Warping (ATW) Correction Raster-based split rendering refers to the case where the XR Server runs an XR engine to generate the XR Scene based on information coming from an XR device. The XR Server rasterizes the XR viewport and does XR pre-rendering. According to Figure Figure 1, the viewport is pre-dominantly rendered in the XR server, but the device is able to do latest pose correction, for example by asynchronuous time-warping (see clause 4.1) or other XR pose correction to address changes in the pose. - XR graphics workload is split into rendering workload on a powerful XR server (in the cloud or the edge) and pose correction (such as ATW) on the XR device - Low motion-to-photon latency is preserved via on device Asynchronous Time Warping (ATW) or other pose correction methods. The following call flow highlights the key steps: 1) An XR Device connects to the network and joins XR application a) Sends static device information and capabilities (supported decoders, viewport) 2) Based on this information, the XR server sets up encoders and formats 3) Loop a) XR Device collects XR pose (or a predicted XR pose) b) XR Pose is sent to XR Server c) The XR Server uses the pose to pre-render the XR viewport d) XR Viewport is encoded with 2D media encoders e) The compressed media is sent to XR device along with XR pose that it was rendered for f) The XR device decompresses video g) The XR device uses the XR pose provided with the video frame and the actual XR pose for an improved prediction using and to correct the local pose, e.g. using ATW. According to TR 26.928, clause 4.2.2, the relevant processing and delay components are summarized as follows: - User interaction delay is defined as the time duration between the moment at which a user action is initiated and the time such an action is taken into account by the content creation engine. In the context of gaming, this is the time between the moment the user interacts with the game and the moment at which the game engine processes such a player response. - Age of content is defined as the time duration between the moment a content is created and the time it is presented to the user. In the context of gaming, this is the time between the creation of a video frame by the game engine and the time at which the frame is finally presented to the player. The roundtrip interaction delay is therefore the sum of the Age of Content and the User Interaction Delay. If part of the rendering is done on an XR server and the service produces a frame buffer as rendering result of the state of the content, then for raster-based split rendering (as defined in clause 6.2.5) in cloud gaming applications, the following processes contribute to such a delay: - User Interaction Delay (Pose and other interactions) - capture of user interaction in game client, - delivery of user interaction to the game engine, i.e. to the server (aka network delay), - processing of user interaction by the game engine/server, - Age of Content - creation of one or several video buffers (e.g. one for each eye) by the game engine/server, - encoding of the video buffers into a video stream frame, - delivery of the video frame to the game client (a.k.a. network delay), - decoding of the video frame by the game client, - presentation of the video frame to the user (a.k.a. framerate delay). As ATW is applied the motion-to-photon latency requirements (of at most 20 ms) are met by XR device internal processing. What determines the network requirements for split rendering is time of pose-to-render-to-photon and the roundtrip interaction delay. According to clause TR 26.928, clause 4.5, the permitted downlink latency is typically 50-60ms.

Categorization

Type: XR, Cloud Computing, GPU Delivery: Interactive, Split, Gaming Device: Phone, HMD, Glasses

Preconditions

On the device, a gaming application may be installed On the network, an XR Server is installed, that runs a gaming application as well GPU based rendering and an encoding.

Requirements in terms of Capabilities and QoS/QoE Considerations

<provides a summary on potential requirements as well as considerations on KPIs/QoE as well as QoS requirements>

Feasibility and Industry Practices

<How could the use case be implemented based on technologies available today or expected to be available in a foreseeable timeline, at most within 3 years? - What are the technology challenges to make this use case happen? - Do you have any implementation information? - Demos - Proof of concept - Existing services - References - Could a reduced experience of the use case be implemented in an earlier timeframe or is it even available today? > Steam Boundless XR: https://zerolight.com/de/news/press-releases/worlds-first-boundless-xr-over-5g-retail-experience

Cost Analysis

< How does the use case scale with an increasing number of users? >

Potential Standardization Status and Needs

<identifies potential standardization needs>

### 5.2.3	User-generated live streaming

Use Case Name

User-generated live streaming

Description

A social influencer starts a live captured media session similar to Facebook Live and publishes the content through 5G Media Uplink Streaming. The content is then distributed live to several or many viewers through 5G Media Downlink Streaming. The application provides several usage scenarios with various configuration options which may change during the live session: - The social influencer may: - be static, occasionally moving or highly mobile, for example in a vehicle, on skis or on a bicycle, - produce different quality of content, depending on lighting conditions, speed, as well as based on the quality of the camera and the available uplink bandwidth, - produce highly-valuable content that requires extra content protection, - want to capitalize on the stream by allowing ad-insertion in the content (targeted pre-roll and/or mid-roll). - The viewers may be quite diverse and changing because: - they may be dynamically joining or leaving the live stream, - their number might be just a few or they may quite many, in range of tens of thousands or more, for example for a popular influencer, - they may be geographically spread with different densities in various areas, and their densities may change during the session, - they may consume the service on different devices, for example on 4K TV sets, mobile phones, in-car receivers or tablets, with different operating systems, DRM capabilities as well as different codec hardware capabilities, - some of them may be mobile, i.e on a car or public transport, - they may react (smilies, comments, likes, audio dubs, images, avatars, and animations) to the content or previous reactions by the viewers who watched the content earlier. - The service requirements may be quite different. The content may need to be: - available for live and/or on-demand consumption, - only available for consumption after the end of the live uplink session, i.e. it is uploaded entirely before being made available to followers, - available with a required target latency with ranges in between capture and display of as low as 1 second up to several tens of seconds, - dubbed into the same or different language, - provided with automatic extraction and addition of captions/subtitles from the audio, - post-processed to improve the audio and/or visual quality, - processed by adding overlays and content tags and other augmented material, - indexed, including the addition of thumbnail navigation in real time, - provided to regional proxies with specific metadata such as black-out information, ad insertion opportunities, language settings or other service metadata, - profanity checked and appropriately altered before being distributed, - available for viewing for some time period (from a few minutes to forever) after the end of the live session. In a simple reference scenario of 5G Media Uplink and Downlink Streaming, the following aspects are supported: - An application manages the service. The server application may be run by external application providers, by MNO, or by a joint collaboration of application provider and MNO. - Uplink streaming is provided through 5G media uplink streaming to the application. - The application may perform one or more of the following processes: - It decodes the received content - It applied the various processes the content such as: - Upscaling - Light correction - Stabilization - Dubbing - Captioning - Overlaying and tagging - Indexing - Navigation improvements. - It encodes the uploaded content to 5GMS downlink streaming formats. - It packages the content and adds appropriate ad metadata. - It applies content protection and DRM. - As soon as the content becomes available, the server application uses downlink streaming for distribution. - It provisions a 5GMS downlink streaming service. - It ingests the content into a 5GMS streaming service. - It gets feedback from consumption reporting on what content is consumed and may change the encoded streaming formats. The following aspects may be critical for the service: - Can the service be provided throughout the session without any interruption considering the dynamic aspect of the service, for example when a highly mobile edge media streaming client makes a transition between two edge processing environments? - Can the desired end-to-end latency be met with the reference scenario and if not, what are possible ways to realize this? - Is the content generation flexible and fast enough to address different user population, yet highly utilized, i.e. the variations of the content is consumed by one or more viewers and is shared as much as possible by many viewers? - Is there a benefit to push certain processing closer to the influencer, certain viewers, or in-between (in terms of bandwidth, latency, and processing requirements)? NOTE: the meaning of flexible content generation needs to be explained.

Categorization

Type: 2D Delivery: 5GMS uplink streaming, 5GMS downlink streaming, live media streaming Device: Phone, HMD, Glasses

Preconditions

On the sending UE, 5GMS Uplink streaming is provided. On every receiving UE, a 5GMS downlink streaming client is provided. An Application Service Provider supports network interfaces for 5GMS Uplink Streaming (M1u and M2u) as well as 5GMS Downlink Streaming (M1d and M2d). Extended functionalities in uplink streaming and downlink streaming may be supported such that the Application Service Provider can delegate certain tasks to the 5G Media Streaming System.

Requirements in terms of Capabilities and QoS/QoE Considerations

Potential new required capabilities of 5G Media Streaming System: - Decoding of content received through uplink streaming - Processing of the content with functionalities such as - Upscaling - Light correction - Stabilization - Dubbing - captioning - Etc. - Transcoding of the uploaded and processed content to 5GMS downlink streaming formats - Packaging/Encrypting the content and adding appropriate ad metadata Relevant KPIs: - Subjective/objective quality of the encoded streams in terms of bitrates/quality. - Latency for the end to end service. - Latency of each processing step. - Seamless integration of ads with the main content. - Scalability and handling of different number of devices. - Quality of the delivered service in terms of streaming metrics. - Computational efficiency in terms of: - Encoding and Decoding workflow - Encryption and D - Manifest generation - Target-based advertising - Seamless session and service continuity of media streaming when the user’s high level of mobility causes a transition to a new edge processing environment.

Feasibility and Industry Practices

Today, the above processing may be achieved by running cloud processing on the application server for all the functionalities mentioned above. Alternatively, third-party cloud-based transcoding services exist. The number of streams, support of multiple codecs, the amount of delay, the number of representations in the encoding ladder varies depending on the service. Codec support evolves with the availability of new codecs. Services like Facebook Live encode the content on demand and on-the-fly. Services like Youtube Live provide channels for professional content providers. They can insert advertisement clips, but the ad insertion occurs using two video elements. Services like TikTok allow the user to add a lip-sync to a song, to add his/her reactions on a small window over the video, and to make short movies using filter and speed change of the video. The user also can capture a video next to a pre-recorded video.

Nominal Cost Analysis

The cost of service increases linearly with the number of ingest streams. The cost of service increases less than linearly with the number of download streaming clients as the encoding and caching requirement will be common with the large number of viewers. Cost analysis needs to take into account if there a benefit or necessity to push certain processing closer to the users/edge (in terms of bandwidth, latency and processing requirements)?

Benefits and Impact

Potential Benefits and Impacts when running services within 5GMS network - tbd Potential Benefits and Impacts when running services on the edge - tbd

Potential Technical Requirements

Potential Requirements: - Establish workflows as defined in the use case with the addition of service parameters. - Distribute the tasks across 5G System and 5GMS components. - Seamless transition of application/service media streaming flows between different edge media processing environments.

Potential Standardization Status and Needs

- tbd

### 5.2.4	Augmented Video Streaming

Use Case Name

Augmented video streaming

Description

Augmented video streaming will help to understand the video and enhance the interest of the video. In live streaming scenes, it can analyze the video content with AI technology, refine the video information, understand the video content and enhance the video quality, such as improving the resolution and color. It may also superimpose AR effect to tag the identified video information interestingly, such as the AR effect of the pass path in a football game or the AR effect on human face or background during streaming, etc. Processing at the edge can reduce the time required for video enhancement processing and network transmission. - Augmented video streaming with AI. Due to limited shooting conditions or the need for new video features, augmented video streaming with AI can provide a service that edge nodes is used to enhance the captured video to improve the user experience. 1) An encoded video streaming is compressed and uploaded to an edge node, and the video resolution, color gamut and other parameters should be specified in configuration. 2) The edge node server decodes the video streaming, then with the installed AI capabilities, sets up the enhancement functions, such as resolution enhancement, color enhancement, signal-to-noise ratio enhancement, video feature recognition, subtitle, etc., for video editing. 3) Augmented Video is re-encoded and distributed to users. - Augmented video streaming with AR. It can provide AR experience to its users. 1) An encoded video streaming is compressed and uploaded to an edge node. 2) The edge node server decodes the video streaming, identifies and understands the video features with AI capability, and loads the corresponding AR special effects on the understanding results. The AR special effects are overlapped with the video streaming. 3) The edge node distributes it to users. - Augmented video streaming with interaction. AI augmented content may consider the user’s preferences. AR augmented content will output according to the user's FOV. Therefore, the workflow of the edge node can involve interactions. According to the user's preference, the edge node server selects the corresponding augmented video streaming to distribute, or processes the video according to the user's FOV. When there is a high requirement for QoE of enhanced video, the terminal device cannot meet this requirement and cloud or edge servers are needed for processing. And in the case of local video streaming transmission, the video streaming can be processed directly at the edge. Compared with uploading to the cloud-based processing, it can reduce roundtrip time delay (RTT) of network and avoid the waste of resources on the core network. For AI video enhancement, users can choose the function of the video enhancement. If the AI video enhancement processing functions are deployed on the cloud, downlink transmission needs to involve multi-video streams with different enhancement effects for users to choose from. The pressure of the downlink will be multiplied. Therefore, it is better for edge nodes to support different video enhancement processing. For AR effects enhancement, users need to use AR devices to watch enhanced AR video and have higher requirement on delay, like MTP (Motion To Photons) latency requirement. It is necessary for split rendering of edge processing to ensure this.

Categorization

Type: XR, AI, Cloud Computing Delivery: Interaction Device: Phone, HMD, TV

Preconditions

On the device side, a video application or a browser supporting XR player with interaction is needed. On the network side, a video server is installed for video data structuring process and its key techniques like deep learning, big data technology and cloud storage. Edge nodes need to store some AR effects in advance.

Requirements in terms of Capabilities and QoS/QoE Considerations

<provides a summary on potential requirements as well as considerations on KPIs/QoE as well as QoS requirements> - Requirements - Real-time adaptation of encoding and transcoding - Overlay graphics. - CDN and content caching - 6DOF interaction - KPI - MTP - Operation response time - The number of users

Feasibility and Industry Practices

<How could the use case be implemented based on technologies available today or expected to be available in a foreseeable timeline, at most within 3 years? - What are the technology challenges to make this use case happen? - Do you have any implementation information? - Demos - Proof of concept - Existing services - References - Could a reduced experience of the use case be implemented in an earlier timeframe or is it even available today? > Intel edge computing solutions At the 2018 China Mobile Global Partner Conference, China Mobile and Intel jointly released a set of video processing unit and intelligent video edge computing solution. https://www.intel.sg/content/www/xa/en/cloud-computing/visual-cloud.html - Media Analytics - AR solutions

Cost Analysis

< How does the use case scale with an increasing number of users? >

Potential Standardization Status and Needs

<identifies potential standardization needs>

### 5.2.5	Generalized Split and Cloud Rendering and Processing

| Use Case Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Generalized Split and Cloud Rendering and Processing |  |  |  |  |  |
| Description |  |  |  |  |  |
| See several use cases in TR 26.928 should be added to the already agreed split rendering use case by reference to TR 26.928. |  |  |  |  |  |
| 3 | Streaming of Immersive 6DoF | VR | 3DoF+, 6DoF | Streaming Interactive Split | HMD with a controller |
| 4 | Emotional Streaming | 2D, AR and VR | 2D, 3DoF+, 6DoF | Streaming Interactive, Split | Phone and HMD |
| 5 | Untethered Immersive Online Gaming | VR | 6DoF | Streaming, Interactive, Split | HMD with a Gaming controller |
| 6 | Immersive Game Spectator Mode | VR | 6DoF | Streaming, Split | 2D screen or HMD with a controller |
| 21 | Immersive 6DoF Streaming with Social Interaction | VR and Social VR | 3DoF+, 6DoF | Streaming Interactive Conversational Split | XR5G-V3 XR5G-V4 |
| 22 | 5G Online Gaming Party | VR | 6DoF | Streaming, Interactive, Split, D2D | XR5G-V3 XR5G-V4 |
| 23 | Spatial Shared Data | AR | 6DoF | Streaming Interactive Conversational Split | XR5G-AX |
| Categorization |  |  |  |  |  |
| See several use cases in TR 26.928 |  |  |  |  |  |
| Preconditions |  |  |  |  |  |
| See several use cases in TR 26.928 |  |  |  |  |  |
| Requirements in terms of Capabilities and QoS/QoE Considerations |  |  |  |  |  |
| See several use cases in TR 26.928 |  |  |  |  |  |
| Feasibility and Industry Practices |  |  |  |  |  |
| See several use cases in TR 26.928 |  |  |  |  |  |
| Nominal Cost Analysis |  |  |  |  |  |
| See several use cases in TR 26.928 |  |  |  |  |  |
| Benefits and Impact |  |  |  |  |  |
| See several use cases in TR 26.928 |  |  |  |  |  |
| Potential Technical Requirements |  |  |  |  |  |
| - See several use cases in TR 26.928 |  |  |  |  |  |
| Potential Standardization Status and Needs |  |  |  |  |  |
| The following information should be added to the agreed use case. Edge/Cloud processing and rendering is a promising technology to support online gaming in power- and resource constrained devices. Relevant aspects for generalized cloud/split rendering include: - A generalized XR cloud and split rendering application framework based on a scene description - Support for 3D formats in split and cloud rendering approaches - Formats and protocols for XR Pose information delivery and possibly other metadata in the uplink at sufficiently high frequency - Content Delivery protocols that support generalized split/cloud rendering - Distributions of processing resources across different resources in the 5G system network, in the application provider domain (cloud) and the XR device. - Supporting the establishment of Processing Workflows across distributed resources and managing those - 5QIs and other 5GS/Radio capabilities that support generalized split/cloud rendering by coordination with other groups - Edge computing discovery and capability discovery based on work in SA2 and SA6 (see clause 4.3.6) |  |  |  |  |  |

### 5.2.6	Photo-realistic AR Rendering in Network

Use Case Name

Photo-realitic AR Rendering in Network

Description

Bob wants to position a complex 3D object with significant amount of data (materials, roughness, texture, transparency) and place it into a real-world environment using an AR device. For this purpose, he uses a local rendering engine, but he is very unsatisfied as natural lights are not well handled and the object looks unrealistic. A cloud service offers a PBR-based cloud rendering, and he gives it a try. He is really happy with the rendering, he gets sparkling effects and starts to move around the object and wants to put it into different angles and positions. However, moving, rotating, interacting the object makes him annoyed again as the latency of the interaction is only executed after several hundred of milliseconds. However, he gets the offering from his 5G Operator that he can get cloud-based rendering in the edge, promising latencies that makes the 3D objects present and interactive. The 5G Operator permits to ingest popular 3D objects into the network and provides an improved rendering for their consumers. Parts of the objects may be static, others may be timed. In certain cases, even several objects may be rendered at the same time. Example: https://docs.microsoft.com/en-us/azure/remote-rendering/overview/features/pbr-materials

Categorization

Type: AR, 3D Delivery: Split rendering, edge rendering Device: Phone, AR devices

Preconditions

On the UE, a 5G modem is available, an AR rendering and a depth camera is available. In the network, advanced rendering functionalities are available that are accessible with the 5GS and 5G Edge enablers and also a data storage to host complex 3D objects. An Application Service Provider supports network interfaces for ingesting popular 3D objects into the network.

Requirements in terms of Capabilities and QoS/QoE Considerations

- Potential new required capabilities of 5G Media Streaming System - Fast uplink streaming of natural environments including protocols and formats to included depth and light conditions. - Discovery and establishment of photo-realistic rendering functionalities that can work with uplink camera information. - Fast Downlink streaming to support interaction with objects including new light conditions, etc. - Formats and protocols that allow post-processing to the latest pose - Relevant KPIs - Quality of rendered scene - Immersiveness and presence of the scene - Latency of each processing step For many details, please refer to TR 26.928.

Feasibility and Industry Practices

tbd

Nominal Cost Analysis

tbd

Benefits and Impact

Potential Benefits and Impacts when running services within 5GMS network - tbd Potential Benefits and Impacts when running services on the edge - tbd

Potential Technical Requirements

Potential Requirements - tbd

Potential Standardization Status and Needs

- tbd

### 5.2.7	Media Services in the Edge

Use Case Name

Media Services in the Edge

Description

In the media production and distribution, the use of cloud services get more and more popular. Examples for standardized approaches are for example documented here: - https://www.fims.tv/, - https://tech.ebu.ch/groups/mcma - https://www.smpte.org/microservices-media-and-entertainment-key-benefits-and-challenges These applications include Artificial Intelligence (AI) enabled services, such as: speech-to-text, automatic subtitling, translation, text-to-speech, face recognition and identification, emotion detection, speech-to-text evaluation, FFMPEG media transformation. Is there a benefit to provide some applications on the edge rather than the cloud?

Categorization

Type: 2D Delivery: Cloud and edge processing Device: phone, TV, tablet

Preconditions

- Potential new required capabilities of 5G Media Streaming System - Ingest - Distribution - Relevant KPIs

Requirements in terms of Capabilities and QoS/QoE Considerations

- Potential new required capabilities of 5G Media Streaming System - tbd - Relevant KPIs - tbd

Feasibility and Industry Practices

See above links

Nominal Cost Analysis

tbd

Benefits and Impact

Potential Benefits and Impacts when running services within 5GMS network - tbd Potential Benefits and Impacts when running services on the edge - tbd

Potential Technical Requirements

Potential Requirements - tbd

Potential Standardization Status and Needs

- tbd

### 5.2.8	Partial delivery of 3D content (point cloud, mesh) for AR/MR device

Use Case Name

Partial delivery of 3D content (point cloud and mesh) for AR/MR device

Description

Service provider provides various types of contents to be overlaid over real environment for augmented reality experience. Certain 3D content is huge as the earth, which is called digital twin of past, current and future of the world. The content is live and continuously updated even though it is past. Therefore, delivering one entire content doesn’t make any sense nor possible. In this case, a partial delivery of 3D content serves user’s demand. For example, user wants to travel oversea. The user can select places to stay and place to visit, then digital twin of hotel room with a view and sightseeing points will be partially delivered to the user. More practically, only the viewport can be delivered then rendered upon user’s position and view direction. Another example is as follows. Certain 3D content has hundreds of millions of vertexes and faces to provide the finest detail as much as possible. For online catalogs of cars, selecting options to try a look and feel of each individual order is always enjoyable. User may hear engine sound and touch dashboard through the playback of such content. Since the level of details for rendering of the vertexes depends on a distance from user’s viewpoint, proper level of density and the only vertexes related with the view can be delivered. The 3D content is encapsulated for the purpose, and MEC does the required post processing to generate partial view then encoding the content.

Categorization

Type: AR, MR, VR, XR, 2D, 3D, Cloud Computing Delivery: Downlink, Interactive, Live streaming, On Demand Streaming, Conversational Device: Phone, HMD, Glasses, Laptop

Preconditions

On the content provider side, delivery or live streaming of 3D content are available. On the UE side, consumption of 3D content and reporting of relevant viewport information are available. On the network side, an MEC server is installed. Performing partial extraction, encapsulation and encoding of the 3D content are available.

Requirements in terms of Capabilities and QoS/QoE Considerations

<provides a summary on potential requirements as well as considerations on KPIs/QoE as well as QoS requirements> Potential requirements - Real-time partial extraction and control level of details of 3D content - Point cloud and mesh encoding/decoding - UE viewport information report KPI - The number of voxels and faces per second for partial extraction - The number of voxels and faces per second for encoding

Feasibility and Industry Practices

<How could the use case be implemented based on technologies available today or expected to be available in a foreseeable timeline, at most within 3 years? - What are the technology challenges to make this use case happen? - Do you have any implementation information? - Demos - Proof of concept - Existing services VR and OMAF have similar cases on sending the only part that user is viewing. For AR and 3D content, there is no such service announced so far. However, we believe it is a matter of maturity of AR ecosystem. - References Could a reduced experience of the use case be implemented in an earlier timeframe or is it even available today? >

Cost Analysis

< How does the use case scale with an increasing number of users? > Similar with split rendering since position and direction of user’s viewport would require different part of the 3D content.

Potential Standardization Status and Needs

<identifies potential standardization needs> How to exchange UE viewport information in 3D space. How to define and signal delivered part out of the total 3D content space. How to measure MEC performance index of the partial extraction, and quality of level of details.

## 5.3	Uplink Streaming Use Cases

### 5.3.1	Multi-Camera Uplink Stream Processing

Use Case Name

Multi-camera uplink stream processing

Description

Some streaming services need multi-camera based scene capture and post-processing. For example, two fisheye lenses or multiple wide-angle lenses used in VR 360 video streaming service can derive 360 ° uplink video stream by using stitching algorithms implemented in the camera or servers located near the camera. As an extension, the edge node can optimize the multi-camera uplink streaming depending on the viewport of the predominant users or the device capabilities that are consuming the content or the number of users. For instance, it can save uplink bandwidth previously occupied by non-highlighted regions. Another example is free angle live streaming, in which an arc-shaped row of time-synchronized cameras are used to collect the images of an object from different perspectives. Through the composition algorithm of adjacent servers, the uplink live stream can be generated for downlink users to freely choose the viewing angle. Another one is telepresence using AR/MR device. Multiple cameras are deployed to collect several streams, based on which the 3D mesh or 3D point cloud generation algorithms are followed. These algorithms are expected to run on edge using its computing power to meet real-time requirements and keep synchronized. The following key steps are highlighted for both of these use cases: - A camera group is set up in correct position and connects to the network, and calibration configurations which describe the relative position relationship are sent to the edge. Each camera’s exposure values are also sent. - Edge nodes set up multiple decoders and algorithms to deal with the uplink streams and output the processed result. Mainly there are two reasons supporting the edge node necessity in this use case. Firstly, VR 360 stitching, free angle video composition, and 3D mesh/point cloud producing algorithms need strong computing CPUs and GPUs rather than those integrated on mobile platforms. Put these servers on edge node instead of capture worksite can reduce space need and transportation costs, more importantly, give mobility to capture device thus clear motion limitations during application. Secondly, multiple cameras bring multiple video streams to uplink network. Taking VR 360 stitching for example, an 8K VR camera rig (KANDAO Obsidian) mostly adopted in China Mobile’s service uses 6 cameras to be stitched and each of them are 4K resolution. That means nearly 25Mbps * 6 bandwidth is the minimum requirements. Cloud based architecture feels difficult to provide stable connection to this requirement compared to edge node, and even if it can be guaranteed, longer network latency makes it difficult for streams to be synchronized under given time threshold, which are very important to all following steps.

Categorization

Type: VR 360, free angle viewing, Delivery: Uplink, Live Streaming Device: Phone, HMD

Preconditions

On devices, 5G connection ability should be installed, and a built-in or external time clock may be used to keep multiple streams to be synchronized. On the network side, multiple decoders and at least one encoder are needed to deal with media streams input and output.

Requirements in terms of Capabilities and QoS/QoE Considerations

<provides a summary on potential requirements as well as considerations on KPIs/QoE as well as QoS requirements> - Requirements - 5G connection ability on each camera - Real-time multiple decoding and encoding instrument on edge - High CPU/GPU computing ability need on edge server to process multiple streams - Real-time Stitching Algorithms for multiple streams and Image processing - KPI - Steady uplink bandwidth more than N*k*(Single-Stream-Bitrate) NOTE: N is the number of streams, and k is the parameter of bitrate to network bandwidth, usually can be set to 1.5 or 2 - Scalability of the solution with the increase of cameras and/or sessions

Feasibility and Industry Practices

<How could the use case be implemented based on technologies available today or expected to be available in a foreseeable timeline, at most within 3 years? - What are the technology challenges to make this use case happen? - Do you have any implementation information? - Demos - Proof of concept - Existing services - References - Could a reduced experience of the use case be implemented in an earlier timeframe or is it even available today? > For VR 360 streaming service, China Mobile has tested cloud-based stitching using 5G SA uplink with professional VR camera system. And there are also some mature business solutions existing, for example cloud stitching product provided by StreamboxVR. Here is the link of introduction and demo: https://www.streamboxvr.com/cloud_stitching/ Using a Ricoh Theta S 360 camera, together with Avenir Micro or Drone, pre-stitched streams can be sent to Streambox Cloud, and users can directly get a VR 360 live streaming service. For free angle live streaming service, China Mobile has made a live streaming application used in “The 2nd Youth Games of the People’s Republic of China”, together with partners including ZTE and Qualcomm. It uses 5GS and Edge computing to realize free angle viewing of wrestling matches. The link of this demo is: https://www.iqiyi.com/v_19rtgqhknc.html Users can choose their view angle freely at any time during streaming service. For telepresence service using AR/MR device, there are also many implementations. https://m.youtube.com/watch?v=4oJJN2eH6U0 For stitching streams reflecting highlight regions as followed:

Cost Analysis

< How does the use case scale with an increasing number of users? > This use case mainly provides uplink solutions, generated media streams are served to subsequent operators, thus the cost for downlink stage and user side does not belong to its costs. For the service itself, it needs to pay for multiple 5G modem-integrated cameras, edge computing hardware and software.

Potential Standardization Status and Needs

<identifies potential standardization needs> - Functionality on the edge node - data transfer between UE and edge node Note: other processing may be added.

### 5.3.2	Cloud/Split Rendering of Immersive Live Events

Use Case Name

Cloud/Split Rendering of Immersive Live Events

Description

In use case 3 in TR26.928, Annex A.4, streaming and consumption of a recorded highlight of basketball match is considered, being technology agnostic in a sense that rendering may be done in the device only or may be done completely or at least partially in the network. In this use case, the scenario from TR26.928 is extended to streaming of immersive live events and rendering is done at least partially in the network. The immersive live event may be a basketball match that is produced by highly-professional multi-camera arrays, possibly several 100s of them and the information is provided to a cloud-environment for pre-processing that data. The immersive 6DoF live media content is offered and shared with a remote audience for streaming access can place the remote audiences within the context of a scene and give them immersive experiences: - Controlling the orientation of the scene and view direction, i.e. a 6DoF pose. For example, when a remote participant with an XR display (e.g., HMD, AR glass) rotates his head, the XR display can extract the 6DoF pose and provides it to a remote rendering. - The content is at least partially pre-rendered in the network according to the latest pose provided from the user in order to minimize the traffic being sent on the downlink as well as to adjust the rendering to the device capabilities. In order to control the immersive latency requirements properly, rendering in the network needs to be timely close to the latest user pose – justifying the usage of an edge node. - The edge nodes process the latest 6DoF pose, and generate a pre-rendered version of the live scene for the user’s predicted pose. This pre-rendered viewport is encoded, sent and decoded at the remote device. The XR display can finally render the scene that he is currently watching based on the received viewport, possibly taking into account local pose corrections to achieve a full immersive experience. . - In addition, the content may further be augmented with overlays, graphics or other data. The data may either be provided for all users or may be user-targeted. For example, the director overlays some value-added information (e.g., the real-time game scores, the sponsor's advertisements) on top of the scene and then distributes the rendered scene to the remote audiences. Especially, if the director can dynamically plant the wonderful slow slots or 3D animated models in a certain region of the scene (e.g., the lower right), a remote viewer can use his finger to zoom or rotate them to look.

Categorization

Type: VR, AR, Edge Computing, GPU Delivery: Downlink, Live Streaming, Split Rendering Device: HMD, AR glass, Smartphone

Preconditions

On the device - Application is installed that permits to consume the scene - The application uses existing HW capabilities on the device, including A/V decoders, rendering functionalities as well as sensors. Inside-out Tracking is available. On the network, - Cloud computation capabilities are available to produce the content - Cloud/edge computation capabilities are available to properly pre-render and encode the content

Requirements in terms of Capabilities and QoS/QoE Considerations

- QoS - Bitrates and latencies in the uplink are sufficient to provide an up-to-date pose to the cloud computation. - Bitrates and Latencies that are sufficient to render the viewport within the immersive limits - Sufficient bandwidth to deliver multiple media streams - QoE - fast reaction to manual controller information, - reaction to head and limited body movement within immersive limits, - seamless experiences when moving across positions - providing sufficient AV experience to enable presence. https://xinreality.com/wiki/Presence

Feasibility and Industry Practices

<How could the use case be implemented based on technologies available today or expected to be available in a foreseeable timeline, at most within 3 years? - What are the technology challenges to make this use case happen? - Do you have any implementation information? - Demos - Proof of concept - Existing services - References - Could a reduced experience of the use case be implemented in an earlier timeframe or is it even available today? >

Cost Analysis

< How does the use case scale with an increasing number of users? >

Potential Standardization Status and Needs

<identifies potential standardization needs>

### 5.3.3	Pandemic Stadium

Use Case Name

Pandemic Stadium

Description

In an extension to the use case 2.2, the stadium is empty because of a pandemic, but the remote users want to interact with the live event, and even more, the teams and competitors in the stadium get aggregated feedback based on the reactions from the remote users in real-time. Due to XR technologies, a remote audience can interact with both the main characters of live events and the other remote audiences to get more immersive experiences as followed: - Interacting with the other remote audiences who are geographically distributed. For example, when a remote audience with his friends who are in the virtual environment are watching the same concert, they are able to have voice conversation with each other. They can even cheer the main singer to the echo which can be fed back into the stadium. A network processing server can aggregate the voices from the remote audiences and reflect them to the stadium. - Interacting with the main characters of live events (e.g., the signer in the concert) remotely. For example, if a remote audience with a mobile device (e.g., a tablet or smartphone) virtually watching the concert where there is a 3D body model of the singer on the top of the scene, he can apply a selected set of 2D/3D AR effects to this 3D body model. He is also able to select his reconstructed 3D model which can reflect his body movements, and overlay it to the scene. It looks like he is with the main singer together. Furthermore, a cloud processing entity can distribute the rendered scene including the two 3D models to the other remote audiences. - In the cases documented above, latencies are critical in order to provide a “presence” feeling for both the audience and the actors/players The use cases are the extensions to those discussed in clause 2.1, the media rendering procedures are similar to these ones descripted in clause 2.1.

Categorization

Type: AR, VR, MR, Edge Computing, GPU, Social Interaction Delivery: Downlink, Live Streaming, Interactive, Split rendering Device: HMD, Glasses, smartphones

Preconditions

On the device - Application is installed that permits to consume the event and participate in the event - The application uses existing HW capabilities on the device, including A/V encoders and decoders, rendering functionalities as well as sensors On the network, - Cloud computation capabilities are available to aggregate the user input - Cloud/edge computation capabilities are available to render At the live event - Recording and playout functionalities to sense the remote audience

Requirements in terms of Capabilities and QoS/QoE Considerations

- QoS - Bitrates and latencies in the uplink are sufficient to provide user interaction to the cloud computation. - Bitrates and Latencies that are sufficient to render the viewport within the immersive limits - Sufficient bandwidth to deliver multiple media streams - Sufficiently low latencies end-to-end to provide a presence sence - QoE - providing sufficient AV experience to enable presence. https://xinreality.com/wiki/Presence - latencies low enough to provide a sense of crowd-reaction remote - Synchronization of user voice communication with the corresponding actions - more details are necessary.

Feasibility and Industry Practices

<How could the use case be implemented based on technologies available today or expected to be available in a foreseeable timeline, at most within 3 years? - What are the technology challenges to make this use case happen? - Do you have any implementation information? - Demos - Proof of concept - Existing services - References - Could a reduced experience of the use case be implemented in an earlier timeframe or is it even available today? >

Cost Analysis

< How does the use case scale with an increasing number of users? >

Potential Standardization Status and Needs

<identifies potential standardization needs>

# 6	Potential 5GMS Architecture Extensions

## 6.1	General

Editor’s Note: This clause will document the identified gaps. Identified gaps may go beyond the scope of the SA4 work, in which case the responsible groups will be contacted.

Editor’s Note: The potential extensions are mainly based on the identified relevant use cases in section 5.

## 6.2	EMSA Architecture

The EMSA architecture is an integration of the 5GMS architecture, the SA6 Edge architecture and the SA5 management architecture. The EMSA architecture is depicted in the following figure:

Figure 7: The reference edge-enabled 5GMS media architecture

The above architecture demonstrates how the edge-enabled 5GMS media architecture leverages the edge functionality and architectural elements specified in [3]. In this illustration, the EEC, EES, and EAS are shown as parts of the MSH, 5GMS AF, and 5GMS AS, respectively.

This architecture implies the following:

- A Media Session Handler (MSH) that is edge-enabled is required to implement the EDGE-5 API.

- A 5GMS AF that is edge-enabled is required to support EES functionality including:

  - EDGE-3 API towards the EAS function of 5GMS AS instances.

  - EDGE-6 API for registering with an ECS function.

  - EDGE-9 API for media session relocation.

  - EDGE-1 API for supporting registration and provisioning of EEC functions, and discovery by them of EAS instances.

- A 5GMS AF that is edge-enabled may perform compute resource allocation using the MnS-C interface.

- A 5GMS AS that is edge-enabled is required to support EAS functionality including EDGE-3 API.

In general, in order to deploy edge support for the 5GMS application, each 5GMS function has to become edge-enabled and a union of each 5GMS function with its corresponding edge function (MSH + EEC, AF + EES, AS + EAS) would satisfy the requirements. In this regard, various deployment scenarios including the standalone existence of the edge functions can be considered.

Editor’s Note: Examples of the different deployment scenarios will be shown in Annex A.

This edge-enabled 5G media streaming architecture supports both client-driven as well as AP-driven management of the edge processing session.

In the client-driven approach, the 5GMS-Aware Application is aware of the support of edge processing in the network and takes steps, such as using the EDGE-5 APIs, to discover and locate an application server in the edge DN.

In Application Provider (AP)-driven approach, the 5GMS Application Provider provisions 5GMS features as normal within the scope of an M1 Provisioning Session, but additionally requests the 5GMS System to accommodate the provisioned features using edge computational resources, where appropriate. The 5GSMS AF acts on behalf of the 5GMS Application Provider to allocate processing resources based on the application’s needs. In order for this transparent resolution to work, SA2 is expected to define a solution to allow the AF to influence the DNS resolution for the domain name(s) related to the media distribution by the Application Provider as described in clause 4.3.2.

## 6.3	Generic Call Flow for Session Establishment

### 6.3.1	General

Two generic call flow for edge-enabled 5G media streaming session establishment are demonstrated below.

### 6.3.2	Client-driven edge discovery

Figure 8 outlines a detailed call flow for client-driven session establishment.

Figure 8: Client-driven session establishment

The steps are:

- Edge Computing Provisioning phase:

  - Spawn ECS

  - Spawn 5GMS AF

  - EES Configuration

  - EES Registration with ECS

- 5GMS Application Provider Provisioning phase:

- Create Provisioning Session

- Provision 5GMS features

- UE Edge Computing Discovery phase:

- Application Initialization

- Locate EAS/5GMS AS

- Locate local EES

- Locate EAS/5GMS AS

Optional subflow to provision an additional 5GMS AS instance (if needed):

- Check resource template

- Instantiate new EAS/5MGS AS

- Spawn 5GMS AS instance

- EAS configuration

- Register EAS with EES

- Configure provisioned features

Completion of UE Edge Computing Discovery phase:

- 5GMS AS location response

- 5GMS AS location response

- 5GMS Session phase

- Start session

- Session starting event

- Retrieve service access information

- Media transfer

- Method calls and notifications

- Reporting, network assistance, and dynamic policy

- End session

- Session ending event

- Final reporting

### 6.3.3	AP-driven management of 5GMS edge processing

Figure 9 outlines a detailed call flow for the AP-driven management of edge processing. In the previous sequence, the optional provisioning of an additional 5GMS AS instance occurs in response to an explicit call from the Application Client to an on-board EEC, whereas in this sequence it occurs (if needed) in response to Application Provider provisioning. It may also be repeated at any time during the lifetime of the 5GMS Provisioning Session in response to satisfy demand levels (not illustrated for brevity).

Figure 9: AP-driven management of 5GMS edge processing

The steps are:

- Edge Computing Provisioning phase (same as before):

  - Spawn ECS

  - Spawn 5GMS AF

  - EES Configuration

  - EES Registration with ECS

B.	5GMS Application Provider Provisioning phase:

  - Create Provisioning Session

  - Provision 5GMS features

Optional sub-flow to provision an additional 5GMS AS instance (if needed, same as before):

  - Check resource template

  - Instantiate new EAS/5MGS AS

  - Spawn 5GMS AS instance

  - EAS configuration

  - Register EAS with EES

  - Configure provisioned features

D:	5GMS Session phase (same as before):

  - Start session

  - Session starting event

  - Retrieve service access information

  - Media transfer

  - Method calls and notifications

  - Reporting, network assistance, and dynamic policy

  - End session

  - Session ending event

  - Final reporting

Annex <A> (normative):
Deployment Scenarios

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 12th February 2021 | 112-e | S4-210313 |  |  |  | Documentation of relocation solutions |  |
| 12th February 2021 | 112-e | S4-210314 |  |  |  | Updates to EAS discovery |  |
| 12th February 2021 | 112-e | S4-210320 |  |  |  | EMSA architecture |  |