3GPP TSG SA WG4#116-e meeting	                                s4-211397

revision of S4aI211245

revision of S4aI211239

10th-19th November, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.502 | CR | – | rev | – | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR to TS 26.502 on reference architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TELUS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBUSA |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification). |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Added text in reference architecture for 5G Multicast-Broadcast User Services |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Several clauses |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes against skeleton document TS 26.502 v0.1.0 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

[7]		3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[8]	IETF RFC 3500: "RTP: A Transport Protocol for Real-Time Applications".

[9]	IETF RFC 2250: "RTP Payload Format for MPEG1/MPEG2 Video".

NEXT CHANGE

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

MBS Application Service: An end-user service for which parts or all of the data are accessible by joining an MBS User Service.

MBS User Service: An abstract transport-level service configured by the MBSF and using one or more MBS Delivery Sessions, possibly in combination with unicast, for the purpose of supporting an MBS-Aware Application via a set of APIs that allows the MBS Client to activate and deactivate reception of the MBS session.

NEXT CHANGE

## 4.2	System description

Editor’s Note: Explanation of fundamental concepts in the MBS User Services architecture.

### 4.2.1	Network architecture

Editor’s Note: How this specification relates to the SA2 architecture in TS 23.247.

Figure 4.2.1-1 depicts the MBS network architecture using the reference point representation as defined in TS 23.247 [5].

Figure 4.2.1-1: Network Architecture for MBS User Service Delivery and Control

The functions and reference points involved in providing User Service Specification within the MBS System are highlighted in green, in particular,

- The Nmb2 interface between MBSF and MBSTF for enabling configuration and control of the Delivery Methods

- The reference point Nmb10 (or Nmbsf service) for provisioning the MBSF

- The reference point Nmb8 (or Nmbstf service) for ingesting content into the MBSTF

### 4.2.2	User Service architecture

MBS User Services enable high-level applications to make use of the low-level features of the MBS System. The MBS User Service is provided by the MBSF and MBSTF working in combination. It presents a complete service offering to an end-user, via a set of APIs that allows the MBS Client to activate or deactivate reception of the service.

MBS User Service architecture in Figure 4.2.2-1 shows MBS-related entities involved in providing MBS User Service delivery and control.

Figure 4.2.2-1: MBS User Service Architecture

### 4.2.2.1	MBSF

MBSF supports the following MBS User Service functions

- User Service announcement

- Data model for MBS services including a session model and its mapping to MBS transport

The User Service Discovery/Announcement function of the MBSF provides session access information, which is consumed by the MBS Client and subsequently used to initiate the reception of a MBS User Service. The session access information may contain information for presentation to the end-user, as well as application parameters used in generating service content to the MBS Client.

The MBS Application Provider and MBSF interact to support MBS session operations and transport in the control plane via Nmb10 reference point. The MBS Application Provider may also interact with NEF for MBS related service exposure, and support MBS procedures including service provisioning, MBS session operations and QoS management.

4.2.2.2 	MBSTF

The MBSTF performs generic packet transport functions available to any IP multicast-enabled application, such as framing, multiple flows and packet FEC (encoding). It also performs multicast/broadcast delivery of input files as objects or object flows. If needed, the MBSTF provides a media anchor point for MBS data traffic and sourcing of IP multicast.

MBSTF supports the following MBS User Service functions

- Delivery Methods to allow MBS User Services to use available MBS capabilities, including support for Group Communication delivery

- File repair

- Reception reporting

The MBS Application Provider and MBSTF interact to support MBS data handling in the user plane via Nmb8 reference point.

### 4.2.3	Delivery methods

Editor’s Note: Explanation of what a delivery method is and what delivery methods are for.

MBS delivery methods make use of MBS sessions to deliver data to MBS Client. A set of MBS Delivery Methods is provided by the MBSTF. These provide functionality such as security and key distribution, reliability control (by means of FEC techniques) and associated delivery procedures.

-	Object delivery method: Delivers discrete binary objects over an MBS Session. This may be used to support real-time delivery of media segments (as special objects) including Low-Latency CMAF delivery.

The use of MBS Sessions by the Object delivery method is specified in clause 6.1.

-	PDU delivery method: Supports IP streaming use cases for which UDP payloads (also referred to as Service Data Unit) are distributed as part of UDP or IP flows carried to the UE over an MBS session. Examples of higher layer protocols are RTP [8] and packetized MPEG-2 Transport Stream [9].

The use of MBS Sessions by the PDU delivery method is specified in clause 6.2.

Editor’s Note: The name of this delivery method is pending to further discussion, for example also transparent mode was suggested. The discussion was around whether to describe the MBS delivery function or the service that is supported to the outside by the delivery.

The above Delivery Methods may use either a multicast MBS session or a broadcast MBS session to deliver content to a receiving application, and may also make use of a set of MBS associated delivery procedures.

NEXT CHANGE

## 4.3	Functional entities

Editor’s Note: Reference architecture for MBS User Services, including client functions.

Figure 4.3-1 shows the complete set of functional entities involved in supporting MBS User Services, including client functions.

Figure 4.3-1 5G Multicast Broadcast User Service Architecture

In the architecture above, MBS-specific functions such as the MBS AS and MBSF are shown as independent and standalone. In deployments, they may be co-located on physical devices with other functions. As an example, the MBS AS may be hosted in the Application Provider domain, or it may be hosted in a 5GMS AS.

Reference points already defined in TS 23.247 [5] that are of relevance for the User Service architecture are:

-	Nmb1: Reference point between the MB-SMF and the MBSF.

-	Nmb2: Reference point between the MBSF and the MBSTF (may be MBS-3).

-	Nmb8: Reference point between the MBSTF and the AF (may be named MBS-2).

-	Nmb9: Reference point between the MB-UPF and the MBSTF.

-	Nmb10: Reference point between the MBSF and the AF (may be named MBS-1).

-	Nmb12: Reference point between the MBSF and the PCF.

The following additional reference points are defined:

-	MBS-5: API from the MBS Client to the MBSF for the purpose of MBS control plane and service handling referred to as interface MBS-5. It is expected that this API has similar functionalities to those of the User Service Description as defined in TS 26.346.

-	MBS-4-MC: dealing with unidirectional and multicast delivery from the MBSTF to the MBS Client. It is expected that this interface has similar functionalities as defined in the delivery methods defined in TS 26.346.

-	MBS-4-UC: dealing with unicast delivery from the MBSTF to the MBS Client. It is expected that this interface has similar functionalities as defined in the delivery methods defined in TS 26.346 provided for unicast.

-	MBS-6: API-based interface exposed by the MBS Client and used by the MBS-Aware Application to manage and control MBS User Services. It is expected that this API has similar functionalities to the control interfaces defined in clause 6 of TS 26.347.

-	MBS-7: API-based interface MBS-7 exposed by the MBS Client and used by the MBS-Aware Application to receive user data information about MBS User Services. It is expected that this API has similar functionalities as the data interfaces defined in clause 7 of TS 26.347.

-	MBS-8: Interface between the MBS Application Provider and the MBS-Aware Application in order to announce MBS User Services.

There are no reference points defined for MBS AS at this stage.

END OF CHANGES