| 3GPP TR 26.806 V0.0.1 (2022-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group SA; Study on Tethering AR Glasses – Architectures, QoS and Media Aspects (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Motivation for Media Service Enablers	7

5	Properties and Functionalities of MSE	7

6	Specification Template for MSE	8

7	Potentially Relevant 5G Media Service Enablers	8

8	Conclusions and Recommendations	8

Annex <A> (informative): <Normative annex for a Technical Specification>	8

Annex <X> (informative): Change history	10

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

For initial AR experiences, an expected prominent setup will be wirelessly tethered of AR glasses, typically connected to a 5G UE. The tethering technology between a UE and an AR glass may use different connectivity, for example provided through WiFi or 5G sidelink. Different architectures for tethering result in different QoS requirements, session handling properties, and also media handling aspects. For enhanced end-to-end QoS and/or QoE, AR glasses may need to provide functions beyond the basic tethering connectivity function. Generally, smartly tethering AR glasses is an important aspect for successful AR experiences using the 5G System.

Based on this, the present document introduces Smartly Tethering AR Glasses (SmarTAR) in such user experience may be maximized using the 5G System.

# 1	Scope

The present document addresses architectures, QoS and media handling aspects of when tethering AR Glasses to 5G UEs based on initial discussions in TR 26.998 [2]. In particular, the following aspects are in scope:

-	Definition of different tethering architectures for AR Glasses including 5G sidelink and non-5G access based on existing 5G System functionalities

-	Documentation of the relationship between AR Glasses tethering and AR glasses considered as PIN (Personal IoT Network) elements according to TR 22.859 [3] and the derived service requirements in TS 22.261 [4].

-	Documentation of end-to-end call flows for session setup and handling

-	Identification media handling aspects of different tethering architectures

-	Identification of end-to-end QoS-handling for different tethering architectures and define supporting mechanisms to compensate for the non-5G link between the UE and the AR glasses

-	Providing recommendations for suitable architectures to meet typical AR requirements such as low power consumption, low latency, high bitrates, security and reliability.

-	Collaboration with relevant other 3GPP groups on this matter

-	Identification of potential follow-up work on this matter

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[3]	3GPP TR 22.859: "Study on Personal Internet of Things (PIoT) networks".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AR	Augmented Reality

PIN 	Personal IoT Network

# 4	Motivation and Background

Editor’s Note:

- Summarize the TR 26.998 major findings.

- Add on or several guiding use cases

- Introduction to PIN

# 5	Tethering Architectures and Call Flows

Editor’s Note:

- Definition of different tethering architectures for AR Glasses including 5G sidelink and non-5G access based on existing 5G System functionalities

- Documentation of end-to-end call flows for session setup and handling

- Identification media handling aspects of different tethering architectures

- Identification of end-to-end QoS-handling for different tethering architectures and define supporting mechanisms to compensate for the non-5G link between the UE and the AR glasses

# 6	Tethering and Personal IoT Network

Editor’s Note

- Documentation of the relationship between AR Glasses tethering and AR glasses considered as PIN (Personal IoT Network) elements according to TR 22.859 [3] and the derived service requirements in TS 22.261 [4].

# 7	Identified Key Issues and Potential Solutions

Editor’s Note

- Providing recommendations for suitable architectures to meet typical AR requirements such as low power consumption, low latency, high bitrates, security and reliability.

# 8	Potential Next Steps

Editor’s Note

- Identify concrete work topics that need to be addressed based on the analysis

# 9	Conclusions and Recommendations

Editor’s Note

- Provide conclusions on how to address the potential next steps in clause 8

Annex <A> (informative):
<Normative annex for a Technical Specification>

Start each annex on a new page.

Annexes are labelled A, B, C, etc. and designated either "normative" or "informative" depending on their content.

Normative annexes only to appear in Technical Specifications. Use style "Heading 8".

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-03 | SA4#118 | S4-220361 |  |  |  | Initial version | V0.0.1 |