3GPP TSG-WG SA4 Meeting #118E e-meeting  	S4-220478

Online, April 6th – 14th, 2022

Title:                      LS Reply on QoS support with PDU Set granularity

# Response to:	LS (S2-2201803/S2-2201851/23700-60)

# Release:	Rel-18

# Work Item:	FS_XRM

Source:	SA4

To:	SA2

Cc:	RAN1, RAN2, RAN3

Contact Person:

Name:	Shuai Zhao

E-mail Address:	shuai.zhao@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

SA4 thanks SA2 about the LS on QoS support with PDU Set granularity. SA4 would like to provide the following feedback:

- provide any feedback/guidance as necessary on the PDU Set definition and Video Slice definition in the attached document S2-2201851.

Answer:

- w.r.t PDU Set definition: The SA4 understanding from TR 23.700-60 is that 5GS would like to optimize the use of radio resources when transporting XR by selective dropping of PDUs in a PDU Set, or an entire PDU Set, in case the delivery of this traffic cannot be used by the XR Client in the receiver. SA4 would like to note that a PDU Set may carry the payload for an encoded video bitstream, as well as audio/haptics. From those perspectives, the definition looks good.

- w.r.t Video slice definition: On a high-level, current definition in TR 23.700-60 is correct.

- provide any input on traffic characteristics of typical media services that rely on e.g., PDU Sets, as necessary.

Answer: Few observations from SA4 regarding media traffic characters which may have an impact on a PDU or PDU sets:

- The size of encoded video payload for each type of video frame such as I/P/B may be bigger than one MTU (ex. 1500bytes).

- One PDU may only be able to carry one type of encoded video frame such as either an I or P/B frame due to packet size limitations.

- The encoded bitstream may be encrypted per PDU.

- clarify what, if any, dependency there is between the IP packets that make up PDU Sets, e.g. a frame/”slice”.

Answer: SA4 understands the question is regarding the dependency between different video frame types in a single PDU set. Feedback below:

- The common size for an I-frame may be over one MTU (e.g., 1500 bytes)

- One PDU may only carry the payload for one type of video frame.

- If a lost PDU contains an I-frame, then the rest of PDUs in that PDU set may also be dropped.

- If a lost PDU contains a P/B frame, then the rest of the PDUs in that PDU set may also be dropped.

- clarify what, if any, dependency there is between PDU Sets that may carry, e.g. different frames/”slices”

Answer: SA4 understands the question is regarding

Answer: SA4 understands the requestion is regarding the dependency on video frame types among multiple PDU sets. Feedback below:

- If a lost PDU contains an I-frame, then the rest of the PDUs in that PDU set may also be dropped. (Same as Q3(c)).

- At the same time, the corresponding PDU set(s) containing referencing P/B frames may also need to be dropped.

- provide feedback on the “Annex X (informative): Traffic characteristics of XR and media services” in the attached document S2-2201807

Answer: In General, the information provided in S2-2201807 Annex is valid.

2. Actions:

To SA2 group.

ACTION: 	SA4 asks SA2 group to take the above reply into account and provide feedback if any.

3. Date of Next TSG SA WG4 Meetings:

SA4#119-e	11 – 20 May 2022		E-Meeting