3GPP TSG-S4 Meeting #119e	S4-220730

Electronic Meeting, 11th to 20th May 2022

Title:	Summary for 5MBUSA “5G multicast-broadcast services User Service architecture”

Type:	Work Item Summary

Agenda Item:	8.13

Source:	TELUS, Peng Tan

Contact:	peng.tan@telus.com

This document is a contribution to TR 21.917 ("Release 17 Description; Summary of Rel-17 Work Items"). It is meant to be endorsed by the plenary.

Summary based on the input provided by TELUS in S4-220730.

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 920010 | 5G Multicast-Broadcast User Service Architecture and related 5GMS Extensions | 5MBUSA | S4 | SP-210376 | PENG TAN, TELUS |

Introduction

In TS 26.502 [1], this work item specifies 5G Multicast-Broadcast User Services architecture, its procedures, Object Distribution Method, Packet Distribution Method, as well as services operations provided by MBSF and MBSTF network functions.

Description

5G Multicast-Broadcast User Services enable high-level applications to make use of the low-level features of the MBS System defined in TS 23.247 [2]. An MBS User Service is provided by the MBSF and MBSTF working in combination to support configuration option 2 and configuration option 3 defined in annex A of TS 23.247. It also support interworking between MBS and eMBMS as described in Clause 5.2 of TS 23.247.

Figure 1: Network architecture for MBS User Services and MBS-eMBMS interworking

In Figure 1, the functions and reference points involved in providing MBS User Services and MBS-eMBMS interworking are highlighted in green. Additional reference points as shown in Figure 2 are defined by TS 26.502 [1].

-	MBS-4-MC: Unidirectional multicast distribution of content from the MBSTF to the MBS Client.

-	MBS-4-UC: File-based unicast repair between the MBS Client and the MBS AS.

-	MBS-5: Interactions between the MBS Client and the MBSF for the purpose of MBS control plane and service handling.

-	MBS-6: API exposed by the MBS Client and used by the MBS-Aware Application to manage and control MBS User Services.

-	MBS-7: API exposed by the MBS Client and used by the MBS-Aware Application to receive user data information distributed using MBS User Services.

-	MBS-8: Announcement of MBS User Services to the MBS-Aware Application by the MBS Application Provider.

Figure 2: MBS User Service and MBS-eMBMS interworking reference architecture

The work item aims to extend the 5G Media Streaming (5GMS) architecture by providing a general description and architecture for extensions of 5GMS via 5G MBS, 5GMS hybrid services, 5GMS via eMBMS and Multicast ABR in TS 26.501 [3], with detailed Protocols and Formats further specified in TS 26.517 [4].

References

[1]	TS 26.502: “5G multicast-broadcast services, User Service architecture”

[2]	TS 23.247: “Architectural enhancements for 5G multicast-broadcast services”

[3]	TS 26.501: “5G Media Streaming (5GMS); General description and architecture”

[4]	TS 26.517: “5G Multicast-Broadcast User Services; Protocols and Formats”