3GPP TSG-S4 Meeting #119-e	TDoc S4-220798

Online, 11th – 20th May 2022

Title:	Presentation of Specification to TSG:
TS 26.532, Version 2.0.0

Source:	Editor (Qualcomm)

Document for:	Agreement

Abstract of document:

The EVEX Work Item identified the creation of TS 26.532 as stage 3 specification for UE data collection and reporting functionality, to complement the stage 2 specification in TS 26.531 on generic data collection and reporting architecture.

TS 26.532 V0.3.0 specifies the following functionality:

1.	Detailed description of network-side, UE-to-network, and UE-internal procedures involving configuration, collection, reporting and event exposure of UE application-specific data (“UE data”) interactions between functional entities and reference points as described by the generic network architecture in TS 26.531.

2.	General aspects of the APIs for UE data collection and reporting including HTTP URL structure and methods, and applicable data types

3.	Definition of the Data Reporting Provisioning service API, used by an Application Service Provider to provision a data collection and reporting configuration in a Data Collection AF.

4.	Definition of the Data Reporting service API, used by clients of a Data Collection AF to obtain their data collection and reporting configuration from, and then report data to, the Data Collection AF.

5.	Definition of the UE-internal or Client API offered by the Direct Data Collection Client to the UE Application for mutual exchange of relevant data.

6.	Specification of the data models associated with the various UE data collection and reporting procedures.

6.	OpenAPI representation of REST APIs associated with UE data collection and reporting.

The APIs associated with event exposure services provided by the Data Collection AF to consumer entities, along with APIs associated with NEF mediation of UE data collection and reporting related interactions between network entities, are specified by CT3 in their relevant specifications.

Changes to presentation of V1.0.0 to SA Meeting #95:

- Addition in clause 4 of call flow diagrams and corresponding descriptions of network-internal, UE-internal and UE-to-network interactions regarding the configuration and reporting of UE data between the Data Collection AF and different types of data collection clients.

- Description in clause 5 of HTTP protocol usage for the access to RESTful and regular HTTP resources associated with UE data collection and reporting and offered by the Data Collection AF.

- Specification in clause 6 of the Ndcaf_DataReportingProvisioning service and associated resources and API methods offered by a Data Collection AF to the Provisioning server (‘Provisioning AF’) of an Application Service Provider. This service and associated API enable the Provisioning AF to provision the Data Collection AF with its UE data collection, reporting and event exposure configuration, a portion of that information is intended to be forwarded by the Data Collection AF to its data collection clients. The provisioning information may contain one or more sets of event exposure restrictions to be followed by the Data Collection AF in controlling the amount or granularity of UE data related information accessible to different consumer entities via Event services offered by the Data Collection AF.

- Corrections, additions to, and reorganization of clause 7 regarding the Ndcaf_DataReporting service and associated resources and associated API methods offered by a Data Collection AF to its data collection clients. This service and associated API enable a data collection client to obtain its UE data collection and reporting configuration from and subsequently send collected UE data reports to a Data Collection AF, according to its configuration.

- Specification in clause 8 of UE-internal procedures between an optionally instantiated UE Application entity and the data collection client regarding the acquisition of a data collection and reporting configuration by the UE Application and subsequent reporting of collected UE data in accordance with that configuration to the data collection client.

- Specification in clause 9 of security and access control mechanisms associated with the use of HTTP methods by a service consumer to access and manipulate resources offered by a service producer with regards to UE data collection, reporting and event exposure functionality.

- Definition in Annex B of the OpenAPI representations of the various REST APIs associated with UE data collection, reporting and event exposure.

Outstanding Issues:

None.

Contentious Issues:

There are no known contentious issues at this point.