Source: 	NTT

Title: 	Proposals for FS_eiRTCW Control Plane

Document for:	Agreement

Agenda item: 	10.6

# Introduction

This contribution gives change proposals on the permanent document of Study on enhancements on immersive Real-Time Communication for WebRTC (FS_eiRTCW).

Note: According to the discussion in the last RTC SWG session in April 2022, this study keeps independent from work item iRTCW in terms of C-Plane signalling and maintains its own permanent document.

# Proposal

It is proposed that the revision of the permanent document for FS_eiRTCW should be created, reflecting all the below changes.

# Changes

All descriptions are added for empty section.

--- Start change#1 ---

- 5	Requirements for C-Plane Signalling

## 5.1	General

This clause identifies requirements for control plane signalling for each reference point. In this study, reference points for control plane are Rs-u, Rs-i, Rs-n, Rs-a, as described in clause 4.

## 5.2	Rs-u

### 5.2.1	Base Protocol

HTTP[RFC7230-7235][RFC7540]/HTTPS and WebSocket[RFC6455] are available options for signalling between UE and WSF so that connection setup procedure could be invoked by JavaScript API as described in IETF RFC 8825 [RFC8825] clause 3. Nevertheless, HTTP/HTTPS is less appropriate for two reasons described in RFC 6455 clause 1.1:

-	Server load caused by http transactions (based on request-response)

-	A connection has two sessions each for sending and receiving signalling packets

In addition, when a notification from the network to the UE is required, for such as an incoming call, an HTTP(S) connection is originated from the network side, but this case has some problem. Generally, NAT box is placed between UE and network entities, therefore NAT-traversal problem should be resolved. Besides, in terms of security configuration, UEs often deny incoming TCP[RFC793] connections.

For that reasons mentioned above, only WebSocket is utilized for the base protocol of signalling in this study. WebSocket can solve the three problems, server load, number of sessions and the NAT-traversal.

### 5.2.2	Upper Layer Protocol over WebSocket

In IETF RFC 8825 [RFC8825], upper layer protocols over the base protocol of control plane are not specified and are thought to be application specific. In the RFC, SIP[RFC3261] and XMPP[RFC6120] are listed as candidate protocols for control plane.

#### 5.2.2.1	SIP

Utilizing SIP for control plane signalling for WebRTC is already described in 3GPP TS 24.371 [TS24.371] clause 5. One of the main advantages of using SIP is the ease of interwork between WebRTC-aware network and IMS network. On the other hand, disadvantages of using SIP are as follows:

-	UE and network must be able to understand both WebRTC and SIP. SIP is not widely used outside of telephony. If SIP must be used in conjunction with WebRTC, the advantage of WebRTC, friendliness to web-based development environments and developers, is to be spoiled.

-	SIP has a strictly managed communication model as SIP dialogue. In principle, the originatedal signalling is transparently relayed through the network and the terminals manage the dialogue with each other. These characteristics are not compatible with the UE-network relation model, which is the scope of this study.

-	UE and network must be able to understand both WebRTC and SIP. SIP is not widely used outside of telephony. If SIP must be used in conjunction with WebRTC, the advantage of WebRTC, friendliness to web-based development environments and developers, is to be spoiled.

-	SIP specifies methods divided by each signalling characteristic (i.e., INVITE, ACK, BYE, CANCEL, PRACK, UPDATE, SUBSCRIBE, NOTIFY, REFER, PUBLISH, INFO). Adding control for a new characteristic may need to start from the method definition.

-	Less affinity with cloud environment which mainly uses HTTP. For example, raw values of the IP addresses related to the SIP dialog (consisting a communication path of SIP trapezoid) are in the protocol header or message body, therefore changing communication elements is difficult once the call session is established.

For those reasons above, SIP is not appropriate except for the applications where the interwork to IMS is expected.

#### 5.2.2.2	XMPP

There is no specification using XMPP for the upper layer protocol of the control plane signalling in 3GPP and no major commercial implementations of WebRTC either. The reason seems that XMPP can be used on its own and does not need to be combined with other protocols. WebSocket encapsulation of XMPP has little benefit except the case that an application using XMPP is implemented using JavaScript.

Therefore, this study will not utilize SIP and XMPP. More optimal (or WebRTC native) signalling protocols for the upper layer of control plane is to be identified in this study.

#### 5.2.2.3	Other Existing Implementations

Among the existing implementations of WebRTC communication, JSON[RFC8259] format is mainly used for the upper layer of control plane. This is because JSON format is easy to handle in JavaScript. In this study, the potential of JSON for the upper layer protocol of control plane signalling is investigated.

In 3GPP specifications, RESTful APIs (such as service-based interface and Northbound APIs) are often defined using OpenAPI 3[OpenAPI] and the message-body of the APIs are based on JSON. However, OpenAPI is mainly suitable for RESTful APIs and not suitable for message-driven APIs such as control plane signalling over WebSocket. For this reason, AsyncAPI[AsyncAPI] as well as OpenAPI are used for identifying API schemas in this study. Those two APIs are managed by Linux Foundation.

### 5.2.3 Protocol Stack

As described above, JSON based protocol over WebSocket is a solution for the protocol of Rs-u. (see Figure 5.2.3-1)

WebSocket can be deployed over several versions of HTTP.

-	WebSocket with HTTP/1.1 is specified in IETF RFC 6455 [RFC6455].

-	WebSocket with HTTP/2 is specified in IETF RFC 8441 [RFC8441].

-	WebSocket with HTTP/3[draft-ietf-quic-http-34] is under IETF draft [draft-ietf-httpbis-h3-websockets-04].

The sub layers of each protocol are according to the existing specifications.

-	TLS under HTTP/1.1 and HTTP/2 is specified in IETF RFC 8446 [RFC8466].

-	QUIC under HTTP/3 is specified in IETF RFC 9000[RFC9000].

-	TCP under TLS is specified in IETF RFC 793[RFC793].

-	UDP under QUIC is specified in IETF RFC 791[RFC791].

-	IPv4 and v6 under TCP/UDP are specified in IETF RFC 791[RFC791](IPv4) and IETF RFC 8200 [RFC8200](IPv6).

Figure 5.2.3-1. Protocol Stack of Rs-u

--- End change#1 ---

--- Start change#2 ---

# 12	References

[TS24.371]		3GPP TS 24.371: “Web Real-Time Communications (WebRTC) access to the IP Multimedia (IM) Core Network (CN) subsystem (IMS); Stage 3; Protocol specification”

[RFC768]		IETF RFC 768: “User Datagram Protocol”

[RFC791]		IETF RFC 791: “Internet Protocol”

[RFC793]		IETF RFC 793: “Transmission Control Protocol”

[RFC3261]		IETF RFC 3261: “SIP: Session Initiation Protocol”

[RFC6120]		IETF RFC 6120: “Extensible Messaging and Presence Protocol (XMPP): Core”

[RFC6455]		IETF RFC 6455: “The WebSocket Protocol”

[RFC7230]		IETF RFC 7230: “Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing”

[RFC7231]		IETF RFC 7231: “Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content”

[RFC7232]		IETF RFC 7232: “Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests”

[RFC7233]		IETF RFC 7233: “Hypertext Transfer Protocol (HTTP/1.1): Range Requests”

[RFC7234]		IETF RFC 7234: “Hypertext Transfer Protocol (HTTP/1.1): Caching”

[RFC7235]		IETF RFC 7235: “Hypertext Transfer Protocol (HTTP/1.1): Authentication”

[RFC7540]		IETF RFC 7540: “Hypertext Transfer Protocol Version 2 (HTTP/2)”

[RFC8200]		IETF RFC 8200: “Internet Protocol, Version 6 (IPv6) Specification”

[RFC8259]		IETF RFC 8259: “The JavaScript Object Notation (JSON) Data Interchange Format”

[RFC8441]		IETF RFC 8441: “Bootstrapping WebSockets with HTTP/2”

[RFC8446]		IETF RFC 8446: “The Transport Layer Security (TLS) Protocol Version 1.3”

[RFC9000]		IETF RFC 9000: “QUIC: A UDP-Based Multiplexed and Secure Transport”

[draft-ietf-quic-http-34]	draft-ietf-quic-http-34: “Hypertext Transfer Protocol Version 3 (HTTP/3)”

[draft-ietf-httpbis-h3-websockets-04]	draft-ietf-httpbis-h3-websockets-04: “Bootstrapping WebSockets with HTTP/3”

[OpenAPI]		OpenAPI Initiative “OpenAPI Specification v3.0.0” https://spec.openapis.org/oas/v3.0.0

[AsyncAPI]	AsyncAPI Initiative “AsyncAPI Specification v2.4.0” https://asyncapi.com/docs/specifications/v2/4/0

--- End change#2 ---