Source:	Intel, Nokia Corporation

Title:	use case of Multiple Video Sources in UE

Agenda Item:	15.3

Document for:	Discussion and Agreement

- Introduction

In this contribution, we would like to introduce a use case where a UE has multiple video sources.

It covers the cases where WebRTC-enabled UE has Multiple Video Sources covering different areas such as 360 degrees of view. In some cases, not all of the current UE’s field-of-view (FoV) is necessary to be streamed or the receiver-side UE does not care what’s behind source’s current FoV. This contribution was previously discussed in the SA4#121 meeting.

- Multiple Video Sources and zone allocations

Figure. X-2.1 illustrates a common use case when a UE has multiple video sources (e.g. 2D/3D-capable). Each camera may have a fixed Field-of-View (FoV) or varied FoV. Each camera may support the same set of video capabilities such as codec support.

A camera ID or zone ID is assigned for each camera. By assigning a zone number, the UE has the flexibility to signal each source by its source ID (e.g. SSRC in case of RTP) or zone/camera ID.

The zone ID may be assigned with a priority based on the areas it covers and may consist of one or more cameras. For example, the area covered by cameras in zone-1 may be more important than the ones located in zone-2 and zone-3 since it covers the front FoV of the UE. This is important information since it enables UE to signal the essential zone areas or high-priority zones. In some cases, all of the zones have to be treated equally, then all the zone will have the same priority assignment.

Figure. X-2.1 Multiple Video Sources With different zone allocations in UE

Note: the number of cameras and the size of UE, i.e., the entire coverage of all cameras, can vary.

In this use case, there are a couple of possible scenarios.

For example, UE-A has multiple media sources under its control. UE-A is communicating with UE-B:

- Each media source belonging to UE-A is able to produce an individual media stream. To set up the media stream with UE-B, UE-A identifies the source of the media (e.g., by camera ID) and exchanges information about the media.

- In each zone, media produced by the sources in the zone may be processed and combined into a new media. UE-A identifies (the source of) the combined media (e.g., by zone ID) and exchanges information about the media.

- Media produced by the sources in all zones may be combined into a new media stream. UE-A identifies (the source of) the combined media and exchanges information about the media.

- The zone IDs or camera IDs may be associated to particular pose information of UE-B when the UE-A is creating or sending immersive content. Streams from individual camera or cameras in certain zones can be paused/resumed depending on the viewing orientation of the receiver UE-B (i.e., for viewport-dependent media). In this case the streams do not have to be combined.

- Proposal

It is proposed to add clause 2 into C.x of the FS_eiRTCW permanent document and work on session management-related procedures for this use case.