3GPP TSG-S4   Meeting #122	S4-230042

# Athens, Greece, 20th–24th February 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0032 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS3] Rel-17 corrections |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-17 |
| Reason for change: |  | A number of errros and omissions have come to light during implementation of this specification in the 5G-MAG Reference Tools initiative.. |  |  |  |  |  |  |  |
| Summary of change: |  | - Allowance of multiple alternative 5GMS AF server endpoint addresses for Network Assistance, in line with Consumption Reporting and Dynamic Policy client configuration. - Support for provisioning Media Entry Points of different types. - Added missing error response for cache purging operation when the supplied regular expression is invalid. - Add missing notifications and errors for AF-based Network Assistance to Media Session Handling API. - Fix Service Operation Point definitions for Media Stream Handling API. - Tighten up specification of absolute URLs versus relative URLs in OpenAPI definitions. - Fix broken OpenAPI definition for cache purging operation. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - AF-based Network Assistance feature in not scalable in deployment. - Transmuxing by the 5GMS AS cannot be provisioned or signalled in Service Access Information. - 5GMS-Aware Application is not notified of events and errors in AF-based Network Assistance. - Automated selection of Dynamic Policy by Media Session Handler is not supported by client APIs. - Cache purge operation cannot specify a regular expression. |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2.3, 4.7.4, 4.7.6, 6.2.2, 6.2.3.3, 7.6.3.1, 7.6.4.3, 7.9.1, 7.9.3.1, 11.2.3.1, 11.3.3.1, 11.6.3.1, 12.2.3, 12.2.6, 12.2.7, 13.2.5, 13.2.6, 17.2, C |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.7.2.3	Retrieve Service Access Information properties

(SNIPPED: NO FURTHER CHANGES TO THIS CLAUSE)

NEXT CHANGE

### 4.7.4	Procedures for consumption reporting

These procedures are used by the Media Session Handler and the Consumption Reporting functions of the 5GMSd Client to submit a consumption report via the M5d interface if Consumption Reporting is applied for a downlink streaming session.

The Service Access Information indicating whether Consumption Reporting is provisioned for downlink streaming sessions is described in clause 11.2.3. When the clientConsumptionReportingConfiguration.samplePercentage value is 100, the Media Session Handler shall activate the consumption reporting procedure. If the samplePercentage is less than 100, the Media Session Handler shall generate a random number which is uniformly distributed in the range of 0 to 100, and the Media Session Handler shall activate the consumption report procedure when the generated random number is of a lower value than the samplePercentage value.

If the consumption reporting procedure is activated, the Media Session Handler shall submit a consumption report to the 5GMSd AF when any of the following conditions occur:

-	Start of consumption of a downlink streaming session;

-	Stop of consumption of a downlink streaming session;

-	Upon determining the need to report ongoing 5GMS consumption at periodic intervals determined by the clientConsumptionReportingConfiguration.reportingInterval property.

-	Upon determining a location change, if the clientConsumptionReportingConfiguration.locationReporting property is set to True.

-	Upon determining an access network change (e.g. unicast to eMBMS, or vice versa), if the clientConsumptionReportingConfiguration.accessReporting property is set to True.

Whenever a consumption report is sent, the Media Session Handler shall reset its reporting interval timer to the value of the reportingInterval property and it shall begin countdown of the timer again. Whenever the Media Session Handler stops the consumption of a downlink streaming session, it shall disable its reporting interval timer.

In order to submit a consumption report, the Media Session Handler shall send an HTTP POST message to the 5GMSd AF. If several 5GMSd AF addresses are listed in the clientConsumptionReportingConfiguration.‌serverAddresses array (see table 11.2.3.1-1), the Media Session Handler shall choose one and send the message to the selected. The request body shall be a ConsumptionReport structure, as specified in clause 11.3.3.1.The server shall respond with a 200 (OK) message to acknowledge successful processing of the consumption report.

NOTE:	If the connection via M5d for consumption reporting is temporarily unavailable, the consumption reports are expected to be stored on the UE for some time until connectivity to 5GMSd AF is restored and 	send as collection later to the 5GMSd AF. Details are left to implementation.

The Consumption Reporting API, defining the data formats and structures and related procedures for consumption reporting, is described in clause 11.3.

A reporting client identifier shall be included in the consumption report. If available to the Media Session Handler, its value should be a GPSI value as defined by TS 23.003 [7]. Otherwise, the reporting client identifier should be represented by a stable and globally unique string.

NEXT CHANGE

### 4.7.6	Procedures for network assistance

This procedure is used by the 5GMS Client to request Network Assistance from one of the 5GMS AF instances listed in the networkAssistanceConfiguration.serverAddresses property of the Service Access Information resource retrieved using the procedure in clause 4.7.2.3.

The 5GMS Client first creates a Network Assistance Session with its chosen 5GMS AF instance. It provides information that will be used by the Network Assistance function to request QoS from the PCF and to recommend a bit rate to the 5GMS Client.

The 5GMS Client may also request a delivery boost to be provided.

After the Network Assistance Session resource is provisioned, the 5GMS Client uses the Network Assistance Session identifier when requesting a bit rate recommendation.

In order to terminate a Network Assistance Session, the 5GMS Client deletes the Network Assistance session resource.

NEXT CHANGE

### 6.2.2	HTTP message bodies for API resources

The OpenAPI [23] specification of HTTP messages and their content bodies is contained in annex C.

NEXT CHANGE

#### 6.2.3.3	Server identification

##### 6.2.3.3.1	5GMS AF identification

The 5GMS AF shall identify itself at reference points M1 and M5 using a Server response header (see section 7.4.2 of RFC 7231  [25]) that includes a product string of the following form:

5GMSAF-{FQDN}/{complianceInformation}

where {FQDN} shall be the Fully-Qualified Domain Name of the 5GMSd AF exposed to the requesting client, and {complianceInformation} should indicate the version number of the present document (without the leading "V") with which the 5GMS AF implementation complies and shall, at minimum, indicate the 3GPP release number with which the implementation complies.

The Server response header may also include comments strings and vendor-specific subproduct strings compliant with the syntax and guidance provided in section 7.4.2 of [25].

EXAMPLE 1:	5GMSAF-vm10664.mno.net/17.4.0 (api=2.1.0) libsbi/2.1 libnf/1.2 libaf/1.1

EXAMPLE 2:	5GMSAF-vm10664.mno.net/17 (api=2.1.0) libsbi/2.1 libnf/1.2 libaf/1.1

NEXT CHANGE

#### 7.6.3.1	ContentHostingConfiguration resource

The data model for the ContentHostingConfiguration resource is specified in table 7.6.3.1-1 below:

Table 7.6.3.1-1: Definition of ContentHostingConfiguration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| name | String | 1..1 | A name for this Content Hosting Configuration. |
| ingestConfiguration | Object | 1..1 | Describes the 5GMSd Application Provider's origin server from which media resources will be ingested via interface M2d. |
| pull | Boolean | 1..1 | Indicates whether to the 5GMSd AS shall use Pull or Push for ingesting the content. |
| protocol | Uri | 1..1 | A fully-qualified term identifier allocated in the name space urn:3gpp:5gms:content-protocol that identifies the content ingest protocol. The set of supported protocols is defined in clause 8. |
| baseURL | AbsoluteUrl | 0..1 | A base URL (i.e. one that includes a scheme, authority and, optionally, path segments) from which content is ingested at reference point M2d for this ingest configuration. In the case of Pull-based content ingest (pull flag is set to True), the base URL shall be provided to the 5GMSd AF to indicate the location from which content is to be pulled for this Content Hosting Configuration. A request received at reference point M4d is mapped by the 5GMSd AS to a URL at reference point M2d whose base is the value of this property. In the case of Push-based content ingest (pull flag is set to False), this property is populated by the 5GMSd AF and returned to the 5GMSd Application Provider to indicate the base URL to which content for this Content Hosting Configuration is to be published. |
| distributionConfigurations | Array(Object) | 1..1 | Specifies the distribution method and configuration for the ingested content. More than one distribution may be configured for the ingested content, e.g. to offer different distribution configurations such as DASH and HLS. |
| entryPoint | Object | 0..1 | The Media Entry Point when this distribution configuration is used to describe a single content item. |
| relativePath | RelativeUrl | 1..1 | A relative path (i.e. without a scheme or any leading forward slash characters) to the resource for the Media Entry Point. The semantics are dependent on the value of ingestConfiguration.protocol, as specified in clause 8. The path shall be valid at reference point M2d when appended to the ingest base URL and at reference point M4d when appended to the distribution base URL. |
| contentType | String | 1..1 | The MIME content type of the Media Entry Point. Used by the 5GMS Client to select a distribution configuration. |
| profiles | Array(Uri) | 0..1 | An optional list of conformance profile identifiers associated with the Media Entry Point, each one expressed as a URI. A profile URI may indicate an interoperability point, for example. Used by the 5GMS Client to select a distribution configuration. If present, the array shall contain at least one item. |
| contentPreparationTemplateId | ResourceId | 0..1 | Indicates that content preparation prior to distribution is requested by the 5GMSd Application Provider. It identifies the Content Preparation Template that shall be used as defined in clause 7.4. |
| supplementary‌Distribution‌Networks | Array(<Distribution‌NetworkType, DistributionMode> | 0..1 | Specifies that the content for this distribution configuration is to be distributed via one of more supplementary networks. Each member of the array is a duple mapping a type of distribution network to a mode of distribution. The same DistributionNetworkType value shall appear at most once in the array. |
| canonicalDomainName | String | 0..1 | All resources of the current distribution shall be accessible through this default Fully Qualified Domain Name assigned by the 5GMSd AF. |
| domainNameAlias | String | 0..1 | The 5GMSd Application Provider may assign another Fully-Qualified Domain Name through which media resources are additionally accessible at M4d. This domain name is used by the 5GMSd AS to select an appropriate Server Certificate to present at M4d, and to set appropriate CORS HTTP response headers at M4d. If this property is present, the 5GMSd Application Provider is responsible for providing in the DNS a CNAME record that resolves domainNameAlias to canonicalDomainName. |
| baseURL | AbsoluteUrl | 0..1 | A base URL (i.e. one that includes a scheme, authority and, optionally, path segments) from which content is made available to 5GMS Clients at reference point M4d for this distribution configuration. The value is chosen by the 5GMSd AF when the Content Hosting Configuration is provisioned. It is an error for the 5GMSd Application Provider to set this. |
| pathRewriteRules | Array(Object) | 0..1 | An ordered list of rules for rewriting the request URL paths of media resource requests handled by the 5GMSd AS. If multiple rules match a particular resource's path, only the first matching rule, in order of appearance in this array, shall be applied. |
| requestPathPattern | String | 1..1 | A regular expression [5] against which the path part of each 5GMSd AS request URL, including the leading "/", and up to and including the final "/", shall be compared. (Any leaf path element following the final "/" shall be excluded from this comparison.) In the case of Pull-based ingest, the M4d download request path is used in the comparison. In the case of Push-based ingest, the M2d upload request path is used in the comparison. In either case, if the request path matches this pattern, the path mapping specified in the corresponding mappedPath shall be applied. |
| mappedPath | String | 1..1 | A replacement for the portion of the 5GMSd AS request path that matches requestPathPattern. In the case of Pull-based ingest, ingestConfiguration.entryPoint is concatenated with the mapped path and any leaf path element from the original M4d download request to form the M2d origin request URL. In the case of Push-based ingest, canonicalDomainName (and, optionally, domainNameAlias) are concatenated with the mapped path and any leaf path element from the original M2d upload request to form the distribution URL(s) exposed over M4d. |
| cachingConfigurations | Array(Object) | 0..1 | Defines a configuration of the 5GMSd AS cache for a matching subset of media resources ingested in relation to this Content Hosting Configuration. |
| urlPatternFilter | String | 1..1 | A pattern that will be used to match media resource URLs to determine whether a given media resource is eligible for caching by the 5GMSd AS. The format of the pattern shall be a regular expression as specified in [5]. |
| cachingDirectives | Object | 1..1 | If a urlPatternFilter applies to a resource, then the provided cachingDirectives shall be applied by the 5GMSd AS at M4d, potentially overwriting any origin caching directives ingested at M2d. |
| statusCodeFilters | Array(Integer) | 0..1 | The set of HTTP origin response status codes to which these cachingDirectives apply. The filter shall be provided as a regular expression as specified in [5]. If the list is empty, the CachingDirectives shall apply to all HTTP origin response status codes at M2d. |
| noCache | Boolean | 1..1 | If set to True, this indicates that the media resources matching the filters shall not be cached by the 5GMSd AS and shall be marked as not to be cached when served by the 5GMSd AS at M4d. |
| maxAge | Integer | 0..1 | The caching time-to-live period that shall be set on ingested media resources matching the filters. This determines the minimum period for which the 5GMSd AS shall cache matching media resources as well as the time-to-live period signalled by the 5GMSd AS at interface M4d when it serves such media resources. The time-to-live for a given media resource shall be calculated relative to the time it was ingested. |
| geoFencing | Object | 0..N | Limit access to the content to the indicated geographic areas. |
| locatorType | Uri | 1..1 | The type of the locators shall be indicated using a fully-qualified term identifier URI from the controlled vocabulary urn:3gpp:5gms:‌locator-type, as specified in clause 7.6.4.6, or else from a vendor-specific vocabulary. |
| locators | Array(String) | 1..1 | Array of locators from which access to the resources is to be allowed. The format of the locator strings shall be determined by the value of locatorType, as specified in clause 7.6.4.6. |
| urlSignature | Object | 0..1 | Defines the URL signing scheme. Only correctly signed and valid URLs will be allowed to access the content resource at M4d. |
| urlPattern | String | 1..1 | A pattern that shall be used by the 5GMSd AS to match M4d media resource URLs. The 5GMSd AS shall not serve a matching media resource at M4d unless it includes a valid authentication token calculated over the portion of the M4d request URL that matches this pattern. The format of the pattern shall be a regular expression as specified in [5]. |
| tokenName | String | 1..1 | The name of the M4d request query parameter that the Media Player should use to present the authentication token when required to do so. |
| passphraseName | String | 1..1 | The name of the query parameter that is used to refer to the passphrase when constructing the authentication token. Note that the token is not included in the cleartext part of the M4d URL query component. |
| passphrase | String | 1..1 | The shared secret between the 5GMSd Application Provider and the 5GMSd AS for this distributionConfiguration. The passphrase is used in the computation and verification of the M4d authentication token but is never sent in-the-clear over that interface. |
| tokenExpiryName | String | 1..1 | The name of the M4d request query parameter that the Media Player should use to present the token expiry field. |
| useIPAddress | Boolean | 1..1 | If set to True, the IP address of the UE is included in the computation of the authentication token for resources that match urlPattern and access to matching media resources shall be allowed by the 5GMSd AF only when the M4d request is made from a UE with this IP address. |
| ipAddressName | String | 0..1 | The name of the M4d request query parameter that is encoded as part of the authentication token if the useIPAddress flag is set to True. Note that the IP address is not passed in the cleartext part of the M4d URL query component. |
| certificateId | ResourceId | 0..1 | When content is distributed using TLS [16], the X.509 [8] certificate for the origin domain is shared with the 5GMSd AF so that it can be presented by the 5GMSd AS in the TLS handshake at M4d. This attribute indicates the identifier of the certificate to use. |

NEXT CHANGE

#### 7.6.4.3	Cache purging

The 5GMSd Application Provider may perform a purge operation to invalidate some or all cached media resources of a particular Content Hosting Configuration. A regular expression describing the set of media resource URLs to be purged from the 5GMSd AS cache for the Content Hosting Configuration in question shall be supplied in the body of the request. The body shall be encoded using the application/x-www-form-urlencoded MIME content type as a key–value pair, with the key being the string pattern and the value being the regular expression.

On receiving a purge request, the 5GMSd AF shall immediately invalidate all media resources in the 5GMSd AS cache matching the regular expression by declaring them as stale. Any request at reference point M4d for a purged media resource will trigger the fetching (and possible caching) of the current version from the origin via M2d in case of a Pull-based ingest. For Push-based ingest, M4d requests for purged content shall be responded to with a 404 (Not Found) HTTP response, until a new version of the object is pushed by the origin to the 5GMSd AS via M2d.

If the procedure is successful, the 5GMSd AF shall respond with one of the following response messages:

-	204 (No Content) if no cache entries were purged, for example because no current cache entries matched the regular expression supplied in the original request.

-	200 (OK) if some cache entries were purged. The body of the response message shall indicate the total number of cache entries purged in all 5GMSd AS instances distributing the Provisioning Session in question.

If the procedure is not successful, the 5GMSd AF shall provide a response code as defined in clause 6.3. In addition, the HTTP response 422 (Unprocessable Entity) shall be returned in the case where the request message body – or the regular expression contained in it – are found by the 5GMSd AF to be syntactically malformed.

NEXT CHANGE

### 7.9.1	Overview

The Policy Templates Provisioning API allow a 5GMS Application Provider to configure a set of Policy Templates within the scope of a Provisioning Session that can subsequently be applied to downlink or uplink media streaming sessions belonging to that Application Provider using the Dynamic Policies API specified in clause 11.5. A Policy Template is used to specify the traffic shaping and charging policies to be applied to these media streaming sessions.

A Policy Template, identified by its policyTemplateId, represents a set of PCF/NEF API parameters which defines the service quality and associated charging for the corresponding downlink or uplink media streaming session(s). The Policy Template is configured as part of the provisioning procedures with the 5GMS AF and is then used by the 5GMS AF to request specific QoS and charging policies for that session from the PCF or NEF.

The state of a Policy Template can be:

-	pending: The Policy Template is awaiting validation, potentially because not all required parameters have yet been provided. This is the default state after Policy Template creation.

-	invalid: One or more of the Policy Template's properties failed validation by the 5GMS AF.

-	ready: After successful validation by the 5GMS AF the Policy Template moves into this state.

-	suspended: The 5GMS AF may move a Policy Template into this state under certain conditions defined within the Service Level Agreement.

When the Policy Template is used for QoS Flows, the qoSSpecification object (of type M1QoSSpecification) shall be present:

-	The qosReference value is obtained with the Service Level Agreement. See TS 23.502 [45] for detailed usage.

-	The maxBtrUl and maxBtrDl properties define the maximal bit rate which can be used for QoS Flows. This value is defined by the 5G System.

-	The maxAuthBtrUl and MaxAuthBtrDl properties define the maximal authorized bit rate values which can be requested by a Media Session Handler. Higher bit rate values are not authorized for use by the 5GMS Application Provider.

-	The minPacketLossRateDl and minPacketLossRateUl properties define the minimal authorized packet loss rate, which can be requested by a Media Session Handler.

When the Policy Template is used for differential charging the chargingSpecification property shall be present.

applicationSessionContext is a mandatory child object, which contains at least the aspId property.

-	The aspId identifies the API invoker.

-	The dnn property contains the Data Network Name of the data network, in which the 5GMS AF is hosted.

-	When Network Slicing is used, the sliceInfo property contains information about the network slice, which is serving the UE.

-	When present, the afAppId property contains an application identifier referencing one or more PFD objects. The value of the afAppId property is provided to the PCF with each new Npcf_PolicyAuthorization service instance.

NEXT CHANGE

#### 7.9.3.1	PolicyTemplate resource

The data model for the PolicyTemplate resource is specified in table 7.9.3-1 below:

Table 7.9.3-1: Definition of PolicyTemplate resource

| Property | Type | Cardinality | Usage | Visibility | Description |
| --- | --- | --- | --- | --- | --- |
| policyTemplateId | ResourceId | 1..1 | C: RO R: RO U: RO |  | Unique identifier of this Policy Template within the scope of the Provisioning Session. |
| state | Enumeration of Strings | 1..1 | C: RO R: RO U: RO |  | A Policy Template may be in the PENDING, INVALID, READY, or SUSPENDED state. Only a Policy Template in the READY state may be instantiated as a Dynamic Policy Instance and applied to media streaming sessions. |
| apiEndPoint | String | 1..1 | C: RW R: RO U: RW | MNO Admin | The API endpoint that should be invoked when activating a Dynamic Policy Instance based on this Policy Template. |
| apiType | Enumeration of Strings | 1..1 | C: RW R: RO U: RW | MNO Admin | N5: Indicates direct invocation of the Npcf_PolicyAuthorization service by the 5GMS AF. N33: Indicates invocation of the Nnef_AsSessionWithQoS or Nnef_ChargableParty services by the 5GMS AF via the Network Exposure Function. |
| externalReference | String | 1..1 | C: RW R: RO U: RW |  | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the media streaming session. |
| qoSSpecification | M1QoSSpecification | 0..1 | C: RW R: RO U: RW |  | Specifies the network quality of service to be applied to media streaming sessions at this Policy Template. |
| applicationSession‌Context | Object | 1..1 |  |  | Specifies information about the application session context to which this Policy Template can be applied. |
| afAppId | AfAppId | 0..1 | C: RW R: RW U: RW |  | As defined in clause 5.6.2.3 of TS 29.514 [34] and clause 5.3.2 of TS 29.571 [12]. |
| sliceInfo | Snssai | 0..1 | C: RW R: RW U: RW |  |  |
| dnn | Dnn | 0..1 | C: RW R: RW U: RW |  |  |
| aspId | AspId | 1..1 | C: RW R: RW U: RW |  |  |
| chargingSpecification | ChargingSpecification | 0..1 | C: RW R: RW U: RW |  | Provides information about the charging policy to be used for this Policy Template. |

NEXT CHANGE

#### 11.2.3.1	ServiceAccessInformation resource type

The data model for the ServiceAccessInformation resource is specified in table 11.2.3.1-1 below. Different properties are present in the resource depending on the type of Provisioning Session from which the Service Access Information is derived (as indicated in the provisioningSessionType property) and this is specified in the Applicability column.

Table 11.2.3.1-1: Definition of ServiceAccessInformation resource

| Property name | Type | Cardinality | Usage | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| provisioningSessionId | ResourceId | 1..1 | RO | Unique identification of the M1 Provisioning Session. | All types |
| provisioningSession‌Type | Provisioning‌Session‌Type | 1..1 | RO | The type of Provisioning Session. | All types. |
| streamingAccess | Object | 0..1 | RO |  | downlink |
| entryPoints | Array(Object) | 0..1 | RO | A list of alternative Media Entry Points for the 5GMS Client to choose between. |  |
| locator | AbsoluteUrl | 1..1 | RO | A pointer to a document at reference point M2 that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |  |
| contentType | String | 1..1 | RO | The MIME content type of this Media Entry Point. |  |
| Profiles | Array(Uri) | 0..1 | RO | An optional list of conformance profile URIs with which this Media Entry Point is compliant. If present, the array shall contain at least one item. |  |
| eMBMS‌Service‌Announcement‌Locator | AbsoluteUrl | 0..1 | RO | A pointer to a document that defines a User Service Announcement for eMBMS where the service announcement file is available. | Downlink |
| clientConsumptionReporting‌Configuration | Object | 0..1 | RO |  | downlink |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between consumption report messages being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the media streaming session has ended. |  |
| serverAddresses | Array(AbsoluteUrl) | 1..1 | RO | A list of 5GMSd AF addresses (URLs) where the consumption reporting messages are sent by the Media Session Handler. See NOTE. (Opaque URL, following the 5GMS URL format.) |  |
| locationReporting | Boolean | 1..1 | RO | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |  |
| accessReporting | Boolean | 1..1 | RO | Stipulates whether the Media Session Handler is required to provide consumption reporting messages to the 5GMSd AF when the access network changes during a media streaming session. |  |
| samplePercentage | Percentage | 1..1 | RO | The percentage of media streaming sessions that shall send consumption reports, expressed as a floating point value between 0.0 and 100.0. |  |
| dynamicPolicyInvocation‌Configuration | Object | 0..1 | RO |  | downlink, uplink |
| serverAddresses | Array(AbsoluteUrl) | 1..1 | RO | A list of 5GMSd AF addresses (URLs) which offer the APIs for dynamic policy invocation sent by the Media Session Handler. See NOTE. (Opaque URL, following the 5GMS URL format.) |  |
| validPolicyTemplateIds | Array(ResourceId) | 1..1 | RO | A list of Policy Template identifiers which the 5GMS Client is authorized to use. |  |
| sdfMethods | Array(SdfMethod) | 1..1 | RO | A list of recommended service data flow description methods (descriptors), e.g. 5-Tuple, ToS, 2-Tuple, etc., which should be used by the Media Session Handler to describe the service data flows for the traffic to be policed. |  |
| externalReferences | Array(String) | 0..1 | RO | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the media streaming session. Example: "HD_Premium". |  |
| clientMetricsReporting‌Configurations | Array(Object) | 0..1 | RO |  | downlink, uplink |
| metricsReporting‌ConfigurationId | ResourceId | 1..1 | RO | The identifier of this metrics reporting configuration, unique within the scope of provisioningSessionId. The value shall be the same as the corresponding identifier provisioned at reference point M1. |  |
| serverAddresses | Array(AbsoluteUrl) | 1..1 | RO | A list of 5GMS AF addresses to which metrics reports shall be sent. See NOTE. (Opaque URL, following the 5GMS URL format.) |  |
| scheme | Uri | 1..1 | RO | The metrics reporting scheme that metrics reports shall use (see clause 4.7.5). |  |
| dataNetworkName | Dnn | 0..1 | RO | The DNN which shall be used when sending metrics reports. If not specified, the name of the default DN shall be used. |  |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between metrics reports being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the media streaming session has ended. |  |
| samplePercentage | Percentage | 1..1 | RO | The percentage of media streaming sessions that shall report metrics, expressed as a floating point value between 0.0 and 100.0. |  |
| urlFilters | Array(String) | 0..1 | RO | A non-empty list of URL patterns for which metrics reporting shall be done. The format of each pattern shall be a regular expression as specified in [5]. If not specified, reporting shall be done for all sessions. |  |
| Metrics | Array(String) | 1..1 | RO | A list of metrics which shall be reported. |  |
| networkAssistance‌Configuration | Object | 0..1 | RO |  | downlink, uplink |
| serverAddresses | Array(AbsoluteUrl) | 1..1 | RO | A list of 5GMS AF addreses (URLs) that offer the APIs for 5GMS AF-based Network Assistance, for access by the 5GMSd Media Session Handler. See NOTE. This address shall be an opaque URL, following the 5GMS URL format. |  |
| Client‌EdgeResources‌Configuration | Object | 0..1 | RO | Present only for Provisioning Sessions with client-driven edge computing management mode provisioned. | Downlink, uplink |
| eligibilityCriteria | Edge‌Processing‌Eligibility‌Criteria | 0..1 | RO | Conditions for activating edge resources for media streaming sessions in the scope of this Service Access Information. (See clause 6.4.3.8.) |  |
| easDiscoveryTemplate | EAS‌Discovery‌Template | 1..1 | RO | A template for the EAS discovery filter that shall be used by the EEC to discover and select a 5GMS EAS instance to serve media streaming sessions in the scope of this Service Access Information. (See clause 11.2.3.2.) |  |
| easRelocation‌Requirements | M5EAS‌Relocation‌Requirements | 0..1 | RO | EAS relocation tolerance and requirements. If absent, the EEC shall assume that relocation is tolerated by all 5GMS EAS instances in the scope of this Service Access Information. (See clause 11.2.3.3.) |  |
| NOTE: In deployments where multiple instances of the 5GMSd AF expose the Media Session Handling APIs at M5, the 5G System may use a suitable mechanism (e.g. HTTP load balancing or DNS resolution) to direct requests to a suitable AF instance. |  |  |  |  |  |

NEXT CHANGE

#### 11.3.3.1	ConsumptionReport format

This type represents the format of consumption report instance. This structure is used by the Media Session Handler to report the consumption.

Table 11.3.3.1-1: Definition of ConsumptionReport format

| Property name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| mediaPlayerEntry | AbsoluteUrl | 1..1 | Identifies the Media Entry Point. In the case of DASH, thisshall be the URL of the MPD at reference point M4d. |
| reportingClientId | string | 1..1 | Identifier of the reporting client that consumed the streaming media service associated with this consumption report. If available to the Media Session Handler, a GPSI value (see clause 28.8 of TS 23.003 [7]); otherwise, a stable and globally unique string. |
| consumptionReportingUnits | Array(Consumption‌Reporting‌Unit) | 1..1 | An array of consumption reporting units. |

NEXT CHANGE

#### 11.6.3.1	NetworkAssistanceSession resource

The NetworkAssistanceSession resource is specified in Table 11.6.3.1-1 below.

Table 11.6.3.1-1: Definition of NetworkAssistanceSession resource

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| naSessionId | ResourceId | 1..1 | C: RO R: RO U: RO | Unique identifier for this Network Assistance Session. |
| serviceDataFlowInformation | Array(ServiceDataFlowDescription) | 0..1 | C: RW R: RO U: RW | Identification of the application flows for the media streaming session for which Network Assistance is to be used, e.g. 2-tuple (IP addresses) or 5-tuple (IP Addresses, protocol and ports). |
| policyTemplateId | ResourceId | 0..1 | C: RW R: RO U: RW | Identification of the policy that is in force for the media streaming session. |
| requestedQoS | M5QoSSpecification | 0..1 | C: RW R: RO U: RW | The requested QoS parameters. |
| recommendedQoS | M5QoSSpecification | 0..1 | C: RO R: RO U: RO | The QoS parameters currently recommended by the 5GMS AF. |
| notficationURL | AbsoluteUrl | 0..1 | C: RO R: RO U: RO | A URL to the MQTT channel over which notifications are to be sent by the 5GMS AF for this session. When set, the Media Session Handler shall subscribe to this channel. The notification messages shall be in the form of the M5QoSSpecification data type. |

NEXT CHANGE

## 12.2	Media Session Handling for Downlink media streaming – APIs and Functions

(SNIPPED)

### 12.2.3	General

Table 12.2.3-1 provides a list status information that can be obtained from the Media Session Handler through reference point M6d.

Table 12.2.3-1: General Status Information

| Status | Type | Parameter | Definition |
| --- | --- | --- | --- |

Table 12.2.3-2 provides a list of general notification events exposed at reference point M6d.

Table 12.2.3-2: General Notification Events

| Event | Definition | Payload |
| --- | --- | --- |
| SESSION_HANDLING_ACTIVATED | Triggered when media session handling was activated for a specific Media Entry Point. | Media Entry Point URL. |
| SESSION_HANDLING_STOPPED | Triggered when media session handling stopped for a specific Media Entry Point. | Media Entry Point URL. |

Table 12.2.3-3 provides a list of general error events exposed at reference point M6d.

Table 12.2.3-3: General Error Events

| Status | Definition | Payload |
| --- | --- | --- |
| ERROR_SESSION_HANDLING | Triggered when there is an error in the media session handling. | Not applicable. |

(SNIPPED)

### 12.2.6	Consumption Reporting Information

Table 12.2.6-1 provides a list status information that can be obtained from the Media Session Handler through reference point M6d.

Table 12.2.6-1: Status Information relating to Consumption Reporting

| Status | Type | Parameter | Definition |
| --- | --- | --- | --- |
| consumptionReport | Object |  | The most recently sent consumption report. |

Table 12.2.6-2 provides a list of general notification events exposed by the Media Session Handler at reference point M6d.

Table 12.2.6-2: Notification Events relating to Consumption Reporting

| Status | Definition | Payload |
| --- | --- | --- |
| CONSUMPTION_REPORTING_ACTIVATED | Consumption reporting has been activated. | Not applicable. |
| CONSUMPTION_REPORTING_STOPPED | Consumption reporting has been stopped. | Not applicable. |
| NEW_CONSUMPTION_REPORT | A new consumption report is available and has been sent. | Not applicable. |

Table 12.2.6-3 provides a list of general error events exposed by the Media Session Handler at reference point M6d.

Table 12.2.6-3: Error Events relating to Consumption Reporting

| Status | Definition | Payload |
| --- | --- | --- |
| ERROR_CONSUMPTION_REPORTING | Error in consumption reporting occurred. | Server address, Provisioning Session Id, HTTP response code Error message. |

NEXT CHANGE

### 12.2.7	Metrics Reporting Information

Table 12.2.7-1 provides a list of status information that can be obtained from the Media Session Handler through M6d.

Table 12.2.7-1: Status Information relating to Metrics Reporting

| Status | Type | Definition |
| --- | --- | --- |
| lastMetricsReport | Object | Status information relating to the last sent metrics report. |
| provisioningSessionId | ResourceId | The Provisioning Seession identifier for this metrics report. |
| metricsReportingConfigurationId | ResourceId | The metrics reporting configuration identifier for this report. |
| Scheme | Uri | The metrics reporting scheme used by this metrics report (see clause 4.7.5). |
| metricsReport | Object | The most recently sent metrics report. |

Table 12.2.7-2 provides a list of general notification events exposed at reference point M6d.

Table 12.2.7-2: Notification Events relating to Metrics Reporting

| Event | Definition | Payload |
| --- | --- | --- |
| METRICS_REPORTING_ACTIVATED | Metrics reporting has been activated. | Not applicable. |
| METRICS_REPORTING_STOPPED | Metrics reporting has been stopped. | Not applicable. |
| NEW_METRICS_REPORT | A new metrics report is available and has been sent. | Not applicable. |

Table 12.2.7-3 provides a list of general error events exposed at reference point M6d.

Table 12.2.7-3: Error Events relating to Metrics Reporting

| Error event | Definition | Payload |
| --- | --- | --- |
| ERROR_METRICS_REPORTING | Error in metrics reporting occurred. | Server address, Provisioning Session Id, Metrics Reporting Configuration Id, HTTP response code Error message. |

Details of status information for RAN-based metrics reporting are for further study.

NEXT CHANGE

### 13.2.5	Notifications and error events

Table 13.2.5-1 provides a list of notification events that are provided by the Media Player.

Table 13.2.5-1: Notification events

| Status | Definition | Payload |
| --- | --- | --- |
| AST_IN_FUTURE | Triggered when playback will not start yet as the MPD's availabilityStartTime is in the future. | Time before playback will start. |
| AVAILABLE_MEDIA_CHANGED | The list of available media has changed. | Media type: video, audio, subtitle, all |
| BUFFER_EMPTY | Triggered when the media playback platform's buffer state changes to stalled. | Media Type |
| BUFFER_LOADED | Triggered when the media playback platform's buffer state changes to loaded. | Media Type |
| CAN_PLAY | Sent when enough data is available that the media can be played. | Not applicable. |
| MANIFEST_LOADED | Triggered when the manifest load is complete | Not applicable. |
| METRIC_ADDED | Triggered every time a new metric is added. |  |
| METRIC_CHANGED | The minimum bit rate that the ABR algorithms will choose. Use NaN for no limit. |  |
| METRIC_UPDATED | Set to true if you would like DASH Client to keep downloading fragments in the background when the video element is paused. |  |
| METRICS_CHANGED | Triggered whenever there is a change to the overall metrics. |  |
| OPERATION_POINT_CHANGED | Triggered whenever there is a change of an operation point parameter. | External reference identifier of currently selected Service Operation Point. |
| PLAYBACK_ENDED | Sent when playback completes. |  |
| PLAYBACK_ERROR | Sent when an error occurs. The element's error attribute contains more information. | Error attribute. |
| PLAYBACK_PAUSED | Sent when playback is paused. |  |
| PLAYBACK_PLAYING | Sent when the media begins to play (either for the first time, after having been paused, or after ending and then restarting). |  |
| PLAYBACK_SEEKED | Sent when a seek operation completes. |  |
| PLAYBACK_SEEKING | Sent when a seek operation begins. |  |
| PLAYBACK_STALLED | Sent when the media playback platform reports stalled |  |
| PLAYBACK_STARTED | Sent when playback of the media starts after having been paused; that is, when playback is resumed after a prior pause event. |  |
| PLAYBACK_WAITING | Sent when the media playback has stopped because of a temporary lack of data. |  |
| SERVICE_DESCRIPTION_SELECTED | sent when the DASH client has selected a service description. |  |
| SERVICE_DESCRIPTION_CHANGED | Sent when the DASH client has changed a service description. |  |
| SERVICE_DESCRIPTION_VIOLATED | Provides notification that the service description parameters are currently not met. | Parameters of service description that are not met. |
| SOURCE_INITIALIZED | Triggered when the source is setup and ready. |  |

NEXT CHANGE

### 13.2.6	Status Information

Table 13.2.6-1 provides a list of dynamically changing status information that can be obtained from the client.

Table 13.2.6-1: Dynamic Status information

| Status | Type | Parameter | Definition |
| --- | --- | --- | --- |
| state | Enumeration |  | An enumerated value from table 13.2.2-1 indicating the current state of the Media Player. |
| averageThroughput | float | none | Current average throughput computed in the ABR logic in bit/s. |
| bufferLength | float | MediaType "video", "audio" and "subtitle" | Current length of the buffer for a given media type, in seconds. If no type is passed in, then the minimum of video, audio and subtitle buffer length is returned. NaN is returned if an invalid type is requested, the presentation does not contain that type, or if no arguments are passed and the presentation does not include any adaption sets of valid media type. |
| liveLatency | float | none | Current live stream latency in seconds based on the latency measurement. |
| mediaSetting[] | MPDAdaptationSet | MediaType "video", "audio" and "subtitle" | Current media settings for each media type based on the CMAF Header and the MPD information based on the selected Adaptation Set for this media type. |
| mediaTime | float | None | Current media playback time from media playback platform. The media time is in seconds and is relative to the start of the playback and provides the media that is actually rendered. |
| playbackRate | float | None | The current rate of playback. For a video that is playing twice as fast as the default playback, the playbackRate value should be 2.00. |
| availableServiceDescriptions[] | Provides the available service descriptions |  | Provides the list of available selectable service descriptions with an id to select from. Those are either configured ones or the ones in the MPD. |
| availableMediaOptions[] | List of Adaptation Set or Preselection ids | MediaType "video", "audio" "subtitle" "all" | Provides the list of available media options that can be selected by the application based on the capability discovery and the subset information. |
| service‌Operation‌Points | Array(Service‌Operation‌Point) |  | The set of Service Operation Points declared in the presentation manifest (e.g. DASH MPD) of the current media presentation. |
| operative‌Service‌Operation‌Point | integer |  | A zero-based index into the service‌Operation‌Points array indicating the Service Operation Point currently operative in the playback session. Set to -1 if the array is empty. |
| metrics[][] | Metrics |  | A data blob of metrics for each configured metrics collecting scheme. |

Table 13.2.6-2 provides a list of configured operation point information that can be obtained from the client. Any change to a parameter below shall be announced with a notification OPERATION_POINT_CHANGED.

Table 13.2.6-2: Service Operation Point Information

| Parameter |  |  | Type | Definition |
| --- | --- | --- | --- | --- |
| ServiceOperationPoint |  |  | Object | The currently configured operation point parameters according to which the DASH client is operating. |
|  | externalIdentifier |  | String | The external identifier uniquely identifying this Operation Point in the presentation manifest (e.g. DASH MPD). |
|  | mode |  | Enum | The following operation modes are defined: live: The DASH client operates to maintain configured target latencies using playback rate adjustments and possibly resync. Vod: The DASH client operates without latency requirements and rebuffering may result in additional latencies |
|  | maxBufferTime |  | Integer | maximum buffer time in milliseconds for the service. |
|  | switchBufferTime |  | Integer | buffer time threshold below which the DASH clients attempts to switch Representations. |
|  | latency |  | Object | Defines the latency parameters used by the DASH client when operating in live mode. |
|  |  | target | Integer | The target latency for the service in milliseconds. |
|  |  | max | Integer | The maximum latency for the service in milliseconds. |
|  |  | min | Integer | The maximum latency for the service in milliseconds. |
|  | playbackRate |  | MediaType audio, video, all | Defines the playback rate parameters used by the DASH client for catchup mode and deceleration to avoid buffer underruns and maintaining target latencies. |
|  |  | max | Real | The maximum playback rate for the purposes of automatically adjusting playback latency and buffer occupancy during normal playback, where 1.0 is normal playback speed. |
|  |  | min | Real | The minimum playback rate for the purposes of automatically adjusting playback latency and buffer occupancy during normal playback, where 1.0 is normal playback speed. |
|  | bitRate |  |  | Defines the operating bandwidth parameters used by the DASH client used for a specific media type or aggregated. The values are on IP level. |
|  |  | target | Integer | The target bandwidth for the service in bit/s that the client is configured to consume. |
|  |  | max | Integer | The maximum bandwidth for the service in bit/s that the client is configured to consume. |
|  |  | min | Integer | The minimum bandwidth for the service in bit/s that the client is configured to consume. |
|  | playerSpecificParameters |  |  | Player-specific parameters may be provided, for example about the used algorithm, etc. |

NEXT CHANGE

## 17.2	MediaStreamingAccessRecord type

As specified in clause 4.11.3, the MediaStreamingAccessRecord type shall be used by the 5GMS AS to report media streaming access.

Table 17.2-1: Definition of MediaStreamingAccessRecord type

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| timestamp | DateTime | 1..1 | The date and time of the media access. (See table 6.4.2-1.) |
| mediaStreamHandler‌EndpointAddress | Endpoint‌Address | 1..1 | The endpoint address of the Media Stream Handler accessing the 5GMS AS. (See clause 6.4.3.8.) |
| applicationServer‌EndpointAddress | Endpoint‌Address | 1..1 | The service endpoint on the 5GMS AS to which the Media Stream Handler is connected. (See clause 6.4.3.8.) |
| sessionIdentifier | string | 0..1 | An identifier for the HTTP session on which the Media Stream Handler request was made. This should not contain any user-identifiable data. It may, for example, be a one-way hash of the transport connection identifier, if available from the underlying transport protocol. Alternatively, it could be a one-way hash of the 5-tuple formed from the Media Stream Handler and 5GMSd AS endpoint addresses and a transport protocol identifier. |
| requestMessage | Object | 1..1 | Details of the HTTP request message submitted to the 5GMS AS by the Media Stream Handler for this media access. |
| method | string | 1..1 | The request method. |
| url | Absolute‌Url | 1..1 | The request URL. (See table 6.4.2-1.) |
| protocolVersion | string | 1..1 | The HTTP protocol version, e.g. "HTTP/1.1". |
| range | string | 0..1 | The value of the Range request header, if present. |
| size | Uinteger | 1..1 | The total number of bytes in the request message. |
| bodySize | Uinteger | 1..1 | The number of bytes supplied by the Media Stream Handler in the HTTP request message body. Zero if there is no request body. |
| contentType | string | 0..1 | The MIME content type of the request message, if any. |
| userAgent | string | 0..1 | A string describing the requesting Media Stream Handler, if it supplies a User-Agent request header. |
| userIdentity | String | 0..1 | A string identifying the user that made the access, if supplied. |
| referer | Absolute‌Url | 0..1 | The URL that the Media Player reports being referred from, if the Referer request header is supplied. (See table 6.4.2-1.) |
| cacheStatus | Cache‌Status | 0..1 | An indication of whether the 5GMS AS is able to serve an object corresponding to requestMessage,url from cache (HIT) or whether there is a stale object cached (EXPIRED) or the requested object is not present in cache (MISS). (See table 6.4.4.4.) For non-caching implementations of the 5GMS AS, the property shall be omitted. |
| responseMessage | Object | 1..1 | Details of the HTTP response message returned by the 5GMS AS to the Media Stream Handler for this media access. |
| responseCode | Uinteger | 1..1 | The HTTP response code. |
| size | Uinteger | 1..1 | The total number of bytes in the response message. |
| bodySize | Uinteger | 1..1 | The number of bytes in the HTTP response message body. |
| contentType | string | 0..1 | The MIME content type of response message, if any. |
| processingLatency | Float | 1..1 | The time, expressed in milliseconds, taken by the 5GMS AS to respond to the Media Stream Handler request, measured from the first byte of the HTTP request being processed by the 5GMS AS to the last byte of the response being sent. |
| connectionMetrics | Object | 0..1 | Metrics about the performance of the transport connection underlying the HTTP session serving this media access. |
| meanNetwork‌RoundTripTime | Float | 1..1 | A rolling mean average, expressed in milliseconds, of the network round-trip time for the HTTP session. |
| networkRoundTrip‌Time‌Variation | Float | 1..1 | The variation in meanNetwork‌RoundTripTime, expressed in milliseconds, during the averaging period. |
| congestion‌Window‌Size | Uinteger | 1..1 | The current size (in bytes) of the congestion window for the transport connection underlying the HTTP session. |

NEXT CHANGE

Annex C (normative):
OpenAPI representation of the 5GMS HTTP REST APIs

# C.1	General

This annex is based on the OpenAPI 3.0.0 specification [23] and provides corresponding representations of all APIs defined in the present document.

NOTE 1:	An OpenAPIs representation embeds JSON Schema representations of HTTP message bodies.

This Annex shall take precedence when being discrepant to other parts of the present document with respect to the encoding of information elements and methods within the API(s).

NOTE 2:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

# C.2	Data Types applicable to several APIs

For the purpose of referencing entities defined in this clause, it shall be assumed that the OpenAPI definitions below are contained in a physical file named "TS26512_CommonData.yaml".

openapi: 3.0.0 info: title: 5GMS Common Data Types version: 2.0.1 description: ¦ 5GMS Common Data Types © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: 5GMS Common Data Types description: '5G Media Streaming: Common Data Types' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' paths: {} components: schemas: ################################# # Clause 6.4.2: Simple data types ################################# ResourceId: type: string description: String chosen by the 5GMS AF to serve as an identifier in a resource URI. Percentage: type: number minimum: 0.0 maximum: 100.0 #DurationSec is defined in TS29571_CommonData #DateTime is defined in TS29571_CommonData #Uri is defined in TS29571_CommonData Url: type: string format: uri-reference description: 'Uniform Resource Locator, comforming with the "URI-reference" production specified in IETF RFC 3986, section 4.1.' RelativeUrl: type: string format: uri-reference description: 'Relatve Uniform Resource Locator, comforming with the "relative-ref" production specified in IETF RFC 3986, section 4.2. Note that both "query" and "fragment" suffixes are permitted by this production.' AbsoluteUrl: type: string format: uri description: 'Absolute Uniform Resource Locator, comforming with the "absolute-URI" production specified in IETF RFC 3986, section 4.3 in which the scheme part is "http" or "https". Note that the "query" suffix is permitted by this production but the "fragment" suffix is not.' ##################################### # Clause 6.4.3: Structured data types ##################################### IpPacketFilterSet: type: object required: - direction properties: srcIp: type: string dstIp: type: string protocol: type: integer srcPort: type: integer dstPort: type: integer toSTc: type: string flowLabel: type: integer spi: type: integer direction: type: string ServiceDataFlowDescription: type: object properties: flowDescription: $ref: '#/components/schemas/IpPacketFilterSet' domainName: type: string M5QoSSpecification: type: object required: - marBwDlBitRate - marBwUlBitRate - mirBwDlBitRate - mirBwUlBitRate properties: marBwDlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' marBwUlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' minDesBwDlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' minDesBwUlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' mirBwDlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' mirBwUlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' desLatency: type: integer minimum: 0 desLoss: type: integer minimum: 0 M1QoSSpecification: type: object properties: qosReference: type: string maxBtrUl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' maxBtrDl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' maxAuthBtrUl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' maxAuthBtrDl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' defPacketLossRateDl: type: integer minimum: 0 defPacketLossRateUl: type: integer minimum: 0 ChargingSpecification: type: object properties: sponId: type: string sponStatus: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/SponsoringStatus' gpsi: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi' TypedLocation: type: object required: - locationIdentifierType - location properties: locationIdentifierType: $ref: '#/components/schemas/CellIdentifierType' location: type: string OperationSuccessResponse: type: object required: - success properties: success: type: boolean reason: type: string CellIdentifierType: anyOf: - type: string enum: [CGI, ECGI, NCGI] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. SdfMethod: anyOf: - type: string enum: [5_TUPLE, 2_TUPLE, TYPE_OF_SERVICE_MARKING, FLOW_LABEL, DOMAIN_NAME] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. ProvisioningSessionType: anyOf: - type: string enum: [DOWNLINK, UPLINK] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. EndpointAddress: type: object required: - portNumber properties: ipv4Addr: $ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr' ipv6Addr: $ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr' portNumber: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uint16' CacheStatus: anyOf: - type: string enum: [HIT, MISS, EXPIRED] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. EdgeProcessingEligibilityCriteria: type: object required: - serviceDataFlowDescriptions - ueLocations - timeWindows - appRequest properties: serviceDataFlowDescriptions: type: array items: $ref: '#/components/schemas/ServiceDataFlowDescription' ueLocations: type: array items: $ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G' timeWindows: type: array items: $ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow' appRequest: type: boolean EASRelocationTolerance: anyOf: - type: string enum: [RELOCATION_UNAWARE, RELOCATION_TOLERANT, RELOCATION_INTOLERANT] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

# C.3	OpenAPI representation of the M1 APIs

## C.3.1	M1_ProvisioningSessions API

openapi: 3.0.0 info: title: M1_ProvisioningSessions version: 2.0.1 description: ¦ 5GMS AF M1 Provisioning Sessions API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ProvisioningSessions description: '5G Media Streaming: Provisioning (M1) APIs: Provisioning Sessions' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions: post: operationId: createProvisioningSession summary: 'Create a new Provisioning Session' responses: '201': description: 'Provisioning Session Created' headers: Location: description: 'URL including the resource identifier of the newly created Provisioning Session.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' /provisioning-sessions/{provisioningSessionId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' get: operationId: getProvisioningSessionById summary: 'Retrieve an existing Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' delete: operationId: destroyProvisioningSession summary: 'Destroy an existing Provisioning Session' responses: '204': description: 'Provisioning Session Destroyed' # No Content components: schemas: ProvisioningSession: type: object description: "A representation of a Provisioning Session." required: - provisioningSessionId - provisioningSessionType - externalApplicationId properties: provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' provisioningSessionType: $ref: 'TS26512_CommonData.yaml#/components/schemas/ProvisioningSessionType' aspId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' externalApplicationId: $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId' serverCertificateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true contentPreparationTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true metricsReportingConfigurationIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true policyTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true edgeResourcesConfigurationIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true eventDataProcessingConfigurationIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true

## C.3.2	M1_ServerCertificatesProvisioning API

openapi: 3.0.0 info: title: M1_ServerCertificatesProvisioning version: 2.1.0 description: ¦ 5GMS AF M1 Server Certificates Provisioning API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ServerCertificatesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Server Certificates Provisioning' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/certificates: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: The resource identifier of an existing Provisioning Session.' post: operationId: createOrReserveServerCertificate summary: 'Create or reserve a Service Certificate resource' description: 'Without the optional csr query parameter, an X.509 certificate is generated and this is returned. If the csr query parameter is present, a Certificate Signing Request is instead generated and returned, allowing the X.509 certificate to be generated by the invoker and later uploaded.' parameters: - in: query name: csr schema: type: string description: 'When present, return a Certificate Signing Request instead of generating an X.509 certificate' responses: '200': description: 'Server Certificate Created' headers: Location: description: 'URL of the newly created Server Certificate resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' content: 'application/x-pem-file': schema: type: string /provisioning-sessions/{provisioningSessionId}/certificates/{certificateId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: The resource identifier of an existing Provisioning Session.' - name: certificateId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Server Certificate' put: operationId: uploadServerCertificate summary: "Upload the X.509 certificate for a previously reserved Server Certificate resource" requestBody: required: true content: application/x-pem-file: schema: type: string responses: '204': description: 'Server Certificate Updated' get: operationId: retrieveServerCertificate summary: 'Retrieve the X.509 certificate representation of the specified Server Certificate resource' responses: '200': description: 'Success' content: 'application/x-pem-file': schema: type: string '204': description: 'Awaiting Upload' '404': description: 'Not Found' delete: operationId: destroyServerCertificate summary: 'Destroy an existing Server Certificate resource' responses: '200': # OK description: 'Server Certificate Destroyed' content: 'application/x-pem-file': schema: type: string '204': # No Content description: 'Server Certificate Destroyed' '404': # Not Found $ref: 'TS29571_CommonData.yaml#/components/responses/404' '409': # Conflict $ref: 'TS29571_CommonData.yaml#/components/responses/409' '410': # Gone $ref: 'TS29571_CommonData.yaml#/components/responses/410'

## C.3.3	M1_ContentPreparationTemplatesProvisioning API

openapi: 3.0.0 info: title: M1_ContentPreparationTemplatesProvisioning version: 2.1.0 description: ¦ 5GMS AF M1 Content Preparation Templates Provisioning API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentPreparationTemplatesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Content Preparation Templates Provisioning' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/content-preparation-templates: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createContentPreparationTemplate summary: 'Create (and optionally upload) a new Content Preparation Template for the specified Provisioning Session' requestBody: description: 'A Content Preparation Template of any type' required: true content: '*/*': schema: type: string responses: '201': description: 'Content Preparation Template Created' headers: Location: description: 'URL of the newly created Content Preparation Template.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' /provisioning-sessions/{provisioningSessionId}/content-preparation-templates/{contentPreparationTemplateId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: contentPreparationTemplateId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Content Preparation Template.' get: operationId: retrieveContentPreparationTemplate summary: 'Retrieve the specified Content Preparation Template of the specified Provisioning Session' responses: '200': description: 'Success' content: '*/*': schema: type: string put: operationId: updateContentPreparationTemplate summary: 'Update the specified Content Preparation Template for the specified Provisioning Session' requestBody: description: 'A Content Preparation Template of any type' required: true content: '*/*': schema: type: string responses: '204': description: 'Updated Content Preparation Template' '404': description: 'Not Found' patch: operationId: patchContentPreparationTemplate summary: 'Patch the specified Content Preparation Template for the specified Provisioning Session' requestBody: description: 'A Content Preparation Template patch of any type' required: true content: application/merge-patch+json: schema: type: string application/json-patch+json: schema: type: string responses: '200': description: 'Patched Content Preparation Templates' content: '*/*': schema: type: string '404': description: 'Not Found' delete: operationId: destroyContentPreparationTemplate summary: 'Destroy the specified Content Preparation Template of the specified Provisioning Session' responses: '200': # OK description: 'Content Preparation Template Destroyed' content: '*/*': schema: type: string '204': # No Content description: 'Content Preparation Template Destroyed' '404': # Not Found $ref: 'TS29571_CommonData.yaml#/components/responses/404' '409': # Conflict $ref: 'TS29571_CommonData.yaml#/components/responses/409' '410': # Gone $ref: 'TS29571_CommonData.yaml#/components/responses/410'

## C.3.4	M1_ContentProtocolsDiscovery API

(SNIPPED: NO CHANGES TO THIS CLAUSE)

## C.3.5	M1_ContentHostingProvisioning API

openapi: 3.0.0 info: title: M1_ContentHostingProvisioning version: 2.2.0 description: ¦ 5GMS AF M1 Content Hosting Provisioning API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentHostingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Content Hosting Provisioning' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createContentHostingConfiguration summary: 'Create (and optionally upload) the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '201': description: 'Content Hosting Configuration Created' headers: Location: description: 'URL of the newly created Content Hosting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' get: operationId: retrieveContentHostingConfiguration summary: 'Retrieve the Content Hosting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' put: operationId: updateContentHostingConfiguration summary: 'Update the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '204': description: 'Updated Content Hosting Configuration' '404': description: 'Not Found' patch: operationId: patchContentHostingConfiguration summary: 'Patch the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '200': description: 'Patched Content Hosting Configuration' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' delete: operationId: destroyContentHostingConfiguration summary: 'Destroy the current Content Hosting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Content Hosting Configuration' '404': description: 'Not Found' /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration/purge: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: A unique identifier of the Provisioning post: operationId: purgeContentHostingCache summary: 'Purge the content of the cache for the Content Hosting Configuration of the specified Provisioning Session' requestBody: description: 'The regular expression pattern for resources to purge from the cache' required: true content: application/x-www-form-urlencoded: schema: properties: pattern: description: 'The regular expression' type: string responses: '200': # OK description: 'Content Purged' content: application/json: schema: description: 'The aggregate number of cache entries purged in all 5GMSd AS instances distributing content for the requested Provisioning Session.' type: integer minimum: 1 '204': # No Content description: 'No Content Purged' '404': # Not Found $ref: 'TS29571_CommonData.yaml#/components/responses/404' '413': # Payload Too Large $ref: 'TS29571_CommonData.yaml#/components/responses/413' '414': # URI Too Long $ref: 'TS29571_CommonData.yaml#/components/responses/414' '415': # Unsupported Media Type $ref: 'TS29571_CommonData.yaml#/components/responses/415' '422': # Unprocessable Entity (e.g. syntactically invalid regular expression in request body) description: 'Unprocessable Entity' content: application/problem+json: schema: $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails' '500': # Internal Server Error $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': # Service Unavailable $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' components: schemas: IngestConfiguration: type: object description: 'A configuration for content ingest.' properties: pull: type: boolean protocol: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' baseURL: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' MediaEntryPoint: description: "A typed entry point for downlink or uplink media streaming." type: object required: - relativePath - contentType properties: relativePath: $ref: 'TS26512_CommonData.yaml#/components/schemas/RelativeUrl' contentType: type: string profiles: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1 PathRewriteRule: type: object description: 'A rule to manipulate URL paths.' required: - requestPathPattern - mappedPath properties: requestPathPattern: type: string mappedPath: type: string CachingConfiguration: type: object description: 'A content caching configuration.' required: - urlPatternFilter properties: urlPatternFilter: type: string cachingDirectives: type: object required: - noCache properties: statusCodeFilters: type: array items: type: integer noCache: type: boolean maxAge: type: integer format: int32 DistributionConfiguration: type: object description: 'A content distribution configuration.' properties: entryPoint: $ref: '#/components/schemas/MediaEntryPoint' contentPreparationTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' canonicalDomainName: type: string domainNameAlias: type: string baseURL: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' pathRewriteRules: type: array items: $ref: '#/components/schemas/PathRewriteRule' cachingConfigurations: type: array items: $ref: '#/components/schemas/CachingConfiguration' geoFencing: type: object required: - locatorType - locators properties: locatorType: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' locators: type: array items: type: string description: 'Format of individual locators depends on the locatorType.' minItems: 1 urlSignature: type: object required: - urlPattern - tokenName - passphraseName - passphrase - tokenExpiryName - useIPAddress properties: urlPattern: type: string tokenName: type: string passphraseName: type: string passphrase: type: string tokenExpiryName: type: string useIPAddress: type: boolean ipAddressName: type: string certificateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' supplementaryDistributionNetworks: type: array items: type: object description: "A duple tying a type of supplementary distribution network to its distribution mode." required: - distributionNetworkType - distributionMode properties: distributionNetworkType: $ref: '#/components/schemas/DistributionNetworkType' distributionMode: $ref: '#/components/schemas/DistributionMode' # Schema for the resource itself ContentHostingConfiguration: type: object description: "A representation of a Content Hosting Configuration resource." required: - name - ingestConfiguration - distributionConfigurations properties: name: type: string ingestConfiguration: $ref: '#/components/schemas/IngestConfiguration' distributionConfigurations: type: array items: $ref: '#/components/schemas/DistributionConfiguration' DistributionNetworkType: description: "Type of distribution network." anyOf: - type: string enum: [NETWORK_EMBMS] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. DistributionMode: description: "Mode of content distribution." anyOf: - type: string enum: [MODE_EXCLUSIVE, MODE_HYBRID, MODE_DYNAMIC] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

## C.3.6	M1_ConsumptionReportingProvisioning API

openapi: 3.0.0 info: title: M1_ConsumptionReportingProvisioning version: 2.0.1 description: ¦ 5GMS AF M1 Consumption Reporting Provisioning API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ConsumptionReportingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Consumption Reporting Provisioning' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/consumption-reporting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: activateConsumptionReporting summary: 'Activate the consumption reporting procedure for the specified Provisioning Session by providing the Consumption Reporting Configuration' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '201': description: 'Consumption Reporting Configuration Created' headers: Location: description: 'URL of the newly created Consumption Reporting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' get: operationId: retrieveConsumptionReportingConfiguration summary: 'Retrieve the Consumption Reporting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' put: operationId: updateConsumptionReportingConfiguration summary: 'Update the Consumption Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '204': description: 'Updated Consumption Reporting Configuration' '404': description: 'Not Found' patch: operationId: patchConsumptionReportingConfiguration summary: 'Patch the Consumption Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '200': description: 'Patched Consumption Reporting Configuration' content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' '404': description: 'Not Found' delete: operationId: destroyConsumptionReportingConfiguration summary: 'Destroy the current Consumption Reporting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Consumption Reporting Configuration' '404': description: 'Not Found' components: schemas: ConsumptionReportingConfiguration: type: object description: "A representation of a Consumption Reporting Configuration resource." properties: reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' locationReporting: type: boolean accessReporting: type: boolean

## C.3.7	M1_MetricsReportingProvisioning API

openapi: 3.0.0 info: title: M1_MetricsReportingProvisioning version: 2.0.1 description: ¦ 5GMS AF M1 Metrics Reporting Provisioning API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_MetricsReportingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Metrics Reporting Provisioning' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/metrics-reporting-configurations: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: activateMetricsReporting summary: 'Activate the Metrics reporting procedure for the specified Provisioning Session by providing the Metrics Reporting Configuration' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '201': description: 'Metrics Reporting Configuration Created' headers: Location: description: 'URL of the newly created Metrics Reporting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' /provisioning-sessions/{provisioningSessionId}/metrics-reporting-configurations/{metricsReportingConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: metricsReportingConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of a Metrics Reporting Configuration.' get: operationId: retrieveMetricsReportingConfiguration summary: 'Retrieve the specified Metrics Reporting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' put: operationId: updateMetricsReportingConfiguration summary: 'Update the specified Metrics Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '204': description: 'Updated Metrics Reporting Configuration' '404': description: 'Not Found' patch: operationId: patchMetricsReportingConfiguration summary: 'Patch the specified Metrics Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '200': description: 'Patched Metrics Reporting Configuration' content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' '404': description: 'Not Found' delete: operationId: destroyMetricsReportingConfiguration summary: 'Destroy the specified Metrics Reporting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Metrics Reporting Configuration' '404': description: 'Not Found' components: schemas: MetricsReportingConfiguration: type: object description: "A representation of a Metrics Reporting Configuration resource." required: - metricsReportingConfigurationId - scheme properties: metricsReportingConfigurationId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' scheme: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' dataNetworkName: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' urlFilters: type: array items: type: string minItems: 1 metrics: type: array items: type: string minItems: 1

## C.3.8	M1_PolicyTemplatesProvisioning API

openapi: 3.0.0 info: title: M1_PolicyTemplatesProvisioning version: 2.0.1 description: ¦ 5GMS AF M1 Policy Templates Provisioning API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_PolicyTemplatesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Policy Templates Provisioning' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/policy-templates: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createPolicyTemplate summary: 'Create (and optionally upload) a new Policy Template' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '201': description: 'Policy Template Created' headers: Location: description: 'URL of the newly created Policy Template resource.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' /provisioning-sessions/{provisioningSessionId}/policy-templates/{policyTemplateId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'A unique identifier of the Provisioning Session.' - name: policyTemplateId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'A resource identifier of a Policy Template.' get: operationId: retrievePolicyTemplate summary: 'Retrieve a representation of an existing Policy Template in the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' '404': description: 'Not Found' put: operationId: updatePolicyTemplate summary: 'Update a Policy Template for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '204': description: 'Updated Policy Template' '404': description: 'Not Found' patch: operationId: patchPolicyTemplate summary: 'Patch the Policy Template for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/PolicyTemplate' application/json-patch+json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '200': description: 'Patched Content Hosting Configuration' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' '404': description: 'Not Found' delete: operationId: destroyPolicyTemplate responses: '204': description: 'Destroyed Policy Template' '404': description: 'Not Found' components: schemas: PolicyTemplate: type: object description: "A representation of a Policy Template resource." required: - policyTemplateId - state - apiEndPoint - apiType - externalReference - applicationSessionContext properties: policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' state: anyOf: - type: string enum: [PENDING, INVALID, READY, SUSPENDED] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. apiEndPoint: type: string apiType: anyOf: - type: string enum: [N5, N33] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. externalReference: type: string qoSSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/M1QoSSpecification' applicationSessionContext: type: object properties: afAppId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AfAppId' sliceInfo: $ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai' dnn: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' aspId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' chargingSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/ChargingSpecification'

## C.3.9	M1_EdgeResourcesProvisioning API

openapi: 3.0.0 info: title: M1_EdgeResourcesProvisioning version: 2.1.1 description: ¦ 5GMS AF M1 Edge Resources Provisioning API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_EdgeResourcesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Edge Resources Provisioning' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 7.10. paths: /provisioning-sessions/{provisioningSessionId}/edge-resources-configurations: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createEdgeResourcesConfiguration summary: 'Create an Edge Resources Configuration within the scope of the specified Provisioning Session' requestBody: description: 'A JSON representation of an Edge Resources Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' responses: '201': description: 'Edge Resources Configuration Created' headers: Location: description: 'URL of the newly created Edge Resources Configuration.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' /provisioning-sessions/{provisioningSessionId}/edge-resources-configurations/{edgeResourcesConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: edgeResourcesConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Edge Resources Configuration.' get: operationId: retrieveEdgeResourcesConfiguration summary: 'Retrieve the Edge Resources Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' put: operationId: updateEdgeResourcesConfiguration summary: 'Update an Edge Resources Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of an Edge Resources Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' responses: '204': description: 'Updated Edge Resources Configuration' '404': description: 'Not Found' patch: operationId: patchEdgeResourcesConfiguration summary: 'Patch the Edge Resources Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Edge Resources Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' responses: '200': description: 'Patched Edge Resources Configuration' content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' '404': description: 'Not Found' delete: operationId: destroyEdgeResourcesConfiguration responses: '204': description: 'Destroyed Edge Resources Configuration' '404': description: 'Not Found' components: schemas: EdgeResourcesConfiguration: type: object description: 'A representation of an Edge Resources Configuration resource.' required: - edgeResourcesConfigurationId - edgeManagementMode - easRequirements properties: edgeResourcesConfigurationId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' edgeManagementMode: $ref: '#/components/schemas/EdgeManagementMode' eligibilityCriteria: $ref: 'TS26512_CommonData.yaml#/components/schemas/EdgeProcessingEligibilityCriteria' easRequirements: $ref: '#/components/schemas/EASRequirements' easRelocationRequirements: $ref: '#/components/schemas/M1EASRelocationRequirements' M1EASRelocationRequirements: type: object description: 'Relocation requirements of an EAS.' required: - tolerance properties: tolerance: $ref: 'TS26512_CommonData.yaml#/components/schemas/EASRelocationTolerance' maxInterruptionDuration: $ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm' maxResponseTimeDifference: $ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm' EASRequirements: type: object description: 'Requirements of an EAS.' required: - easProviderIds - easType - easFeatures - serviceAvailabilitySchedule - serviceContinuityScenarios properties: easProviderIds: type: array items: type: string easType: type: string easFeatures: type: array items: type: string serviceKpi: $ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASServiceKPI' serviceArea: $ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/GeographicalServiceArea' serviceAvailabilitySchedule: type: array items: $ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime' serviceContinuityScenarios: type: array items: $ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario' serviceContinuitySupport: type: array items: $ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario' EdgeManagementMode: description: 'The management mode of an EAS.' anyOf: - type: string enum: [EM_AF_DRIVEN, EM_APP_DRIVEN] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

## C.3.10	M1_EventDataProcessingProvisioning API

openapi: 3.0.0 info: title: M1_EventDataProcessingProvisioning version: 2.0.1 description: ¦ 5GMS AF M1 Event Data Processing Provisioning API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_EventDataProcessingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Event Data Processing Provisioning' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/event-data-processing-configurations: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createEventDataProcessingConfiguration summary: 'Supply an Event Data Processing Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Event Data Processing Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' responses: '201': description: 'Event Data Processing Configuration Created' headers: Location: description: 'URL of the newly created Event Data Processing Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' /provisioning-sessions/{provisioningSessionId}/event-data-processing-configurations/{eventDataProcessingConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: eventDataProcessingConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an Event Data Processing Configuration.' get: operationId: retrieveEventDataProcessingConfiguration summary: 'Retrieve the specified Event Data Processing Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' put: operationId: updateEventDataProcessingConfiguration summary: 'Update the specified Event Data Processing Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Event Data Processing Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' responses: '204': description: 'Updated Event Data Processing Configuration' '404': description: 'Not Found' patch: operationId: patchEventDataProcessingConfiguration summary: 'Patch the specified Event Data Processing Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Event Data Processing Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' responses: '200': description: 'Patched Event Data Processing Configuration' content: application/json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' '404': description: 'Not Found' delete: operationId: destroyEventDataProcessingConfiguration summary: 'Destroy the specified Event Data Processing Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Event Data Processing Configuration' '404': description: 'Not Found' components: schemas: EventDataProcessingConfiguration: description: 'A representation of an Event Data Processing Configuration resource.' type: object required: - eventDataProcessingConfigurationId - eventId - dataAccessProfiles properties: eventDataProcessingConfigurationId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' eventId: $ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AfEvent' authorizationUrl: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' dataAccessProfiles: type: array items: $ref: 'TS26532_Ndcaf_DataReportingProvisioning.yaml#/components/schemas/DataAccessProfile'

# C.4	OpenAPI representation of the M5 APIs

## C.4.1	M5_ServiceAccessInformation API

openapi: 3.0.0 info: title: M5_ServiceAccessInformation version: 2.2.0 description: ¦ 5GMS AF M5 Service Access Information API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_ServiceAccessInformation description: '5G Media Streaming: Media Session Handling (M5) APIs: Service Access Information' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /service-access-information/{provisioningSessionId}: parameters: - name: provisioningSessionId description: 'The resource identifier of an existing Provisioning Session.' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveServiceAccessInformation summary: 'Retrieve the Service Access Information resource' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ServiceAccessInformationResource' '404': description: 'Not Found' components: schemas: MediaEntryPoint: description: "A typed entry point for downlink or uplink media streaming." type: object required: - locator - contentType properties: locator: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' contentType: type: string profiles: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1 ServerAddresses: description: "A set of application endpoint addresses." type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' minItems: 1 ServiceAccessInformationResource: description: "A representation of a Service Access Information resource." type: object required: - provisioningSessionId - provisioningSessionType properties: provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' provisioningSessionType: $ref: 'TS26512_CommonData.yaml#/components/schemas/ProvisioningSessionType' streamingAccess: type: object properties: entryPoints: type: array items: $ref: '#/components/schemas/MediaEntryPoint' eMBMSServiceAnnouncementLocator: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' clientConsumptionReportingConfiguration: type: object required: - serverAddresses - locationReporting - samplePercentage properties: reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' serverAddresses: $ref: '#/components/schemas/ServerAddresses' locationReporting: type: boolean accessReporting: type: boolean samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' dynamicPolicyInvocationConfiguration: type: object required: - serverAddresses - validPolicyTemplateIds - sdfMethods properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' validPolicyTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 0 sdfMethods: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/SdfMethod' minItems: 0 externalReferences: type: array items: type: string minItems: 1 clientMetricsReportingConfiguration: type: array items: type: object required: - serverAddresses - scheme - samplePercentage - urlFilters - metrics properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' scheme: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' dataNetworkName: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' urlFilters: type: array items: type: string minItems: 0 metrics: type: array items: type: string networkAssistanceConfiguration: type: object required: - serverAddresses properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' clientEdgeResourcesConfiguration: type: object required: - easDiscoveryTemplate properties: eligibilityCriteria: $ref: 'TS26512_CommonData.yaml#/components/schemas/EdgeProcessingEligibilityCriteria' easDiscoveryTemplate: $ref: '#/components/schemas/EASDiscoveryTemplate' easRelocationRequirements: $ref: '#/components/schemas/M5EASRelocationRequirements' M5EASRelocationRequirements: description: 'Relocation requirements of an EAS.' type: object required: - tolerance properties: tolerance: $ref: 'TS26512_CommonData.yaml#/components/schemas/EASRelocationTolerance' maxInterruptionDuration: $ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm' EASDiscoveryTemplate: description: 'A template for discovering an EAS instance .' type: object required: - easType - easProviderIds - serviceFeatures properties: easType: type: string easProviderIds: type: array items: type: string serviceFeatures: type: array items: type: string

## C.4.2	M5_ConsumptionReporting API

(SNIPPED: NO CHANGES TO THIS CLAUSE)

## C.4.3	M5_MetricsReporting API

(SNIPPED: NO CHANGES TO THIS CLAUSE)

## C.4.4	M5_DynamicPolicies API

openapi: 3.0.0 info: title: M5_DynamicPolicies version: 2.0.1 description: ¦ 5GMS AF M5 Dynamic Policy API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_DynamicPolicies description: '5G Media Streaming: Media Session Handling (M5) APIs: Dynamic Policies' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /dynamic-policies: post: operationId: createDynamicPolicy summary: 'Create (and optionally upload) a new Dynamic Policy resource' requestBody: description: 'An optional JSON representation of a Dynamic Policy resource' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '201': description: 'Created Dynamic Policy Resource' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' headers: Location: description: 'The URL of the newly created Dynamic Policy resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' '400': description: 'Bad Request' '401': description: 'Unauthorized' /dynamic-policies/{dynamicPolicyId}: parameters: - name: dynamicPolicyId description: 'The resource identifier of a Dynamic Policy resource' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveDynamicPolicy summary: 'Retrieve an existing Dynamic Policy resource' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' put: operationId: updateDynamicPolicy summary: 'Update an existing Dynamic Policy resource' requestBody: description: 'A replacement JSON representation of a Dynamic Policy resource' required: true content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' patch: operationId: patchDynamicPolicy summary: 'Patch an existing Dynamic Policy resource' requestBody: description: 'A JSON patch to a Dynamic Policy resource' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/DynamicPolicy' application/json-patch+json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '200': description: 'Patched Dynamic Policy' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' '204': description: 'Patched Dynamic Policy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' delete: operationId: destroyDynamicPolicy summary: 'Destroy an existing Dynamic Policy resource' responses: '204': description: 'Destroyed Dynamic Policy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' components: schemas: DynamicPolicy: description: "A representation of a Dynamic Policy resource." type: object required: - dynamicPolicyId - policyTemplateId - serviceDataFlowDescriptions - provisioningSessionId properties: dynamicPolicyId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' serviceDataFlowDescriptions: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ServiceDataFlowDescription' provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' qosSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' enforcementMethod: type: string enforcementBitRate: type: integer

## C.4.5	M5_NetworkAssistance API

openapi: 3.0.0 info: title: M5_NetworkAssistance version: 2.0.1 description: ¦ 5GMS AF M5 Network Assistance API © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_NetworkAssistance description: '5G Media Streaming: Media Session Handling (M5) APIs: Network Assistance' externalDocs: description: 3GPP TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols. url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: apiRoot as defined in subclause 4.4.1 of 3GPP TS 29.501. paths: /network-assistance/: post: operationId: createNetworkAssistanceSession summary: 'Create a new Network Assistance Session.' responses: '201': description: 'Created Network Assistance Session' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' headers: Location: description: 'The URL of the nely created Network Assistance Session resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' '400': description: 'Bad Request' '401': description: 'Unauthorized' /network-assistance/{naSessionId}: parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveNetworkAssistanceSession summary: 'Retrieve an existing Network Assistance Session resource' responses: '200': description: 'A JSON representation of a Network Assistance Session resource' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' put: operationId: updateNetworkAssistanceSession summary: 'Update an existing Network Assistance Session resource' requestBody: description: 'A replacement JSON representation of a Network Assistance Session resource' required: true content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' responses: '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' patch: operationId: patchNetworkAssistanceSession summary: 'Patch an existing Network Assistance Session resource' requestBody: description: 'A JSON patch to a Network Assistance Session resource' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' application/json-patch+json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' responses: '200': description: 'Patched Network Assistance Session' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' '204': description: 'Patched Network Assistance Session' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' delete: operationId: destroyNetworkAssistanceSession summary: 'Destroy an existing Network Assistance Session resource' responses: '204': description: 'Destroyed Network Assistance Session' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' /network-assistance/{naSessionId}/recommendation: get: operationId: requestBitRateRecommendation summary: 'Obtain a bit rate recommendation for the next recommendation window' parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Success' content: application/json: schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' /network-assistance/{naSessionId}/boost-request: post: operationId: requestDeliveryBoost summary: 'Request a delivery boost' parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Delivery Boost Request Processed' content: application/json: schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/OperationSuccessResponse' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' components: schemas: NetworkAssistanceSession: description: "A representation of a Network Assistance Session resource." type: object required: - naSessionId properties: naSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' serviceDataFlowDescription: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ServiceDataFlowDescription' minItems: 1 policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' requestedQoS: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' recommendedQoS: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' notficationURL: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl'

# C.5	OpenAPI representation of data reporting records

## C.5.1	R4 data reporting records

For the purpose of referencing entities defined in this clause, it shall be assumed that the OpenAPI definitions below are contained in a physical file named "TS26512_R4_DataReporting.yaml".

openapi: 3.0.0 info: title: 5GMS Data Reporting data types version: 1.0.1 description: ¦ 5GMS Data Reporting data types © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: 5GMS Data Reporting data types description: '5G Media Streaming: Data Reporting data types' externalDocs: description: 'TS 26.512 V17.4.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' paths: {} components: schemas: MediaStreamingAccessRecord: allOf: - $ref: 'TS26532_Ndcaf_DataReporting.yaml#/components/schemas/BaseRecord' - type: object required: - mediaStreamHandlerEndpointAddress - applicationServerEndpointAddress - requestMessage - responseMessage - processingLatency properties: mediaStreamHandlerEndpointAddress: $ref: 'TS26512_CommonData.yaml#/components/schemas/EndpointAddress' applicationServerEndpointAddress: $ref: 'TS26512_CommonData.yaml#/components/schemas/EndpointAddress' sessionIdentifier: type: string requestMessage: type: object required: - method - url - protocolVersion - size - bodySize properties: method: type: string url: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' protocolVersion: type: string range: type: string size: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger' bodySize: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger' contentType: type: string userAgent: type: string userIdentity: type: string referer: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' cacheStatus: $ref: 'TS26512_CommonData.yaml#/components/schemas/CacheStatus' responseMessage: type: object required: - responseCode - size - bodySize properties: responseCode: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger' size: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger' bodySize: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger' contentType: type: string processingLatency: $ref: 'TS29571_CommonData.yaml#/components/schemas/Float' connectionMetrics: type: object required: - meanNetworkRoundTripTime - networkRoundTripTimeVariation - congestionWindowSize properties: meanNetworkRoundTripTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/Float' networkRoundTripTimeVariation: $ref: 'TS29571_CommonData.yaml#/components/schemas/Float' congestionWindowSize: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

END OF CHANGES