3GPP TSG-SA WG4 Meeting #122	S4-230062

# Athens, Greece, 20 – 24 February 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.247 | CR | 0178 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to DASH quality metric and QoE configuration and reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eDASH, QOED, NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2023-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following issue related to the DASH quality metric „Playout Delay for Media Start-up“ needs to be fixed in clause 10.4: - The DASH quality metric „Playout Delay for Media Start-up“ was introduced in Rel-13 by CR0074r2 (was approved in SA#66 in CR pack SP-140737). But in clause 10.4 on „Quality metrics for DASH“ this metric is missing. The following issues related to the QoE configuration and reporting need to be fixed in Annex L.1: - The “Within-area Indication” is not supported in the AT commands +CAPPLEVMC and +CAPPLEVMCNR. Therefore, the corresponding description should be removed. - The “Recording Session Indication” is not supported in the AT command +CAPPLEVMR. Therefore, the corresponding description should be removed. - With regards to the relase of QoE configuration the parameter <start-stop_reporting> should be corrected to <start-stop_measurement> for the AT command +CAPPLEVMCNR. - To be consistent with the description, the Figures L-2 and L-3 should be revised to replace the term „DASH Handler“ by “DASH Client” and “QAM” by “OAM”. Furthermore, the hyperlinks in the Figures can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 10.4 the DASH quality metric „Playout Delay for Media Start-up“ has been added. - In Annex L.1 the description of “Within-area Indication” has been removed. - In Annex L.1 the description of “Recording Session Indication” related to the AT command +CAPPLEVMR has been removed. - In Annex L.1 with regards to the relase of QoE configuration the parameter <start-stop_reporting> has been corrected to <start-stop_measurement> for the AT command +CAPPLEVMCNR. - In Annex L.1 revised Figures L-2 and L-3 have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | To change 1): The list of quality Metrics for DASH remains incomplete. To change 2) to 4): The description of QoE configuration and reporting remains misaligned with TS 27.007. To change 5): Issues remain in the Figures L-2 and L-3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4, Annex L.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The change 1) is applicable from Rel-13. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 10.4	Quality Metrics for DASH

The following metrics shall be supported by 3GP-DASH clients supporting the QoE reporting feature:

-	List of Representation Switch Events (Clause 10.2.3),

-	Average Throughput (Clause 10.2.4),

-	Initial Playout Delay (Clause 10.2.5),

-	Buffer Level (Clause 10.2.6),

-	Play List (Clause 10.2.7),

-	MPD Information (Clause 10.2.8),

-	Playout Delay for Media Start-up (Clause 10.2.9),

-	Device information (Clause 10.2.10).

The @metrics attribute contains a list of quality metric keys listing all metrics that the DASH shall collect and report.

The semantics of the attributes within the Metrics element are provided in Table 32. The XML-syntax of a Metrics element is provided in Table 33.

Table 32: Semantics of Metrics element

| Element or Attribute Name |  |  |  |  |  | Use | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Metrics |  |  |  |  | DASH metric element |
|  |  |  |  | @metrics |  | M | This attribute lists all quality metrics (as a list of quality metric keys as defined in section 10.2, separated by a whitespace) that the client shall report. Certain keys allow specifying a measurement interval or period over which a single value of the metric is derived and potentially also other parameters controlling the collection of the metrics. The parameters, if any, are included in parenthesis after the key and their semantics are specified in clause 10.2 with the metric definition itself. |
|  |  |  |  | Range |  | 0..N | When specified, it indicates the time period during which quality metric collection is requested. When not present, quality metric collection is requested for the whole duration of the content. |
|  |  |  |  |  | @starttime | O | When specified, it indicates the start time of the quality metric collection operation. When not present, quality metric collection is requested from the beginning of content consumption. For services with MPD@type "Live", the start time of quality metric collection can be obtained in wallclock time by adding the value of this attribute indicated in media time to the value of the MPD@availabilityStartTime attribute. For services with MPD@type "OnDemand" , the start time is indicated in media time and is relative to the PeriodStart time of the first period in this MPD. |
|  |  |  |  |  | @duration | O | When specified, it indicates the duration of the quality metric collection period. The value of this attribute is expressed in media time. |
|  |  |  |  | Reporting |  | 1...N | Descriptors that provide information about the requested Quality Reporting method and formats, and Auxiliary Reporting method and format. See clause 10.5 for the 3GP-DASH quality reporting schemes, and clause 14 for the 3GP-DASH auxiliary reporting scheme. |
| Legend: For attributes: M=Mandatory, O=Optional, OD=Optional with Default Value, CM=Conditionally Mandatory. For elements: <minOccurs>...<maxOccurs> (N=unbounded) Elements are bold; attributes are non-bold and preceded with an @. |  |  |  |  |  |  |  |

Table 33: XML-Syntax of Metrics element

<!-- QoE Collection and Reporting --> <xs:complexType name="MetricsType"> <xs:sequence> <xs:element name="Reporting" type="DescriptorType" maxOccurs="unbounded"/> <xs:element name="Range" type="RangeType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="metrics" type="xs:string" use="required"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType> <xs:complexType name="RangeType"> <xs:sequence> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="startTime" type="xs:duration" use="optional"/> <xs:attribute name="duration" type="xs:duration" use="required"/> <xs:anyAttribute namespace="##other" processContents="lax"/> </xs:complexType>

Next change

Annex L (normative):
QoE Measurement Collection Functionalities

# L.1	Configuration and reporting

As an alternative to configuration via MPD or OMA-DM, the QoE configuration can optionally be specified by the QoE Measurement Collection (QMC) functionality. In this case the QoE configuration is received via specific RRC [53] messages for UMTS, RRC [59] messages for LTE, and RRC messages for NR [70] over the control plane, and the QoE reporting is also sent back via RRC messages over the control plane.

If QMC is supported, the UE shall support the following QMC functionalities:

-	QoE Configuration: The QoE configuration will be delivered via RRC to the UE as a container according to "Application Layer Measurement Configuration" (see [53]) for UMTS, "measConfigAppLayer" (see [59]) for LTE and “AppLayerMeasConfig” (see [70]) for NR. The container is an octet string with gzip-encoded data (see [18]) stored in network byte order . The maximum size of the container is 1000 bytes for UMTS (see [53]) and LTE (see [59]), and 8000 bytes for NR (see [70]). The container shall be uncompressed, and is then expected to conform to XML-formatted QoE configuration data according to clause L.2 in the current specification. This QoE Configuration shall be forwarded to the DASH client. The interface towards the RRC signalling is handled by the AT command +CAPPLEVMC for UMTS and LTE, and AT command +CAPPLEVMCNR for NR [61].

-	QoE Metrics: QoE Metrics from the DASH client shall be XML-formatted according to clause 10.6 in the current specification. The XML data shall be compressed with gzip (see [18]) and stored in network byte order into an octet string container. The maximum size of the container is 8000 bytes for UMTS (see [53]) and LTE (see [59]). For NR [70], the maximum size is 8000 bytes if RRC segmentation is not enabled, and 144000 bytes if enabled. The container shall be delivered via RRC to the RNC according to "Application Layer Measurement Reporting" (see [53]) for UMTS, to the eNB according to "measReportAppLayer" (see [59]) for LTE, and to the gNB according to “MeasurementReportAppLayer” (see [70]) for NR. The behaviour if the compressed data is larger than the maximum container size is unspecified in this version of the specification. The interface towards the RRC signalling is handled by the AT command +CAPPLEVMR for UMTS and LTE, and AT command +CAPPLEVMRNR for NR[61].

-	The UE shall also set the QMC capability "QoE Measurement Collection for streaming services" (see [53]) to TRUE for UMTS, include the QMC capability "qoe-MeasReport" (see [59]) for LTE and include the QMC capability “qoe-Streaming-MeasReport” (see [70]) for NR.

-	When a new session is started, the QoE reporting AT command +CAPPLEVMRNR [61] shall be used to send a Recording Session Indication. Such an indication does not contain any QoE report, but indicates that QoE recording has started for a session.

-    When the QoE configuration is to be released, an unsolicited result code, associated with the AT command +CAPPLEVMC or AT command +CAPPLEVMCNR [61] and containing the parameter <start-stop_reporting> or <start-stop_measurement> set to "1" shall be sent to the DASH client as notification of a discard request. Then the DASH client shall stop collecting quality metrics and discard any already collected information [63].

For NR, the RAN visible QoE may be supported. The gNB can use RAN visible QoE configurations to instruct the UE to collect application layer measurements for network optimization.

-	The RAN visible QoE configuration generated by the gNB shall be forwarded to the DASH client, including the required RAN visible QoE metrics, service type, the RRC identifier and optionally reporting periodicity. The set of RAN visible QoE metrics is a subset of the QoE metrics defined in clause 10.6. The reporting periodicity follow the baseline NR QoE configuration when the reporting periodicity is not included.

-	Based on the RAN visible QoE configuration, the RAN visible QoE report shall be directly delivered to the UE AS layer and the collected metrics shall be sent to the gNB via the “MeasurementReportAppLayer” message.

-	When the RAN visible QoE measurement is deactivated by the gNB, the DASH client shall be notified to terminate and release the RAN visible QoE measurement.

NOTE: The RAN visible QoE metrics collection can be configured only if baseline NR QoE measurements are configured for the same service type. When the baseline NR QoE measurements are released, the RAN visible QoE configuration shall also be released.

The exact implementation is not specified here, but example signalling diagrams for UMTS, LTE and NR below show the QMC functionality with a hypothetical "QMC Handler" entity.

Figure L-1: Example signalling diagram for UMTS

Figure L-2: Example signalling diagram for LTE

Figure L-3: Example signalling diagram for NR

Note that the QMC Handler is only shown here as one possible implementation, and it need not be implemented as such. The corresponding QMC functionality could be built into the DASH client or into other UE entities. In this version of the specification the detailed implementation of the above functionalities is left to the UE vendor.

End of changes