3GPP TSG-SA WG4 Meeting #122	S4-230070

# Athens, Greece, 20 – 24 February 2023

Source:	China Mobile Com. Corporation

Title:	A Use Case for XR Streaming over WebRTC

Agenda item:	14.3

Document for:	Discussion and Agreement

- Introduction

TR 26.928 identified a use case and requirements for XR Streaming (A.21 in TR 26.928). The video content (may be 2D or 3D content) can be synched among a group of users.  Any user within the group can interact with the video content (e.g., pause, rewind, fast-forward), and this affects the playback for all the members of the group. This use case requires a streaming server that can distribute and ensure synchronized content playback for multiple XR users.

In this contribution, we proposed a XR Streaming solution over WebRTC peer-to-peer communication framework. It runs an XR application remotely, and streams rendered video frames and audio to multiple users. Users can view that broadcast media stream in any standard web browser that supports the WebRTC connection model on their own computer or mobile device. Users can also interact with the XR application from their browser by sending commands (e.g., keyboard, mouse, touch events, and other custom events) emitted from the player Web page back to the XR application.

- A Use Case for XR Streaming over WebRTC

- 2.1 Components and Functionalities

- XR Application: The XR Application runs the game logic and renders every frame in real-time on a remote server. It continuously encodes (e.g., H.264,VP 8, VP 9 video compression) the rendered video frames along with the game audio into a media stream, and distributes that stream to connected browsers over WebRTC direct peer-to-peer connections.

The Remote Server: The Remote Server can be a GPU-capable server, such as a virtual machine provided by a custom cloud-hosted platform.

Signaling and Web Server: The Signaling and Web Server negotiates connections between browsers and the XR Application and handles the WebRTC streaming from the XR application to the user.

STUN and TURN Sever: The STUN and TURN Sever tells each endpoint what its publicly visible IP address and make up the ICE framework.

XR Clients: The XR Clients (e.g., mobile phones, HMD, computers) send the user’s commands back to the Remote Server. The user’s commands can be generic (e.g., keyboard, mouse, touch events) or App specific.

- 2.2 The Signaling Process

The signaling process is summarized below:

- Web browser (XR Client) sends Offer SDP to the Signaling and Web Server.

- XR Application checks the Signaling and Web Server for unprocessed Offer SDPs

- XR Application sends Answer SDP to the Signaling and Web Server.

- Web browser (XR Client) checks the Signaling and Web Server for unprocessed Answer SDPs.

- Web browser (XR Client) sends ICE Candidate to the Signaling and Web Server.

- XR Application checks the Signaling and Web Server for unprocessed ICE Candidates.

- XR Application sends ICE Candidates to the Signaling and Web Server.

- Web browser (XR Client) checks the Signaling and Web Server for unprocessed ICE Candidates.

- The connection is established between XR Application and Web browser (XR Client);

  - 9.1. XR Application sends media streams to Web browser (XR Client).

  - 9.2. Web browser (XR Client) sends users’ commands back to XR Application.

- Proposal

We propose to include section 2 of this document into clause 5.2 of the permanent document as a use case for discussion.

- References

- Unity Unity Render Streaming 	https://docs.unity3d.com/Packages/com.unity.renderstreaming@3.1/manual/index.html

- Unreal Pixel Streaming

- https://docs.unrealengine.com/4.27/en-US/SharingAndReleasing/PixelStreaming/PixelStreamingOverview/

- Unreal Pixel Streaming in Azure

- https://learn.microsoft.com/en-us/gaming/azure/reference-architectures/unreal-pixel-streaming-in-azure