3GPP TSG SA WG4#122	S4-230091

Athens, Greece, 20th – 24th February 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119-PD | CR | pseudo | rev | - | Current version: | 4.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Updates to Device Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 14/02/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A few aspects are missing - Audio is not mentioned in the description - Control session client is missing - XR Source Management to MAF not described - API to MAF is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implements the above |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1, 4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 4.1.1	General

The XR Baseline Client represents the functionalities, the peripherals, and the interfaces that are present on a generic XR UE. The actual device may be realized by a single device, or a combination of devices linked together. The details on how to instantiate an XR Baseline Client in the context of a service or deployment scenario is left for the respective Work Items and Study Items to define.

In terms of functionalities, an XR Baseline Client is composed of:

- An XR application: a software application that integrates audio-visual content into the user’s real-world environment

- An XR Runtime: a set of functions that interface with a platform to perform commonly required operations, such as accessing the controller/peripheral state, getting current and/or predicted tracking positions, performing spatial computing, as well as submitting rendered frames to the display processing unit and rendered audio to the speakers with a late stage re-projection to the latest pose.

- An XR Source Management: management of data sources provided through the XR runtime such as microphones, cameras, trackers, etc, for example making the information available to the XR application or providing it to the MAF for sending in the uplink.

- A Media Access Function: A set of functions that enables access to media and other XR-related data that is needed in the Scene manager or XR Runtime to provide an XR experience as well to create delivery formats for information provided by the XR Source Management.

- A Scene Manager: a set of functions that supports the application in arranging the logical and spatial representation of a multisensorial scene based on support from the XR Runtime.

- A Presentation Engine: a set of composite renderers, rendering the component of the scenes, based on the input from the Scene Manager.

In addition, those functional blocks are integrated together via interfaces. Interfaces may be made of APIs and/or data formats and collectively act as a contract between the two sides of the interface.

The XR Baseline Client contains the following interfaces:

- IF-1 lies between the XR Runtime on one side and the XR Source Management and the Presentation Engine. IF-1 is implement as an API that exposes functions provided by the XR Runtime

- IF-2 lies between the Media Access Function and the 5G System for user plane date.

- IF-3 lies between the XR Source Management and the Media Access Function.

- IF-4 lies between the Scene Manager and the Media Access Function.

- IF-5 lies between the Scene Manager and the XR Application.

- IF-6 lies between the XR Application and the Media Access function to configure Media Access. This is typically implemented as an API that exposes functions of the MAF.

- IF-7 lies between the UE and the 5G System implementing control sessions (such as 5G Media Streaming, IMS)

=====  CHANGE  =====

#### 4.1.2	Architecture

Figure 11 - XR Baseline Client architecture