3GPP TSG SA WG4#122	S4-230095

Athens, Greece, 20th – 24th February 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119-PD | CR | pseudo | rev | - | Current version: | 4.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Metrics Framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 14/02/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 5.2	Metrics Observation Points and KPIs

## 5.2.1	Metrics Observation Points

In a similar fashion as for operation points and interface, metrics observation points may be defined to specify basic metrics as shown in Figure 2.

Editor’s Note: the above diagram is expected to be further updated and refined based on the agreements of the XR baseline client.

Figure 14 – Metrics Observation Points of interest for TS 26.119

In the following potential KPIs as documented in TR 26.998, clause 4.5.2 are summarized. Specifically, the latencies as introduced in TR 26.998 are rediscussed in the context of the MeCAR work item

### 5.2.2	Relevant KPIs for XR and AR according to TR 26.998

In TR 26.928 [2], some high-level statements on experience KPIs for AR are provided. To achieve Presence in Extended and Augmented Reality, seamless integration of virtual content and physical environment is required. Like in VR, the virtual content has to align with user's expectations. For truly immersive AR and in particular MR, it is expected that users cannot discern virtual objects from real objects.

Also relevant for VR and AR, but in particular AR, is not only the awareness for the user for the environment. This includes, safe zone discovery, dynamic obstacle warning, geometric and semantic environment parsing, environmental lighting and world mapping.

Table 5.2.2-1 provides the KPIs from TR 26.998, Table 4.5.2-1 with  focus on AR and in particular glasses. For some background and additional details refer for example to [10], [11], [49], [50], and [51].

[10]	Daniel Wagner, Louahab Noui, Adrian Stannard, "Why is making good AR displays so hard?", LinkedIn Blog, August 7, 2019, https://www.linkedin.com/pulse/why-making-good-ar-displays-so-hard-daniel-wagner/

[11]	Daniel Wagner, "MOTION TO PHOTON LATENCY IN MOBILE AR AND VR", Medium Blog, August 20, 2018, https://medium.com/@DAQRI/motion-to-photon-latency-in-mobile-ar-and-vr-99f82c480926

[49]	Oscar Falmer: “AR Headsets Landscape”, 10th September 2021, https://docs.google.com/spreadsheets/d/1aUO8nuXWCnL1xYnpe9tvSgCphifhMRLrFj1enayv0X8/edit#gid=0

[50]	Oscar Falmer: “Mobile AR Features Landscape”, 20th September 2021, https://docs.google.com/spreadsheets/d/1S1qEyDRCqH_UkcSS4xVQLgcMSEpIu_mPtfHjsN02GNw/edit#gid=0

[51]	Microsoft, Coordinate Systems, https://docs.microsoft.com/en-us/windows/mixed-reality/design/coordinate-systems

Table 4.5.2-1 AR KPIs from TR 26.998 with relevance for MeCAR

| Feature |  | KPIs for AR glasses |  |  |
| --- | --- | --- | --- | --- |
| Tracking |  |  |
| Freedom Tracking |  | 6DoF |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Translational Tracking Accuracy |  | Sub-centimeter accuracy - tracking accuracy of less than a centimeter |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Rotational Tracking Accuracy |  | Quarter-degree-accurate rotation tracking is desired |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| AR tracking space |  | In AR, the tracking space is theoretically unlimited. However, when moving, tracking accuracy may not be assured beyond a certain level of space or trajectory distance. SLAM based methods quickly introduce a large drift in large scale mapping. To correct the scaling issues, a loop closure technique [12] needs to be applied in order to continuously harmonize the local coordinate systems with global ones. |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| World-scale experience |  | World-scale experiences that let users wander beyond - orientation-only or seated-scale experiences - standing-scale or room-scale experiences To build a world-scale experience, techniques beyond those used for room-scale experiences, namely creating an absolute room-scale coordinate system that is continuously registered with the world coordinate system, typically requiring dynamic sensor-driven understanding of the world, continuously adjusting its knowledge over time of the user's surroundings. |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Tracking frequency |  | At least 1000 Hz |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Latency (for more details refer to clause 4.5.3) |  |  |
| motion-to-photon latency |  | Less than 20 ms, and preferably even sub 10ms for AR as you may observe movement against the real world. |  | This latency is run-time internal. It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| pose-to-render-to-photon latency |  | 50-60ms for render to photon is desired in order to avoid wrongly rendered content with late warping applied. |  | This is a relevant metric and measuring this should be in scope of MeCAR. For details refer to clause 5.2.3. |
| Video Rendering and Display |  |  |
| Persistence – Duty time |  | Turn pixels on and off every 2 - 4 ms to avoid smearing / motion blur |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Display refresh rate |  | 60 Hz minimum 90 Hz acceptable 120 Hz and beyond desired 240 Hz would allow always on display at 4ms |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Colour |  | RGB colours Accurate colours independent of viewpoint. |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Spatial Resolution per eye |  | for 30 x 20 degrees - 1.5K by 1K per eye is required - 1.8K by 1.2K per eye is desired for 40 x 40 degrees - 2K by 2K required - 2.5 K by 2.5 K desired ultimate goal for display resolution is reaching or going slightly beyond the human vision limit of roughly one arcmin (1/60°) |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. However, the resolution of the content is relevant and is a metric that should be computed. |
| Content frame rates |  | Preferably matching the display refresh rate for lowest latency Lower frame rates for example 60 fps or 90 fps may be used but add to overall end to end delay. |  | This is a feature that can be handled together with latency. For more details refer to clause 5.2.3. |
| Brightness |  | 200-500 nits for indoor Up to 2K for state-of-the-art devices in 2021 [49] 10K to 100K nits for full outdoor experiences |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Optics |  |  |
| Field of View |  | Augmentable FoV - typically, 30 by 20 degrees FoV acceptable - 40 by 40 degrees desired maximize the non-obscured field of view |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Eye Relief |  | the minimum and maximum eye-lens distance wherein a comfortable image can be viewed through the lenses. at least 10mm, ideally rather 20mm |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Calibration |  | correction for distortion and chromatic aberration that exactly matches the lens characteristics |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Depth Perception |  | Avoid vergence and accommodation conflict (VAC) for accommodation being different for the real and virtual object |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. However, it relates also to the content and may be checked for metrics. |
| Physics |  |  |
| Maximum Available Power |  | AR Glass: below 1 W, typically 500mW For less design-oriented devices, additional power may be available. |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Maximum Weight |  | AR Glass: around 70g. However, if the weight is well distributed, several hundred grams may be acceptable. |  | It is a device feature and an assumption/requirement for the device, but not a metric to be computed. |
| Audio Related metrics |  |  |
| Tbd | Tbd | tbd |

### 5.2.3	Typical Latencies in networked AR Services and their Measurements

Building on top of the architectures introduced in TR 26.998, in this document as well as the latency considerations in TR 26.928, Figure 4.5.3-1 provides a summary of different latencies involved networked AR services. Based on TR 26.928 as well as Table 5.2.3-1, two relevant latency requirements for adequate user experience matter:

-	motion-to-photon latency being less 20ms, but preferably even single digit latency below 10ms.

-	pose-to-render-to-photon latency: as small as 50-60ms

It is important to note that the motion-to-photon latency is primarily a function of the device implementation as it is basically covered within the AR runtime. What matters and is relevant is the time used to provide the pose information from the AR runtime to the renderer and the renderer using this pose to generate the displayed media. Final pose correction to the latest pose may always be done in the AR runtime.

Figure 5.2.3-1 (based on Figure 4.5.3-1 from TR 26.998) provides different latency critical uplink and downlink operations, depending on where the rendering is done, locally, in the edge or in the cloud. If done in the edge or cloud, rendered data needs to be delivered in low-latency and high-quality over the network. The typical operations in this case include:

-	pose detection in the UE

-	sending the pose through a 5G uplink network to the edge of cloud.

-	rendering the scene in the edge or cloud

-	compressing and encrypting the rendered scene and delivering to the UE

-	decrypting and decompressing the rendered scene

-	composition of the scene

-	applying the latest pose in the pose correction and display the immersive media.

Note that Figure 5.2.3-1 also adds buffers that are typically handled by the AR Run time, namely eye and depth as well as sound buffers.

Figure 5.2.3-1: Typical Latencies in networked AR services with OPs

It is ultimately relevant that in case of networking the rendering loop, the processes in the loop are executed such that the end-to-end latency requirements for the pose-to-render-to-photon latency are ensured. Clearly the “closer” the rendering happens at the AR UE, the easier it is to meet latency requirements. However, with proper support of 5G system and media functionalities, these networked AR challenges are solved. This is subject of the remaining discussion of this report.

With reference to TR 26.928 [2], other types of latencies impact the user experience, for example when used for cloud gaming, interactive scenes or in case of real-time network-based processing of sensor data. These aspects are not specific to AR but are also relevant.

Figure 5.2.3-1 also adds the OPs.

Editor’s Note: the above diagram is expected to be further updated and refined based on the agreements of the XR baseline client.

Based on the OPs, a subset of the latencies can be measured.

Detailed algorithms are ffs.